3GPP TSG-RAN WG4 Meeting # 107	R4-2308223

# Incheon, KR, May. 22 – May 26, 2023

Source:	LG Electronics

Title:	Discussion for FR2-1 UL256QAM

Agenda item:	8.7.2

Document for:	Discussion

Introduction

During the FR2-1 UL256QAM meetings, there were detailed discussions and agreements regarding the MPR simulation assumptions. In this contribution, we would like to provide our phase noise simulation results and our point of view on the UL 256QAM MPR simulation.

Discussion

- 2.1 Phase noise simulation

During the FR2-1 UL256QAM meetings, there were detailed discussion and agreements regarding the MPR simulation assumptions for phase noise profiles evaluation, and agreed simulation assumption is shown in Table 1.

Table 1 Agreed MPR simulation assumptions for FR2-1 UL256QAM

##### [1]R4-2306603 MPR Issue 1-1-3 Simulation assumption for phase noise profiles evaluation Agreement Parameters; Value. Frequency; 29, 39 GHz. SCS; 60kHz, 120kHz. BW; 100MHz. RB Size; 128RB for 60kHz, 64RB for 120kHz. Background AWGN; No additional noise. Time offset/Frequency offset; 0. Antenna configuration; 1T1R. Modulation; 256QAM. Waveform type; CP-OFDM / DFT-s-OFDM. DMRS; 3 symbols per slot (UL DMRS add-pos = 2). PTRS configuration; OFF/ON For CP-OFDM: L-PTRS (Time density) = 1 (every 1 symbol) K-PTRS (Freq density) =2 (every 2 RBs) For DFTs-OFDM: L-PTRS (Time density) = 1 (every symbol) N_group = 8, N_samp = 4 Other PTRS configurations can be considered for narrow RB allocations, especially for DFT-s-OFDM.. EVM measurement; Data aided EVM calculation, based on ideal data signal. Phase noise profiles; Refer to Issue 1-1-4. Issue 1-1-4 How to apply the phase noise profiles in MPR simulation Agreement: - For 29GHz: - Example 1 in TR38.803 for 29 GHz. - Example 2 in TR38.803. - New phase noise model using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from Qualcomm PSD0; 32 dB. 1; 3e3; 2.37; 1; 3.3. 2; 6e5; 4.7; 8e5; 5.3. 3; 8e5; 2.0; 1.1e6; 2.5. - Parameters from MTK - 39GHz - Example 2 in TR38.803. - Example 1-based using following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from vivo - Parameters from Anritsu - New phase noise model using the pole-zero method based on following function: Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m - Parameters from MTK - Adopt min(example1, example2) as the phase noise profile for UL256QAM, where ‘example2’ refers to the example phase noise profiles in TR38.803 and ‘example1’ refers to Example 1-based for 39GHz.

Table 2 simulation assumption for phase noise

| Parameters | Value |
| --- | --- |
| Frequency | 29, 39 GHz |
| SCS | 120kHz |
| BW | 100MHz |
| RB Size | 64RB for 120kHz |
| Background AWGN | No additional noise |
| Time offset/Frequency offset | 0 |
| Antenna configuration | 1T1R |
| Modulation | 256QAM |
| Waveform type | CP-OFDM |
| DMRS | 3 symbols per slot (UL DMRS add-pos = 2) |
| PTRS configuration | ON For CP-OFDM: L-PTRS (Time density) = 1 (every 1 symbol) K-PTRS (Freq density) =2 (every 2 RBs) |
| EVM measurement | Data aided EVM calculation, based on ideal data signal |
| Phase noise profiles | For 29 GHz Case 1: Example 1 in TR 38.803 for 29 GHz % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 29.35; Case2: Parameters from QC % fz = [3e3 7e5 9e5]; % fp = [1 1e6 1.1e6]; % alphaz = [2.37 4.7 2]; % alphap = [3.3 5.3 2.5]; % PSD0 = 33; Case 3: Prameters from MTK % fz = [3e3 550e3 280e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 29.35; For 39 GHz Case 4: Parameters from Vivo % fz = [3e3 494e3 418e6]; % fp = [1 1.58e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 34.41; Case 5: Parameters from Anritsu % fz = [3e3 499e3 3.9e8]; % fp = [1 1.54e6 3e7]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 34.4; Case 6: Parameters from MTK % fz = [3e3 620e3 240e6]; % fp = [1 1.6e6 30e6]; % alphaz = [2.37 2.7 2.53]; % alphap = [3.3 3.3 1]; % PSD0 = 31.76; |

Table 3 CP-OFDM [29 GHz, 120KHZ, 100 MHz, 64RBs, (L=1, K=2)]

|  | EVM (dB) |
| --- | --- |
| Case 1 | -30.75 |
| Case 2 | -31.57 |
| Case 3 | -31.37 |

Table 4 CP-OFDM [39 GHz, 120KHZ, 100 MHz, 64RBs, (L=1, K=2)]

|  | EVM (dB) |
| --- | --- |
| Case 4 | -28.56 |
| Case 5 | -28.77 |
| Case 6 | -30.96 |

We simulate EVM according to phase noise profiles and simulation results are shown in table 3 and table 4.

For 29 GHz simulation results, we can observe the feasibility in case 1, case 2 and case 3. But, case 2 and case 3 phase noise profiles can achieve more EVM margin than case 1.

Observation 1: For 29 GHz simulation results, we can observe the feasibility in case 1, case 2 and case 3. But, case 2 and case 3 phase noise profiles can achieve more EVM margin than case 1.

For 39 GHz simulation results, we can observe the feasibility in only case 6. Case 4 and case 5 can not achieve below 3.5% EVM.

Observation 2: For 39 GHz simulation results, we can observe the feasibility in only case 6. Case 4 and case 5 can not achieve below 3.5% EVM.

Proposal 1: For 29 GHz, We can use the case 1,case 2 and case 3 for MPR simulation.

Proposal 2: We need to discuss more how to get the reasonable EVM margin for 39 GHz.

- 2.2 MPR simulation results according to phase noise profiles

Table 5 Simulated MPR results for 29 GHz PC2 UL256QAM

| CBW (100 MHz) | PN profile | Region 1 [22, 20] | Outer1 [0, 64] | Outter 2 [0, 1] | Outter 3 [0, 20] |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Case 1 | 7.54 | 7.54 | 6.21 | 6.50 |
|  | Case 2 | 7.22 | 7.34 | 5.96 | 6.18 |
|  | Case 3 | 7.22 | 7.22 | 6.00 | 6.19 |
| CBW (100 MHz) | PN profile | Region 1 [23, 20] | Outter1 [0, 66] | Outter 2 [0,1] | Outter 3 [0, 20] |
| CP-OFDM | Ex1 | 9.48 | 9.50 | 8.27 | 9.05 |
|  | QC’s | 9.26 | 9.27 | 8.04 | 8.72 |
|  | MDK’s | 9.26 | 9.26 | 7.74 | 8.83 |
| CBW (400 MHz) | PN profile | Region 1 [88, 80] | Outter1 [0, 240] | Outter 2 [0, 1] | Outter 3 [0, 80] |
| DFT-s-OFDM | Ex1 | 8.91 | 8.91 | 7.19 | 7.84 |
|  | QC’s | 8.06 | 8.06 | 7.10 | 7.33 |
|  | MDK’s | 8.48 | 8.06 | 7.11 | 7.44 |
| CBW (400 MHz) | PN profile | Region 1 [88, 88] | Outter1 [0,264] | Outter 2 [0,1] | Outter 3 [0, 80] |
| CP-OFDM | Ex1 | 10.96 | 10.84 | 9.04 | 10.09 |
|  | QC’s | 10.30 | 10.30 | 9.10 | 9.76 |
|  | MDK’s | 10.19 | 10.30 | 8.96 | 9.75 |

[start RB position, allocated RB number]

Table 6 MPR results for 29 GHz PC2 UL256QAM with margin

| CBW (100 MHz) |  | Inner RB allocations, Region 1 | Edge RB allocations |
| --- | --- | --- | --- |
| DFT-s-OFDM | Example1 | 9 | 9 |
|  | QC | 8.5 | 9 |
|  | MDK | 8.5 | 8.5 |
| CP-OFDM | Example1 | 11 | 11 |
|  | QC | 11 | 11 |
|  | MDK | 11 | 11 |
| CBW (400 MHz) |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Example1 | 10.5 | 10.5 |
|  | QC | 9.5 | 9.5 |
|  | MDK | 10 | 9.5 |
| CP-OFDM | Example1 | 12.5 | 12.5 |
|  | QC | 12 | 12 |
|  | MDK | 11.5 | 12 |

In the last meeting, there was a concern that EVM 3.5% could not be achieve due to phase noise, or even if it was achieved, it would have a very large MPR, so new phase noise profiles that are more relaxed than the exisiting phase noise profile were proposed. In this clause, the simulated MPR values of PC2 UL 256QAM according to the phase noise profile are shown in Table 5 and Table 6. It was found that the MPR value for 29 GHz can be improved by 0~1 dB by using the phase noise profile of case 2 and case 3 compared to the exisiting case1.

Observation 3: The MPR value for 29 GHz can be improved by 0~1 dB by using the phase noise profile of case 2 and case 3 compared to the case 1.

We observed the insufficient margin of dynamic range for EVM test with the analysis in clause 2.3, and considering this, we need to discuss about reasonable MPR vaule based on MPR simulation results.

Proposal 3: We need to discuss about reasonable MPR value based on the MPR simulation results and EVM test dynamic range analysis.

- 2.3 Dynamic range

Table 6 WF for the minimum EIRP requirements for EVM test

Issue 2-1 The minimum EIRP requirements for EVM test - Down select from Option 1 and Option 2 in next meeting. - Option 1: The minimum EIRP for UL 256 QAM for EVM test could be relaxed by 14 dB based on the difference between the SNR of 256QAM (29.1dB) and the SNR of QPSK(15.1dB). Parameter; Unit; Level for PC1; Level for PC2; Level for PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 18; 1; 8. Operating conditions; Normal Conditions. NOTE 1: PTRS is configured for 256 QAM. - Option 2: Use a “-1dB/dB” relation to calculate the minimum EIRP requirement for 256QAM and consider 1dB correction factor. Parameter; Unit; PC1; PC2; PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 19.5; 2.5; 9.5.

Table 6 Dynamic range for 256QAM EVM test using 12 dB MPR value

|  | band | dynamic range w/o MPR issue 2-1 option 1 | dynamic range w/o MPR issue 2-1 option 2 | dynamic range with MPR issue 2-1 option 1 | dynamic range with MPR issue 2-1 option 2 |
| --- | --- | --- | --- | --- | --- |
| PC1 | n257 | 22 | 20.5 | 10 | 8.5 |
|  | n258 | 22 | 20.5 | 10 | 8.5 |
|  | n260 | 20 | 18.5 | 8 | 6.5 |
|  | n261 | 22 | 20.5 | 10 | 8.5 |
|  | n262 | 16.2 |  | 4.2 |  |
| PC2 | n257 | 28 | 26.5 | 16 | 14.5 |
|  | n258 | 28 | 26.5 | 16 | 14.5 |
|  | n259 | 24 | 22.5 | 12 | 10.5 |
|  | n260 | 24 | 22.5 | 12 | 10.5 |
|  | n261 | 28 | 26.5 | 16 | 14.5 |
|  | n262 | 21.9 |  | 9.9 |  |
| PC5 | n257 | 22 | 20.5 | 10 | 8.5 |
|  | n258 | 22.4 | 20.9 | 10.4 | 8.9 |
|  | n259 | 19 | 17.5 | 7 | 5.5 |

In the last meeting of the WF [1], there were two option for issue 2-1 the minimum EIRP requirements for EVM test. The EVM test dynamic range analysis for option 1 and option2, assuming an MPR value of 12 dB, is shown in Table 6. With option 1, the dynamic range was less than 10 dB in two cases, and with option2, it was less than 10 dB in seven cases. If tolerance is also considered in EVM testing, a dynamic range of less than 10 dB may be insufficient. Therefor, option 1, which provides more dynamic range margin, is suitable for minimum EIRP requirements

Proposal 4: Option 1, which provides more dynamic range margin, is suitable for minimum EIRP requirements.

Conclusion

Observation 1: For 29 GHz simulation results, we can observe the feasibility in case 1, case 2 and case 3. But, case 2 and case 3 phase noise profiles can achieve more EVM margin than case 1.

Observation 2: For 39 GHz simulation results, we can observe the feasibility in only case 6. Case 4 and case 5 can not achieve below 3.5% EVM.

Observation 3: The MPR value for 29 GHz can be improved by 0~1 dB by using the phase noise profile of case 2 and case 3 compared to the case 1.

Proposal 1: For 29 GHz, We can use the case 1,case 2 and case 3 for MPR simulation.

Proposal 2: We need to discuss more how to get the reasonable EVM margin for 39 GHz.

Proposal 3: We need to discuss about reasonable MPR value based on the MPR simulation results and EVM test dynamic range analysis.

Proposal 4: Option 1, which provides more dynamic range margin, is suitable for minimum EIRP requirements.

- References

[1]R4-2306603, WF on FR2-1 UL 256QAM (Xiaomi)

- Annex

EVM budget from LGE

-For CP-OFDM (L=1, K=2), (64 RB,120 kHz)

| Tx EVM contributor | EVM (%) | SNR (dB) |
| --- | --- | --- |
| Phase Noise+IQ Imbalance | 2.95 | 30.6 |
| PA Non-linearity & Transmitter | 1.88 | 34.5 |
| Total | 3.50 | 29.1 |

-For DFT-s-OFDM(no PTRS correction), (64 RB,120 kHz)

| Tx EVM contributor | EVM (%) | SNR (dB) |
| --- | --- | --- |
| Phase Noise+IQ Imbalance | 2.30 | 32.8 |
| PA Non-linearity & Transmitter | 2.64 | 31.6 |
| Total | 3.50 | 29.1 |

PA

Using the 27 dBm PA model considering the Tx post loss for PC2

Phase noise profile

Case 1:

Example 1 in TR 38.803 for 29 GHz

%                 fz = [3e3 550e3 280e6];

%                 fp = [1 1.6e6 30e6];

%                 alphaz = [2.37 2.7 2.53];

%                 alphap = [3.3 3.3 1];

%                 PSD0 = 29.35;

Case2:

Parameters from QC

%                 fz = [3e3 7e5 9e5];

%                 fp = [1 1e6 1.1e6];

%                 alphaz = [2.37 4.7 2];

%                 alphap = [3.3 5.3 2.5];

%                 PSD0 = 33;

Case 3:

Prameters from MTK

%                 fz = [3e3 550e3 280e6];

%                 fp = [1 1.6e6 30e6];

%                 alphaz = [2.37 2.7 2.53];

%                 alphap = [3.3 3.3 1];

%                 PSD0 = 29.35;

PTRS configuration for CPE correction

Using the PTRS (K=2, L=1)