# 3GPP TSG-RAN WG4 Meeting #107	R4-2308270

# Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	TP on system parameters for SL unlicensed operation for single CC

Agenda Item:	8.31.2.1.1

Document for:	Approval

Introduction

In the last meeting, TR 38.786 v0.1.0 for SL evolution [1] was agreed.

In this contribution, we provide the text proposal for TR 38.786 for system parameters for SL-U based on the agreements made in earlier meetings [2][3].

# References

[1] R4-2305422,TR38.786 v0.1.0 for SL evolution, OPPO, RAN4#106bis-e.

[2] R4-2303568, WF on NR sidelink evolution, LGE, RAN4#106.

[3] R4-2306630, WF on SLU single CC system parameters, vivo, RAN4#106bis-e.

# Text proposal

## <<Start of Change for TR 38.786>>

# 5	Operating bands and channel arrangement for SL evolution

## 5.1	Operating bands

### 5.1.1	Operating bands for single carrier operation in unlicensed band

NR Sidelink is designed to operate in the unlicensed operating bands in FR1 defined in Table 5.1.1-1.

Table 5.1.1-1. NR SL-U operating bands in FR1

| NR SL-U operating band | Sidelink (SL) Transmission operating band | Sidelink (SL) Reception operating band | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |
| n461 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | HD | PC5 |
| n961 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | HD | PC5 |
| n1021 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | HD | PC5 |
| NOTE 1: Client devices are prohibited from connecting directly to another client device in the frequency range 5850 – 5895 MHz and 5925 – 7125 MHz in USA, and in the frequency range 5925 – 7125 MHz in Canada per regulatory rules. Direct connection between client devices in the shared spectrum band is subject to country-specific conditions. NOTE 2: For bands n96 and n102, SL-U operation is excluded for vehicular UEs. |  |  |  |  |

### 5.1.2	Operating band combinations for inter-band con-current operation

## 5.2	Channel bandwidth

### 5.2.1	Channel bandwidth for single carrier operation in unlicensed band

Channel bandwidths defined for NR SL-U operating bands are in Table 5.2.1-1.

Table 5.2.1-1 NR SL-U channel bandwidth

|  |  | SL-U band /channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Operating Band | SCS kHz | 101 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n46 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1002 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1002 |
| n96 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1002 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1002 |
| n102 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1002 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1002 |
| NOTE 1: Channel bandwidth 10MHz is not defined for single carrier operation for SL-U bands n46, n96 and n102. NOTE 2: Channel bandwidth 100MHz is optionally supported for SL-U. |  |  |  |  |  |  |  |  |  |  |

### 5.2.2	Channel bandwidth for inter-band con-current operation

## 5.3	Channel arrangement

### 5.3.1 Channel spacing

The general requirements in clause 5.4.1 of TS 38.101 are applied for SL-U.

For SL-U bands n46, n96, n102, the nominal channel spacing is defined as follows:

-	For NR operating bands with 15 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-5 kHz, 0 kHz, 5 kHz} for ∆FRaster equals 15 kHz

### 5.3.2	Channel raster

The general requirements in clause 5.4.2 of TS 38.101 are applied for SL-U.

Channel raster points for 10MHz CBW in n46 are not applicable for SL-U.

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both Tx and Rx for SL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

### 5.3.2	Synchronization raster

## <<End of Change>>