3GPP TSG-RAN WG4 Meeting #107	R4-2308965
Incheon, KR, May 22 – May 26, 2023

Title:	R18 8Rx FWA

Source:	OPPO

Agenda item:	8.5.1.2

Document for:	Approval

- Introduction

In Rel-18, 8Rx for FWA/CPE/vehicle/industrial devices was one of the objectives for FR1 enhancement. And in last meeting, the WF [1] was approved with several open issues as below, this paper further discusses them.

Issue 1-2-1: ΔRIB for TDD bands <Way forward/Agreement>: RAN4 stop the discussion on the AL issue and directly discuss ΔRIB. Further discuss ΔRIB for TDD bands with the following values as a baseline: - - [4.0-4.5] dB for n41 - - [4.0] dB for n77/n78/n79 Issue 1-3-1: ΔRIB for FDD band n7 <Way forward/Agreement>: - [4.5] dB for n7 Issue 2-2-1: Further analysis for indication of ΔTRxSRS <Way forward>: No decision and agreements are made in this meeting, future discussion in RAN4 will depend on RAN1 feedback if any. Issue 2-2-2: Applicability for 2Rx/4Rx <Way forward>: No decision and agreements are made in this meeting, future discussion in RAN4 will depend on RAN1 feedback if any. Issue 3-1-1: Whether to remove ΔPpowerclass for PCMAX_H,f,c <Way forward>: Further discuss whether to remove the ΔPPowerClass=3dB applied for PCMAX_H,f,c for a PC2 capable UE with the support of TxD, if it further indicates the support of 1TxR AS-SRS. Issue 3-2-1: whether to remove or not the guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage <Way forward>: Open issue needs further discussion Issue 3-3-1: Which release 8Rx can be release independent from. <Way forward>: FFS the following approach can be agreeable in next meeting. - 8Rx without SRS-AS can be release independent from Rel-15. - 8Rx with SRS-AS can be release independent from Rel-X. - Further discuss “Rel-X” by clarifying which SRS-AS patterns were introduced from which release. - Further discuss how to describe it in TS 38.307. Issue 3-4-1: Other RF Rx single carrier requirements (Proposal 1). <Way forward/Agreement> Further discuss the following options: For UE equipped with 8Rx, the UE RF Rx requirements other than REFSENS are - Option 1: Verified with 4Rx - Option 2: Verified with 8Rx Issue 3-4-1: CA requirements. <Way forward>: For 8Rx UE RF CA requirement, encourage interested companies to bring contributions in RAN4#107 with the following aspects: - Expected clarification/revision on the objective in WID if needed considering whether to consider band combinations in this WI. - For a band supporting 8Rx in single band operation, whether the band is mandated to support 8Rx in a band combination. - For whether example CA/DC band combinations (including EN-DC) is needed or not, check if proposal 2 is agreeable in next meeting: - Proposal 2: 8Rx requirements should apply to the band implemented with 8Rx in all existing band combinations including intra/inter band CA and DC (including EN-DC). - For whether additional MSD is needed, check if proposal 3 is agreeable in next meeting: - Proposal 3: Clarify the relationship between MSD and ΔRIB, 8R by adding the following sentence in section 7.3A.1. - For operations with 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,8R in Table 7.3.2-x when MSD > 0.

- Discussion

2.1 Delta Rib for 8Rx

8Rx for LTE was introduced in Rel-15 and the only change was for Refsens requirement which delta Rib for 8Rx was defined [2]. And the delta RIB,8R is defined as -4dB for b41/42/43 which considered the different ILs and also potential couplings among Rx paths in the RFFE due to the different antenna locations. This was similar as delta RIB,4R where these issues were also considered but is even worse for 8Rx.

These similar issues were also existing in NR, the difference comparing to LTE is that CPE/FWA will have larger size than smartphone, and some factors could be improved like the coupling among Rx paths, while some factors will be worse like the IL differences. For the factor of antenna correlation, it doesn’t impact the RF requirements. Therefore, overall in our view same requirement as LTE 8Rx can be applied for NR n41/n77/n78/n79 delta RIB,8R.

For n7, the frequency range is also similar as n41, in principle same value can be applied, however, as compromised value, -4.5dB can be applied.

Observation 1:   -4dB for delta RIB,8R was defined for LTE b41/42/43 considering the IL differences among different Rx paths and coupling among Rx paths, these issues also exist in NR CPE/FWA and no difference from LTE handheld UE.

Proposal 1:         Delta RIB,8R for NR CPE/FWA can be defined as -4dB for bands n41/n77/n78/n79 and -4.5dB for n7 as a package.

2.2 Pcmax,h improvement

The potential Pcmax,h improvement issue was also raised in previous meetings, i.e. whether it is possible to not reduce the 3dB power level for Pcmax,h when UE supports 2Tx but transmit with 1Tx as seen in below table.

For a UE that supports 2Tx and 1T8R SRS AS, further study whether 3dB power back off at main antenna defined for TxD (ΔPPowerClass) is applied for PCMAX_H,f,c or not.

The 3dB power back off at main antennas was defined in Rel-17 for TxD. And it considers UE support TxD but transmit with 1Tx for SRS signal then only one of the PAs will be activated which caused the 3dB power back off at main antenna. And the 3dB power back off will be applied to both Pcmax,L and Pcmax,H. In our view, this is the normal handling of Pcmax boundaries, however, we are open to the potential enhancement in the improvement of Pcmax,h if UE can transmit higher power. One example is that if UE supports 23+26 PAs but indicate TxD capability, but actually the 26 dBm PA can be applied in SRS antenna switching which may require to remove the 3dB reduction in the higher limit, but of course for this UE the lower limit should also remove the 3dB reduction. Following this logic it seems we need to differentiate whether TxD UE can transmit with PC2 in SRS antenna switching.

Observation 2:   3dB power back off was defined in Rel-17 for the case that UE support TxD but only one PA transmit in t1r4 then the max power at main antenna will be reduced by 3dB. And this 3dB will be applied to both Pcmax,L and Pcmax,H.

Observation 3:   If consider the case that UE supporting TxD but can transmit with 1Tx full power in SRS antenna switching, then probably new signaling is needed to indicate whether TxD UE can do that.

Proposal 2:         Discuss whether RAN4 need to consider the case that UE supporting TxD but can transmit with 1Tx full power in SRS antenna switching, and the corresponding signaling to differentiate TxD UE capabilities if needed.

2.3 Which release 8Rx can be release independent from

For which release 8Rx can be applied, the SRS antenna switch capabilities like 1t8r, etc. was introduced in Rel-17, and normally it cannot be applied earlier than that release. And if not consider the SRS antenna switching, our preference is no impact the Rel-15/16 where UEs already on the market.

Proposal 3:         For 8Rx with SRS-AS it can be release independent from Rel-17, and for 8Rx without SRS-AS from release 17 is preferred and doesn’t impact the Rel-15/16.

2.4 Other RF Rx single carrier requirements

The only change to current spec for 8Rx is the REFSENS which means for other requirements the general single CC requirement will be applied. For these 8Rx capable requirements UE also supports 4Rx, e.g. mandatory support 4Rx for n7/n41/n77/n78/n79, and optional support 4Rx for other bands, therefore, the 4Rx requirements can be applied.

Observation 4:   The only change to current spec for 8Rx is the REFSENS which means for other requirements the general single CC requirement will be applied.

Proposal 4:         For other RF Rx requirements, the 4Rx requirements are applied for 8Rx.

2.5 CA requirements for 8Rx

In our understanding CA is not part of this WI according to the WID. Therefore, no more discussion is needed until the WID clearly include 8Rx CA.

Proposal 5:         Stop discussion of CA 8Rx until the WID include it.

- Conclusion

In this paper, we discussed requirements for 8Rx got the observations and proposals.

Observation 1:   -4dB for delta RIB,8R was defined for LTE b41/42/43 considering the IL differences among different Rx paths and coupling among Rx paths, these issues also exist in NR CPE/FWA and no difference from LTE handheld UE.

Proposal 1:         Delta RIB,8R for NR CPE/FWA can be defined as -4dB for bands n41/n77/n78/n79 and -4.5dB for n7 as a package.

Observation 2:   3dB power back off was defined in Rel-17 for the case that UE support TxD but only one PA transmit in t1r4 then the max power at main antenna will be reduced by 3dB. And this 3dB will be applied to both Pcmax,L and Pcmax,H.

Observation 3:   If consider the case that UE supporting TxD but can transmit with 1Tx full power in SRS antenna switching, then probably new signaling is needed to indicate whether TxD UE can do that.

Proposal 2:         Discuss whether RAN4 need to consider the case that UE supporting TxD but can transmit with 1Tx full power in SRS antenna switching, and the corresponding signaling to differentiate TxD UE capabilities if needed.

Proposal 3:         For 8Rx with SRS-AS it can be release independent from Rel-17, and for 8Rx without SRS-AS from release 17 is preferred and doesn’t impact the Rel-15/16.

Observation 4:   The only change to current spec for 8Rx is the REFSENS which means for other requirements the general single CC requirement will be applied.

Proposal 4:         For other RF Rx requirements, the 4Rx requirements are applied for 8Rx.

Proposal 5:         Stop discussion of CA 8Rx until the WID include it.

- References

[1] R4-2306597, WF on UE RF for 8Rx, NTT DoCoMo, RAN4#106bis-e, April 2023

[2] R4-1803450, CR for 36.101: Reference sensitivity power level, HW