3GPP TSG-RAN4 Meeting #107		R4-2309075

Incheon, South Korea, May 22nd – 26th, 2023

Agenda item:	8.28.2.4

Source:	Apple

Title:	On Rel-18 coverage enhancement

Release:	Rel-18

Document for:	Approval

# 1  Introduction

The WI [1] on NR coverage enhancements aims to reduce MPR/PAR through diverse techniques. During RAN4#106bis-e simulation data has been collected from several companies and combined into a large excel document. This contribution provides detailed analysis of available data and makes observations and draws conclusions.

# 2  Discussion

2.1 Shaping function and frequency extension

One discussion in RAN4#106bis-e was revolving around the gNB receiver implementation. In case of fully transparent schemes the receiver does not require any modifications to achieve maximum demod performance. For non-transparent techniques modifications at receiver side are required for optimum performance. If a conventional receiver (which only considers the in-band portion of the signal) is deployed at gNB then some of the waveform energy is excluded during the demodulation process and therefore information which would improve the result. In case of spectrum extension, the extension contributes to the total output power which also contributes to the OBO. With ignoring the extension, the loss in power is strongly dependent on shaping filter and the extension factor as displayed in Figure 2. If a conventional receiver would be deployed in the field, then the net-gain analysis needs to consider this loss by adjusting OBO accordingly.

| Fig. 1: Frequency domain response of shaping filter | Fig. 2: Power loss of conventional receiver |
| --- | --- |

Observation 1: If a conventional receiver (which only considers the in-band portion of the signal) is deployed at gNB then some of the waveform energy is excluded during the demodulation process and therefore information which would improve the result. In case of spectrum extension, the extension contributes to the total output power which also contributes to the OBO. If a conventional receiver would be deployed in the field, then the net-gain analysis needs to consider this loss by adjusting OBO accordingly.

2.3 Evaluation of simulation results

This section provides analysis on the simulations results which have been collected during RAN4#106bis-e and post meeting. The net-gain for transparent and non-transparent schemes are calculate and condensed to allow making conclusions. For details on the calculation and analysis the reader is asked to check the companion excel sheet. In the following the results are provided and the reasoning on data crunching.

In the following the company names are directly mentioned (instead of company A, B etc.) to make comparison easier and to honor the tremendous effort which has been spent in generating and in providing the results.

The net-gain has been calculated using the contributions from Ericsson, Huawei, Nokia, Qualcomm and vivo. Both frequency ranges (700MHz and 4GHz) are evaluated. In the following the focus lies on 4GHz and the details on 700MHz are provided in the annex.

In most cases companies provide several data points for a certain RB configuration. The setup could differ in deployed shaping filter, use of alternative transparent peak cancelation methods, extension factor or MCS. For each data point the net-gain is calculated by utilizing the agreed net-gain equation from WF [2]. The net-gain results of all the individual data points can be found in the companion excel sheet. In the following the tremendous dataset is reduced by aggregating the data points of each RB configuration. For this aggregation the max() operator is used. As example, if there exist four data points for inner allocations with 16 RBs then the maximum net-gain value is used for further processing. The reasoning is that a UE vendor would optimize its setup in terms of shaping filter and peak cancelation to achieve optimum net-gain. With this approach the max of the net-gain is collected for inner allocations in Table 1 and outer allocations in Table 2.

Table 1: Max of Net-Gain - Inner Allocations- Transparent Scheme (4GHz)

| RB | Nokia | Qualcomm | vivo | Huawei | Ericsson | Average |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  |  |  | -0.02 |  | 0.00 |
| 8 | 0.04 | 1.50 |  | -0.13 | -0.12 | 0.38 |
| 16 | 0.12 | 1.50 | -3.09 |  |  | 0.54 |
| 24 | 0.19 | 1.20 | -3.14 |  |  | 0.46 |
| 32 | 0.31 | 1.20 | -3.68 |  |  | 0.50 |
| 36 |  |  |  | 0.64 |  | 0.64 |
| 40 | 0.32 | 1.00 | -3.65 | -0.14 | -0.24 | 0.26 |

Table 2: Max of Net-Gain - Outer Allocations- Transparent Scheme (4GHz)

| RB | Nokia | Qualcomm | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- | --- |
| 8 | 1.32 | 1.00 |  | 0.69 | 1.00 |
| 16 | 1.14 | 1.00 | -1.87 |  | 0.71 |
| 24 | 1.47 | 0.90 | -2.02 |  | 0.79 |
| 32 | 1.39 | 0.90 | -2.28 |  | 0.76 |
| 40 | 1.42 | 0.50 | -2.21 | 0.76 | 0.67 |

Viewing Table 1 it can be easily observed that there is certain discrepancy between individual company results.  In case of inner allocations Ericsson, Huawei, Nokia and vivo consistently provide results below 0.5dB with only one exception being 0.64dB. Vivo reports the lowest net-gain overall. Outstanding are the results from Qualcomm as those report up to 1.5dB. Qualcomm results for transparent schemes are obtained by using other peak cancelation techniques which differ from FDSS.

Observation 2: Outstanding for transparent schemes are the results from Qualcomm as those report up to 1.5dB. Qualcomm results are obtained by using other peak cancelation techniques which differ from FDSS. It seems that transparent schemes other than FDSS should be considered to find optimum performance.

When observing Table 2 it can be concluded that outer allocations have good power boosting potential in general with using FDSS or other peak cancelation techniques for transparent deployment. Data from vivo is an exception as the net-gain is strongly negative.

The average is calculated by summation of all results and dividing through the number of companies which provided results for the specific RB configuration. In case negative net-gain is reported then the value is replaced by zero in the summation. The reasoning is that the optimum net-gain for transparent techniques is obtained by optimizing the shaping filter or using other sophisticated peak cancelation techniques. In case of shaping filter, a UE vendor would explore different filter types (e.g. 2-tap, 3-tap, tRCC) and also optimize its coefficients. The filter choice is important as strong filter such as [0.335 1 0.335] can introduce large EVM distortion. If strong filter result in a negative net-gain then lighter filter (such as [0.17 1 0.17]) might be a better choice. Even if a light filter would also result in negative net-gain the last resort would be to do nothing by simply deploying a one-tab filter with union energy. This would result into baseline performance with a net-gain of zero. Another option would be to utilize different peak cancelation techniques. Therefore, the net-gain of transparent techniques can never fall below zero if optimization is done by considering all options. In worst case the net-gain for transparent schemes can only approach zero.

Observation 3: The net-gain of transparent techniques can never fall below zero if optimization is done by considering all options. The worst case for the net-gain is that it approaches zero.

Next the net-gain of FDSS with spectrum extension is analyzed. The Table 3 and Table 4 contain the results for inner and outer allocations. The same approach as for transparent techniques is used by using the max() operator to condense the multitude of data points. The majority of companies results show that there is positive net-gain for FDSS-SE. Those results consistently indicate gains in the range of 1 to 2dB dependent on the RB configuration.

Table 3: Max of Net-Gain: Inner Allocations- FDSS-SE (4GHz)

| RB | Nokia | Qualcomm | vivo | Huawei | Ericsson | Average |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  |  |  | 1.56 |  | 1.56 |
| 8 | 0.47 | 1.50 | 1.04 | 1.43 | -0.21 | 0.89 |
| 16 | 0.50 | 1.30 | 0.71 |  |  | 0.84 |
| 24 | 0.69 | 2.10 | 0.97 |  |  | 1.25 |
| 32 | 0.83 | 1.00 | 1.88 |  |  | 1.24 |
| 36 |  |  |  | 1.13 |  | 1.13 |
| 40 | 0.96 | 1.50 | 0.73 | 1.84 | 0.14 | 1.03 |

Table 4: Max of Net-Gain: Outer Allocations- FDSS-SE (4GHz)

| RB | Nokia | Qualcomm | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- | --- |
| 8 | 1.46 | 1.50 | 2.07 | 1.12 | 1.54 |
| 16 | 1.05 | 1.10 | -0.13 |  | 0.72 |
| 24 | 2.07 | 1.90 | 1.82 |  | 1.93 |
| 32 | 1.52 | 0.90 | 1.48 |  | 1.30 |
| 40 | 2.01 | 1.30 | 1.12 | 1.65 | 1.52 |

The next step is to compare the net-gain of transparent techniques with the net-gain from FDSS-SE. The comparison is achieved by using ‘delta net-gain’ as metric. The delta net-gain is calculated as follows:

Delta Net-Gain = Net-Gain(FDSS-SE) – max(Net-Gain(transparent), 0)

The delta net-gain is calculated by considering the FDSS-SE gain and subtracting the gain from transparent techniques. The max() operator is deployed as some net-gain results are negative. As explained above, the net-gain for transparent schemes cannot become negative if properly optimized. The lowest value for net-gain of transparent schemes can only be zero. As consequence the delta net-gain equation limits the transparent net-gain to a minimum of zero and prevents an overestimation of non-transparent schemes.

Table 5 and Table 6 present the delta net-gain of inner and outer allocations. Please note that there is no transparent result for inner and outer values with 8RBs from vivo but there are results for FDSS-SE. For calculating delta net-gain the transparent gain was simply set to zero for this specific case. The average is calculated by summation of individual values and dividing by the number of contributing companies.

Table 5: Delta Net-Gain - Inner Allocations (comparison between FDSS-SE and TS) (4GHz)

| RB | Nokia | Qualcomm | vivo | Huawei | Ericsson | Average |
| --- | --- | --- | --- | --- | --- | --- |
| 6 |  |  |  | 1.56 |  | 1.56 |
| 8 | 0.43 | 0.00 | 1.04 | 1.43 | -0.21 | 0.54 |
| 16 | 0.38 | -0.20 | 0.71 |  |  | 0.30 |
| 24 | 0.50 | 0.90 | 1.88 |  |  | 0.79 |
| 32 | 0.52 | -0.20 | 1.88 |  |  | 0.73 |
| 36 |  |  |  | 0.49 |  | 0.49 |
| 40 | 0.63 | 0.50 | 0.73 | 1.84 | 0.14 | 0.77 |

Table 6: Delta Net-Gain - Outer Allocations (comparison between FDSS-SE and TS) (4GHz)

| RB | Nokia | Qualcomm | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- | --- |
| 8 | 0.14 | 0.50 | 2.07 | 0.43 | 0.78 |
| 16 | -0.09 | 0.10 | -0.13 |  | -0.04 |
| 24 | 0.59 | 1.00 | 1.82 |  | 1.14 |
| 32 | 0.13 | 0.00 | 1.48 |  | 0.54 |
| 40 | 0.58 | 0.80 | 1.12 | 0.89 | 0.85 |

The delta net-gain value is majorly positive for the different RB allocations. For inner allocations Qualcomm and Ericsson results show the lowest delta. In case of Qualcomm this is the consequence of the transparent scheme already showing good net-gain results. The average ‘delta net-gain’ across all analyzed RB allocations is approximately 0.78dB. For outer allocations the average across all RB allocations is approximately 0.65dB.

Observation 4: The average advantage of FDSS-SE versus transparent techniques across all analyzed RB allocations is approximately 0.74dB. For outer allocations the average advantage is approximately 0.65dB. The largest RB allocation size consists of 40 RBs.

Qualcomm results are interesting as those indicate that by using advanced peak cancelation techniques the net-gain for transparent schemes can be considerably higher than what is achievable with FDSS. Transparent techniques have the advantage that those are far simpler to be implemented. Impact on baseband processing is lower. Also, the optimization effort during implementation is reduced. With transparent techniques a UE vendor has only to consider the amount of RBs and their location for finding an optimum setup. In case of non-transparent techniques such as FDSS-SE the UE vendor must additionally consider the extension factor and the adapted MCS. The extension factor introduces another dimension which needs to be handled when optimizing the net-gain. The various data needs to be stored in the UE with large tables creating additional costs as more memory is required. Considering the low advantage of FDSS-SE versus existing transparent schemes it is proposed to lay the focus on optimizing transparent techniques.

Observation 5: Transparent techniques have the advantage that those are far simpler to be implemented. Impact on baseband processing is lower. Also, the optimization effort during implementation is reduced. With transparent techniques a UE vendor has only to consider the amount of RBs and their location for finding an optimum setup. In case of non-transparent techniques such as FDSS-SE the UE vendor must additionally consider the extension factor and the adapted MCS. The extension factor introduces another dimension which needs to be handled when optimizing the net-gain. The various data needs to be stored in the UE with large tables creating additional costs as more memory is required.

Proposal: Considering the low advantage of FDSS-SE versus existing transparent schemes it is proposed to lay the focus on optimizing transparent techniques.

# Conclusions

This contribution provides several simulation results for coverage enhancement with the use of spectrum shaping and spectrum extension. The following observations and proposals are made:

Observation 1: If a conventional receiver (which only considers the in-band portion of the signal) is deployed at gNB then some of the waveform energy is excluded during the demodulation process and therefore information which would improve the result. In case of spectrum extension, the extension contributes to the total output power which also contributes to the OBO. If a conventional receiver would be deployed in the field, then the net-gain analysis needs to consider this loss by adjusting OBO accordingly.

Observation 2: Outstanding for transparent schemes are the results from Qualcomm as those report up to 1.5dB. Qualcomm results are obtained by using other peak cancelation techniques which differ from FDSS. It seems that transparent schemes other than FDSS should be considered to find optimum performance.

Observation 3: The net-gain of transparent techniques can never fall below zero if optimization is done by considering all options. The worst case for the net-gain is that it approaches zero.

Observation 4: The average advantage of FDSS-SE versus transparent techniques across all analyzed RB allocations is approximately 0.74dB. For outer allocations the average advantage is approximately 0.65dB. The largest RB allocation size consists of 40 RBs.

Observation 5: Transparent techniques have the advantage that those are far simpler to be implemented. Impact on baseband processing is lower. Also, the optimization effort during implementation is reduced. With transparent techniques a UE vendor has only to consider the amount of RBs and their location for finding an optimum setup. In case of non-transparent techniques such as FDSS-SE the UE vendor must additionally consider the extension factor and the adapted MCS. The extension factor introduces another dimension which needs to be handled when optimizing the net-gain. The various data needs to be stored in the UE with large tables creating additional costs as more memory is required.

Proposal: Considering the low advantage of FDSS-SE versus existing transparent schemes it is proposed to lay the focus on optimizing transparent techniques.

# 4  References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- R4-2306627, “WF on remaining open issues for MPR/PAR reduction for coverage enhancement”, Nokia, RAN4#104-bis-e

- R4-2220842, “LS on RF simulation parameters to be used in RAN4 for power domain enhancements”, Nokia, RAN4#105

# 5  Annex

The annex shall provide a quick overview on the results for 700MHz. For details on the calculation the reader is asked to check the companion excel sheet containing all the data and calculations.

The Table A1 and Table A2 contain the results for inner and outer allocations with transparent schemes. As explained in the section 2 the max() operator is used to condense the multitude of data points.

Table A1: Delta Net-Gain - Inner Allocations – Transparent Scheme (700MHz)

| RB | Nokia | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- |
| 8 | 0.25 |  | -0.14 | 0.08 |
| 16 | 0.00 | -2.59 |  | 0.00 |
| 24 | -0.00 | -2.36 |  | 0.00 |
| 32 | -0.02 | -2.39 |  | 0.00 |
| 40 | 0.07 | -2.32 | 1.40 | 0.49 |

Table A2: Delta Net-Gain - Outer Allocations – Transparent Scheme (700MHz)

| RB | Nokia | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- |
| 8 | 1.53 |  | 0.50 | 1.01 |
| 16 | 1.22 | -0.78 |  | 0.61 |
| 24 | 1.25 |  |  | 0.62 |
| 32 | 1.39 | -1.11 |  | 0.69 |
| 40 | 1.42 | -1.12 | 0.71 | 0.71 |

The results for inner and outer allocations with FDSS-SE can be found in Table A3 and Table A4.

Table A3: Max of Net-Gain: Inner Allocations- FDSS-SE (700MHz)

| RB | Nokia | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- |
| 8 | 0.71 | 1.03 | -0.12 | 0.58 |
| 16 | 0.87 | 0.02 |  | 0.45 |
| 24 | 0.97 | 0.43 |  | 0.70 |
| 32 | 0.56 | 0.92 |  | 0.74 |
| 40 | 0.80 | 0.41 | 0.86 | 0.69 |

Table A4: Max of Net-Gain: Outer Allocations- FDSS-SE (700MHz)

| RB | Nokia | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- |
| 8 | 1.80 | 1.60 | 1.29 | 1.56 |
| 16 | 1.63 | 0.07 |  | 0.85 |
| 24 | 2.08 | 1.41 |  | 1.74 |
| 32 | 1.42 | 0.72 |  | 1.07 |
| 40 | 1.85 | 1.33 | 2.05 | 1.74 |

The delta net-gain is found in Table A5 and A6. Calculation of delta net-gain is described in section 2.

Table A5: Delta Net-Gain - Inner Allocations (comparison between FDSS-SE and TS) (4GHz)

| RB | Nokia | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- |
| 8 | 0.47 | 1.03 | -0.12 | 0.46 |
| 16 | 0.87 | 0.02 |  | 0.44 |
| 24 | 0.97 | 0.43 |  | 0.70 |
| 32 | 0.56 | 0.92 |  | 0.74 |
| 40 | 0.73 | 0.41 | -0.54 | 0.20 |

Table A6: Delta Net-Gain - Outer Allocations (comparison between FDSS-SE and TS) (4GHz)

| RB | Nokia | vivo | Ericsson | Average |
| --- | --- | --- | --- | --- |
| 8 | 0.27 | 1.60 | 0.79 | 0.89 |
| 16 | 0.42 | 0.07 |  | 0.24 |
| 24 | 0.83 | 1.41 |  | 1.12 |
| 32 | 0.03 | 0.72 |  | 0.38 |
| 40 | 0.43 | 1.33 | 1.34 | 1.03 |

Using the results an average gain over all RB allocations is approximately 0.51dB for inner allocations and approximately 0.73dB for outer allocations.