3GPP TSG-RAN WG4 Meeting #107	R4-2310245

Incheon, KR, 22 ‒ 26 May, 2023

Title: 	WF on PC2 A-MPR for band n8 and n28

Agenda Item:	7.22.2

Source: 	Huawei, HiSilicon, Apple, China Unicom

Document for:	Approval

# <Background>

The PC2 A-MPR requirements for band n8 and n28 have been discussed for several meetings. Based on the simulation results in [1-4], the proposals are merged into one set of requirements, which are recommended to be adopted into the spec.

References:

- R4-2304518 PC2 A-MPR for band n8 NS_43, Huawei, HiSilicon, RAN4#106bis-e

- R4-2304519 PC2 A-MPR for band n28 NS_18, Huawei, HiSilicon, RAN4#106bis-e

- R4-2309059 On PC2 A-MPR Proposal for NS_43, Apple, RAN4#107

- R4-2309060 On PC2 A-MPR Proposal for NS_18, Apple, RAN4#107

# <Band n8 NS_43 PC2 A-MPR>

<Agreement>:

The existing A-MPR regions for PC3 as defined in TS38.101-1 are adapted for PC2, which are shown as the table below. The new table is applicable for both power classes without impact to the PC3 requirements.

Table 6.2.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | ≥ 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: Only 15 kHz SCS is applicable for power class 3; both 15kHz and 30kHz SCS are applicable for power class 2. NOTE 3: Void |  |  |  |  |  |  |  |

The A-MPR values for PC2 1Tx are defined as in the tables below.

Table 6.2.3.6-2a: A-MPR for NS_43 (Power Class 2, 1Tx)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | 1 | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A |
|  | QPSK | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  |
|  | 64 QAM | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 5 |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | ≤ 4 |  |  |  |  |  | ≤ 4 |  | ≤ 6 |  |
|  | 16 QAM | ≤ 4 |  |  |  |  |  | ≤ 4.5 |  | ≤ 6 |  |
|  | 64 QAM | ≤ 4 |  | ≤ 5 |  | ≤ 6.5 |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.6-2b: A-MPR for NS_43 (Power Class 2, 1Tx) Cont.

| Modulation/Waveform |  | A6 | A7 |  |
| --- | --- | --- | --- | --- |
|  |  | Outer / Inner (dB) | Outer (dB) | Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 12 | ≤ 4 | 3 |
|  | QPSK | ≤ 12 | ≤ 4 | 3 |
|  | 16 QAM | ≤ 12 | ≤ 5 | 4 |
|  | 64 QAM | ≤ 12 | ≤ 5.5 | 5.5 |
|  | 256 QAM | ≤ 12 | ≤ 6.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 12 | ≤ 6 | 4.5 |
|  | 16 QAM | ≤ 12 | ≤ 6 | 5 |
|  | 64 QAM | ≤ 12 | ≤ 6.5 | 6 |
|  | 256 QAM | ≤ 12 | ≤ 7 | 7 |

<Way forward>:

Companies are encouraged to bring contributions to the next meeting on PC2 A-MPR for NS_43U as well as the requirements for 2Tx PC2.

# <Band n28 NS_18 PC2 A-MPR>

<Agreement>:

The existing A-MPR regions for PC3 as defined in TS 38.101-1 are adapted for PC2, which are shown as in the tables below. The new tables are applicable for both power classes without impact to the PC3 requirements.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 for PC3; Table 6.2.3.13-2, A1 for PC2 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 for PC3; Table 6.2.3.13-2, A2 for PC2 |
|  |  |  | 25, 30 |  | Table 6.2.3.13-1, A3, A4, A5 for PC3; Table 6.2.3.13-2, A3, A4, A5, A6 for PC2 |

Table 6.2.3.13-0: Band n28 and n83 25MHz and 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

The A-MPR values for 1Tx PC2 are defined as in the table below.

Table 6.2.3.13-2: A-MPR for NS_18 (Power Class 2, 1Tx)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | 3.5 | 10 | 4.5 | 2 |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | 3.5 | 10 | 4.5 | 2 |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | 3.5 | 11 | 5 | 2 |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | 3.5 | 11 | 5.5 | 2.5 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | 3.5 | 11 | 5.5 |  |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | 5 | 11.5 | 6 | 4 |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | 5 | 11.5 | 6 | 4 |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | 5 | 11.5 | 6 | 4 |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | 5 | 11.5 | 7.5 |  |

Note that BW=25MHz is missing for NS_18 in Table 6.5.3.3-1 of TS38.101-1 and will be added.

Table 6.5.3.3.3-1: Additional requirements for "NS_18"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 25, 30 |  |
| 692-698 | -26.2 | 6 MHz |

<Way forward>:

Companies are encouraged to bring contributions to the next meeting on PC2 A-MPR for NS_17 as well as the requirements for 2Tx PC2.