3GPP TSG-RAN WG4 Meeting # 108		R4-2311052

# Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.4.1.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Power imbalance for SRS antenna switching

Document for:	Approval

# Introduction

Although RAN4#107 approved WF [1], there have been no concrete agreement. In our understanding, this situation comes from the fact that companies may not have a common understanding about UE behavior in terms of power control of each port in the same SRS resource set not only in RAN4 but also in RAN1. This paper shares way forwards on how to move on.

# Discussion

## Interpretation of 38.213

A FL summary of [2] captures following questions and corresponding views of each of the companies.

What we can observe from the above table is that at least companies’ interpretations of expected UE behavior in terms of power control of each port in the same SRS resource set is diverse. With this condition, just introducing allowance of reporting ∆TRxSRS,p (p=port numbering) not meaningful, since even if ∆TRxSRS,p is reported, network has no idea on if the UE is supplementing the lost power due to ∆TRxSRS,p or not. If the network takes ∆TRxSRS,p into account, but the UE supplements the ∆TRxSRS,p, this just brings more channel estimation errors.

Observation 1: The current specifications do not provide readers with a unified interpretation in terms of power control per port in the same SRS resource set.

- It’s noted that at least our understanding is that UE must perform the same power control across ports in the same SRS resource set.

Observation 2: Just introducing allowance of reporting ∆TRxSRS,p (p=port numbering) not meaningful considering the Observation 1.

Coming back to the above table in FL, specifically for Q1, it may have been difficult for companies to response to the Q1 in a way of Yes or No, since it may not be feasible for UE to supplement the lost powers across ports due to ILs up to e.g., an advertised power class, as we pointed out in our previous contribution [3]. For more specific, assume that a PC3 UE has two ports where a primary path has 3 dB RFFE loss and a secondary path has 3 dB additional insertion loss, i.e., 6 dB RFFE loss, compared to that for primary path. The UE may be able to supplement the lost power in the secondary port by 3 dB up to 20 dBm, but the UE may not be able to do that in case the power exceeds 20 dBm as can be seen in case 3 in Figure 1. It is noted that our understanding is that a UE has to achieve the same power across the ports by supplementing the lost power in the secondary port given that the two ports are treated in the same SRS resource set.

Figure 1: No power imbalance

From Observations 1 and 2, it is clear that there is no meaning to discuss resolutions alone without clear UE behaviours on whether a UE supplements the lost powers due to insertion losses across the ports to achieve the same power or not, or even if it does, if the supplement is feasible up to the advertised power class or not. Whichever RAN1 takes, it is important to share the fact with RAN1 that the power supplement may not be always feasible for the entire power range, i.e., between min and max output power equivalent to an advertized power class. Overall, roughly there are following UE implementation options.

Option 1: Supplement the lost power(s) across ports up to the advertised power class.

Option 2: Supplement the lost power(s) across ports up to “the advertised power class -  max(∆TRxSRS,p)”

Option 3: Not supplement the lost power(s) at all across port and maintain the power imbalances across ports according to ∆TRxSRS,p, i.e., P0, P1 - ∆TRxSRS,1, …., Pp - ∆TRxSRS,p.

It is up to RAN1 if specifications allow all the options as implementations together with suitable resolutions or selects one or some of them, but it is needed that not all the UEs cannot implement the option 1 so that RAN1 needs to take UE behavior of the option 2 into account in their discussion. In fact, according to options, future accompanied specification works must be different. For example, if the option 1 is selected, there would be no need to change the current power control equation since in any case, all the ports in the same SRS resource set shall be able to achieve the same target power across the ports. If option 2 is selected, however, then, power control per port may be needed as proposed in [4]. Or alternatively, specifications may define that the power control is for the primary (referenced) antenna port, where ∆TRxSRS,0 is zero dB. Then, when the primary port power is approaching to the power of  “the advertised power class -  max(∆TRxSRS,p)”, the network commences to need to consider ∆TRxSRS,p to correct power imbalances, e.g., for PC3 with two ports, ∆TRxSRS,1 is 3 dB, when the primary port power is 21, 22, or 23 dBm, then, the secondary port power is 20 dBm, 20 dBm, 20 dBm, respectively so that actual deltas are considered as 1, 2 or 3 dB depending on the power of the reference port meaning that even if the reported ∆TRxSRS,1 is 3 dB, values that network uses for correcting DL channel estimation may not always exactly follow ∆TRxSRS,p.

Proposal: Send an LS to inform RAN1 of at least following possible options in their future discussion.

Option 1: Supplement the lost power(s) across ports up to the advertised power class.

Option 2: Supplement the lost power(s) across ports up to “the advertised power class -  max(∆TRxSRS,p)”

Option 3: Not supplement the lost power(s) at all across port and maintain the power imbalances across ports according to ∆TRxSRS,p, i.e., P0, P1 - ∆TRxSRS,1, …., Pp - ∆TRxSRS,p.

## Power supplement accuracy

Some companies have shared simulation results to show gains thanks to the introduction of reporting ∆TRxSRS,p. In our understanding, the simulation must be executed assuming that power imbalance perfectly follows reported ∆TRxSRS,p. For example, if ∆TRxSRS,1 is 3 dB, the powers at port 0 is X dBm and port 1 is always X – 3 dBm. We would like to question on the accuracy of the power control across ports. Since if the amount of accuracy is not what we expect, the expected gain may not be able to be obtained. The accuracy of the power per port is normally not good when the power is low, while even if the power is high, the tolerance is +2/-2 at best. Then, another question arises whether power setting errors across ports have a high correlation or not. For more specifically, if power at port 0 is + 1 dB error, is power at port 1 also + 1 dB error? If this is always the case, the power setting errors across ports have a high (perfect with the example) correlation, the reported value may be utilized by network apart from real gain and usefulness. But if not, errors on errors may happen after network corrects channel estimation with ∆TRxSRS,p, hence, the reported value cannot be used by the network.

When it comes to the above aspect, requirement in 6.4D.4 may be a possible reference. According to 6.4D.4, 4 dB power difference is allowed between ports. With this large amount of allowed error, just reporting ∆TRxSRS,p may or may not improve DL channel estimation.

Observation 3: UE’s ability to accurately set powers across ports to the same should be studied, e.g., how much accuracy is achieved.

Observation 4: In case, high accuracy to make the report meaningful requires a special implementation, a specific requirement and UE capability is needed.

## Rx IL imbalance

As summarized in RAN1 FL summary, Rx IL imbalance doesn’t always exist. It means that Rx IL imbalance may exist in some cases. If a UE has Rx IL imbalance, the imbalance also impacts on DL channel estimation and the amount of the impact depends on correlation between Tx ILs per port and Rx ILs per corresponding port. For instance, assume following cases.

Case 1: Tx IL (0 dB@port 0, 3 dB@port 1) and Rx IL (0 dB@port 0, 3 dB@port 1)

Case 1: Tx IL (0 dB@port 0, 3 dB@port 1) and Rx IL (0 dB@port 0, 1.5 dB@port 1)

For Case 1, there is no need to report ∆TRxSRS,p or network should not take ∆TRxSRS,p into account at all, since Tx power imbalance impact on channel estimation is fully offset by Rx IL imbalance (since Rx signal at port 1 experiences 3 dB power attenuation due to Rx IL anyway). If a network takes the Tx IL into account, propagation loss including RFFE via port 1 is overestimated by 3 dB.

For Case 2, if a network takes the reported Tx IL into account, then propagation loss via port 1 is underestimated by 1.5 dB. If the network doesn’t take it into account, the loss via port 1 is overestimated by 1.5 dB.

As such, in case Rx IL imbalance exists, this must impact on channel estimation and that impact must be taken into account unless the Rx imbalance is purely zero.

Observation 5: Rx IL imbalance must impact on channel estimation as Tx IL imbalance does. The amount of impact can change depending on relation between Tx IL imbalance and Rx IL imbalance. In case, the resolution on Tx IL imbalance is addressed, Rx IL imbalance must be also considered unless the Rx imbalance is zero.

# Conclusion

This document has made the following observations and proposal.

For interpretation of TS38.213

Observation 1: The current specifications do not provide readers with an unified interpretation in terms of power control per port in the same SRS resource set.

- It’s noted that at least our understanding is that UE must perform the same power control across ports in the same SRS resource set.

Observation 2: Just introducing allowance of reporting ∆TRxSRS,p (p=port numbering) not meaningful considering the Observation 1.

Proposal: Send an LS to inform RAN1 of at least following possible options in their future discussion.

Option 1: Supplement the lost power(s) across ports up to the advertised power class.

Option 2: Supplement the lost power(s) across ports up to “the advertised power class -  max(∆TRxSRS,p)”

Option 3: Not supplement the lost power(s) at all across port and maintain the power imbalances across ports according to ∆TRxSRS,p, i.e., P0, P1 - ∆TRxSRS,1, …., Pp - ∆TRxSRS,p.

For Power supplement accuracy

Observation 3: UE’s ability to accurately set powers across ports to the same should be studied, e.g., how much accuracy is achieved.

Observation 4: In case, high accuracy to make the report meaningful requires a special implementation, a specific requirement and UE capability is needed.

For Rx IL imbalance

Observation 5: Rx IL imbalance must impact on channel estimation as Tx IL imbalance does. The amount of impact can change depending on relation between Tx IL imbalance and Rx IL imbalance. In case, the resolution on Tx IL imbalance is addressed, Rx IL imbalance must be also considered unless the Rx imbalance is zero.

# Reference

- R4-2310279, WF on UE 8Rx requirements, NTT DOCOMO, TSG-RAN WG4 #108

- R1-2306200, FL summary on UE SRS IL imbalance issue, Moderator (Huawei), TSG-RAN WG1 #113

- R4-2307122, Power imbalance indication for SRS antenna switching, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #107

- R4-2309724, On Correction of SRS-Based Downlink Channel Estimates, Lenovo, TSG-RAN WG4 #107