3GPP TSG-RAN WG4 Meeting # 108	R4-2312344

# Toulouse, France, August 21 – August 25, 2023

Source:	LG Electronics

Title:	Discussion on RRM core requirement for carrier phase positioning

Agenda item:	8.22.3.6

Document for:	Discussion

Introduction

RAN4 continued discussing RRM requirements for carrier phase positioning in RAN4#107 Further agreements and open issues were captured in a WF[1]. In this contribution, we provide our views on carrier phase positioning measurements and procedures related to RRM issues based on the approved WF [1] in RAN4#107.

Discussion

Issue 2-3-1: Report mapping for DL RSCP/RSCPD:

Agreements: - Reporting range: - [0, 360) degrees for DL RSCP. - FFS for DL RSCPD. - Granularity is FFS - Option 1: Fixed value - Option 2: Up to NW configuration and UE capability

In the last meeting, RAN4 agreed reporting range of DL RSCP. But some more informations for CPP method to work normally are needed. For example, let’s suppose that the UE knows the initial phase of gNB and the distance from gNB with 0.1 meter accuracy, from 30 meters to 30.1 meters. And the carrier frequency is 3GHz. Then the UE can know the exact distance from the gNB by measuring the carrier phase. If the phase is 0 degree, then the distance is 30 meters. Or if the phase is 90 degree, then the distance is 30.025 meters.

However, it’s not realistic to expect to know distance information to 0.1 meter accuracy in advance. So, let’s suppose that the UE knows the distance information to 1 meter accuracy in advance, from 30 meters to 31 meters. If the carrier phase which measured by UE is 0 degree, then the UE’s distance can not detect only with carrier phase. Because there are 10 candidates of distances. 30, 30.1, 30.2, 30.3, 30.4, 30.5, 30.6, 30.7, 30.8 and 30.9. This is an integer ambiguity. The integer ambiguity problem maybe solved by RAN1. So, RAN4 is not necessary to address the integer ambiguity. But RAN4 should discuss how to report the integer information.

Proposal 1: RAN4 to discuss how to report the integer information with carrier phase.

Option 1: Report the integer information and carrier phase separately.

Carrier phase ϕ ([0, 360) degrees ), integer Ι (0,1,2,…, [63])

Option 2: Report the value X after combining the integer information and carrier phase.

X = Ι×360+ϕ

where, Carrier phase ϕ ([0, 360) degrees ), integer Ι (0,1,2,…, [63])

According to WID[1], the target resolution of carrier phase positioning can be a few centimeter levels. And the resolution depends on carrier frequency. Here is an example of the carrier frequency 2GHz and 4GHz cases.

| Phase [degree] | Resolution [cm] @ carrier frequency 2GHz | Resolution [cm] @ carrier frequency 4GHz |
| --- | --- | --- |
| 1 | 0.0417 | 0.0208 |
| 3 | 0.125 | 0.0625 |
| 5 | 0.208 | 0.104 |
| 10 | 0.417 | 0.208 |
| 15 | 0.625 | 0.3125 |

From the example, for the unit for the carrier phase report, 1, 3, 5, 10, and 15 degrees can be considered.

Proposal 2: RSCP and RSCPD should have different report mapping tables and the candidates of the units 1, 3, 5, 10, and 15 degrees for the carrier phase report can be possible.

Conclusion

In this contribution, we provide our views on carrier phase positioning measurements and procedures related to RRM issues based on the approved WF [1] in RAN4#107.

Proposal 1: RAN4 to discuss how to report the integer information with carrier phase.

Option 1: Report the integer information and carrier phase separately.

Carrier phase ϕ ([0,360] degrees ), integer Ι (0,1,2,…, [63])

Option 2: Report the value X after combining the integer information and carrier phase.

X = Ι×360+ϕ

where, Carrier phase ϕ ([0,360] degrees ), integer Ι (0,1,2,…, [63])

Proposal 2: RSCP and RSCPD should have different report mapping tables and the candidates of the units 1, 3, 5, 10, and 15 degrees for the carrier phase report can be possible.

Reference

[1] R4-2310163, “WF on NR Positioning – SL positioning and CPP,” CATT.