3GPP TSG RAN WG4 Meeting #108	R4-2312364

Toulouse, 21 Aug. 2023 – 25 Aug. 2023

Source: 	Ericsson

Title:  	General aspects for In-Device Co-existence (IDC) enhancements

Agenda Item:	8.36.1

Document for:	Discussion

# Discussion

RAN4 shall take the agreements in LS R2-2306594 as follows into consideration.

1. Overall Description: According to the RAN2#122 meeting discussion, RAN2 discussed the UE behaviours regarding how to count the number of dropped UL slots for autonomous denial across all CC(s) within the same CG and the combination configuration of the LTE autonomous denial configuration and the NR autonomous denial configuration in EN-DC. RAN2 achieved the following agreements for autonomous denial solution: The LTE autonomous denial configuration is only for the LTE frequency in EN-DC. The UE sums up the denied UL slots together across all CC(s) in the CG. The UE sums up the UL slots together across all CC(s) in the CG as validity time period. The dropped/denied UL slots across CCs at the same time are counted as a single slot (based on longest slot). The details are up to UE implementation. 2. Actions: To RAN4 ACTION: RAN2 kindly asks RAN4 to take the above information into account for the further work, and define the corresponding RRM requirements if needed.

With respect to above LS, some interpretations shall be noted in TS 38.133. Referring to content of TS36.133, the IDC solution influence to measurement requirements is defined generally as follows: ﻿‘If IDC autonomous denial is configured, an additional delay can be expected.’ Since the statement spreads among different clauses in TS 38.133, adding note for each IDC autonomous denial requirement seems too redundant. Therefore, we suggest adding a chapter to present the interpretations in clause 3.6 ﻿‘Applicability of requirements in this specification version’.

Proposal 1: Add a chapter to present the interpretations (as LS listed) for autonomous denial solution in clause 3.6 ﻿‘Applicability of requirements in this specification version’ of TS 38.133.

# Conclusion

Proposal 1: Add a chapter to present the interpretations (as LS listed) for autonomous denial solution in clause 3.6 ﻿‘Applicability of requirements in this specification version’ of TS 38.133.

# References

- R4-2310162, WF on IDC enhancements, Xiaomi