3GPP TSG-RAN WG4 Meeting #108	R4-2312792

Toulouse, France, 21 – 25 August 2023

Agenda Item:	8.12.5.2

Source:	Ericsson

Title:	Simulation results of CA PDSCH demodulation requirements for FR2 HST

Document for:	Discussion

# 1	Introduction

RAN4#107 agreed with the way forward on PDSCH demodulation requirements for Rel-18 FR2 HST [1]. This contribution provides our updated PDSCH simulation results according to the simulation assumption [2] and discusses the remaining open issues.

# 2	Simulation results

Issue 2-1-6: Simulation results alignment and requirement derivation Agreement: - Companies are encouraged to provide the simulation result in the next meeting for further alignment according to the simulation assumption R4-2305893 - The requirement derivation and whether to add additional margin will be discussed based on the collected simulation results.

Table 1 shows our simulation results for FR2 HST-DPS according to the simulation assumption [2]. Note we have revised the FRCs as shown in Appendix, where we found the number of DMRS symbols in special slots did not follow RAN1 specification (See [3] for the correction CR of Rel-17 FR2 HST-DPS).

Table 1	PDSCH CA simulation results for alignment.

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |

# 3	Open issues

## 3.1	Applicability of CA test cases

Way Forward - FFS on Rel-18 FR2 HST PDSCH CA requirements are release independent from Rel-17.

The remaining open issue on the CA demodulation requirements is the PDSCH CA requirements are applicable from Rel-17 as the release independent, or applicable from Rel-18.

RAN4 had the similar discussion in Rel-17 FR1 HST WI on the applicable releases for CA PDSCH demodulation requirements. Rel-17 FR1 HST concluded that HST-DPS CA demodulation requirements are applicable from Rel-15 according to the UE capability, although HST-SFN CA demodulation requirements are applicable from Rel-16, as specified in TS 38.307 V17.10.0 below.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| … | … | … | … |
| HST-SFN CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 3) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-SFN CA demodulation enhancement: see Table C.4-1 |
| HST-DPS CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-15 (NOTE 4) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-DPS CA demodulation enhancement: see Table C.4-1 |
| […] NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. |  |  |  |

If we follow the conclusion of Rel-17 FR1 HST, we think Rel-17 FR2 HST-DPS CA demodulation requirements should be applicable from Rel-17 UE according to the UE capability.

Proposal: Rel-18 FR2 HST-DPS CA PDSCH demodulation requirements should be applicable from Rel-17 according to the UE capability.

# 4	Summary

Proposal: Rel-18 FR2 HST-DPS CA PDSCH demodulation requirements should be applicable from Rel-17 according to the UE capability.

# References

- R4-2309825, “WF for FR2 HST demodulation requirements”, Samsung.

- R4-2305893, “Simulation assumption for PDSCH requirements with CA”, Ericsson.

- R4-2312786, “[NR_HST_FR2-Perf] CR: Correction of FRC for FR2 HST-DPS UE demodulation requirements”, Ericsson, Anritsu.

# Appendix

Table 2	Corrected FRC for test 1 (HST-DPS-FR2-BI-B, single TCI states)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.2 TDD (For information) | FRC 1-1 | FRC 1-2 | FRC 1-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 | 13 |
| Allocated slots per 2 frames |  | 126 | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | N/A (Note 4) |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 7 | [2] | [4] | [15] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 180720 | [32640] | [75384] | [391392] |
| For Slots i = 3 and 83 (Note 3) | Bits | 107856 | [14976] | [38952] | [245664] |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,79,82,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Max. Throughput averaged over 2 frames | Mbps | 515.629 | [123.200] | [257.839] | [1031.409] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

Table 3	Corrected FRC for test 2 (HST-DPS-FR2-UNI-A, two TCI state)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD (For information) | FRC 2-1 | FRC 2-2 | FRC 2-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 | 13 |
| Allocated slots per 2 frames |  | 127 | 127 | 127 | 127 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | 90176 | [21504] | [45096] | [180376] |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 7 | [2] | [4] | [14] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | 11 | [3] | [6] | [22] |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slots i = 3 and 83 (Note 3) | Bits | 122832 | [24192] | [53928] | [260640] |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,79,82,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | 210672 | [51072] | [105336] | [421344] |
| Max. Throughput averaged over 2 frames | Mbps | 520.138 | [124.275] | [260.094] | [1040.428] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |