3GPP TSG-RAN WG4 Meeting # 108	R4-2313495

# Toulouse, France, August 21 – August 25, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Corrections needed due to not implemented R17 draftCR R4-2220561

Agenda Item:	7.1.1.1	band combinations with UL configurations including intra-band ULCA with IMD or triple beat issues	[WI code]

Document for:	Approval

# Introduction

In RAN4#105, a Draft Release 17 CR was endorsed but not implemented before the Release 18 specification was available. In the meantime, Release 18 and 17 specifications have evolved but are missing the corrections. In this meeting, we could cover some of those changes in the Release 17 and 18 CRs [2, 3] but some cannot be implemented as there are difference between R17 and R18 and the initial CR. In this contribution, we explain the remaining corrections that are needed, and assuming the support from the group, we could then implement those in revisions of CRs [2,3].

# Discussion

## Corrections needed in R4-2220561 but not implemented

This draft CR was making essential corrections to MSDs due to IMDs of intra-band UL configurations and adding explanatory notes. Added or corrected MSD in Table 7.3A.5-1 for following band combinations and UL configurations:

- CA_n3-n41 MSD with n41C UL

- CA_n71-n77 MSD with n77(2A) UL

- CA_n66-n77 MSD with n77(2A) UL

- CA_n1-n78 MSD with n78(2A) UL

- CA_n3-n78 MSD with n78(2A) UL

- CA_n7-n78 MSD with n78(2A) UL

- CA_n28-n77 MSD with n77(2A) UL

- CA_n13-n77 MSD with n77(2A) UL

The changes are in following table: Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA.

The changes mostly added missing IMDs from intra-band ULCA as UL configuration and explanatory notes about possible regional frequency restrictions.

But to add these corrections now that we also have the Release 18 specification and contradicting test points in places, picking the right one is not trivial.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  |  | n78 |  | 3710 |  | 10 |  | 50 | 3710 |  | N/A |  | TDD |  | N/A |  |
|  |  | n1 |  | N/A |  | 5 |  | N/A | 2167.5 |  | 1.7 |  | FDD |  | IMD7Y |  |
|  |  | n7812 |  | 3305 |  | 10 |  | 1 (RBstart=0) | 3305 |  | N/A |  | TDD |  | N/A |  |
|  |  |  |  | 3675 |  | 10 |  | 1 (RBstart=44) | 3675 |  |  |  |  |  |  |  |
| CA_n3-n41 |  | n3 |  | 1740 |  | 5 |  | 25 | 1835 |  | 8.2 |  | FDD |  | IMD4 |  |
|  |  | n41 |  | 2657.5 |  | 10 |  | 50 | 2657.5 |  | N/A |  | TDD |  | N/A |  |
|  |  | n3 |  | N/A |  | 5 |  | N/A | 1877.5 |  | [2.7] |  | FDD |  | IMD9X |  |
|  |  | n41 |  | 2541 |  | 90 |  | 1 (RBstart=18) | 2541 |  | N/A |  | TDD |  | N/A |  |
|  |  |  |  | 2636 |  | 100 |  | 1 (RBstart=272) | 2636 |  |  |  |  |  |  |  |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7Y |
|  | n7812 | 3305 3780 | 10 10 | 1 (RBstart=3) 1 (RBstart=0) | 3305 3780 | N/A | TDD | N/A |
| CA_n5-n77 | n5 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD413 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  |  | n5 |  | 829 |  | 5 |  | 25 | 874 |  | 5.5 |  | FDD |  | IMD513 |  |
|  |  | n77 |  | 4190 |  | 10 |  | 50 | 4190 |  | N/A |  | TDD |  | N/A |  |
| CA_n7-n78 |  | n7 |  | N/A |  | 5 |  | N/A | 2687.5 |  | 15 |  | FDD |  | IMD5 |  |
|  |  | n7812 |  | 3405 |  | 10 |  | 1 (RBstart=10) | 3405 |  | N/A |  | TDD |  | N/A |  |
|  |  |  |  | 3755 |  | 10 |  | 1 (RBstart=0) | 3755 |  |  |  |  |  |  |  |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  |  | n7712 |  | N/A |  | N/A |  | N/A | N/A |  | N/A |  | TDD |  | N/A |  |
|  |  | n28 |  | 705.5 |  | 5 |  | 25 | 760.5 |  | 5.5 |  | FDD |  | IMD5 |  |
|  |  | n77/n78 |  | 3582.5 |  | 10 |  | 50 | 3582.5 |  | N/A |  | TDD |  | N/A |  |
|  |  | n28 |  | N/A |  | 5 |  | N/A | 705.5 |  | [8.6] |  | FDD |  | IMD4 |  |
|  |  | n7712 |  | 3455 |  | 10 |  | 1 (RBstart=17) | 3455 |  | N/A |  | TDD |  | N/A |  |
|  |  |  |  | 3805 |  | 10 |  | 1 (RBstart=0) | 3805 |  |  |  |  |  |  |  |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBstart=10) | 3455 | N/A | TDD | N/A |
|  |  |  |  | 3875 |  | 10 |  | 1 (RBstart=0) | 3875 |  |  |  |  |  |  |  |
|  |  | n66 |  | N/A |  | 5 |  | N/A | 2197.5 |  | 15 |  | FDD |  | IMD513 |  |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  |  |  | 3855 |  | 10 |  | 1 (RBstart=8) | 3855 |  |  |  |  |  |  |  |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD513 |
|  |  | n77 |  | 3309 |  | 10 |  | 50 | 3309 |  | N/A |  | TDD |  | N/A |  |
|  |  | n71 |  | N/A |  | 5 |  | N/A | 619.5 |  | [8.6] |  | FDD |  | IMD4 |  |
|  |  | n7712 |  | 3455 |  | 10 |  | 1 (RBstart=10) | 3455 |  | N/A |  | TDD |  | N/A |  |
|  |  |  |  | 3765 |  | 10 |  | 1 (RBstart=0) | 3765 |  |  |  |  |  |  |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| NOTE X: For a UE which supports this band combination only when the Band n41 frequency range restriction of 2515 – 2675 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE Y: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 – 3800 MHz or 3300 – 3600 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

## Corrections implemented in R4-2311139 and mirrored in R4-2311138

After changes in R4-2311139 and mirrored in R4-2311138 here is the list of changes and remaining changes:

| Combination | R4-2220561 | R4-2311139 | R4-2311138 |
| --- | --- | --- | --- |
| CA_n1-n78 | New MSD due to n78(2A) IMDs Note Y | Implemented Note Y=Note 15 | Implemented Note Y=Note 16 |
| CA_n3-n41 | New MSD due to n41C (190MHz) IMDs Note X | Missing but should be aligned with R18 which considers 160MHz spectrum in China. Note X= Note 14 | considers 160MHz spectrum in China. Note X= Note 14 |
| CA_n3-n78 | Note 12 and Y | Implemented Note 12 and Y=Note 15 | Implemented Note 12 and Y=Note 16 |
| CA_n5-n77 | Removed Note 12 added Note 13 | Implemented | Implemented |
| CA_n7-n78 | New MSD due to n78(2A) IMDs based on Canada? | Not implemented Chose which test point is valid: R4-2220561 or R18 | Already there but different test point: aligned for China? Chose which test point is valid: R4-2220561 or R18 |
| CA_n28-n77 | New MSD due to n77(2A) test point for Japan | Implemented | Implemented |
| CA_n66-n77 | Additional IMD5 test point for n77(2A) valid for US+ corrections | Implemented | Implemented |
| CA_n71-n77 | Additional IMD4 test point for n77(2A) valid for US+ corrections | Missing Chose and add which test point is valid: R4-2220561 or R18 | Already there but different test point: Chose which test point is valid: R4-2220561 or R18 |

Other missing cases in R4-2220561: CA_n13-n77/n77(2A) mentioned but not implemented and not in R17/R18 => Need to check conclusions from RAN4#105 to see if it is an omission or not.

Proposals on discussions with experts in “not for block approval” on corrections for IMDs due to intra-band ULCA in R17 and R18 due to missing implementation of R4-2220561:

- CA_n3-n41/CA_n41C: add MSD test points from R4-2311138 into revision of R4-2311139

- CA_n7-n78/CA_n78(2A): Discuss if R4-2220561 or R4-2311138 test point should be used and implement in revision of R4-2311139 and R4-2311138

- CA_n71-n77/CA_n77(2A): Discuss if R4-2220561 or R4-2311138 test point should be used and implement in revision of R4-2311139 and R4-2311138

- CA_n13-n77/n77(2A): check conclusions from RAN4#105 to see if it is an omission or not

- Note that CRs are posted to the SimBC thread and will need coordination.

# Conclusions

In this contribution, we analysed the remaining issue related to the missed implementation of draft CR R4-2220561 after corrections in R4-2311139 and R4-2311138 and make the following proposal.

Proposals on discussions with experts in “not for block approval” on corrections for IMDs due to intra-band ULCA in R17 and R18 due to missing implementation of R4-2220561:

- CA_n3-n41/CA_n41C: add MSD test points from R4-2311138 into revision of R4-2311139

- CA_n7-n78/CA_n78(2A): Discuss if R4-2220561 or R4-2311138 test point should be used and implement in revision of R4-2311139 and R4-2311138

- CA_n71-n77/CA_n77(2A): Discuss if R4-2220561 or R4-2311138 test point should be used and implement in revision of R4-2311139 and R4-2311138

- CA_n13-n77/n77(2A): check conclusions from RAN4#105 to see if it is an omission or not

- Note that CRs are posted to the SimBC thread and will need coordination.

# References

- R4-2220561DraftCR to 38.101-1 on correcting REFSENS exceptions due to intermodulation interference of intra-band ULCA, Skyworks Solutions Inc., RAN4#105

- R4-2311138 CR to TS 38.101-1 Rel-18 Removing triple uplink and corrections to 2UL CA exceptions, Skyworks Solutions Inc., RAN4#108

- R4-2311139 CR to TS 38.101-1 Rel-17 Removing triple uplink and corrections to 2UL CA exceptions, Skyworks Solutions Inc., RAN4#108