3GPP TSG-RAN WG4 Meeting #	108                                         R4-23xxxxx

Toulouse, France, August 21 – August 25, 2023

Agenda item:		8.28.2.2

Source:	Dell Technologies

Title:	Further Discussion on RF Requirements for NCR

Document for:	Discussion

# Introduction

Way forward/agreement

Issue 3-1-2   Transmitter ON-OFF power and transition period

- Agreement: Further discuss below two options:

  - Option 1: IAB-MT approach

  - Option 2: Legacy UE approach

Issue 3-1-5   OBUE/SEM

- Agreement:

  - For wide area NCR-MT: BS requirements applied

  - For local area NCR-MT: FFS

  - Proposal 3 agreed

Issue 3-2-3    ACS/IBB

- Agreement:

  - For wide area NCR-MT: reusing IAB-MT requirements

  - For local are NCR-MT: FFS

Issue 3-2-4  OOBB requirements

- For wide area NCR-MT: reusing IAB requirements

- For local area NCR-MT: FFS

In RAN4-#107 meeting, the WF on system parameters and RF requirements for NCR, R4-2308893 [1], is approved. In this contribution, the RF requirements for NCR-MT and NCR-fwd are further discussed with our views based on the WF.

- Discussions

  - Issue 3-1-2 Transmitter ON-OFF power and transition period

For transmitter ON/OFF power and transition period requirements, it may consider the cases of shared and non-shared RF circuits for NCR-fwd uplink and NCR-MT.

For transmitter ON/OFF power in shared RF circuits case, the transmitter ON/OFF status is switched the same for NCR-fwd and NCR-MT whereas in non-shared RF circuits case, the transmitter ON/OFF status could be switched different.

Observation 1: Based on whether NCT-fwd and NCT-MT share RF circuits, NCR-fwd and NCR-MT could be switched ON/OFF simultaneously or separately and define requirements accordingly.

Observation 2: For transmitter transition period, both cases can be the same requirement due to ignorable delay between NCT-fwd and NCT-MT switches

Proposal 1: It is suggested to reuse UE transmitter ON/OFF power and transition period requirement for NCR-MT.

  - Issue 3-1-5   OBUE/SEM

For NCR-MT OBUE/SEM requirements, it has been agreed to apply BS requirements for wide area class of NCR- MT in last meeting. Since OBUE/SEM requirement similar as ACLR as unwanted emission requirements for the wide area and local area NCR-MT and considering in Local Area NCR-MT ACLR has been agreed to apply UE  requirements in last meeting and NCR-MT and NCT-fwd are the same class and transmitted in uplink based on UE power level, therefore, for the local area class NCR-MT OBUE/SEM, it is reasonable to reuse the UE SEM requirements accordingly.

Proposal 2: For the Local Area class NCR-MT OBUE/SEM, it is reasonable to reuse UE SEM requirements.

  - Issue 3-2-3 ACS/IBB

Based on the agreement for NCR-MT ACS requirements in last meeting [1], the Wide Area class NCT-MT will reuse IAB-MT requirements defined on the BS specifications [2, 3]. For Local Area class NCR-MT, it functionalises similar as UE to BS. Therefore, the ACS requirement for Local Area NCR-MT can use UE ACR/IBB requirements.

Proposal 3: It is proposed to reuse the UE ACE/IBB requirements for Local Area NCR-MT.

  - Issue 3-2-4 OOBB requirements

For OOBB which is similar as ACS/IBB and NCR-MT is functionalized as UE, it can be reuse UE requirements for Local Area NCR-MT.

Proposal 4: It is suggested to reuse UE requirements for Local Area NCR-MT OOBB requirements.

- Conclusions

In this contribution, we provide our views on   and present our observation and proposals as followings:

Observation 1: Based on whether NCT-fwd and NCT-MT share RF circuits, NCR-fwd and NCR-MT could be switched ON/OFF simultaneously or separately and define requirements accordingly.

Observation 2: For transmitter transition period, both cases can be the same requirement due to ignorable delay between NCT-fwd and NCT-MT switches

Proposal 1: It is suggested to reuse UE transmitter ON/OFF power and transition period requirement for NCR-MT.

Proposal 2: For the Local Area class NCR-MT OBUE/SEM, it is reasonable to reuse UE SEM requirements.

Proposal 3: It is proposed to reuse the UE ACE/IBB requirements for Local Area NCR-MT.

Proposal 4: It is suggested to reuse UE requirements for Local Area NCR-MT OOBB requirements.

- References

[1] R4-2309783 WF on system parameters and RF requirements

[2] 3GPP 38.104 38104 BS radio transmission and reception

[3] 3GPP 38.174 38.174 IAB radio transmission and reception

[4] 3GPP 38.101-1/2 UE radio transmission and reception

[5] R4-2310450 Pre_1st_round_summary_[107][312] NR_netcon_repeater_RF_v2