3GPP TSG-RAN WG4 Meeting #108-bis	R4-23xxxxx

# Xiamen, 9th - 13th Oct, 2023

Agenda item:			5.26.9

Source:	Moderator (Huawei, HiSilicon)

Title:	Offline meeting minutes on NTN UE and SAN demod

Document for:	Information

Introduction

This contribution summarises the open issues for NR_NTN_enh_SAN_UE_demod under AI 5.26.8 at RAN4#108-bis.

This topic is introduced in RAN4 demodulation at RAN4#108-bis with a completion by RAN#104 in June 2024.

Three topics are captured:

- Topic #1: Workplan

- Topic #2: UE demodulation performance requirements

- Topic #3: SAN demodulation performance requirements

Topic #1: Workplan

Open issues summary

Sub-topic 1-1: Workplan

Issue 1-1: Workplan

- Proposals

  - Option 1: (Huawei)

| RAN4#108-bis Oct 2023 |  |
| --- | --- |
|  | Discussion and agreement on work plan. Initial round of discussion on performance requirements scope. Initial round of discussion on simulation assumptions. |
| RAN4#109 Nov 2023 |  |
|  | Second round of discussion on performance requirements scope. Second round of discussions on simulation assumptions. Initial round of simulation results collection and alignment. |
| RAN4#110 Feb 2024 |  |
|  | Finalize discussions on performance requirements scope. Discussions on CR split. Second round of simulation results collection and alignment. |
| RAN4#110-bis Apr 2024 |  |
|  | Finalize discussions on simulation assumptions. Third round of simulation results collection and alignment. Endorse draft CRs. |
| RAN4#111 May 2024 |  |
|  | Final round of simulation results collection and alignment. Agree formal CRs. |

- Recommended WF

  - Workplan to be agreed at RAN4#108-bis meeting.

Topic #2: UE demodulation performance requirements

Open issues summary

    - Sub-topic 2-1: Test scope

Issue 2-1-1: Whether to define UE PDSCH demodulation performance requirements for above 10 GHz bands

- Tentative agreement: Agree to define UE PDSCH demodulation performance requirements for above 10 GHz bands.

Issue 2-1-1c: HARQ processes for above 10 GHz bands

- Proposals

  - Candidate case 1: Enabled

    - Option 1a: 16 HARQ process

    - Option 1b: 32 HARQ process

    - Option 1c: Both 16 and 32 HARQ process

  - Candidate case 2: Disabled

    - Option 2a: Introduce requirement

    - Option 2b: Not introduce requirements

- Tentative agreement

  - FFS 16 HARQ process, 32 HARQ process, HARQ process disabled.

  - Interested Companies can provide further analysis for 16 HARQ process, 32 HARQ process, HARQ process disabled.

Issue 2-1-2: Whether to define UE PDCCH demodulation performance requirements for above 10 GHz bands

- Tentative agreements:

  - Ask satellite companies to confirm if VSAT can be used in TN network.

    - If VSAT can be used in TN network,

      - Reuse TN PDCCH requirements

      - Define new PDCCH requirements for NTN

      - Do not define new PDCCH requirements for NTN

    - If VSAT cannot be used in TN network,

      - Define new PDCCH requirements for NTN

      - Do not define new PDCCH requirements for NTN

    - Other options are not precluded.

- There is no VSAT NTN-TN UE capability discussion in RAN2 from THALES’s input.

Issue 2-1-3: Whether to define UE PBCH demodulation performance requirements for above 10 GHz bands?

- Tentative agreements:

  - Ask satellite companies to confirm if VSAT can be used in TN network.

    - If VSAT can be used in TN network,

      - Reuse TN PBCH requirements

      - Define new PBCH requirements for NTN

      - Do not define new PBCH requirements for NTN

    - If VSAT cannot be used in TN network,

      - Define new PBCH requirements for NTN

      - Do not define new PBCH requirements for NTN

    - Other options are not precluded.

- There is no VSAT NTN-TN UE capability discussion in RAN2 from THALES’s input.

Issue 2-1-4: Whether to define UE CSI reporting requirements for above 10 GHz bands?

- Tentative agreements

  - Do not consider PMI reporting and RI reporting.

  - Companies can provide feasibility analysis on CQI reporting under AWGN or fading channels.

    - Sub-topic 2-2: General issues for above 10 GHz bands

Issue 2-2-1: Channel model

- Proposals

  - Option 1: (Nokia)

    - RAN4 to discuss propagation channel models and applicability to deployments above 10 GHz.

  - Option 2: (Apple)

    - Consider NTN-TDLA and NTN-TDL-C

    - Further discuss Doppler to be used

  - Option 3: (Ericsson)

    - Take NTN-TDLA100 and NTN-TDLC5 for Rel-18 FR1 demodulation initial discussion.

    - Prioritize LOS channel model NTN-TDLC for Rel-18 NTN FR2 demodulation requirements.

    - Taken rural LOS scenario with 30o SAN elevation angle as worst case for FR2 channel model and take K = 8.4dB, delay spread = 3ns and the maximum Doppler spread = 833Hz for LOS channel configuration in initial simulations.

    - For Rel-18 FR2 NTN demodulation discussion, take mobile UE for GSO as the worst case, and take the NTN UE speed as 30km/h for initial discussion.

  - Option 4: (Huawei)

    - Consider NTN-TDLA and NTN-TDL-C

    - Consider 10ns delay spread and 2000Hz Doppler.

- Recommended WF

  - Further discussion is needed.

Issue 2-2-2: Frequency/timing drift

- Proposals

  - Option 1: (Apple)

    - Reuse the methodology used in R17 NTN demod requirements for R18 eNTN

      - Assume UE compensation for Doppler shift prior to DL demodulation, i.e. no Doppler shift model

      - Do not consider timing drift and sampling frequency offset for UE demod requirements

  - Option 2: (Ericsson)

    - The NTN UE capability of pre-compensation for UL transmission on frequency error and timing error is also applied for FR2 demodulation discussion.

    - The NTN UE capability of compensation for DL reception on frequency error for high Doppler scenario is assumed for FR2 demodulation discussion.

    - Assume the residual frequency error and timing error is small for initial analysis at current stage. Furtherly discussion is needed if final error requirements are much worse than FR1.

    - Do not consider frequency drift model and sampling drift model in Rel-18 NR NTN demodulation initial discussion. If much worse frequency error requirement and timing error requirement are agreed in RF and RRM, RAN4 could fatherly discuss corresponding channel model modification.

- Recommended WF

  - Further discussion is needed.

Issue 2-2-3: SCS (except PBCH testing) and channel bandwidth

- Proposals

  - Option 1: 120kHz (Qualcomm)

  - Option 2: 120kHz/100MHz (Apple, Ericsson)

  - Option 3: 120kHz/200MHz (Huawei)

- Recommended WF

  - Only consider 120kHz SCS.

  - Further discussion of channel bandwidth is needed.

Issue 2-2-4: Antenna configuration

- Proposals

  - Option 1: (Ericsson)

    - Take 1Tx1Rx for parabolic VSAT antenna configuration for initial demodulation discussion and input from satellite companies is needed.

- Recommended WF

  - Further discuss is needed.

Issue 2-2-5: Receiver assumption

- Proposals

  - Option 1: MMSE-IRC. (Apple)

- Recommended WF

  - Option 1

Issue 2-2-6: Duplex

- Proposals

  - Option 1: FDD (Qualcomm)

- Recommended WF

  - Option 1

Issue 2-2-7: Payload

- Proposals

  - Option 1: Transparent payload for both GSO and NGSO scenarios. (Qualcomm)

- Recommended WF

  - Option 1

Issue 2-2-8: Beamforming and beam steering

- Proposals

  - Option 1: Discuss potential beamforming and beam steering mechanism for VSAT devices for NR NTN enhancements. (Qualcomm)

- Recommended WF

  - Further discuss is needed

Issue 2-2-9: Rx phase noise

- Proposals

  - Option 1: Take Rx phase noise impact into impairment results and companies could give proper values based on preferred PN model. (Ericsson)

- Recommended WF

  - Further discuss is needed

Issue 2-2-10: Applicability rule

- Proposals

  - Option 1: Adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE. (Ericsson)

- Recommended WF

  - Postpone it after the test scope is stable.

    - Sub-topic 2-3: Test setup for above 10 GHz bands

Issue 2-3-1: MCS for PDSCH

- Proposals

  - Option 1: 16QAM as baseline, FFS 64QAM based on link budget analysis. (Qualcomm)

  - Option 2: QPSK, 16QAM (Apple, Ericsson)

  - Option 3: MCS4 (QPSK, 0.30) and MCS13 (16QAM, 0.48) (Huawei)

- Recommended WF

  - Both QPSK and 16QAM can be considered, FFS detailed MCS.

  - Further discuss 64QAM.

Issue 2-3-2: Rank for PDSCH

- Proposals

  - Option 1: Rank 1. (Qualcomm, Apple, Ericsson, Huawei)

- Recommended WF

  - Option 1

Issue 2-3-3: PDSCH mapping type

- Proposals

  - Option 1: PDSCH mapping type A. (Apple, Ericsson)

- Recommended WF

  - Option 1

Issue 2-3-4: PDCCH aggregation level (If agreed to be introduced)

- Proposals

  - Option 1: (Ericsson)

    - Aggregation level: 4

- Recommended WF

Issue 2-3-5: Configuration for PBCH test (If agreed to be introduced)

- Proposals

  - Option 1: (Ericsson)

    - SSB index is known and unknown.

    - SCS: 120kHz and 240kHz SCS

- Recommended WF

  - Further discussion is needed.

- Topic #3: SAN demodulaton requirements

  - Open issues summary

    - Sub-topic 3-1: Test scope

Issue 3-1-1: Which feature can be considered for SAN PUSCH demodulation performance requirements for above 10 GHz bands?

- Tentative agreements:

  - Define PUSCH with transform precoding disabled and enabled demodulation performance requirements for above 10 GHz bands.

  - FFS PUSCH repetition Type A, companies can do some analysis on the link budget.

  - Do not consider UL timing adjustment

Issue 3-1-2: Which format can be considered for SAN PUCCH demodulation performance requirements for above 10 GHz bands?

- Tentative agreement

  - Define PUCCH format 0/1/2/3/4 demodulation performance requirements for above 10 GHz bands.

Issue 3-1-3: Which format can be considered for SAN PRACH demodulation performance requirements for above 10 GHz bands?

- Tentative agreement

  - Define PRACH burst format B4/C2 demodulation performance requirements for above 10 GHz bands.

  - Do not consider PRACH burst format A2.

Issue 3-1-4: Whether to define SAN PUSCH demodulation performance requirements for DMRS bundling?

- Tentative agreement

  - Define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1 only if RF confirm that phase continuous is feasible.

Issue 3-1-5: Whether to define SAN PUCCH demodulation performance requirements for Msg4 HARQ-ACK?

- Proposals

  - Option 1: Yes (Nokia, Samsung)

  - Option 2: No (Ericsson, Huawei)

- Way forward:

  - Option 1: Add new requirements under LOS channel for multi-slot PUCCH format 1.

  - Option 2: Do not define PUCCH requirements Msg4 HARQ-ACK for LOS channel.

Issue 3-1-6: Whether to define SAN demodulation performance requirements for network verified UE location?

- Tentative agreement: Do not define SAN demodulation performance requirements for network verified UE location.

Issue 3-1-7: Whether to define SAN demodulation performance requirements for NTN-TN and NTN-NTN mobility and service continuity enhancement?

- Tentative agreement:

  - Do not define SAN demodulation performance for NTN-TN and NTN-NTN mobility and service continuity enhancement.

Issue 3-1-8: WID update

- Tentative agreement:

  - Update the impacted specification TS 38.108 for the performance part in the WID.

  - Update the objective of performance part WI to include coverage enhancement if necessary.

    - Sub-topic 3-2: General issues for above 10 GHz bands

Issue 3-2-1: Channel model

- Proposals

  - Option 1: (Nokia)

    - RAN4 to discuss propagation channel models and applicability to deployments above 10 GHz.

  - Option 2: (Ericsson)

    - Doppler shift and delay spread for both GEO and LEO deployment should be discussed in SAN (satellite access node) demodulation to enable pre-compensation for Doppler shift.

    - Same channel model might be used for both GEO and LEO if the Doppler error after pre-compensation does not have significant difference.

  - Option 3: (Huawei)

    - Consider NTN-TDLA and NTN-TDL-C

    - Consider 10ns delay spread and 3000Hz Doppler.

  - Option 4: (Samsung)

    - The following channel model can be considered as starting point

      - [NTN-TDLA10-3000]

      - [NTN-TDLC5-3000]

- Tentative agreement:

  - For FR2, NTN-TDLA and NTN-TDL-C

  - FFS delay and Doppler

Issue 3-2-1a: Frequency and timing drift

- Tentative agreement:

  - General

    - Assumption: UE will do pre-compensation for frequency and timing drift.

    - No frequency and timing drift are modelled in Rel-18 NTN SAN requirements.

Issue 3-2-2: SCS and channel bandwidth

- Proposals

  - Option 1: Prioritize 120kHz, the selection of bandwidth could depend on the simulation results. (Ericsson)

  - Option 2: Prioritize 120kHz/200MHz. Additional consider 60kHz/50MHz and 120kHz/50MHz. (Huawei)

  - Option 3: 60kHz/50MHz, 120kHz/50MHz (Samsung)

- Recommended WF

  - 120kHz SCS can be considered.

  - Further discussion of 60kHz SCS and channel bandwidth.

Issue 3-2-3: Antenna configuration

- Proposals

  - Option 1: 1Tx, 1/2Rx (Ericsson)

- Recommended WF

  - Further discuss is needed.

Issue 3-2-4: MCS & rank

- Proposals

  - Option 1: Rank 1 with MCS4 (QPSK, 308/1024) and MCS5 (QPSK, 99/1024, Table 3, for PUSCH repetition Type A case only). (Huawei)

- Recommended WF

  - Further discuss is needed.

Issue 3-2-5: SAN type

- Proposals

  - Option 1: Prioritize to define SAN demodulation requirement for Type 2-O. FFS to define BS demodulation requirement for Type 2-H, pending on conclusion of RF core requirement. (Samsung)

- Recommended WF

  - Option 1.

Issue 3-2-6: Duplex

- Proposals

  - Option 1: FDD (Samsung)

- Recommended WF

  - Option 1

    - Sub-topic 3-3: Test setup for above 10 GHz bands

Issue 3-3-1: Test setup for SAN PUSCH demodulation performance requirements

- Proposals

  - Option 1: (Samsung)

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0, 1} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2, |
| configuration | Time density (LPT-RS) | 1, |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

- Recommended WF

  - Further discuss is needed.

Issue 3-3-2: Test setup for SAN PUCCH demodulation performance requirements

- Proposals

  - Option 1: (Samsung)

Table 3:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |
| Channel Model | [NTN-TDLA10-3000] |

Table 4:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 5   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

- Recommended WF

  - Further discuss is needed.

Issue 3-3-3: Test setup for SAN PRACH demodulation performance requirements

- Proposals

  - Option 1: (Samsung)

    - PRACH format: C2 and B4

    - Test preamble configuration

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
|  | 60 | 69 | 0 | 0 |
| B4, C2 | 120 | 69 | 0 | 0 |

    - Frequency offset

      - AWGN: 0Hz

      - Fading: 3000Hz

    - Time error tolerance

      - 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel

    - Channel Model: [NTN-TDLA10-3000]

- Recommended WF

  - Further discussion is needed.

    - Sub-topic 3-4: Test setup for UL coverage enhancement

Issue 3-4-0: Frequency and timing drift

- Wayford:

  - Option 1: Further discuss whether assumption of no timing drift modelling can be applied for DMRS bundling requirement pending on RF conclusion.

Issue 3-4-1: Test setup for SAN PUSCH demodulation performance requirements for DMRS bundling

- Proposals

  - Option 1: (Ericsson)

    - Demodulation performance for DMRS bundling depends on actual window length in which NTN UE can maintain it’s phase and power.

    - To define BS PUSCH demodulation requirements for DMRS bundling, with detailed parameters such as slot number, configured window length, frequency hopping, repetition type, waveform etc., RF discussion should be followed.

  - Option 2: (Huawei)

    - The Channel model, Channel bandwidth & SCS and MCS & Rank can be reused from the existing FR1-NTN cases in TS 38.108, at the same time the other parameters can be reused from the existing PUSCH DMRS bundling cases in TS 38.104.

  - Option 3: (Samsung):

    - FFS on introduce the timing drift model for specifying the PUSCH requirement with DMRS bundling in NTN scenario.

    - The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

      - Actual TDW and PUSCH-TimeDomainWindowLength = 2 or 4 for FDD

      - Actual TDW and PUSCH-TimeDomainWindowLength 2 for TDD

    - The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

      - PRB: 6RBs for both 15KHz and 30KHz

    - Introduce PUSCH requirement with mapping type A for NTN-specific UE with DMRS bundling

    - Introduce PUSCH requirement with CP-OFDM waveform for NTN-specific UE with DMRS bundling

    - Introduce PUSCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with DMRS bundling. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

    - Introduce PUSCH requirement for NTN-specific UE with 1+1 DMRS symbols.

    - Apply the existing test parameters for specifying PUSCH requirement with DMRS bundling for NTN-specific UE with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern [Note 1] |  | 15 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and TDD 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 2] | 0, 3, 0, 3 for FDD 0, 3, 0, 3 for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | [n2] or [n8] for FDD n2 for TDD |
| pusch-TimeDomainWindowLength |  | [2] or [4] slots for FDD 2 slots for TDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots. Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

- Recommended WF

  - Further discuss is needed.

Issue 3-4-2: Test setup for SAN PUSCH demodulation performance requirements for Msg4 HARQ-ACK (If agreed to be introduced)

- Proposals

  - Option 1: (Nokia)

    - RAN4 shall define requirements for 4 and 8 PUCCH repetitions.

  - Option 2: (Samsung)

    - Only PUCCH format 1 requirement should be specified

    - Introduce multi-slot PUCCH requirement with format 1 under 2 repetition for NTN-TDLC5-200 channel

    - Introduce PUCCH requirement with both 1Tx1Rx and 1Tx2Rx for NTN-specific UE with repetition. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test

    - Using the following test parameters for specifying PUCCH requirement for Msg4 HARQ-ACK

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

- Recommended WF

  - Further discuss is needed.