3GPP TSG-RAN4 Meeting # 108bis	R4-2317010

Xiamen, China, October 09 – October 13, 2023

Source: 	CMCC

Title: 	Draft TR 38.858

Agenda Item:	5.19.4

Document for:	endorsement

- Introduction

This is Big TP of TR 38.858 to merge all agreed TPs/pCRs in AI 5.19 until RAN4#108bis meeting. Following list all endorsed TPs in this meeting.

- R4-2316383, Comments to Section 10.1 Background for analysis, Huawei, HiSilicon. RAN4#108bis

- R4-2316531, Maintenance TP to TR 38.858 Clause 9.3~9.4, ZTE, RAN4#108bis

- R4-2316532, Maintenance TP to TR 38.858 Clause 9.5, ZTE, RAN4#108bis

- R4-2316936, TP to TR 38.858 on Feasibility of FR1 UE aspects, MediaTek Inc., RAN4#108bis

- R4-2316937, Maintenance TP to TR 38.858 Clause 9.7, ZTE, RAN4#108bis

- R4-2316938, TP to TR 38.858 on UE RF impacts for full duplex operation, vivo, RAN4#108bis

- R4-2316943, TP to TR 38.858 Annex E: Comparison of RAN1 and RAN4 simulation methodology and assumptions, Nokia, Nokia Shanghai Bell, Spark NZ Ltd, Ericsson, CableLabs, Charter Communications Inc., Korea Testing Laboratory, RAN4#108bis

- R4-2316958, TP for TR 38.858: “Feasibility of FR1 Local Area BS aspects”CATT, RAN4#108bis

- R4-2316959, TP to TR 38.858: Feasibility of FR1 MR BS aspects, Nokia, Nokia Shanghai Bell, RAN4#108bis

- R4-2316961, TP to TR 38.858:  Feasibility of FR2 wide area BS, Huawei, HiSilicon, RAN4#108bis

- R4-2316962, TP to TR 38.858 Impact on BS RF requirements, ZTE Corporation, Samsung,CMCC, RAN4#108bis

- R4-2316973, TP to TR 38.858: Updates to RAN4 coexistence simulation assumptions in Annex E, Ericsson, RAN4#108bis

- R4-2316975, Draft TP to TR 38.858 on Chapter 11, Samsung, CMCC, Qualcomm Inc., CableLabs, Ericsson, ZTE, Huawei, HiSilicon, CATT, Charter Communications, RAN4#108bis

- R4-2316976, TP for TR 38.858 to add regulatory requirements, CableLabs, Charter Communications, CMCC, Ericsson, HiSilicon, Huawei, Korea Testing Laboratory, Nokia, Nokia Shanghai Bell, Qualcomm, Samsung, Spark, ZTE, RAN4#108bis

- R4-2317009, SBFD implementation feasibility of FR1 BS and text proposal to TR 38.858, Samsung

# Text proposal

## <<Start of Change for TR 38.858>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-213591, New SI: Study on evolution of NR duplex operation.

[3]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[4]	3GPP TR 36.843: "Study on LTE Device to Device Proximity Services; Radio Aspects".

[5]	3GPP TR 38.802: "Study on New Radio Access Technology Physical Layer Aspects".

[6]	3GPP TR 36.873: "3D channel model for LTE".

[7]	3GPP TR 36.814: "Further advancements for E-UTRA physical layer aspects".

[8]	3GPP TR 36.889: "Study on Licensed-Assisted Access to Unlicensed Spectrum".

[9]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[10]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[11]	Report ITU-R M.2412: "Guidelines for evaluation of radio interface technologies for IMT-2020".

[12]	RP-180524, Summary of calibration results for IMT-2020 self evaluation.

[13]	3GPP TR 38.830: "Study on NR coverage enhancements".

[14]	R1-2304212, Summary on SLS calibration results for NR duplex evolution

[15] 	R1-2307274, On evaluations for NR duplex evolution, Apple

[16] 	R1-2307083, SBFD evaluation results, CATT

[17] 	R1-2307192, Evaluation on NR duplex evolution, CMCC

[18] 	R1-2307324, Evaluation of NR duplex evolution, Ericsson

[19] 	R1-2308336, Discussion on evaluation and methodologies on evolution of NR duplex operation, Huawei, HiSilicon

[20] 	R1-2306695, Discussion on evaluations on NR duplex evolution, InterDigital, Inc.

[21] 	R1-2306885, Study on Evaluation for NR duplex evolution, LG Electronics

[22] 	R1-2306814, Discussion on evaluation of NR duplex evolution, MediaTek Inc.

[23] 	R1-2306400, Discussion for Evaluation on NR duplex evolution, New H3C Technologies Co., Ltd.

[24] 	R1-2306874, On the evaluation methodology for NR duplexing enhancements, Nokia, Nokia Shanghai Bell

[25] 	R1-2307571, Discussion on evaluation on NR duplex evolution, OPPO

[26] 	R1-2307922, On Deployment scenarios and evaluation Methodology for NR duplex evolution, Qualcomm Incorporated

[27] 	R1-2306642, Discussion on evaluation on NR duplex evolution, Spreadtrum Communications, BUPT, New H3C

[28] 	R1-2307674, Discussion on evaluation for NR duplex evolution, Samsung

[29] 	R1-2307817, Evaluation of NR duplex evolution, Sharp

[30] 	R1-2306906, SBFD System Level Simulation Results, Sony

[31] 	R1-2307381, Discussion on evaluation on NR duplex evolution, Xiaomi

[32] 	R1-2306981, Prototype and Simulation Results for SBFD, ZTE

[33] 	R1-2306746, Evaluation on NR duplex evolution, vivo

[34] 	R1-2308001, LLS for evaluation of coverage performance in TDD and SBFD systems, CEWiT

[35] 	R1-2308101, Evaluation on NR duplex operation, China Unicom, Huawei, HiSilicon

[36] 	R1-2307471, Discussion on evaluation on NR duplex evolution, NTT DOCOMO, INC.

[37] 	R1-2307330, Discussion on evaluation on NR duplex evolution, Panasonic

[38] 	R1-2307159, Evaluation on NR duplex evolution, Fujitsu

[39] 	R1-2306835, Evaluations on NR Duplex Evolution, Intel Corporation

[40] 	R1-2307084, Discussion on subband non-overlapping full duplex, CATT

[41] 	R1-2307325, Subband non-overlapping full duplex, Ericsson

[42] 	R1-2307619, Field test for dynamic/flexible TDD, China Telecom, ZTE

[43] 	R1-2306983, Discussion of enhancements on dynamic/flexible TDD, ZTE, China Telecom

[44]	R1-2307326, Potential enhancements of dynamic TDD, Ericsson

[45]	R1-2305035, SBFD System Level Simulation Results, Sony

[46]	R4-2214376, Reply LS on interference modelling for duplex evolution, Samsung, CMCC, Qualcomm

[47]	R4-2220244, WF for the feasibility from BS aspect, Samsung

[48]	R4-2300690, Further considerations on SBFD BS RF aspects, Nokia, Nokia Shanghai Bell

[49]	R4-2305917, WF on implementation feasibility of SBFD: Co-channel Co-site/inter-site interference, Ericsson

[50]				R4-2219633, BS RF feasibility considerations, Ericsson

[51]		ECC Report 281, Analysis of the suitability of the regulatory technical conditions for 5G MFCN operation in the 3400-3800 MHz band, approved 6 July 2018. Online available: https://docdb.cept.org/download/3419.

[52]	ECC Recommendation(20)03, Frame structures to facilitate cross-border coordination of TDD MFCN in the frequency band 3400-3800 MHz, approved 23 October 2020. Online available: https://docdb.cept.org/download/1738.

[53]		ECC Decision(11)06, Harmonised frequency arrangements and least restrictive technical conditions (LRTC) for mobile/fixed communications networks (MFCN) operating in the band 3400-3800 MHz, amended 26 October 2018. Online available: https://docdb.cept.org/download/1531.

[54] 	FCC-21-32, “FCC Opens 100 Megahertz of Mid-Band Spectrum for 5G,” March 2021. Online available: https://www.fcc.gov/document/fcc-opens-100-megahertz-mid-band-spectrum-5g-0.

[55] 	FCC, “Part 27 – Miscellaneous Wireless Communications Services,” last amended on August 7, 2023. Online available: https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-27.

[56] 	FCC, “Part 96 – Citizens Broadband Radio Service,” last amended on August 7, 2023. Online available: https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-96.

[57] 	FCC-20-22, “Expanding Flexible Use of the 3.7 to 4.2 GHz Band,” March 3, 2020. Online available: https://www.fcc.gov/document/fcc-expands-flexible-use-c-band-5g-0.

[58] 	OnGo Alliance, “CBRS Coexistence Technical Specifications,” OnGo-TS-2001 V4.1.0, March 8, 2022. Online available: https://ongoalliance.org/wp-content/uploads/2022/03/OnGo-TS-2001_v4.1.0_-Published-March-08-2022.pdf.

[59] 	ISED, “Decision on the Technical and Policy Framework for the 3650-4200 MHz Band and Changes to the Frequency Allocation of the 3500-3650 MHz Band,” May 2021. Online available: https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/spectrum-allocation/3650-4200-mhz/decision-technical-and-policy-framework-3650-4200-mhz-band-and-changes-frequency-allocation-3500.

[60] 	MSIT, “Ministry of Science and ICT Announcement No. 2018-235,” May 4, 2018. Online available: https://www.msit.go.kr/bbs/view.do?sCode=user&mId=123&mPid=122&bbsSeqNo=96&nttSeqNo=1381869

[61] 	MSIT,” Enforcement Decree Of The Radio Waves Act,” June 7, 2023. Online available: https://www.law.go.kr/lsSc.do?section=&menuId=1&subMenuId=15&tabMenuId=81&eventGubun=060101&query=%EC%A0%84%ED%8C%8C%EB%B2%95+%EC%8B%9C%ED%96%89%EB%A0%B9#undefined.

[62] 	Radio Spectrum Management, “Technical Arrangement of the 3.5 GHz Band,” June 2019. Online available: https://www.rsm.govt.nz/assets/Uploads/documents/consultations/2019-technical-arrangements-of-the-3-5-ghz-band/2babbb72ec/technical-arrangements-of-the-3-5-ghz-band-discussion-document.pdf.

## <<Next Change for TR 38.858>>

9	Implementation feasibility of SBFD

Editor's note: This section captures the feasibility considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE, as well as feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

9.1	Background for analysis

Editor's note: This section captures the overall description of all potential approaches and key enablers for SBFD

In an SBFD-capable BS, the receiver in the uplink sub-band potentially experiences interference from multiple sources. In the study these sources include:

- The base stations own transmitter transmitting in the downlink sub-bands;

- Other sectors transmitting in the downlink sub-bands;

- Other base stations of the same operator transmitting in the downlink sub-bands;

- Other operators base stations transmitting on other carriers within the same band.

Throughout the study, it is assumed that all other sectors and other base stations belonging to the same operator do not transmit in the uplink sub-band during SBFD slots, that is, that SBFD is deployed in all sectors and sites and the SBFD configuration is the same in all sectors and sites. The case where SBFD is not deployed at all sites or SBFD configurations could differ has not been examined as part of the study.

Self-interference has been assessed using a framework named as “Residual Self Interference Calculation”, RSIC in a table. It is important to take into account that although the table is a useful and convenient means to break down the analysis of self-interference mitigation steps for the purposes of proposing an overall self-interference mitigation solution, the table should be interpreted with care. The individual components in the table are not independent of one another and are not independent of the total TX power level and RX parameters. Although the table expresses the interference suppression as a dB value, the dB interference suppression would change for other TX power or RX parameters. Thus, the values in the table should not be seen as a generic approach to scaling interference, but rather as a breakdown of how the SBFD may operate under a certain set of assumptions that is used to visualize how the analysis of feasibility has been worked out.

Furthermore, the overall impact of analogue components is considered rather than the behavior of each individual transmitter/receiver. Due to production variations between radios and deviations in the exact level of isolation between TX and RX some transceivers may perform better and others worse in reality. The RSIC table should be seen as an overall example of the distribution of interference mitigation performances, but not an exact calculation based on individual transmitter/receivers.

An obvious baseline for assessing the radio performance is a transmitter / receiver performance that just meets RAN4 minimum requirements. However, it is to be expected that to a reasonable extent the performance could be extended beyond the minimum needed for meeting for 3GPP requirements and thus examples are provided considering both the baseline and some improvement. The associated complexity of improvement should be considered.

It is important to note that the antenna isolation can vary depending on the beam direction although this variation with beam direction may reduce after beam nulling is applied. In some companies’ input, worst case is considered. Meanwhile some figures presented in the tables from other companies are “typical”. For “good” beam directions, the residual interference may be reduced. However, there may be other beam steering directions for which the power may greater than used in these estimates. Individual RSIC descriptions capture the extent of expected variation with beam direction.

The impact of and ability to suppress inter-sector interference is captured in a similar manner to the self-interference cancellation with a table termed the co-channel co-site inter-sector interference table. The same considerations apply to the table for inter-sector interference as for the self-interference table.

The impact of interference between different gNBs on the same SBFD carrier at other sites is not considered as part of the RAN4 study on feasibility, because the relevant analysis highly depends on the practical network deployment. However, RAN4 have advised RAN1 on how to consider inter-subband leakage and the receiver blocking model and RAN1 then consider the impact of such interference as a performance issue, on which the relevant conclusion has been captured.

The impact of interference from other operators may be considered by RAN1 in the performance assessment and is considered by RAN4 as part of the co-existence study.

The downlink sub-band transmissions can impact the performance in the uplink sub-bands by two basic mechanisms:

- Unwanted emissions from the downlink sub-bands occur at frequencies corresponding to the uplink sub-band and cause interference. The impacts of such effects are described qualitatively below and assessed quantitatively in subsequent sections.

- A high downlink power from the downlink sub-bands passing through the uplink receiver can induce interference in the uplink sub-band due to non-linearities in the RF. The impacts of such effects are described qualitatively below and assessed quantitatively in subsequent sections.

When considering the RSIC for self-interference and inter-sector interference, the following aspects and self-interference mitigation techniques have been considered:

Antenna isolation

For self-interference suppression, isolation between the transmitter and receiver antennas can be used to reduce both the transmitter leakage power transferred to the uplink sub-band and the power level of the TX sub-band signals that must be handled by the receiver. The antenna isolation may be impacted by the beamforming applied at both the transmitter and receiver.

Physical distance between the transmitter and receiver sub-arrays produces isolation. The isolation can be significantly increased by means of isolation structures, isolating materials and/or RF chokes.

The antenna isolation needs to be achieved over at least a bandwidth covering all of the carriers that the basestation transmits. Since BS hardware is generally built with the ability to configure or re-configure carriers within the 3GPP band(s) or sub-band(s) covered by the BS, and only applies analogue filtering to suppress the transmitter outside of the band(s) then the isolation may need to be achieved between a TX carrier or sub-band anywhere within the 3GPP (sub-)band(s) transmitted by the BS and the UL sub-band.

Inter-sector interference occurs due to propagation of side/grating lobes and backlobes between sectors. The positioning of sectors is likely to be limited by site constraints. Potentially inter-sector isolation may be increased by increasing physical distance between sectors or by means of adding isolation materials (which may depend on factors such as weight, wind-load, weather resistance etc.).

TX beam nulling

SBFD assumes an AAS array at the BS with a large antenna array. The degrees of freedom available from the array can be used for steering transmitter nulls towards the receive sub-panel, such that the transmitter leakage to the receiver is reduced. A possible cost of applying TX beam nulling is that the EIRP towards downlink users is reduced, and that downlink MIMO performance may be impacted. The RSIC analysis provides details of additional isolation achieved and the downlink impact of the TX beam nulling.

Suppression of transmitter leakage (frequency isolation)

The unwanted emissions from the transmitter that fall into the receiver sub-band arise from the spectral widening of the OFDM signal and from intermodulation products in the transmitter RF. Spectrum widening of the OFDM signal can be managed by means of digital filtering. Non-linearity in the transmitter RF chain is typically mitigated by techniques including Crest Factor Reduction (CFR) and Digital Pre-Distortion (DPD). Using these techniques, the interference in the UL sub-band due to transmitter leakage can be suppressed. The amount of suppression is captured in the RSIC tables.

Analogue interference cancellation

Analogue interference cancellation refers to techniques that involve capture of the transmitted signal in the downlink (including both the DL sub-bands and potentially also the leakage signal to the UL sub-band at the transmitter) and feedback to and removal from the received signal. This can involve both cancellation of the leakage signal in the UL sub-band and removal of the signal in the downlink sub-band in order to reduce the total power entering the analogue receive chain.

The analogue cancellation theoretically requires feedback from every transmitter to every receiver, although techniques have been proposed that reduce the number of TX-RX paths that need to be implemented. The feedback needs to take into account the coupling between the TX and RX (S parameters), any filtering or other processing that take place in between the TX capture and RX cancellation points and any impact of scattering or reflections from the environment around the base station.

Digital interference cancellation

Digital interference cancellation refers to the use of digital processing to remove interference in the uplink sub-bands. Digital processing can refer to several different techniques. One technique is to capture and sample the leakage from the transmitter in order to subtract the leakage signal in the digital domain at the receiver. In this case, the impact of coupling within the array, receiver RF and scattering / reflection needs to be modelled in the cancellation signal. The complexity of such cancellation depends on the number of transmitters and number of receivers. Digital cancellation of this type can attenuate transmitter interference but cannot reduce the magnitude of the TX signal entering the receiver RF analogue front end.

Another type of digital processing is the use of the receiver equalization and combining to mitigate interference due to the transmitter, potentially including the leakage signal and non-linearities created within the receiver. A channel estimate is needed for the wanted received signal. For the interference components, it may be possible to estimate the characteristics of the coupling and distortions between transmitter and receiver. Alternatively, co-variance estimations of the interference can be used in the receiver equalization processing. The receiver algorithm should aim to maximize SINR at the receiver.

Receive beamforming

The receiver beamforming can be used to steer nulls towards sources of interference, such as the transmitter or other sectors. The process of receiver beamforming is implicitly captured in the receiver MMSE combining as described above. The receiver combining will aim to maximize RX SINR by means of optimizing the beamforming gain towards the wanted signal and rejection towards interferers.

Receiver linearity performance

The receiver analogue chain contains RF imperfections. For a large input signal, the receive chain may be driven towards compression and create intermodulation products. The self-interference signal from the transmitter (and other nearby sectors) is a high-power signal that enters the receiver and can cause such effects.

Several approaches may be considered to mitigate the impact of receiver non-linearity, each of which will have trade-offs. One possibility is to use components, in particular LNAs with an increased linearity. This will mitigate receiver non-linearity behaviors at a cost of some increase in energy consumption and potentially size. A second possibility is to use automatic gain control to reduce the receiver gain and hence the magnitude of the received signals. Reduction of gain will increase the receiver noise figure, and so careful consideration is needed on the balance between mitigating RX interference and increasing the noise figure (which decreases coverage).

A further possibility is the use of filtering before or within the RF front end. Filtering prior to the LNA or in the first stages of the receiver chain that filters out the downlink power from the transmitter can reduce interference. There may be drawbacks from filtering such as filter insertion loss, which can increase noise figure. The need to be specifically tuned to the RF frequency of the UL sub-band and impacts to size and integration. More analysis of filtering solutions is provided in the feasibility sub-sections. Filtering further down the chain, such as between LNA stages has less impact on noise figure but requires early stages of the RX chain to have high linearity.

The RSIC tables present estimates of linearity performance considering the advantages and disadvantages of mitigation approaches.

Phase noise

Phase noise in the receiver can lead to interference. Reciprocal mixing of receiver phase noise on a high-power signal in the DL sub-band can cause interference into an UL sub-band. In general, phase noise is not a significant contributor to cause interference into an UL sub-band unless phase noise is high and the DL signal is received at very high power

Other considerations

Another factor that can impact the self- and inter-sector interference suppression can be the dynamic range of the ADC in the receiver. The dynamic range of the ADC needs to be sufficient to accommodate both the TX signal in the DL sub-band and the lower power RX signal in the UL sub-band. It is generally straightforward to reduce the signal power in the TX sub-band by means of filtering at an intermediate frequency immediately prior to the ADC and so ADC dynamic range is not seen as a problem. (A more advanced solution could be needed for direct conversion receiver architectures).

## 9.2	Feasibility of FR1 wide area BS aspects

9.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

9.2.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

Based upon RAN4 agreements, the RSIC capability is broken down into four aspects: (1) spatial isolation; (2) frequency isolation; (3) beam nulling/isolation and (4) digital IC. Accordingly, based upon the inputs from companies, the ranges for values of (1)-(4) are summarized in reply LS [46]; however, the detailed ranges are the supersets of results provided from source companies which require further feasibility analysis. Therefore, RAN4 further carried out the study based on a more detailed self-interference analysis framework [47], which is used to capture inputs from companies.

Table 9.2.1.1-1: FR1 WA BS Self-interference Analysis Summary

| FR1 |  |  |  | Samsung | Samsung | Ericsson | Huawei | Huawei | Qualcomm | CATT | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS (subband filter-1) | Wide Area BS (subband filter-2) | Wide Area BS | Wide Area BS example 1 | Wide Area BS example 2 | Wide Area BS | Wide Area BS | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm | 49 dBm | 53 dBm | 47 | 53 | 49 dBm | 49 | 54 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI−frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 | 45 | 45 dBc | 45 | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD utilized | Digital filtering, CFR, DPD | DPD | DPD | DPD | CFR、DPD | Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion |
|  | Spatial isolation | Spatial isolation capability dBαSI−spatial = ③ dBc |  | 80 dBc | 80 dBc | 70 dB | 80 | 80 | 80 dBc | 70 | 65 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | TX/RX panel separation and RF barrier structure | A combination of spatial isolation, chokes, absorption, mushroom EBG. 70dB is indicative average; isolation varies from around 55dB to more than 80dB depending on beam direction. | Spatial separation between TX/RX panel with absorbing material and choke structure. | Spatial separation between TX panel with absorbing material and choke structure. | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation | TX/RX panel separation、 isolation structures, isolation material, cross polarization | Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels 65dB is indicative average; isolation varies from around 53 dB to 73dB depending on beam direction.. |
|  | TX Beam nulling /isolation in TX sub-band dBαSI−beam = ④ dBc |  |  | 10 dBc | 10 dBc | 10 dBc TX beam nulling reduces the variation with beam direction, and hence spatial isolation + TX nulling can be around 80dB for most directions. | 10 | 10 | 15 dBc | 10 | 12 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | Limited, ~0dB | Up to 1-5dB EIRP loss, depending on beam direction | Less than 0.5 dB loss | Less than 0.5 dB loss |  |  | 0.8 dB maximum |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -86 dBm Note: provided by ①-②-③-⑨ dBm | -86 dBm Note: provided by ①-②-③-⑨ dBm | -72 dBm | -94 | -88 | -91 dBm (=①-②-③-④) | -89.01 ①-②- ③- ⑨-⑦-10*log10(40/20) | -62 dBm/20 MHz |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI−RFIC = ⑤ dBc |  | 15 dBc | 11.8 dBc | 0 dBc | N/A | 10 |  | 0 | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI−RFIC = ⑧ dBc |  | 0dBc | 0dBc | 0 dBc | N/A | N/A |  | 0 | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | subband filtering (20MHz passband, 2* 5PRB transition band used for roll-off between passband/stopband) | subband filtering (24.8MHz passband, 2*3.8MHz transition band used for roll-off between passband/stopband) | None; see section 9.2.1.2.2 for analysis. | N/A | Analog filter is put after LNA |  |  | None apply due to feasibility concerns |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | Limited | Limited | N/A | N/A |  |  | 0 | N/A dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -56 dBm Note: provided by ①-③-④-⑤dBm | -52.8 dBm Note: provided by ①-③-④-⑤dBm | -27 dBm | -43 | -37 |  | -31 Rx is blocked ①-③-④-⑤ | -23 dBm with some variations depending on TX beam |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI−frequency, RX= ⑥ dBc |  | 40 dBc | 40 dBc | 0 dBc | digital filter: 60-80 dB | sub-band analog filter: 10 dB digital filter: 60-80 dB | 15 dBc | 65 | 0 dBc |
|  |  | Frequency isolation techniques |  | Filtering | Filtering | The receiver is in high non-linearity; no possibility for interference mitigation as part of the digital receive combining algorithms. . | digital filtering | sub-band analog filter and digital filtering | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). | Digital filter for ACS | None apply due to feasibility concerns |
|  |  | RX IMD | Rx IIP3 capability (dBm) | With subband filtering, RX non-linearity impact is neglectable | With subband filtering, RX non-linearity impact is neglectable | -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS) | -10 | 0 | Not a significant contributor on the gNB Rx capability. | -10 | -10 dBm at maximum sensitivity; +10 dBm at maximum linearity (at NF penalty) |
|  |  |  | Rx IM3 contribution (dBm) |  |  | Even without ADC overload: -17 dBm (RAN4 minimum receiver) -37 dBm (Realistic) -61 dBm (Optimistic) | -109 | -121 |  | -73 (①-③-④-⑤)-2*(IIP3-(①-③-④-⑤)) | Negligible (at NF penalty) |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | N/A | ADC overload can be mitigated with filtering prior to ADC except for direct conversion architectures. | ADC noise: -109 reciprocal phase noise mixing:-112 | ADC noise: -113 reciprocal phase noise mixing:-116 | Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model. | -116 | Negligible |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -96 dBm Note: provided by ①-③-④-⑤-⑥dBm | -92.8 dBm Note: provided by ①-③-④-⑤-⑥dBm | Receiver in high non-linearity | -105 | -111 |  | -96 ①-③-④-⑤-⑥ | -73 dBm/20MHz (at 50 dBc ACS) |
|  | RX Beam nulling /isolation in RX sub-band dBαSI−beam = ⑨ dBc |  |  | 10 dBc | 10 dBc | RX processing does not mitigate analogue non-linearity | 10 | 10 |  | 10 | 0 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | Limited, ~0dB | Receiver saturated | Less than 0.5 dB loss | Less than 0.5 dB loss |  |  | 0 dBc; should not assume further UL beamforming loss to maintain any UL gains |
|  | Digital IC dBαSI−digtial = ⑦ dBc |  |  | 20 dBc | 20 dBc | Digital IC not possible due to receiver non-linearity and would anyhow be highly complex due to large number of TX/RX for wide area. | 15 | 15 | 15 dB | 10 | 0 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 154.6 dBc | 154.2 dBc | Transmitter: 125 dB Receiver: N/A due to receiver saturation | -150.6 | -155.3 | 155 dB (②+③+④+⑦) | 121.86 | 122 dBc |
| Noise floor ⑩dBm |  |  |  | -96dBm/20MHz | -96dBm/20MHz | -96 dBm/CBW | -96 dBm/20 MHz | -96 dBm/20 MHz | -96 dBm/20 MHz @ 5dB noise figure | -95.99 | -96 dBm/CBW (20 MHz) |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm | -102 dBm | -102 dBm | -102 dBm | -102 dBm | -102 dBm | -101.99 | -102 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 151 dBc | 151 dBc | 155 dBc | 149 | 155 | 151 dBc | 150.99 | 156 dBc |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | DUD(40-20-40MHz) | 40-20-40 MHz | DUD [40, 20, 40] | DUD [40, 20, 40] | DUD | 40-20-40 | DUD (40/20/40 MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB | 5 PRB | 5 PRB. | Existing SU | Existing SU | 5 PRBs |  | 5 RB (1.8 MHz) |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | 100MHz | >300 MHz | Several hundred MHz | Several hundred MHz | 100MHz |  |  |
| Others |  |  |  |  |  |  |  |  |  |  |  |

9.2.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

9.2.1.2.1	Samsung

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 9.2.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

When SBFD is implemented at the gNB, the received UL signal at the gNB is subject to co-channel self-interference from the gNB side transmitter. Methods to cancel the self-interference include passive methods which rely on the antenna isolation between Tx and Rx antennas, active methods which utilize RF or digital signal processing, hybrid methods using a combination of these, and filtering.

Achieving a sufficient level of residual self-interference suppression and cancellation is the most critical part when implementing SBFD at the gNB. Without adequate SIC capability, the interference from the transmitted DL signal would corrupt the received UL signal as illustrated in Figure 9.2.1.2.1-1 (a). To solve this problem, various SIC schemes can be used. Using the example of Figure 9.2.1.2.1-1 (b), SIC capability can be provided through the antenna or panel design (A), can be applied in RF domain to the RF signal (B) or in digital signal domain (C), or a combination of these.

Figure 9.2.1.2.1-1: gNB transceiver architecture with self-interference cancellation capability

For example, antenna SIC can be used to minimize the leakage power from the Tx ports to the Rx ports of the panel, and digital SIC is then used to handle any residual interference after antenna SIC. DL out-band signal power flowing into the UL Rx path can be effectively suppressed below the noise floor level to guarantee the UL receiver performance. Also, by combining digital pre-distortion (DPD) at the Tx path and digital SIC at the Rx path, the out-band interference from the DL signal to the UL signal can be effectively mitigated by the gNB such that the need for a guard band between the UL and DL signals is minimized.

Spatial Isolation by Antenna Design

In the analysis it’s assumed separate panels for simultaneous downlink transmission and uplink reception as separate-TX/RX antenna array for evaluation of SBFD operation. The basic spatial isolation between RX and TX antenna panels can be achieved by directional isolation.

Firstly, Tx/Rx isolation can be increased by increasing the spatial distance. Furthermore, an additional RF barrier structure could be useful to further improve Tx/Rx isolation performance, and using the RF barrier between the Tx and Rx panels could also affect the required spatial distance separating the Tx and Rx panels. A well-designed RF barrier can minimize the need for large spatial separation and mostly preserve the existing antenna form factor and enclosed volume comparable to legacy TDD. To design an efficient RF barrier, various electromagnetic resonator structures can be incorporated into the antenna design, e.g., wall(s), gap(s), or a combination of them. These result in surface wave nulling and can further block the undesired leakage signals from the Tx panel to the Rx panel.

Figure 9.2.1.2.1-2 demonstrates the S21 measurement results with respect to the distance between upper and lower antenna panels in our FR1 3.5 GHz SBFD testbed.

Figure 9.2.1.2.1-2: FR1 testbed and SIC performance when varying distance between upper and lower panel

While it can be expected that spatial isolation numbers vary depending on the form and particular layout configuration of antenna elements in the upper and lower panels, we have shown that >80 dB antenna isolation is possible between the Tx and Rx panels in FR1, with reasonable separation distance between upper and lower panel.

An important design consideration for increased spatial isolation provided by the RF barrier is whether such stopband performance is stable over a wide enough frequency range. Electromagnetic (EM) isolators and resonant structures are designed around a specific center frequency, e.g., 3.5 GHz. Therefore, design of the resonant structure must account properly for the channel bandwidth and NR operating band under consideration to provide a sufficiently large stopband between Tx and Rx panel. Another consideration is that undesired Tx/Rx interference is created by multiple EM sources, e.g., antenna elements in the Tx panel. Therefore, diffusion of the corresponding surface waves is more challenging when isolating the Tx and Rx panel. Despite these challenges, our FR1 3.5 GHz testbed have achieved isolation performance that show almost uniform antenna and panel isolation performance with respect to frequency for the 100 MHz channel BW of the NR carrier in 3.5 GHz.

According to the applied mechanisms and measurement results, the achievable level for TX and RX spatial isolation without impact on radiation pattern based on compact antenna size is around 80dB for FR1.

TX and RX beam nulling/isolation

The effect of beam nulling for isolation depends on implementation and antenna array size. For both TX and RX panels, the large number of antenna elements for TX/RX beamforming can provide the ability to provide nulling to mitigate the self-interference by increasing the isolation. For FR1 up to 10dB isolation by beam nulling can be contributed to residual interference suppression.

Frequency isolation at TX

For SBFD, in which the Tx signal and the Rx signal are respectively allocated to non-overlapping frequency-domain resources on the same time-domain symbol for simultaneous transmission and reception, at least the waveform roll-off therefore reduces the magnitude of the Tx-Rx interference to which the Rx signal is subjected. Additionally, BB filtering can be applied to further increase the achievable isolation. The use of frequency-domain isolation between the Tx and Rx signal allocations is primarily an approach that serves the purpose of reducing the amount of self-interference which must be further cancelled by a digital cancellation stage.

In the case of gNB-side SBFD operation, the SBFD UL subband can be considered as out-of-channel with respect to the 1 or 2 SBFD DL subband(s). Undesired spectral leakage from the DL Tx signal in the gNB into the Rx path are reduced similar to the case of out-of-channel leakage, e.g., comparable to the gNB Tx-side Adjacent Channel Leakage Ratio (ACLR) for coexistence between two operators on adjacent channels in the same NR band. Note that ACLR is determined by the non-linear characteristics of the PA and corresponding RF requirements are set by RAN4, e.g., 45 dBc for the gNB Tx.

While it can be assumed that the achievable Tx-to-Rx interference from the SBFD DL subband to the UL subband can only guarantee performance according to the less stringent in-channel RF requirements, our FR1 3.5 GHz testbed implementation shows that the use of digital pre-distortion (DPD) techniques to improve upon the non-linearity characteristics of the PA can achieve 45 dBc isolation between the SBFD DL and UL subbands. Figure 9.2.1.2.1-3  shows the achievable isolation in frequency domain for FR1 SFBD when Tx-to-Rx leakage is also compensated for by DPD based on the FR1 3.5 GHz testbed.

Figure 9.2.1.2.1-3: FR1 testbed and PSD for SBFD DL and UL SBs after antenna isolation and digital pre-distortion

Frequency isolation at RX and RF SIC

Note that TDD gNB radio unit design must also account for ADC and LNA in the receiver path, e.g., to prevent Rx saturation or blocking by the spectral leakage created from the undesired Tx signal. To prevent ADC saturation in the Rx path of the gNB radio unit supporting SBFD, Rx filtering can be used to suppress the leakage from the Tx side interfering signal. Additional Rx filters can provide protection to avoid potential dynamic range and saturation issues for ADC or LNA when demodulating the UL subband in the Rx path of the gNB. Note that for RF filters with sharp roll-off’s, the order of the filter must increase, and so must then the size of the filter. Additional insertion losses are incurred which negatively affect the link budget.

High-Q value RF filter can provide enough attenuation towards high power level interference in the DL subband(s), not only for the self-interference but also other co-channel interference sources from co-site inter-sector and inter-site gNBs. As illustrated in the below figure, for RF direct-sampling receiver (which shall be regarded as the receiver architecture more difficult to implement subband filter compared to super heterodyne and homodyne/zero-IF receivers) to have the RF subband filter be located between the two-stage cascaded LNAs, the normal design is to have the UL subband as passband and reserve a few number of PRBs (e.g., 5PRB assumed) for transition band(s) to allow a certain suppression to filter out interference signals over DL subband(s).

Figure 9.2.1.2.1-4: Improved direct-RF sampling receiver with subband filtering between the two-stage cascaded LNAs

The key difficult is to design a high Q-value RF subband filter, which should also be restricted by the limited space in the integrated base station design. The RF filter performance for Q-values of 1500 and 5000 has been studied by using RF simulation tool as provided as below, by providing the transmission S21 and reflection S11 goal for the targeted 20MHz passband, 20dB return loss, stopband and 25dB attenuation.

Figure 9.2.1.2.1-5: Analog filter performance for Q=1500 (left) and 5000 (right), for 3.5GHz operating freq. and 20MHz passband

The feasibility of high Q-value RF subband filter with reasonable size/weight to be integrated into current gNB implementation has been challenged by some companies in previous RAN4 meetings. On the other hand, it should be noted that some novel designs are recently proposed, which could be based on some new structure for ceramic dielectric filter to have very good RF filtering performance as requested, and there are some preliminary results simulated by HFSS, which are based on the ceramic dielectric filter with the cascaded quardruplet structure dimensioned by 19.5mm*19.5mm*6mm illustrated in the below figure, that shall be regarded as reasonable small size/weight and feasible to be integrated in current gNB design.

Figure 9.2.1.2.1-6: New cascaded quardruplet structure for ceramic dielectric filter

Furthermore, one alternative solution with relaxed Q-value subband filter but with more flexibility for subband configuration is also studied. As illustrated by the below figure, subband filter can still be implemented between the two-stage cascaded LNAs, and what different is the designed filter shall have a passband wider than the configured UL subband and the transition band could be much relaxed from 5PRB. For example, to support 20MHz UL subband, we can implement a subband filter easier to be implemented, e.g., {larger passband than 20MHz, more PRB for transition band} being considered.

Figure 9.2.1.2.1-7: Alternative solution with relaxed Q-value subband filtering

With the above design, the motivation of introducing UL subband filter is to reduce DL interference level to avoid RX blocking, rather than to remove all DL interference signals, thus making the filtering passband to be equal to UL subband unnecessary. If the subband filter with larger passband could prevent RX blocking, the residual interference not filtered by the subband filter can be further handled by the operation in the digital domain, including digital filtering and digital interference cancellation.

For instance, we designed the filter with <25MHz passband and <4MHz used for roll-off transition band between passband/stopband and 25dB suppression (better suppression performance, but still easier to be implemented because of larger transition bands). We would also like to use HFSS-based RF simulation to demonstrate the feasibility of this design. There are some numerical results of the well-designed advanced RF filter for which we evaluate the performance by HFSS-based RF simulation. The filter is also based on the ceramic dielectric filter with the cascaded quardruplet structure with the same dimension as previous filter design (i.e., 19.5mm*19.5mm*6mm) but different structure illustrated as the below figure, that shall also be regarded as reasonable small size/weight and feasible to be integrated in current gNB design.

Figure 9.2.1.2.1-8: New cascaded quardruplet structure for ceramic dielectric filter for filter design with 24.8MHz passband (intentionally larger than 20MHz UL subband)

Figure 9.2.1.2.1-9: RF simulation results for filter design with 24.8MHz passband (intentionally larger than 20MHz UL subband)

As demonstrated in the above figure, for this RF filter design, the passband (the point m2 to m3 in the above figure) is 24.8MHz, which is intentionally larger 20MHz as UL subband bandwidth. Even by considering 25dB suppression, the transmission bands are less than 3.8MHz for both lower and higher frequency sides.

We can assume the worst case that 4.8MHz DL interference signals (24.8MHz passband – 20MHz UL subband BW) are not filtered out at all, and the DL interference at 2x 3.8MHz transition bands is filtered out by -14dB (for the worst case estimation by separating 3.8MHz into several parts). Therefore, we can derive the residual self-Interference signal in gNB RX subband (caused by non-ideal RX selectivity) gain-normalized dBPSI_Selectivity = as 49dBm - 80dB - 5dB - 11.8dB (subband filtering) - 40dB = -87.8dB, which is still 6.8dB smaller than the residual self-interference leakage in UL subband due to non-ideal TX. It should be noted that the equivalent suppression provided by subband filtering can be calculated as 10*log_10((4.8MHz/80MHz)*10^(0dB/10) + (2*3.8MHz/80MHz)*10^(-14dB/10) +  (67.6MHz/80MHz)*10^(-25dB/10)) = -11.8dB. Therefore, with the alternative solution with the subband filtering having a larger passband than the configured UL subband and larger transition bands for roll-off, the RF filter will be easier to be design.

Additionally, analog filters such as IF and BB filters can be employed. For example, when the receiver is designed to use zero IF architecture, the receiver can use the lowpass filter to further remove the leakage signal after applying the mixer. By combining multiple LNAs, filter loss can be compensated more easily.

Digital IC

As aforementioned theoretically, the digital IC should be with the capability to remove all remaining self-interference if the total level to be handled by ADC input is within its dynamic range. For 12bit ADC with assumption of 12dB PRPA signal, the dynamic range is >50dB.

The desired received signal is mixed with the undesired DL leakage signal in the Rx path of the gNB radio, e.g., after ADC. The unwanted DL leakage signal must be removed by receiver processing using digital SIC. It is necessary to estimate the interference channel between the Tx panel and the Rx panel. Digital SIC performance is helped when synchronization to accurately remove the Tx signal from the Rx signal can be obtained. In principle, two methods exist to estimate the interference channel. One approach is to store information on a Tx signal that has passed through the PA with a feedback link and then estimate the interference channel over-the-air to remove the interference from the Rx signal. Another approach is to use only over-the-air estimation. Without a feedback link, the whole combined channel can still be estimated through the Rx panel. We used the first approach in the FR1 3.5 GHz testbed.

9.2.1.2.2	Ericsson

The corresponding technique input in the summary Table 9.2.1.1-1 presents an overview of the self-interference mitigation potential for a wide area BS with 53 dBm transmit power.

When considering the transmitter sub-band emissions that leak into the RX sub-band, the emissions are suppressed to a level of around -72dBm using transmitter and analog suppression techniques, which is around 24dB above the noise floor. In principle, digital techniques could to some extent be used to further suppress the TX interference, however the receiver is blocked. From the receiver perspective, the input power is too high and the receiver is blocked. A detailed description is provided in the summary table.

The following are more detailed considerations of modelling and techniques captured in table 9.2.1.2.2-1.

TX – RX isolation

Transmitter to receiver isolation is achieved by means of separating the transmit and receive panels. Spatial separation alone achieves in the order of 30-40 dB isolation. However, a number of techniques exist to significantly improve the TX-RX panel isolation including chokes, absorption, mushroom EBG etc. A detailed electromagnetic simulation of these techniques is presented in R4-2216404, which demonstrates that the isolation between a TX panel and RX sub-array varies depending on beam direction between 55 to 80dB. An example of the electromagnetic simulations is depicted in figure 9.2.1.2.2-1 and 9.2.1.2.2-2. The first figure visualizes the EM propagation between the sub-arrays, whereas the second figure indicates the TX panel to RX sub-array isolation for several TX beam steering directions. The simulations take into account an advanced suppression structure between the sub-arrays.

Figure 9.2.1.2.2-1: Full-wave averaged E-field magnitude on an XZ plane cut based on EM simulation

| - 0 degree (boresight) | - 15 degrees (TX steers main beam toward RX) | - -15 degrees (TX steers main beam away from RX) |
| --- | --- | --- |
| (d) 0 degree (boresight) | (e) 15 degrees (TX steers main beam toward RX) | (f) -15 degrees (TX steers main beam away from RX) |

Figure 9.2.1.2.2-2: TX panel to RX sub-array coupling magnitude curves considering co-polarized (top) and cross polarized (bottom) ports. Each curve represents the coupling magnitude of the TX panel to a single RX sub-array. Each sub-figure corresponds to a specific elevation angle.

The specific value depends on the scheduled users, and 70dB has been taken as a representative value. With TX beam nulling, as described in the subsequent section the variation can be reduced and the achievable isolation becomes around 80dB.

TX beam nulling

The transmit panel has a large number of transmit elements and hence a high number of degrees of freedom to perform beamforming. Beam Nulling can be used in the transmit panel to reduce the power at the receive panel. It is not clear that beam nulling has the same impact on both the transmitted signal and the transmitter leakage, however for simplicity this has been assumed. A simulation investigation has been presented in R4-2219633, which demonstrated the possibility to increase the spatial isolation to around 80dB using beam nulling. Furthermore, beam nulling reduces the variation of the spatial isolation due to beam direction. Thus, 80dB of spatial isolation is assumed.

The beam nulling has an impact on DL EIRP depending on the beam direction. The impact to DL MIMO performance was not investigated.

Analogue interference cancellation

Analogue interference cancellation could be considered as a means for suppressing both interference in the RX sub-band and power in the TX sub-band entering the receiver. Analogue IC requires and inter-connections/routing paths to detect the signal at each transmitter as well as remove the interference in the receiver. In order to ensure that the removed signal is not impacted by receiver processing, the interference subtraction must take place in the first stage of the receiver chain, before the LNA and thus insertion losses caused by the coupling will degrade the noise figure.

Analogue interference cancellation is a promising technology for some smaller BS types and simulations demonstrate potential for mitigating interference for larger arrays. However, the complexity of interconnections between all TX and RX elements in a large commercial BS and the losses associated with the combining and subtracting would lead to a performance decrease and size and weight increase for an AAS to the level of a doubtful feasibility. Thus, analogue interference cancellation has not been considered for a high power, large array AAS.

Receiver analogue filtering

For the wide area BS, the main performance issue is the large power in the TX sub-bands entering the receiver, -27dBm. The minimum RAN4 requirement for a receiver is to produce a 6dB desensitization when a carrier of -43dBm is applied in a 2nd adjacent channel, and thus the application of -27dBm directly next to the RX sub-band is very challenging. Although the linearity performance can be improved, the LNA linearity cannot be directly improved to become sufficient.

A possible solution is to use analogue filters before the LNA to remove the DL sub-band power. Investigations in R4-2219633 demonstrate that it is not possible to build analogue filters with an achievable Q-factor without a large insertion loss.

Figure 9.2.1.2.2-3: Analogue filter performance for Q=1500

Table 9.2.1.2.2-2: Insertion losses for Q=1500 filters

| Filter type | Edge insertion loss | Average insertion loss |
| --- | --- | --- |
| 10 MHz UL sub-band | 7.6 dB | 4.4 dB |
| 20 MHz UL sub-band | 7.4 dB | 3.1 dB |
| 30 MHz UL sub-band | 7.8 dB | 2.6 dB |

Another alternative is to place analogue filters between a first stage and a second stage LNA. Using this approach, the linearity of the receiver chain could in principle be improved with a much-reduced impact to the noise floor. However, there are a number of significant problems with such an approach:

- The first LNA stage needs high linearity and becomes very power consuming.

- The tight integration needed to achieve power and weight efficient AAS BS is no longer feasible due to the filter size. Hence there will be further increases in power consumption due to reduced integration and thermal management issues.

- The filters would need to be tuned specifically to the UL sub-band if implemented in RF. Hence, non-reconfigurable, operator specific hardware would be needed for every BS. The alternative is to use a mixer to bring the signal down to IF or baseband, but then the mixer linearity would compromise the receiver performance and the blocking performance would not be achieved.

- There would need to be a number of filters for every branch due to e.g. 2 polarizations, support for DL slots, UL slots and SBFD slots with different filtering requirements (even more filters if there would be multi-carrier support). Also switches would be needed, which would compromise linearity and add further space. It is doubtful all of the filters could be accommodated without further losses.

Due to the above reasons, analogue filtering is not considered to be a realistic approach for a commercially relevant BS and so is not considered the feasibility analysis.

Digital interference cancellation and digital processing

Digital TX interference cancellation and subtraction, and RX combining taking into account interference covariance have the potential to mitigate interference in the receiver. Digital processing has not been considered because the power level in the analogue front end of the receiver is high enough to saturate the receiver. Digital interference subtraction would required a very high computational complexity for a wide area AAS due to the large number of TX-RX combinations.

9.2.1.2.3	Huawei

For FR1 our analysis and evaluation are provided in the summary table, we show two examples for Wide Area BS to consider different max TX power. One is 47 dBm max TX power and the other is 53 dBm. The major difference for WA example 2 is the adoption of analog filter to counteract the higher RX blocking from TX sub-band.

Analogue filter prior to the LNA would introduce also insertion loss which will cause sensitivity loss. Hence it is not suitable. Meanwhile putting analogue filter after LNA could tolerance the high insertion loss. The filter can be RF analog filter in the front-end or base-band analogue filter before the ADC. In the example we use RF analog filter which is put after LNA. The filter performance can be found in Figure below. A Q-value of 1500 and 5 poles are assumed in the simulator. It can be found that the insertion loss is less than 5 dB and ~15 dB suppression is achievable, with some margin to address the manufacturing accuracy and temperature drift.

Figure 9.2.1.2.3-1: performance of an example sub-band filter

As shown in the example below, putting analogue filter after LNA, since the front-end LNA can provide substantial gain on the wanted signal, the impact to overall noise figure is negligible (2.21dB vs 2.24 dB), and the RX IIP3 prior to the filter is high enough to cope with high blocking level, such as ~ -30 dBm. The OIP3 of gain block in LNA might need to be increases a bit, which results the addition of power consumption. However the addition is quite limited, which is less than 0.5% of overall power consumption.

Table 9.2.1.2.3-1: Cascaded NF and IIP3

| Receiver | Band Filter | LNA | ATT | subband Filter |
| --- | --- | --- | --- | --- |
| GAIN(dB) | -1.20 | 25.00 | -2.00 | -5.00 |
| NF(dB) | 1.20 | 1.00 | 2.00 | 5.00 |
| OIP3(dBm) | 100.00 | 34.00 | 100.00 | 100.00 |
| C_gain(dB) | -1.20 | 23.80 | 21.80 | 16.80 |
| C_NF(dB) | 1.20 | 2.20 | 2.21 | 2.24 |
| C_OIP3(dBm) | 100.00 | 34.00 | 32.00 | 27.00 |
| C_IIP3(dBm） | 101.20 | 10.20 | 10.20 | 10.20 |

Analogue sub-band filter after LNA can provide the needed suppression for the receiver parts after the filter, and the impacts to RX sensitivity due to insertion loss is negligible. If analogue sub-band filter is adopted in the solution, the blocking performance can be improved at least 10 dB.

9.2.1.2.4	Qualcomm

To enable proper reception of the uplink signal at the gNB receiver with simultaneously transmission DL signal, gNB should mitigate the direct self-interference ‘leakage’ and any significant clutter reflections. The self-interference could be mitigated by different techniques such as spatial isolation, analog subband filter, analog interference cancellation, beamforming and digital interference cancellation. In the following, we discuss in detail the knobs for gNB transceiver that enable the mitigation of both component of self-interference, namely direct leakage and clutter reflections.

Antenna techniques and spatial isolation

For SBFD deployments, gNB antenna configurations should be based on two panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception as shown in Figure 9.2.1.2.4-1. on the other hand, for legacy TDD deployments, gNB antenna configuration is based on single panel for downlink transmission or uplink reception. With the split panel architecture, the gNB can enable larger spatial isolation is an essential component to mitigate self-interference. In addition, the physical separation between the two panels could be used to add electro-magnetic spatial duplexer that enhances the spatial isolation between the panels.

Figure 9.2.1.2.4-1: gNB antenna/panels configuration in TDD and SBFD modes

RF measurements for the Tx-Rx spatial has been conducted and results are shown in Figure 9.2.1.2.4-2. Each curve represents the spatial isolation measured between all transmit chains of one array to one receiver chain of the other array. This includes the near field transmit and receive antenna gains. The results show more than 80 dB of isolation is achieved at the band of interest.

Figure 9.2.1.2.4-2: RF measurements of Tx-Rx spatial isolation between for FR1

Frequency isolation

DL and UL transmissions can be separated in the frequency domain via multiplexing of the DL and UL using non-overlapping DL and UL sub-bands. As a result, large frequency isolation for the UL signal reception is attained as shown Figure 9.2.1.2.4-3. For RAN4 further considerations, the frequency isolation represents the ratio of the power of non-linear leakage into the UL subband to the power of the DL signal at the DL subband, which can be approximated by the ACLR requirements specified by RAN4. RAN1 has requested RAN4 to provide value range for the frequency isolation capability of the gNB as well as the accompanying assumptions to those values.

Figure 9.2.1.2.4-3: Frequency isolation

A guardband may be needed at the gNB to protect UL reception within the UL subband and reduce the impact of self-interference. In some scenarios, depending on the gNB implementation, a very small guardband or even no guardband may be needed at all. However, from UE perspective, given that there is no UE selectivity, a guardband may be needed to protect the DL reception from the inter-UE CLI. To further analyse this, 80 MHz system bandwidth, the 60 MHz DL subband is allocated with 161 RBs (starting from first RBs at band edge) and the 20 MHz UL subband is allocated with 51 RBs. A guard band of 5RBs in between UL and DL subband. The Tx waveform is pushed to the PA to derive max Tx power of 47 dBm. The subband frequency isolation is defined at the ratio between the power leakage within the 20 MHz UL subband as compared to the transmit signal power within the 60 MHz DL subband as shown in Figure 9.2.1.2.4-4.

Figure 9.2.1.2.4-4: PSD of DL waveform and frequency isolation

Beam isolation and beamforming/nulling

In FR1, the DL precoder and UL combiner weights could be optimized to provide some beamform nulling for the clutter and/or self-interference. The massive MIMO antenna has large number of degrees of freedom in both digital and analog (i.e., hybrid beamforming) that provide the ability to create some spatial nulls. Beamforming nulling is an efficient technique for clutter mitigation.

Digital self-interference mitigation

The nonlinearities introduced within the gNB front’s end due to non-ideal components of the Tx chain will lead to residual non-linear self-interference that cannot be fully captured in the RF or analog domain due to the associated high complexity, high sensitivity of the canceler and the system’s stability. In this regard, leveraging adaptive filtering and non-linear modeling of the residual self-interference to accurately model and cancel the residual self-interference is performed to provide additional mitigation in the digital domain and enable higher MCS. With the knowledge of the DL samples and the non-linear model, an adaptive filter can be used to synthesize the non-linear leakage and cancel it out from the Rx signal as shown in Figure 9.2.1.2.4-5. This technique can be used for cancellation of both self-interference and clutter echo by having multiple taps cancellation.

Figure 9.2.1.2.4-5: Digital self-interference cancellation

9.2.1.2.5	CATT

For FR1 Wide Area BS SI analysis, the following assumptions are used.

Table 9.2.1.2.5-1: SI analysis assumptions for Wide area BS

| Parameters | Wide Area BS |
| --- | --- |
| Reference sensitivity level degradation due to SI | 1dB |
| Channel bandwidth | 100MHz |
| Subband configuration | {DUD} |
| DL subband width | 40MHz*2 |
| UL subband width | 20MHz |
| Tx output power over whole channel | 49dBm |
| Adjacent subband Leakage Power Ratio | 45dB |
| Noise Figure | 5dB |

In SI capability analysis, the following techniques are used,

- CFR is used to improve component efficiency, DPD is used for high power equipment to optimize ACLR.

- Tx antennas panel and Rx antennas panel are separate, there are also some isolation materials between them, and cross polarization is also used.

- RF subband RF filter is assumed for Tx and Rx path.

- Digital filter is used to resolve the adjacent subband (i.e. Tx subband) interference issue.

- Beam nulling is used to improve isolation between Tx and Rx.

- Digital IC is used to reduce interference in the UL sub-bands. The interference is leakage from the transmitter, similar as DPD, digital domain needs to capture and sample the interference signals, then subtract the interference at receiver.

From the analysis provided in the summary table, the receiver may be blocked. Some specific techniques need to be used to improve the spatial ISO. Requirement of IIP3 is relatively high and the ACS requirement is improved largely compared with the legacy BS. It seems wide area SBFD BS design may need much improvement from several aspects to make it feasible.

9.2.1.2.6	Nokia

The Nokia input in the summary table presents the company’s view on the self-interference mitigation analysis for a wide area base station with 54 dBm total output power. The RSIC capability corresponds to 110 dBc for the Rx subband and between 122 dBc for TX sub-band which are achieved using a combination of spatial isolation (65 dBc), frequency separation (45 dBc), and Tx beam nulling (5-10 dBc, only applicable to the Tx subband isolation). Under such considerations, the self-interference observed in the UL subband is -62 dBm/20 MHz which is more than 60 dB above the noise floor. Such level of interference would result in high desensitization of the receiver which makes it unpractical for wide-area deployments where coverage is one of the main KPIs. A detailed description of the assumed techniques and other assumptions is provided in the summary table.

BS TX Power

To study the feasibility for wide area base stations, including powerful mMIMO base stations, an output power of 55 dBm (as e.g. in the case of 64 TX paths with 5 W each) is assumed. Considering 80%/20% DL/UL frequency resource split in an SBFD configuration, this amounts to 54 dBm.

If lower power is assumed for wide area base stations, correspondingly the deployment scenario would require a denser ISD.

Frequency isolation at TX

We think 45 dB frequency isolation is feasible. This is in line with the 45 dB ACLR requirement that is typical for base stations, albeit for D-U-D sub-band configuration, slightly more difficult to achieve.

The techniques to achieve sufficient frequency isolation may include:

- Transmitter digital filtering or windowing to clean the UL sub-band. This is required to clean the IFFT output of the linear leakage of the signal, otherwise the sinc spectrum of the IFFT will dominate the emissions on the UL sub-band. Requires new filter design with potentially tighter suppression requirements compared to the channel filter, due to the desire to minimize guard bands between DL and UL sub-bands.

  - Tighter filter suppression requirements may mean longer filter impulse response and lead to signal EVM degradation.

- Transmitter digital pre-distortion to linearize the transmit chain and suppress PA distortion components. Achieving the same performance for sub-band leakage ratio as for ACLR will be more challenging, as the UL sub-band is closer to the DL sub-band(s) than the adjacent channel. The ACLR is averaged over the same bandwidth as the DL channel, with emissions likely decaying somewhat with offset. For inter-sub-band leakage, the offset is generally small, indicating tighter DPD requirements for the same absolute level of emissions. Moreover, the DUD frequency configuration will be challenging due to spectral regrowth from both sides of the UL sub-band, compared to DU configuration or the ACLR case.

  - Higher DPD complexity translates to higher energy consumption.

  - Higher energy consumption leads to increased heating, worse PA performance and thermal management issues. This may require larger and heavier cooling solutions.

Spatial isolation

For the achievable spatial isolation for separate TX and RX antenna arrays, we find that 65 dB may be a reasonable assumption for a well-designed antenna in an average case, if assuming EM shielding structures between the arrays.

The techniques to achieve sufficient spatial isolation may include:

- Separate TX and RX antennas or antenna arrays. Increased separation from TX to RX will improve isolation.

  - To maintain the same or similar physical size of the antenna, the number of elements per array need to be halved. This reduces the achievable array gain by at least 3 dB in both link directions and has been demonstrated by simulations to degrade the system performance.

  - To maintain baseline system performance, the number of antenna elements per array must be maintained, leading to an increased antenna size by at least 2x. This in turn means higher weight and wind load, increased complexity, increased trace losses which may need to be compensated, and in general higher cost.

  - Separate TX and RX antenna arrays requires separate PWBs for the TX and RX, leading to a higher cost.

  - Separate TX and RX antenna arrays leads to loss of reciprocity in the DL and UL channels and makes reliable channel state measurements more difficult and complicated. The extent of this loss has not been studied.

- EM shielding techniques such as wave traps or chokes between the TX and RX arrays.

TX Beam nulling /isolation in TX sub-band

The beamforming coefficients of the transmit beamforming may be modified so that the energy coupled to the receive antenna elements is minimized. We have conducted EM simulations that measure the TX-RX isolation based on the most exposed RX antenna element/subarray, where this element/subarray is determined separately, with and without beam nulling. Those findings are found in contribution [48]. We observe that the level of self-interference depends on the beam direction. We have measured a minimum of 8 dB of beam nulling gain for all the Tx beams and up to 14 dB of gain for the beam with lowest isolation to the receive antenna elements as illustrated in Figure 9.2.1.2.6-1. The observed DL EIRP impact was between 0dB - 0.8 dB depending on beam direction, although 90% of the transmit beams experienced a DL EIRP loss below 0.3 dB. Note also that:

  - The modification of TX beamforming coefficients reduces the transmitted EIRP toward the intended UE, leading to further reduced DL performance unless compensated by increased conducted power.

  - Based on simulations, some TX beams may be affected more than others, leading to potential scheduler restrictions in which UEs may be scheduled during the SBFD time slots.

  - The transmit beam nulling is most effective on the DL sub-band, for which the transmit signal is known and can be beamformed. It is not assumed that transmit beam nulling is effective on the UL sub-band, which contains only unwanted emission components from the transmitter.

Figure 9.2.1.2.6-1: Empirical CDF of isolation between each Tx beam towards worst-affected Rx port. Tx beams are generated within ±45° azimuth and elevation angles.

RF IC and other tech. (before LNA)

The following techniques have been evaluated, but have significant challenges in FR1 wide area base station implementations:

- Analog cancellation in the RF domain. This method may be used to subtract the unwanted coupled TX signal components from the RX signal path, before the LNA. With proper phase shift and amplitude scaling, the TX component may be cancelled.

  - There has been a proposal of an AFIR type of canceler solution in which the canceler unit is made of M+N individual cancelers, where M is the number of TXs and N the number of RXs. The canceler unit is connected to TX and RX antenna panels. If M=N=64 there will be 64 RF connections from TX panel to the canceler unit and 64 RF connections from canceler unit to RX panel. That might be doable in a common mechanics but in the case that all three units are separate, it is not feasible anymore. Since the distance of the TX and RX antenna panels need to be quite high (due to needed spatial isolation) and receiver and transmitter chains need to be located close to antenna filters (to avoid excessive insertion losses that cannot be compensated), separate TX and RX PWBs are needed.

  - Valid mathematics have been presented that M+N cancelers are sufficient for RF IC but that it is true only when one set of beamforming coefficients are valid at a time e.g. in the mmW arrays. For MU MIMO there is a need for (M+N)xUxL cancelers where M and N are as above and U is the number or users and L is the average number of layers per use. For a typical 5G case that would be starting from 5120 cancelers. On top of that, all of them have to be updated every time when the beamforming coefficients are updated. That leads to an intolerable complexity and processing burden when solving mathematics for each of those individually.

Frequency isolation at RX

Sub-band filtering techniques before or after the LNA may improve frequency isolation, but have significant challenges in FR1 wide area base station implementations and therefore are not assumed to be feasible:

- High insertion loss before the LNA will increase the receiver noise figure and negate any system gains of SBFD.

- If a sub-band filter would be used between the two LNAs, i.e. as an inter-stage filter:

  - Very high linearity is required from the first stage LNA, which will lead to increased cost, power consumption and thermal management issues.

  - Inter-stage filter does not help to protect the first LNA from direct ACLR from the aggressor. If the first stage LNA is saturated, the receiver would not work.

- High insertion loss placed after the LNA does not reduce LNA linearity requirements. Improved linearity LNAs are needed that add to cost and power consumption, which leads to thermal management issues. Other challenges include:

  - Significant insertion loss → NF increase that can hardly be accommodated in the 1 dB desensitization budget.

  - Considerable transition band and temperature dependency of the passband position → larger guard band between the sub-bands needed and less BW is usable for UL

  - Group delay distortion close to the cut-off frequencies

  - Incompatibility with a typical multi-carrier gNB design

  - Increased complexity as switches are needed for by-pass in UL slots for full BW

  - Additional space needed in RX chain that is not available in typical gNB design

  - Overall additional power consumption which leads to thermal management issues

  - Frequency drift over temperature that will impact filter insertion loss and rejection performance, hence impacts the RX lineup performance

- The new sub-band specific filters would be operator’s spectrum specific and locking the spectrum configuration for any further changes or tuning. The existing bandpass filters for the operating band would anyway be required, suggesting that the new filters would double the filter size for the UL antenna panel. This all means higher cost and complexity.

Besides, some preliminary analyses with simulations of filters with Q-values of 1500 and even up to 5000 have been presented by companies. In one of the analyses, it has been shown that the guard bands would become very wide or excessive losses would be obtained otherwise. Also, it is important to note that these simulations have not taken the manufacturing tolerances into consideration nor the temperature drift, which have an effect of paramount importance when implementing a working filtering solution. Last but not least, these filtering solutions would be too large to be suited for mMIMO BS, and they add large amount of cost to the BOM of a BS.

RX Beam nulling / isolation in RX sub-band

We assume 0 dB for RX beam nulling; as the SBFD feature is about enhancing uplink performance, we do not think the UL beamforming can be compromised further than the loss of channel reciprocity (due to separate TX and RX arrays) brings.

RX beamforming operates in the digital domain in a mMIMO system. The digital signal streams of the relevant receivers are combined using suitable amplitude and phase coefficients. Hence RX beam nulling will not relax the receiver dynamic range and linearity requirements.

Digital IC

The following techniques have been evaluated, but have significant challenges in FR1 wide area base station implementations:

- Digital cancellation. With knowledge of the TX signal, a properly scaled and phase shifted TX component may be subtracted from the RX signal to improve cancellation performance.

  - The TX signal may be available from observation receiver that is used in the DPD processing. For a mMIMO implementation, the DPD system may utilize only a few observation receivers, that sample the TX chains sequentially. It may be necessary to multiply the number of observation receivers to be able to sample each TX chain, leading to increased cost and energy consumption.

  - Each RX chain contains signals that are coupled from every TX chain. This means that the cancellation signal for each RX chain must be formed of every TX chains. The complexity can easily become extreme in a mMIMO implementation, with 32 or 64 TRXs. The complexity of the cancellation results in high energy consumption.

  - The cancellation may work with different performance for the DL signal fundamental components (i.e. the DL PRBs) than for the unwanted emission components (i.e. leakage on UL sub-band). The DL signal may be easier to cancel than the unwanted emissions. For the unwanted emissions, it is more efficient to cancel them at the TX DPD.

9.2.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

Based on the self-interference analysis provided in Section 9.2.1 for FR1 wide area BS, it can be observed that the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects including:

- Maximum BS transmit power

- Spatial isolation capability

- beam nulling/ isolation capability

- Blocker suppression at the RX

- Frequency isolation at the TX and RX

- The digital interference suppression/cancellation capabilities

[Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 6 companies’ technical inputs, 3 companies have come to the conclusion that 1dB receiver desensitization target is achievable by self-interference cancellation capability, while other 3 companies have come to the conclusion that 1dB receiver desensitization target is not achievable or is challenging based on technology roadmaps that are viewed by the 3 companies as viable in the current time or foreseeable future.]

9.2.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results.

9.2.2.1	Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the summary table which is based on co-channel inter-sub-band co-site inter-sector interference analysis framework.

RAN4 has carried out the study based on an analysis framework as provided in the following table to capture co-site inter-sector co-channel interference impact [49], which is used to capture inputs from companies.

Table 9.2.2.1-1: FR1 WA BS Co-site Inter-sector Co-channel Interference Analysis Summary

| FR1 |  |  |  | Samsung | Samsung | Ericsson | Huawei | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS (subband filter-1) | Wide Area BS (subband filter-2 and EM conjugated structure) | Wide Area BS | Wide Area BS | Wide Area BS |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm | 49 dBm | 53 dBm | 53 dBm | 54 dBm |  |
| Number of co-site co-channel sectors considered |  |  |  | 1 | 1 | 2 | 2 | 2 (plus 1 sector for self-interference) |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI−frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc |  |
|  |  | Frequency isolation techniques used |  | DPD utilized | DPD utilized | DPD, CFR | DPD | DPD, digital filtering |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI−spatial = ③ dBc |  | 75 dBc | 100 dBc | 75-90 dBc (Varies depending on scheduled beam directions) | 85 dBc | 60-80 dBc |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 75dB for typical spatial isolation | Based on 75dB for typical spatial isolation and additional 25dB by installing EM conjugated structure between sectors | Typical site layout with around 400mm between sectors | Spatial separation between TX panel with absorbing material and choke structure. | Spatial separation between TX/RX panel; cross polarization |  |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI−beam = ④ dBc |  |  | 10 dBc | 10 dBc | 0 dBc | 10 dB | 0 dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | Neglectable | Neglectable | 0 dB | Less than 0.5 dB loss | n.a |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -81dBm | -106dBm | -79 to -64 dBm (Varies depending on scheduled beam directions) | -74 dBm | -74…-54 dBm |  |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -51dBm (-36dBm and further suppressed by 15dB subband filter) | -72.8dBm (-61dBm and further suppressed by 11.8dB subband filter) | -34 to -19 dBm (Varies depending on scheduled beam directions) | -39 dBm | -23…-3 dBm (receiver will be blocked) |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI−frequency, RX= ⑥ dBc |  | 40 dBc | 40 dBc | 0 dBc | sub-band analog filter: 10 dB digital filter: 60-80 dB | 46 dBc |  |
|  |  | Frequency isolation techniques |  | Filtering | Filtering | None; see section 9.2.1.2.2 for analysis.. | sub-band analog filter put after LNA; digitla filter | Digital filtering, FFT frequency selectivity |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | With subband filtering, RX non-linearity impact is neglectable | With subband filtering, RX non-linearity impact is neglectable | -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS) | 0 dBm | Included in the NF model |  |
|  |  |  | Rx IM3 contribution (dBm) |  |  | -18 to +9 dBm (Minimum RAN4 receiver) -38 to -12dBm (Realistic for AAS) -62 to -35 dBm (optimistic for AAS) (Varies depending on scheduled beam directions) | -127 |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | N/A | (IM3 due to inter-sector interference only, assuming the presence of both self- and inter-sector interference in the receiver) | Negligible |  |  |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | -91dBm | -112.8dBm | -18 to +9 dBm (Minimum RAN4 receiver) -38 to -12dBm (Realistic for AAS) -62 to -35 dBm (optimistic for AAS) (Varies depending on scheduled beam directions) | Note: Each company to explain if/how they have separated CSSI and SI when considering IM3 | Inf dBm (receiver will be blocked above -25 dBm input level) |  |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI−beam = ⑨ dBc |  |  | 10 dB | 10 dB | 0 dB | 10 dB | 0 dB |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Neglectable | Neglectable | Receiver saturated; RX processing not feasible | Less than 0.5 dB loss | n.a. |  |
|  | Digital processing interference supression capability |  |  | 20dB | 20dB | No digital cancellation between sectors. RX Saturated | 12 | 0 dB |  |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | -100.6 dBm | -125.2 dBm | -18 to +9 dBm (Minimum RAN4 receiver) -38 to -12dBm (Realistic for AAS) -62 to -35 dBm (optimistic for AAS) (Additional interference due to inter-sector interference only) (Varies depending on scheduled beam directions) | -102 dBm/20 MHz | Inf dBc |  |
| Noise floor ⑩dBm |  |  |  | -96 dBm/CBW | -96 dBm/CBW | -96 dBm/CBW | -96 dBm/CBW | Inf dBm/CBW |  |
| Calculated Desensitization (dB) |  |  |  | 1.29 dB relative to normal RX REFSENS (1.05 dB relative to normal RX REFSENS) | Neglectable | Receiver saturated (>> 30dB) | 1 dB | Inf dB |  |
| SBFD configuration |  |  |  | DUD(40-20-40MHz) | DUD(40-20-40MHz) | 40-20-40 MHz | DUD [40, 20, 40] | DUD (40-20-40 MHz) |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB | 5 PRB | 5 PRB. | Existing SU | 5 PRB |  |
| bandwidth over which suppression is achieved |  |  |  | 20MHz | 20MHz | >300 MHz | Several hundred MHz |  |  |
| Others |  |  |  | subband filtering (20MHz passband, 2* 5PRB transition band used for roll-off between passband/stopband) | subband filtering (24.8MHz passband, 2*3.8MHz transition band used for roll-off between passband/stopband) |  |  |  |  |

9.2.2.2	Feasibility study on co-channel inter-sub-band co-site inter-sector interference

Editor's note: This section captures the feasibility study on co-channel inter-sub-band co-site inter-sector interference based on individual companies’ analysis.

9.2.2.2.1	Samsung

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 9.2.2.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

The achievable antenna isolation is key factor to analyze the co-site inter-sector co-channel gNB-gNB CLI. For the below interested scenario, antenna isolation (with the achievable coupling loss) is to be evaluated:

- 3 sector scenario is under consideration:

  - The angle between every two sectors’ boresight directions is 120 degrees;

  - Sector antenna panel’s width is 180mm;

  - Between two sectors’ antenna panel:

    - The center-to-center distance is: 150mm;

    - The nearest distance between edge to edge is: 60mm;

  - Three antenna elements are used to form the antenna port.

  - 3.5GHz operating frequency with 100MHz bandwidth.

The above simulation scenario can be illustrated in Figure 9.2.2.2.1-1. In the right part of below figure, the top view is presented with the concerned panels of sector 1 and sector 2.

Figure 9.2.2.2.1-1: (Left figure) 3-sector scenario for co-channel co-site inter-sector antenna isolation study; 
(Right figure) top view for the 2-sector scenario.

Accordingly, we have performed HFSS-based RF simulation for the above 3-sector scenario, by evaluating the isolation from sector 2 to sector 1. Specifically, S-parameters between two antenna ports from two sectors are evaluated, by considering each pair of antenna ports with co/cross-polarization relationships, which is illustrated in Figure 9.2.2.2.1-2. The RF evaluation results have been provided in the Table 9.2.2.2.1-1.

Figure 9.2.2.2.1-2: Illustration of S-parameters for antenna port pair.

Table 9.2.2.2.1-1: S-parameter evaluation results.

Based on the numerical results, the variance of spatial isolation for different antenna port pairs and different co-/cross-polarization relationships can be demonstrated. Moreover, the edge effect (the wave traversing the surface of antenna panel is condensed and reflected arbitrary at the edge of the antenna panel or any obstacles) further complicates the results.

By comparing the same pair of antenna ports but with co-polarization and cross-polarization, it is hard to have a simple observation for which one is higher, but different observations depend on the designated antenna pair. The results could be explainable by the +45degree and -45degree placement for two polarizations. Within a panel, the co-pol and cross-pol can be guaranteed, while 3-sector case may make the alignment disappear.

The RF simulation has shown the antennal isolation for co-channel co-site inter-sector gNB-gNB CLI is in the range of 62-93dB, depending on different antenna pair and co/cross-polarization, and 75dB can be regarded as typical value as RAN4 agreement.

It is worth noting that the above spatial isolation values (typical value for 75dB) based on HFSS simulation have not yet reflected EM conjugated structure as used in the testbed for two panels within a sector. In the testbed to evaluate self-interference within a sector, the EM conjugated structure can improve around 20~30dB additionally. It is anticipated that the similar improvement if the EM conjugated structure is installed between two-sector antennas. Hence, with the EM conjugated structure, it is expected that the achievable antenna isolation shall be improved by around 25dB.

9.2.2.2.2	Ericsson

The input provided in Table 9.2.2.1-1 presents an analysis of the inter-sector interference effects for wide area FR1 BS. It should be noted that, as demonstrated in section 9.2.1.2.2, the receiver is already driven into saturation due to self-interference. In addition to the self-interference, the inter-sector interference in the TX sub-bands is also very high power and would drive the receiver into saturation.

Inter-sector isolation

Due to capacity optimization and site costs, outdoor sites will typically host several sectors, as well as potentially co-located base stations. Some examples of different types of deployment are depicted in figure 9.2.2.2-1. Site space constraints (considering zoning, rental, weight, wind-load and other factors) typically mean that the potential to increase distance between sectors or to mount additional structures between sectors and base stations can be very limited.

To avoid direct interference to the SBFD receive resources, all sectors using the same carrier must apply SBFD simultaneously in the same slots. If this is the case, then the SBFD receiver will still experience significant power from the TX sub-band of the other sectors, and from other base stations. The isolation between the TX sub-band of other sectors and the RX panel can potentially be less than the TX sub-band within the own base station since the possibilities for building an efficient isolating structure between sectors is less than within a BS.

Figure 9.2.2.2.2-1: Examples of outdoor BS deployments

Isolation between sectors has been simulated using electromagnetic simulations in R4-2301885 with an assumption of 400mm sector separation. The isolation varies to some degree with separation, but not to an extent that would change the overall results. For most practical site deployments, addition of materials between sectors is not likely to be feasible (and may reduce network performance). The simulation set-up is depicted in figure 9.2.2.2.2-2. In additional to a horizontal separation, a height separation between TX and RX panels is assumed.

| (a) | (b) |
| --- | --- |

Figure 9.2.2.2.2-2: EM simulation setup for 3-sector site

Figure 9.2.2.2.2-3 depicts the EM simulation results. The left hand plot shows the isolation with azimuth steering and elevation in boresight and the right hand plot with elevation steering and azimuth on boresight. The insolation between sectors is highly dependent on the beam direction. Although an “average” isolation can be given, this would mask the fact that for certain beam directions isolation is good and for others it is not good. Since the beam direction depends on the physical positions of users, advanced co-ordination of beam directions may not be possible if other constraints such as capacity and latency are to be optimized. Even with an optimization, the isolation would be less than 80dB.

| - Azimuth beam steering with elevation at boresight | - Downtilt elevation beam steering with azimuth at boresight |
| --- | --- |

Figure 9.2.2.2.2-3: Inter-sector isolation (between two sectors) results from EM modelling.

It should also be considered that there are likely to be two interfering sectors, as well as potentially other co-located BS (for example, from other operators).

Beam nulling

There may be some potential for beam nulling to mitigate interference between sectors. However, it is not sufficient to avoid that the power into the receiver drives the receiver into saturation for the wide area scenario.

Receiver filtering

Analogue filtering in the receiver is not assumed for reasons described in section 9.2.1.2.2.

9.2.2.2.3	Huawei

On digital IC aspect, in our view, since the information of non-linear product is already got in the digital domain, and it can be exchanged between sectors within a BBU, hence digital IC can be applied for this case.

On the achievable coupling loss in the case of co-site inter-sector gNB-gNB, RF measurements for the spatial isolation has been conducted. Figure 9.2.2.2.3-2 shows measurements results for two 120° sectors which is 2.5 m distance in horizontal. The curve represents the spatial isolation between a TX full power transmission of one sector to one receiver chain of the other sector. Form the measurements it can be found that the larger angle beam steering, the isolation becomes smaller. The isolation at worst case is ~76 dB. It is a test on legacy AAS BS. There are some methods can be adopted to improve the isolation for a BS capable of SBFD operation, e.g. mounting EM absorber materials next to the antenna array in the base station. ~10 dB improvement is foreseen based on our evaluations. For co-site inter-sector case better spatial isolation than RSI case is achievable.

Figure 9.2.2.2.3-1: spatial isolation measurements

Figure 9.2.2.2.3-2: measurements results at different beam directions

9.2.2.2.4	Nokia

The co-site inter-sector interference analysis of values for FR1 Wide Area BS is provided in the summary table.

Frequency isolation techniques

The spatial isolation mechanisms for co-site inter-sector case are in principle similar to the self-interference case. In addition, these aspects need to be considered:

- Transmit beam nulling across different sectors is theoretically possible, but the practical implementation may be too costly since it may be necessary to calculate the beamforming vector for each subcarrier.

Spatial isolation techniques

The spatial isolation mechanisms for co-site inter-sector case are in principle similar to the self-interference case. In addition, these aspects need to be considered:

- Element-to-element isolation is easier to manage within a single antenna enclosure, where all parameters and physical dimensions can be controlled. Isolation between sectors occurs due to unwanted radiation towards the back of the antenna, which is more difficult to control. The geometry between the antennas of different sectors can be difficult to adjust precisely, meaning that the element coupling can be difficult to predict.

- It has been suggested that EM shielding material between sectors may be used. This is not possible in all installations but may possibly be an option in some installations. The effectiveness of the EM shielding has not been studied.

9.2.2.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on co-channel inter-sub-band co-site inter-sector interference based on RAN4 agreement.

Based on the analysis on co-site inter-sector co-channel inter-subband interference provided in Section 9.2.2 for FR1 wide area BS, it can be observed that the implementation feasibility of controlling the co-site inter-sector co-channel inter-subband interference to meet the target (i.e., being less than certain level of receiver desensitization) depends on the implementation aspects including:

- Maximum BS transmit power

- Number of co-site, co-channel sectors and the separation between them and other site constraints

- The achievable spatial isolation and use of absorbing material and choke structure depending on site constraints

- Beam nulling/isolation capability

- Tx frequency isolation

- Frequency isolation at the RX and the implementation of subband filtering

- The digital interference suppression/cancellation capability.

[Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 4 companies’ technical inputs, 2 companies have come to the conclusion that the implementations can achieve reasonable residual level for co-site inter-sector co-channel inter-subband interference, up to 1dB desensitization, while other 2 companies have come to the conclusion that the implementations are not able to achieve that because the receiver is saturated, and the RX processing is not feasible, based on co-site deployment limitation and technology roadmaps that are viewed by the 2 companies as viable in the current time or foreseeable future.]

9.2.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

On the feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity, RAN4 agree that

- The same transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation.

  - TX leakage baseline: gNB ACLR

  - Receiver impairment can be studied with gNB ACS as baseline for system level simulation and feasibility study, and further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

9.2.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

Based on RAN4 feasibility study on FR1 wide area BS, specifically the analysis on self-interference, co-site inter-sector co-channel inter-subband interference and inter-site inter-subband interference, RAN4 concluded that:

- For self-interference analysis, the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects mentioned in clause 9.2.1. [Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 6 companies’ technical inputs, 3 companies have come to the conclusion that 1dB receiver desensitization target is achievable by self-interference cancellation capability, while other 3 companies have come to the conclusion that 1dB receiver desensitization target is not achievable or is challenging based on technology roadmaps that are viewed by the 3 companies as viable in the current time or foreseeable future.]

- For co-site inter-sector co-channel inter-subband interference, that the implementation feasibility of controlling the co-site inter-sector co-channel inter-subband interference to meet the target (i.e., being less than certain level of receiver desensitization) depends on the implementation aspects mentioned in clause 9.2.2. [Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 4 companies’ technical inputs, 2 companies have come to the conclusion that the implementations can achieve reasonable residual level for co-site inter-sector co-channel inter-subband interference, up to 1dB desensitization, while other 2 companies have come to the conclusion that the implementations are not able to achieve that because the receiver is saturated, and the RX processing is not feasible, based on co-site deployment limitation and technology roadmaps that are viewed by the 2 companies as viable in the current time or foreseeable future.]

- For inter-site co-channel inter-subband interference, since the feasibility is deployment-dependent, RAN4 has provided the inter-site gNB-gNB CLI modelling used for coexistence study by considering unwanted emission and receiver selectivity modelling.

## 9.3	Feasibility of FR1 medium range BS aspects

### 9.3.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results.

#### 9.3.1.1	Summary table for self-interference analysis

Table 9.3.1.1-1 summarizes the analysis from different companies on the FR1 MR base station. This section is based on the self-interference analysis framework.

Table 9.3.1.1-1: Summary of FR1 medium range BS self-interference analysis

| FR1 |  |  |  |  | Nokia | Ericsson |  |  |  | ZTE | Huawei | Samsung |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  |  | Medium Range BS | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) | MR | Medium Range BS | Medium Range BS |
| BS TX Power dBPTX = ① dBm |  |  |  |  | 36 dBm | 38 dBm | 38 dBm | 38 dBm | 35 dBm | 30 | 38 dBm | 38 dBm |
| Component capability and parameters | Frequency isolation at TX |  | Frequency isolation capability dBαSI−frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 51 | 45 | 45 dBc |
|  |  |  | Frequency isolation techniques used |  | Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion | Digital filtering, CFR, DPD |  |  |  | DPD utilized | DPD | DPD utilized |
|  | Spatial isolation |  | Spatial isolation capability dBαSI−spatial = ③ dBc |  | 60 dBc (if omnidirectional antennas are used, this would be less) | 65-70 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc | 60 | 80 | 80 dBc |
|  |  |  | Spatial isolation techniques used |  | Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels | A combination of spatial isolation, chokes, absorption, mushroom EBG. 70dB is indicative average; isolation varies from around 55dB to more than 80dB |  |  |  | TX/RX panel separation | spatial separation between TX/RX panel | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI−beam = ④ dBc |  |  |  | 0-5 dBc | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |  | 0 | 5 | 5 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  | 1 dB maximum | Up to 5dB EIRP loss, depending on beam direction |  |  |  | N/A | Limited | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  | -75 dBm/20 MHz | -87 dBm | -87 dBm | - 87 dBm | -90 dBm | -101 | -98 | -92 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) |  | RF IC capability and other tech. in TX sub-band dBαSI−RFIC = ⑤ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 20 | N/A | 0 dBc |
|  |  |  | RF IC capability and other tech. in RX sub-band dBαSI−RFIC = ⑧ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 | N/A | 0 dBc |
|  |  |  | RF IC techniques and other tech. (before LNA) |  | None apply due to feasibility concerns (depends on the number of TRX) | None; see section 9.3.1.2.2 for analysis. |  |  |  | Subband filtering | N/A | N/A |
|  |  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A dBc | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |  |  | N/A | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  |  | -24 dBm to -29 dBm depending on TX beam | -42 dBm | -42 dBm | -42 dBm | -45 dBm | -50 | -47 | -47 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX |  | Frequency isolation capability dBαSI−frequency, RX= ⑥ dBc |  | 0 dBc | 0 dBc |  |  |  | 60 | digital filtering | 40 dBc |
|  |  |  | Frequency isolation techniques |  | None apply due to feasibility concerns | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. Digital baseband combining may improve self-interference suppression, at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |  | Digital Filtering | digital filtering | Filtering |
|  |  |  | RX IMD | Rx IIP3 capability (dBm) | -10 dBm at maximum sensitivity; +10 dBm at maximum linearity (at NF penalty) | -27.6 dBm | -17.6 dBm | -13 dBm | -17.6 dBm | -5 | -5 | -20 |
|  |  |  |  | Rx IM3 contribution (dBm) | Negligible | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm | -140 | -131 | -101dBm |
|  |  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Negligible | No significant issues for medium range BS power level other than mentioned above |  |  |  | Marginal | negligible | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  |  | -74 to -79 dBm/20MHz (at 50 dBc ACS) | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm | -110 | negligible | -87 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI−beam = ⑨ dBc |  |  |  | 0 dBc | 0-10 dBc RX beam nulling is in effect part of the digital baseband combining. See “frequency isolation techniques” |  |  |  | 0 | 5 | 5 dBc |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  | 0 dBc; should not assume further UL beamforming loss to maintain any UL gains |  |  |  |  | N/A | Limited | Limited, ~0dB |
|  | Digital IC dBαSI−digtial = ⑦ dBc |  |  |  | 5-10 dBc (Improved linearization could provide here additional 5dB. Digital IC depends on the implementation) | 10-15 dBc (Transmitter) |  |  |  | 20 | 10 | 10 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  |  | 110 dBc to 120 dBc | 109 dBc | 128-138 dBc | 135 dBc | 134 dBc | 130.5 | 146 | 134 dBc |
| Noise floor ⑩dBm |  |  |  |  | -92 dBm/CBW (20 MHz) | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -91.0 | -90 dBm/20 MHz | -90 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  | -98 dBm | -96 dBm | -96 dBm | -96 dBm | -96 dBm | -97.0 | -96 dBm | -96dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  |  | 134 dBc | 134 dBc | 134 dBc | 134 dBc | 131 dBc | 127.0 | 134 | 134 dBc |
| SBFD configuration |  |  |  |  | DUD (40/20/40 MHz) | 40-20-40 |  |  |  |  | 40-20-40 | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  |  | 5 RB (1.8 MHz) | 5 PRB |  |  |  |  | Existing SU | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  |  |  | <300MHz |  |  |  |  |  | 100MHz |
| Others |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.2	Feasibility study on self-interference

Editor’s note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

##### 9.3.1.2.1	Nokia

Table 9.3.1.1-1 presents the company’s view on the self-interference mitigation analysis for a medium range base station with 36 dBm total output power. The RSIC capability corresponds to 110 to 120 dBc which is achieved using a combination of spatial isolation (60 dBc), Tx beam nulling (0-5 dB, only applicable in the Tx subband), frequency separation (45 dBc), and digital IC techniques (5-10 dBc). Under such considerations, the self-interference observed in the 20 MHz UL subband is dominated by transmitter ACLR and receiver selectivity with a magnitude of approximately -74 dBm prior to digital IC and up to -84 dBm after digital IC, while the noise floor is -91.0 dBm. This results in a non-negligible desensitization of the receiver which would degrade more or less the system performance depending on the deployment ISD, UL link budget, etc.

The assumptions and corresponding justification are presented below with focus on the main differences with respect to the wide-area analysis in Section 9.2.1.2.6.

Spatial isolation

The same techniques to achieve spatial isolation in FR1 wide-area base stations can be applied to medium-range base stations as well. Nevertheless, the following differences should be noted:

- Medium range BS have typically smaller form-factor than the wide-area BS, e.g. down to 30x30x10 cm. The absolute physical separation between Tx and Rx panels needs to be smaller than for wide-area BS to keep the relative increase of the BS enclosure to a reasonable level.

- Medium range base stations may have a lower number of TRXs as compared to wide-area base stations. This could result in higher coupling per Rx chain.

Considering these two aspects, 60 dBc of spatial isolation is considered.

TX Beam nulling / isolation in TX sub-band

With only a relatively low number of TRXs, the potential of Tx beam nulling techniques is reduced. Only 0-5 dBc is considered, under the assumption of at most 1 dB of EIRP loss.

RF IC and other tech. (before LNA)

If the number of TRX is significantly lowered compared to massive MIMO, there is a possibility for RF IC to be feasible. However, this all depends on the implementation. With a low number of TRXs, the spatial isolation to a single RX port would likely be lower than for an implementation with a large number of TRXs – possibility necessitating RF cancellation paths to compensate. The overall SIC capability would be similar in both cases.

Frequency isolation at RX

This all depends on the implementation.

Digital IC

If the number of TRX is significantly lowered compared to massive MIMO, there is a possibility for digital IC to be feasible. However, this all depends on the implementation. Digital IC can be achieved by improving the DPD in the transmitter side to reduce the unwanted transmitter leakage components that fall in the UL subband, while receiver-side digital IC techniques are also required to supress the self-interference signal in gNB RX subband caused by non-ideal RX selectivity.

##### 9.3.1.2.2	Ericsson

For the medium range BS, four implementation scenarios are presented in Table 9.3.1.1-1. In the first scenario, the RF is assumed to perform at the absolute minimum needed to meet the 3GPP requirements. In the second scenario, an improved RF performance that is still likely to be a reasonable commercial implementation is considered (called realistic). Two further scenarios are considered; one in which the receiver linearity is assumed to exceed the realistic scenario, which could lead to more significant compromises in power consumption, size etc. The other scenario is one in which realistic RF performance is assumed, but the transmitter is assumed to be 3dB lower than the maximum transmitter limit in power.

It can be observed that for a BS only built to meet 3GPP minimum requirements, the receiver performance is not sufficient to operate SBFD without significant desensitization. To operate SBFD, either receiver digital processing is needed with the realistic assumptions (the feasibility of achieving sufficient gain with such processing depends on the wider deployment scenario), or somewhat better receiver performance, or lower transmit power than the 3GPP maximum limit.

An explanation for the assumptions in the table are provided below.

Spatial isolation and beam nulling

For a medium range size AAS array, simulations show a spatial isolation of around 65-70dB, depending on the beam direction. With beam nulling, the isolation can be lifted to around 80dB, with in general less than 1dB cost in the downlink.

Analogue interference cancellation

With a smaller array size, analogue interference cancelation may be more feasible for a MR BS than for a WA BS. However, the number of interconnections is still significant. Furthermore, analogue IC requires that the same beam steering is applied on all RBs and all carriers, preventing sub-band precoding and multi-carrier operation. Also, in general analogue IC is not needed to avoid receiver saturation. Analogue IC has not been taken into account.

Analogue filtering

Analogue filtering is not realistic in an implementation for the same reasons described for the WA BS in section 9.2.1.2.2. Furthermore, analogue filtering is not really needed as the input power level to the MR receiver is generally manageable.

Digital interference cancellation and receiver processing

Digital sampling of the TX leakage interference and subtraction at RX is more feasible for a MR BS than for a WA BS. An alternative to digital cancellation of TX leakage could be an improved PA linearization at the transmitter side. The possibility of either improved PA linearization or digital IC is captured as 15dB digital IC.

At the receiver, the interference co-variance matrix can be estimated and the receiver MMSE-IRC algorithm can mitigate interference. The extent to which the receiver can mitigate interference depends on the overall interference structure, which depends on the profile of interfering UEs, other sectors and other base stations as well as the fading channel profiles in the environment. The study has not considered the deployment environment when considering feasibility of self-interference suppression, and hence a specific number for the suppression by means of receiver processing is not provided. However, it is noted that for the MR to operate with the “realistic” receiver and the maximum allowed transmit power, several dB of suppression would need to be achieved by digital processing.

##### 9.3.1.2.3	ZTE

Table 9.3.1.1-1 presents the company’s view on the self-interference mitigation analysis for a medium range base station.

##### 9.3.1.2.4	Samsung

Table 9.3.1.1-1 presents the company’s view on the self-interference mitigation analysis for a medium range base station.

9.3.1.3	 	Conclusion

Based on the self-interference analysis provided in Section 9.3.1 for FR1 medium range BS, it can be observed that the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects, including:

- Maximum BS transmit power

- Spatial isolation capability

- Beam nulling/ isolation capability

- Blocker suppression at the RX

- Frequency isolation at the TX and RX

- The digital interference suppression/ cancellation capabilities

Based on the different assumptions and/ or techniques adopted for the above-mentioned implementation aspects, based on 5 companies’ technical inputs, 4 companies have concluded that it is feasible to achieve the 1 dB desensitization target. In addition, 2 companies have shown that it is not feasible to achieve the 1 dB desensitization target for a base station that strictly meets the RAN4 minimum requirements and/or operates with the maximum BS transmit power.

### 9.3.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results

#### 9.3.2.1	Summary table for co-channel inter-subband co-site inter-sector interference  analysis

Table 9.3.2.1-1 summarizes the analysis on the co-channel inter-subband co-site inter-sector interference for FR1 MR base station. This section is based on the co-channel inter-subband co-site interference analysis framework.

Table 9.3.2.1-1: Summary of FR1 medium range BS co-channel inter-subband co-site inter-sector interference analysis

| FR1 |  |  |  | Ericsson |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Medium Range BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 dBm |
| Number of co-site co-channel sectors considered |  |  |  | 2 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI−frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD, CFR |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI−spatial = ③ dBc |  | 75-90 dBc (Varies depending on scheduled beam directions) |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Typical site layout with around 400mm between sectors |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI−beam = ④ dBc |  |  | 0 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | 0 dB |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -94 to -79 dBm (Varies depending on scheduled beam directions) |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -49 to -34 dBm (Varies depending on scheduled beam directions) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI−frequency, RX= ⑥ dBc |  | 0 dBc |
|  |  | Frequency isolation techniques |  | Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -13 dBm (“Optimistic” value) |
|  |  |  | Rx IM3 contribution (dBm) | -101 dBm to -74 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | (IM3 due to inter-sector interference only, assuming the presence of both self- and inter-sector interference in the receiver) |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | -101 dBm to -74 dBm (Additional interference due to inter-sector interference only) (Depending on beam direction) |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI−beam = ⑨ dBc |  |  | 0 dB |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | 0 dB |
|  | Digital processing interference supression capability |  |  | No digital cancellation between sectors. |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | -94 To -74 dBm (Additional interference due to inter-sector interference only) (Depending on beam direction) |
| Noise floor ⑩dBm |  |  |  | -96 dBm/CBW |
| Calculated Desensitization (dB) |  |  |  | 4 to 22 dB (Depending on beam direction. Additional degradation due to inter-sector interference only) |
| SBFD configuration |  |  |  | 40-20-40 MHz |
| Guardband assumption (if exist) |  |  |  | 5 PRB. |
| bandwidth over which suppression is achieved |  |  |  | >300 MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |

#### 9.3.2.2	Feasibility study on co-channel inter-subband co-site inter-sector interference  analysis

##### 9.3.2.2.1	Nokia

Medium range base stations are commonly deployed with omnidirectional antennas, but some deployments use directional antennas also with the target to boost coverage or capacity. In case of 3-sector site deployment with directional antennas, the techniques described in Section 9.2.2.2.4 apply here as well, although it is important to note that it may be more difficult to add large horizontal separation between sectors if the site footprint is small.

##### 9.3.2.2.2	Ericsson

Table 9.3.2.1-1 presents an analysis of the inter-sector interference effects for medium range FR1 BS. The table considers the “optimistic” receiver considered in section 9.3.2.1.2; i.e., a receiver that is more capable than in a current BS. If the “realistic” receiver would be considered, then the indicated desensitizations would be greater.

The level of inter-sector interference only varies depending on the beam direction from around 4dB to 22dB. The total desensitization would include self-interference and would be around 1dB greater. There may be some potential to increase inter-sector isolation using TX beam nulling (not shown in the table), however clearly mitigating inter-sector interference will be a challenge also for MR BS.

Inter-sector isolation

Isolation between sectors has been simulated using electromagnetic simulations in R4-2301885 with an assumption of 400mm sector separation. The isolation varies to some degree with separation, but not to an extent that would change the overall results. For most practical site deployments, addition of materials between sectors is not likely to be feasible (and may reduce network performance).

The insolation between sectors is highly dependent on the beam direction. Although an “average” isolation can be given, this would mask the fact that for certain beam directions isolation is good and for others it is not good.

Beam nulling

There may be some potential for beam nulling to mitigate interference between sectors. To achieve complete isolation between sectors, more than 20dB beam nulling would be needed.

Receiver filtering

Analogue filtering in the receiver is not assumed for reasons described in section 9.3.2.1.2.

#### 9.3.2.3 Conclusion

### 9.3.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

On the feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity, RAN4 agree that

- The same transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation.

  - TX leakage baseline: gNB ACLR

  - Receiver impairment can be studied with gNB ACS as baseline for system level simulation and feasibility study, and further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

### 9.3.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

9.4	Feasibility of FR1 local area BS aspects

9.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

#### 9.4.1.1	Summary table for self-interference analysis

Table 9.4.1.1-1 summarizes the self-interference analysis results from companies.

Table 9.4.1.1-1: SI feasibility study for FR1 Local Area BS

| FR1 |  |  |  | Ericsson (preliminary) |  | ZTE | CATT | Huawei | Nokia | Samsung |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Local Area BS (3GPP minimum) | Local Area BS (Realistic RX) | LA | LA | LA | Local Area BS | Local Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 24 | 24 | 23 | 24 | 24 | 23 | 24 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI−frequency, TX = ② dBc |  | 45 | 45 | 45 | 45 | 45 | 45 | 45 |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  | DPD utilized | CFR | DPD | Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion | DPD utilized |
|  | Spatial isolation | Spatial isolation capability dBαSI−spatial = ③ dBc |  | 70 | 70 | 60 | 70 | 70 | 60 (if omnidirectional antennas are used, this would be less) | 80 |
|  |  | Spatial isolation techniques used |  | Physical distance, isolation structures |  | TX/RX panel separation | TX/RX panel separation, isolation structures, isolation material, cross polarization | Spatial separation between TX panel to single RX | Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI−beam = ④ dBc |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | TX beam nulling not assumed due to array size |  | N/A | TX Beam nulling is not assumed due to the array size | N/A | TX Beam nulling is not assumed due to the array size | N/A |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 | -91 | -101 | -104.01 | -97 | -88 (①-②-③-④- 10*log10(80/20)) | -101 Note: provided by ①-②-③-⑨ |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI−RFIC = ⑤ dBc |  | 0 | 0 | 0 | 0 | N/A | 0 | 0 |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI−RFIC = ⑧ dBc |  | 0 | 0 | 0 | 0 | N/A | 0 | 0 |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed. |  | subband filtering | 0 | N/A | 0 | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 | 0 | 0 | 0 | N/A | 0 | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -46 | -46 | -50 | -46 | -46 | -37 | -56 Note: provided by ①-③-④-⑤ |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI−frequency, RX= ⑥ dBc |  | xxx dBc |  | 60 | 55 | 60-80 |  | 40 |
|  |  | Frequency isolation techniques |  | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. |  | Digital Filtering | Digital filter for ACS | Digital filtering |  | Digital filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -24.6 | -10 | -5 | -16 | -5 | -10 | -20 |
|  |  |  | Rx IM3 contribution (dBm) | -88.8 | Negligible | -140 | -106 | -128 | Negligible | -128 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No other significant impacts other than those mentioned above |  | Marginal | No other significant impacts | negligible | No other significant impacts | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) (dBm) |  |  | -88.8 | -110 | -110 | -101 | negligible | -87 (at 50 dB ACS) | -96 Note: provided by ①-③-④-⑤-⑥ |
|  | RX Beam nulling /isolation in RX sub-band dBαSI−beam = ⑨ dBc |  |  | 0-3 dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  | 0 | 0 | N/A | 0 | 0 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  | N/A |  | N/A |  | N/A |
|  | Digital IC dBαSI−digtial = ⑦ dBc |  |  | 15 dBc on transmitter |  | 20 | 10 | N/A | 5 to 10 | 10 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 112 dBc | 128 dBc | 130.5 | 122.33 | 121 | 110 to 115 | 129 |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW | -87 dBm/CBW | -91.0 | -87.99 | -87 dBm/20 MHz | -91.0 | -87 |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm | -93 dBm | -97.0 | -93.99 | -93 dBm | -97.00 | -93 |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117 dBc | 117 dBc | 127.0 | 117.99 | 117 | 120.0 | 117 |
| SBFD configuration |  |  |  | 40-20-40 |  | 40-20-40 | 40-20-40 | 40-20-40 | 40-20-40 | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  | 5 PRB |  |  |  | Existing SU | 5 PRB | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |  |  |  |  |  | 20MHz |
| Others |  |  |  | The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed). |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |  |  |  |  |

#### 9.4.1.2	Feasibility study on self-interference

##### 9.4.1.2.1 Ericsson

The self-interference suppression expectations for the LA BS class are provided in table 9.4.1.1-1. Two examples are provided. The first is an LA BS with RF performance just enough to meet 3GPP minimum requirements. The second is an LS BS with improved receiver performance, but still likely sufficient for a commercially viable solution.

The table suggests that, depending on variation in the achievable spatial isolation, some degree of improved receiver and/or digital cancellation may be needed for LA SBFD. However, in general self-interference suppression appears feasible.

Spatial isolation and beam nulling

The spatial isolation for LA is more difficult to quantify, as the array size and form-factor for a LA BS may vary significantly. LA SBFD could even be operated by placing TX and RX LA base stations at some distance. 70dB isolation has been assumed, and obviously the conclusions may vary if a smaller isolation is achieved. Beam nulling is not assumed, since a small array size is assumed.

Analogue interference cancellation

Analogue interference cancellation is more feasible for a LA BS with a smaller array size. The drawback would be the need to beamform on the same way on all RB and all carriers for a LA BS. Potentially analogue IC could be considered if the isolation for a particular design would be substantially less than 70dB.

Analogue filtering

Analogue filtering is not really needed for a LA BS due to the lower power in the TX sub-bands.

Digital interference cancellation and receiver processing

Digital sampling of the TX leakage interference and subtraction at RX is feasible for a LA BS. An alternative to digital cancellation of TX leakage could be an improved PA linearization at the transmitter side. The possibility of either improved PA linearization or digital IC is captured as 15dB digital IC.

Receiver combining could also potentially mitigate some interference although with a smaller array size, the degrees of freedom with which to do so would be lower than other BS classes.

##### 9.4.1.2.2 CATT

In SI capability analysis, the following techniques are used,

- CFR is used to improve equipment efficiency, DPD is used for high power equipment to optimize ACLR.

- Tx antennas panel and Rx antennas panel are separate, there are some isolation material between them, and cross polarization is also used.

- Digital filter is used to resolve the adjacent sub-band (i.e. TX subband) interference issue.

- Digital IC is used to reduce interference in the UL sub-bands.

From the analysis in Table 9.4.1.1-1, the following capabilities are needed,

- Frequency isolation capability of frequency isolation at TX is 45dB

- Spatial isolation is 70dB

- ACS is 55dBc

- IIP3 is -16dBm

- Digital IC is 10dB

The above capabilities except frequency isolation are improved compared with legacy LA BS, but it’s feasible from implementation point of view.

##### 9.4.1.2.3 Nokia

The Nokia input in the Table 9.4.1.1-1 presents the company’s view on the self-interference mitigation analysis for a local area base station with 24 dBm total output power. The RSIC capability corresponds to 110 to 115 dBc for the Rx which is achieved using a combination of spatial isolation (60 dBc), frequency separation (45 dBc), and digital IC techniques (5-10 dBc). Under such considerations, the self-interference observed in the 20 MHz UL subband is dominated by transmitter ACLR and receiver selectivity with a combined magnitude of approximately -85 dBm prior to digital IC and -90 dBm to -95 dBm after digital IC, while the noise floor is -91.0 dBm. Even though there is a small desensitization of the receiver, it is feasible to operate the base station with acceptable performance as the coverage is not the primary target of local area deployments where the UL link budget and received UL power is generally very high.

A detailed description of some of the key assumptions is provided below.

Spatial isolation and beam nulling

Approximately 60 dB of spatial isolation can be achieved for ceiling- or wall-mounted base stations with directive antenna radiation patterns although at the expense of larger form-factor to reach sufficient physical separation between Tx and Rx. For typical LA base stations with omni-directional antennas, the spatial isolation would be significantly lower, e.g., 30 dB or less, thus SBFD operation would be much more challenging or potentially unfeasible.

Tx beam nulling is not assumed due to relatively low number of TRXs.

Digital Interference cancellation techniques

Since the number of TRX is significantly lowered compared to mMIMO, there is a possibility for digital IC to be feasible. Digital IC can be achieved by improving the DPD in the transmitter side to reduce the unwanted transmitter leakage components that fall in the UL subband, while receiver-side digital IC techniques are also required to supress the self-interference signal in gNB RX subband caused by non-ideal RX selectivity.

##### 9.4.1.2.4 ZTE

Table 9.4.1.1-1 presents the company’s view on the self-interference mitigation analysis for a local area base station.

#### 9.4.1.3	Conclusion

Based on the feasibility study on self-interference in Section 9.4.1.2 and the summary table provided in Section 9.4.1.1, it can be concluded that with the proper implementation of component techniques including spatial isolation, frequency isolation, beam nulling, digital IC or a combination of these, the SBFD residual self-interference for FR1 LA BS can be controlled to the level of 6dB below the noise floor, which results in 1dB sensitivity degradation.

9.4.2	Co-channel inter-sub-band inter-site interference analysis

This section captures the typical assumption of RF requirements and analysis results

On the feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity, RAN4 agree that

- The same transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation.

  - TX leakage baseline: gNB ACLR

  - Receiver impairment can be studied with gNB ACS as baseline for system level simulation and feasibility study, and further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

9.4.3	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

For self-interference of FR1 LA BS, it can be concluded that the SBFD self-interference 1dB sensitivity degradation is achievable.  It is assumed that LA scenarios are not sectorized deployments.

For inter-site co-channel inter-subband interference, since the feasibility is deployment-dependent, RAN4 has provided the inter-site gNB-gNB CLI modelling used for coexistence study by considering unwanted emission and receiver selectivity modelling.

In summary, from implementation point of view, it is feasible for FR1 LA SBFD BS.

9.5	Feasibility of FR2-1 BS aspects

### 9.5.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability and analysis results.

#### 9.5.1.1	Summary table for self-interference analysis

The self-interference analysis from companies’ inputs for FR2 SBFD-capable gNB are summarized in Table 9.5.1.1-1. Both self-interference leakage in gNB RX sub-band due to non-ideal TX and Self-Interference signal in gNB RX sub-band caused by non-ideal RX selectivity are studied in the analysis framework. The self-interference cancellation techniques including spatial isolation, frequency isolation, beam nulling and digital are considered.

Table 9.5.1.1-1: self-interference analysis

| FR2-1 |  |  |  | Samsung | Huawei | Qualcomm | Ericsson |  |  | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS | FR2-1 BS | FR2-1 BS | FR2-1 BS |  |  | FR2-1 BS (preliminary) |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm | 35 dBm | 30 dBm | 40 dBm | 35 dBm | 30 dBm | 37 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI−frequency, TX = ② dBc |  | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD | DPD | DPD | Digital Filtering, CFR |  |  | Without DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI−spatial = ③ dBc |  | 87 dBc | 85-95 dBc | 85-95 dBc | 80 dBc | 80 dBc | 80 dBc | 80 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX panel, with absorbing material and choke structure. | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation | A combination of spatial isolation, chokes, absorption, mushroom EBG. |  |  | Separate TX and RX panels with absorptive RF barriers, physical walls and chokes |
|  | TX Beam nulling /isolation in TX sub-band dBαSI−beam = ④ dBc |  |  | 10 dB | 10 dBc | 5-10 dBc | 10 dBc | 10 dBc | 10 dBc | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | Less than 0.5 dB loss |  | Up to 5dB EIRP loss, depending on beam direction |  |  | < 0.5 dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -95 dBm Note: provided by ①-②-③-⑨ dBm | -94~-104 dBm | -88 dBm | -78 dBm | -83 dBm | -88 dBm | -87 dBm Note: provided by ①-②-③-④-10*log10(160/40) dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI−RFIC = ⑤ dBc |  | 0 dBc | N/A |  | 0 dBc | 0 dBc | 0 dBc | Not applicable |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI−RFIC = ⑧ dBc |  | 0 dBc | N/A |  | 0 dBc | 0 dBc | 0 dBc | Not applicable |
|  |  | RF IC techniques and other tech. (before LNA) |  | Not applicable | N/A |  |  |  |  | Not applicable |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | No impact | N/A |  | 0 dBc | 0 dBc | 0 dBc | Not applicable |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -67 dBm Note: provided by ①-③-④-⑤dBm | -60 ~ -70 |  | -50 dBm | -55 dBm | -60 dBm | -53 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI−frequency, RX= ⑥ dBc |  | 24 dBc | N/A | 15 dBc |  |  |  | Not applicable |
|  |  | Frequency isolation techniques |  | Filtering | - | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). |  |  |  | Not Applicable Filtering would not prevent in-band blocking in any case |
|  |  | RX IMD | Rx IIP3 capability (dBm) | IM3 contribution is Neglectable | negligible | Similar conclusion as FR1 (i.e., IIP3 and IM3 are not dominant). | -35 dBm | -35 dBm | -35 dBm | -35 dBm |
|  |  |  | Rx IM3 contribution (dBm) |  | negligible |  | -80 dBm | -95 dBm | -110 dBm | Negligible |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | negligible | Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model. | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -100dBm | Reciprocal phase noise mixing will add noise at around -105dBm |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -91 dBm Note: provided by ①-③-④-⑤-⑥dBm | negligible |  | -80 dBm | -94 dBm | -104 dBm | -77 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI−beam = ⑨ dBc |  |  | 10 dBc | 10 | 5-10 dBc | TBC dBc |  |  | TBC |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | Less than 0.5 dB loss |  |  |  |  | - |
|  | Digital IC dBαSI−digtial = ⑦ dBc |  |  | 10 dBc | - | 10 dB | 10 dBc | 10 dBc | 10 dBc | - |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 129.5 dBc | 129 ~ 139 | 128 dB | 119 dBc | 125,5dBc | 127 dBc | 114 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz | -88 dBm/40 MHz | -88 dBm/40 MHz | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/40 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -89 dBm | -94 dBm | -94 dBm | -93 dBm | -93 dBm | -93 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc | 129 | 124 dBc | 133 dBc | 128 dBc | 123 dBc | 130 dBc |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) | DUD [80,40,80] | DUD | 75-50-75 |  |  | DUD [80,40,80] |
| Guardband assumption (if exist) |  |  |  | 5PRB | Existing SU | 5 PRBs | 3 RB |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | Several GHz | 200MHz | Several GHz |  |  |  |

#### 9.5.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

##### 9.5.1.2.1	Samsung

Different from FR1 counterpart, the difference of feasibility analysis on self-interference for FR2 BS will be provided here for different factors.

Spatial Isolation by Antenna Design

It’s observed that similar and even better antenna isolation performance with the FR2-1 26 GHz testbed where panel separation can be exploited.

Figure 9.5.1.2.1-1 shows the FR2-1 testbed using 2 Tx panels and 2 Rx panels. Like described in the case of the FR1 3.5 GHz testbed, the Tx panel and the Rx panel in the FR2-1 26 GHz testbed are separated by a separation distance. Additional Tx/Rx isolation performance is then enabled by using an RF barrier, e.g., an additional EM resonant between the panels. In the case of FR2-1 26 GHz, since each panel can perform more directive beam in analog domain than possible in FR1 using mMIMO panels, the FR2-1 antenna isolation performance is better than what is achievable in FR1.

Figure 9.5.1.2.1-1: FR2-1 testbed and SIC performance when varying the operating frequency

Similar as FR1, an important design consideration for increased spatial isolation provided by the RF barrier is whether such stopband performance is stable over a wide enough frequency range. Our FR2-1 26 GHz testbeds have achieved isolation performance that show almost uniform antenna/ panel isolation performance with respect to frequency for 100 MHz CC BW in 26 GHz. Figure 9.5.1.2.1-1 shows measurement results from the FR2-1 testbed with respect to achievable antenna isolation as a function of the operating frequency.

According to the applied mechanisms and measurement results, the achievable level for TX and RX spatial isolation without impact on radiation pattern based on compact antenna size is around 87dB for FR2.

Frequency isolation at TX

In the case of FR2-1, frequency-domain isolation for SBFD is of particular importance. Non-linear characteristics of mmWave PAs are worse than those of FR1 mid-band PAs. RAN4 ACLR requirements are more relaxed in FR2-1 when compared to FR1. This is due to beamforming providing isolation in FR2-1, implying that the probability of a blocker coming from the same direction is much lower than in FR1. Another consideration is that in FR1, the difference between the out-of-channel requirements like the ACLR and in-channel requirements like EVM is large. The PA linearity requirement is therefore dominated by out-of-channel emission requirements, e.g., ACLR. In FR2-1, these are at comparable levels. Spectral regrowth due to IM3 is dominant for in-channel requirements and as such, PA linearity requirements are rather driven by EVM and possibly in-band emissions. Another design challenge for DPD in FR2-1 is that PA characteristics must be carried through a feedback link from the output of the PA. In the case of mmWave, it is more difficult than in FR1 to create such a feedback link due to signal attenuation. Therefore, it is significantly more challenging to exploit DPD in FR2-1 such as done for FR1.

Despite these design challenges for gNB-side SBFD operation, our FR2-1 26 GHz testbed measurement results in Figure 9.5.1.2.1-2 show that 28 dBc leakage ratio between DL and UL subband (or component carriers) are still possible, e.g., similar to ACLR as existing out-of-channel requirement for FR2-1.

Figure 9.5.1.2.1-2: FR2-1 testbed and PSD for SBFD DL and UL subbands after antenna isolation and filtering

Frequency isolation at RX and RF SIC

Considering the input power at LNA for FR2 BS could be much less than FR1 WA BS, because of the better spatial isolation and lower BS output power, there is no necessity to have subband filtering to improve the linearity of LNA.

Digital IC

Similar as FR1, the digital SIC performance is helped when synchronization is achieved between Tx and Rx to accurately remove the Tx signal from the Rx signal can be obtained. As described in our analysis for FR1 WA BS, two methods exist to estimate the interference channel. One approach is to store information on a Tx signal that has passed through the PA with a feedback link and then estimate the interference channel over-the-air to remove the interference from the Rx signal. Another approach is to use only over-the-air estimation. Without a feedback link, the whole combined channel can still be estimated through the Rx panel. We used the second approach in the FR2-1 26 GHz testbed.

##### 9.5.1.2.2	Huawei

As shown in the 9.5.1.1, it can be found that the blocking level to RX pannel is weak and should be even lower at each LNA input. And the power at the Hence the IM3 is not a limited factors. Due to the same reason that the blocking level is relatively weak, the other RX impacts due to blocker in Tx sub-band can also be negligible. Hence we think that the evaluation on Self-Interference leakage in gNB RX subband would be sufficient for FR2.

##### 9.5.1.2.3	Qualcomm

In the following, we discuss in detail the knobs for FR2 gNB transceiver that enable the mitigation of both component of self-interference, namely direct leakage and clutter reflections.

Antenna techniques and spatial isolation

Similar to FR1, FR2 gNB Radio unit architecture with two physically separated panels for simultaneous transmission and reception enable large spatial isolation. To highlight this more, RF measurements for the Tx-Rx spatial has been conducted at 28 GHz frequency with two separate panels. The Tx and Rx measurement setup of the full duplex antenna array is shown in Figure 9.5.1.2.3-1. This measurement setup is on top of the building roof with antenna pointing to the sky, in which case could be without clutter impact or with negligible clutter impact. In this setup, the Tx and Rx beam sweeping is synchronized which is the worst-case scenario - without including clutter. The measurement results, presented in Figure 9.5.1.2.3-2, show at least 80-90 dB spatial isolation can be achieved between the two Tx and Rx panels. If the antenna array center-to-center distance is 65 cm, the spatial isolation could be achieved at -86.9 dB or better.

Figure 9.5.1.2.3-1: Measurement setup for Tx-Rx spatial isolation of the full duplex antenna array at 28 GHz

Worse case isolation -86.9dB.

Figure 9.5.1.2.3-2: RF measurements of Tx-Rx spatial isolation between two subarrays for FR2

Beam isolation and beamforming/nulling

In FR2, spatially isolated and narrow Tx and Rx beam could be selected to provide extra ‘beam’ isolation, which is a combined factor with the antenna isolation. For direct leaked self-interference, it is less related to the beam direction although there is still some dependency. However, for clutter, the signal transmitted from the Tx panel goes through the wireless medium, scattered by the reflectors and then gets back to the Rx panel, which generally has longer delay compared with direct leaked self-interference. The clutter is direction specific, in which case proper selection of Tx and Rx beam pair can alleviate such clutter impact. For FR2, clutter measurements have been conducted for a typical conference room. The measurement setup and results are shown in Figure 9.5.1.2.3-3. In the measurement setup, the Tx and Rx beam sweeping is synchronized, which is the worst-case scenario.

Figure 9.5.1.2.3-3: Clutter measurements for FR2 indoor deployments

Digital self-interference mitigation

In FR2, non-linear interference cancellation (NLIC) measurements have been conducted at 60 GHz for a typical conference room setup. For FR2, the measurement results are shown in for both low MCS and high MCS showing that NLIC could provide ~10 dB improvement on SNR performance.

##### 9.5.1.2.4	Ericsson

An overview of the self-interference mitigation potential for FR2-1 (28GHz) for three output power levels can be found in Table 9.5.1.1-1. It can be seen that mitigation of self-interference with less than 1dB receiver desensitization appears feasible for 30dBm. With 35dBm, the desensitization is around 1.5dB from self-interference. There may be some potential to further refine the performance. For 40dBm output power, mitigation of self-interference becomes significantly more challenging.

TX – RX isolation

A number of techniques exist to improve the TX-RX panel isolation including chokes, absorption, mushroom EBG etc. A detailed electromagnetic simulation of these techniques is presented in [50], which demonstrates that the isolation between a TX panel and RX sub-array is around 80dB. Unlike for FR1, the self-isolation does not seem to vary significantly with beam direction. Figure 9.5.1.2.4-1 shows EM simulation results and a visualization of the EM isolation.

| - boresight | - +15 deg. toward RX | - -15 deg. away from RX |
| --- | --- | --- |

| - boresight | - +15 deg. toward RX | - -15 deg. away from RX |
| --- | --- | --- |

Figure 9.5.1.2.4-1: EM simulation results for FR2 self-isolaiton (top) co-polarized (middle) cross polarized elements (bottom) visualization of the EM propagation

TX beam nulling

The transmit panel has a large number of transmit elements and hence a high number of degrees of freedom to perform beamforming. Beam Nulling can be used in the transmit panel to reduce the power at the receive panel. It is not clear that beam nulling has the same impact on both the transmitted signal and the transmitter leakage, however for simplicity this has been assumed.

Interference cancellation

Analogue and digital interference cancellation have not been considered due to the complexity and losses of building interconnectors for FR2, the high bandwidth and large amount of digital processing required.

Digital processing

Digital RX combining taking into account interference convariance have the potential to mitigate interference in the receiver. The gains from digital combining depend on the BS environment considering other interference sources such as other sectors, gNB and UEs.

Phase noise reciprocal mixing

Reciprocal mixing of phase noise causes a more significant amount of noise in the receiver in FR2-1 compared to FR1. However, the analysis suggests that also for FR2-1, reciprocal mixing of phase noise will not cause a significant degradation.

##### 9.5.1.2.5	Nokia

Nokia’s views on the self-interference analysis for FR2-1 base stations are presented in Table 9.5.1.1-1.

#### 9.5.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

Based on the provided studies in clause 9.5.1.1, it can be concluded that it is feasible to meet the 1 dB desensitization target considering self-interference supression for FR2-1 BS with TX output power levels up to around 33dBm.

### 9.5.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

#### 9.5.2.1	Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the summary table which is based on co-channel inter-sub-band co-site inter-sector interference analysis framework.

The Co-channel inter-sub-band co-site inter-sector interference analysis from companies’ input for FR2 SBFD-capable gNB is summarized in Table 9.5.2.1-1. Both self-interference leakage in gNB RX subband due to non-ideal TX and Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity are studied in the analysis framework. The interference cancellation techniques including spatial isolation, frequency isolation, beam nulling and digital/RF cancellation are considered in the analysis framework.

Table 9.5.2.1-1: Co-channel inter-sub-band co-site inter-sector interference analysis

| FR1 (or FR2-1) |  |  |  | Samsung | Huawei | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |  |  |  |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm | 35 dBm | 30 dBm | 35 dBm | 40 dBm |
| Number of co-site co-channel sectors considered |  |  |  | 1 | 2 | 2 | 2 | 2 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI−frequency, TX = ② dBc |  | 28 dBc | 28 dB | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD | DPD | Digital Filtering, CFR | Digital Filtering, CFR | Digital Filtering, CFR |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI−spatial = ③ dBc |  | 87 dBc | 100 dB | 75-98 dBc (depending on beam steering directions) |  |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 87dB for typical spatial isolation | Spatial separation between TX/RX panel and surface wave suppression techniques | Typical site deployment with 400mm between sectors |  |  |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI−beam = ④ dBc |  |  | 20 dBc | - | 0 dB | 0 dB | 0 dB |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | Neglectable | - | 0 dB | 0 dB | 0 dB |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -105 dBm | -96 | -93 dBm to -70 dBm (depending on beam steering directions) | -88 to -65 dBm (depending on beam steering directions) | -83 to -60dBm (depending on beam steering directions) |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -77 dBm | -62 | -65 dBm to -42 dBm(depending on beam steering directions) | -60 to -37 dBm (depending on beam steering directions) | -55 to -32dBm (depending on beam steering directions) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI−frequency, RX= ⑥ dBc |  | 24 dBc | - | 0 dB | 0 dB | 0 dB |
|  |  | Frequency isolation techniques |  | Filtering | - | 0 dB | 0 dB | 0 dB |
|  |  | RX IMD | Rx IIP3 capability (dBm) | IM3 contribution is Neglectable | - | -35 dBm | -35 dBm | -35 dBm |
|  |  |  | Rx IM3 contribution (dBm) |  | - | -109 to -56 dBm | -101.5 to -41 dBm | -91 to -26 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | - | RX phase noise is -105 dBm | RX phase noise is -100 dBm | RX phase noise is -95 dBm |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | -101 dBm | - | -103.5 to -56 dBm | -97.5 to -41 dBm | -89.5 to -26 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI−beam = ⑨ dBc |  |  | 20 dB | - | 0 dB | 0 dB | 0 dB |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Neglectable | - | 0 dB | 0 dB | 0 dB |
|  | Digital processing interference supression capability |  |  | 0 dB | - | 0 dB | 0 dB | 0 dB |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | -99.5 dBm | -96 dBm | -93 to -56 dBm | -87.5 to -41 dBm | -82 to -26 dBm |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz | -88 dBm/40 MHz | -87 dBm | -87 dBm | -87 dBm |
| Calculated Desensitization (dB) |  |  |  | 0.1dB | 0.6 dB | 1 to 31 dB (depending on beam direction) | 2.8 to 46 dBm (Depending on beam direction) | 6 to 61 dB (Depending on beam direction) |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) | DUD | 75-50-75 |  |  |
| Guardband assumption (if exist) |  |  |  | 5PRB | Existing SU | 3 RB |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | Several GHz | Several GHz |  |  |
| Others |  |  |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |  |  |  |  |

#### 9.5.2.2	Feasibility study on co-channel inter-sub-band co-site inter-sector interference

##### 9.5.2.2.1	Samsung

Different from the self-interference, the digital IC is not of necessity for the co-channel interference from co-site inter-sector BS. Furthermore, the isolation achieved by TX and RX beamforming nulling will be larger due to the different beamforming directions from different sectors.

##### 9.5.2.2.2	Huawei

For FR2 inter-sector isolation, some measurements based on existing modules are proceed as below. The AAS mounting in the same mast would be worse cases for radiated isolation. The following two cases are the typical site deployments for the worse cases. The left one (Case 1) is up-down installation and the right one (case 2)is 120° installation.

Figure 9.5.2.2.2-1: Measurements of inter-sector isolation

For each case, 60*60=3600 beam combinations are measured. The results are shown in Table 9.5.2.2.2-1 for existing modules and Table 9.5.2.2.2-2 for improved modules. Some surface wave suppression measures can be used to improve the isolation, e.g. reflection and absorption structure. Using these measures, more than 10 dB improvement can be achieved for the case with poor isolation. As shown in in the measurements, 100 dB inter-sector isolation is achieveable for FR2

Table 9.5.2.2.2-1: Inter-sector isolation for existing modules

| Test cases | Existing modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 96 | 99 | 103 |
| Case 2 | 120°installation | 85 | 90 | 100 |

Table 9.5.2.2.2-2: Inter-sector isolation for improved modules

| Test cases | Improved modules | 95% CDF | 90% CDF | 50% CDF |
| --- | --- | --- | --- | --- |
| Case 1 | up-down installation | 101 | 103 | 104 |
| Case 2 | 120°installation | 100 | 101 | 102 |

##### 9.5.2.2.3	Ericsson

An analysis of the inter-sector interference effects for FR2-1 BS can be found in Table 9.5.2.1-1. The analysis suggests that inter-sector interference can lead to substantial degradations. Apart from with the most optimal beam directions settings and 30dBm TRP, degradations due to inter-sector interference are substantial. Although TX beam nulling may be considered to reduce inter-sector interference, in excess of 30dB-60dB suppression (depending on output power) would be needed to mitigate interference for all beam directions.

For 30dBm TRP, RX processing or improvements in the TX interference suppression may improve the inter-sector performance. However, the degradation will not be reduced to well below 1dB (taking into account also degradation from self-interference will occur) for all beam directions.

Thus co-sectorization is likely to pose technical challenges for SBFD deployments in FR2.

Inter-sector isolation

Isolation between sectors has been simulated using electromagnetic simulations in R4-2301885 with an assumption of 400mm sector separation. The isolation varies to some degree with separation, but not to an extent that would change the overall results. For most practical site deployments, addition of materials between sectors is not likely to be feasible (and may reduce network performance). Figure 9.5.2.2.3-1 depicts the EM simulation setup.

Figure 9.5.2.2.3-1: Simulation setup for FR2 multi-sector EM modelling

Figure 9.5.2.2.3-2 depicts the achievable isolation. The left hand plot shows the isolation with azimuth steering and elevation in boresight and the right hand plot with elevation steering and azimuth on boresight. The insolation between sectors is highly dependent on the beam direction. Although an “average” isolation can be given, this would mask the fact that for certain beam directions isolation is good and for others it is not good. Since the beam direction depends on the physical positions of users, advanced co-ordination of beam directions may not be possible if other constraints such as capacity and latency are to be optimized.

| - Azimuth beam steering with elevation at boresight | - Downtilt elevation beam steering with azimuth at boresight |
| --- | --- |

Figure 9.5.2.2.3-2: FR2 antenna with 400 mm edge to edge coupling magnitude

Beam nulling

There may be some potential for beam nulling to mitigate interference between sectors. To achieve complete isolation between sectors, more than 30dB beam nulling would be needed.

#### 9.5.2.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on co-channel inter-sub-band co-site inter-sector interference based on RAN4 agreement.

Based on the provided studies in clause 9.5.2.1, two companies conclude that it is feasible to supress inter-sector interference to a level that it meets 1 dB desensitization target, whereas one company concludes that it is not feasible for a proportion of beam directions. The difference between the conclusions is mainly due to differences in assumption on whether it is possible to build isolation materials within sites and whether sufficient beam nulling is possible in all directions.

9.5.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

9.5.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## 9.6 FR1 Feasibility of UE aspects

### 9.6.1	Interference analysis

#### 9.6.1.1 General

In the objective of this study item, half duplex operation at UE side is assumed. In this part of feasibility of UE aspects, FR1 is considered.

In the UE feasibility study, existing UE RF requirements in TS 38.101-1 can be applied as default assumption for study phase conclusion if no issues identified by co-existence study. Detailed UE RF requirements if any should be discussed during WID phase.

In the UE feasibility study in FR1, the co-channel inter-subband UE-UE CLI model and adjacent channel UE-UE CLI model are mainly discussed. Co-channel/adjacent channel interference models at the UE side are summarized in Table 9.6.1.1-1. For co-channel interference models, the UE IBE requirement can be used for the interference modelling at Tx side; for receiver in-channel adjacent subband selectivity performance, no rejection/attenuation due to RF/BB filtering is assumed. For adjacent channel models, UE ACLR and adjacent channel selectivity requirements can be used for the interference modeling at Tx and Rx side, respectively.

Table 9.6.1.1-1: Existing UE interference models based on RF requirements in RAN4

| Co-channel RF interference models |  | Adjacent channel RF interference models |  |
| --- | --- | --- | --- |
| Tx side | Rx side | Tx side | Rx side |
| UE IBE for Tx | In-channel adjacent subband selectivity (Note 1) | Power dependent ACLR as described in Section 9.6.1.3.2 | Adjacent channel selectivity |
| Note 1. For legacy UE, there is no existing UE RF requirement for in-channel adjacent subband selectivity. It is only used in SBFD feasibility study purpose and no indication that this requirement shall or shall not be defined in the normative phase. |  |  |  |

#### 9.6.1.2	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

##### 9.6.1.2.1 Overview and analysis framework

The objectives of UE-UE co-channel inter sub-band CLI modeling is to analyze the impact of interference that occurs between two UEs in close proximity, operating on adjacent sub-bands within the same channel. This interference occurs when UL transmission of an aggressor UE in the channel interferes with the DL reception of a victim UE in the same channel.

For this SI RAN4 has decided to use typical UE parameters in the analysis, as different from worst-case parameters that are commonly used to define minimum performance requirements.  The evaluation of gNB performance improvements is underway in RAN1, taking into account a population of typical UEs rather than worst-case UEs.

In-channel adjacent subband selectivity is a measure of a receiver’s ability to receive an NR signal on its assigned downlink subband in the presence of an interference power on the adjacent uplink subband. The value of in-channel adjacent subband selectivity is the ratio of the receiver attenuation on the assigned downlink subband to the receiver attenuation on the adjacent uplink subband. In an ideal scenario, the UL transmission of the aggressor UE should not impact the DL reception of the victim UE due to the OFDM wave orthogonality. However, non-ideal FFT suppression can cause interference to the victim UE, particularly when the UL sub-band has frequency tracking errors and is not perfectly time-synchronized with the DL sub-band. The analysis indicates that the IBE interference could be more significant than the in-channel adjacent subband selectivity, and frequency and time offset are not significant factors influencing UE-UE interference. It is worth noting that the RF degradations can cause inter-subband interference as well and the impact will depend on the targeted Rx IM and EVM performance. The measurement data submitted by one company shows the achievable in-channel adjacent subband selectivity of FR1 UE can be 33 dB. Nonetheless, the typical interference will normally not be any worse than this value. For this reason, the 33 dB was agreed for modeling the in-channel adjacent subband selectivity.

For legacy UE, no sub-band filtering is considered.

To model the NF for co-channel CLI in a system level simulation, a fixed value noise figure of 9 dB is used. AGC is not modeled if a fixed NF model is used.

Apart from the in-channel adjacent subband selectivity, it is important to mention that degradation can also be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel cases, the leakage was agreed to be modelled based on IBE based model with details provided in clause 9.6.1.2.2.

##### 9.6.1.2.2 UE co-channel Tx model

In-Band Emissions

For UE co-channel Tx model, RAN4 has decided to use the IBE minimum requirements from 38.101-1 clause 6.4.2.3 as shown in Table 9.6.1.2.2-1 in the feasibility study. This IBE-based model consists of three parts: General, IQ image, and Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation. The granularity of this model is 1RB and it is not pursued to simplify this model to a frequency flat model. It is understood that these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters. Since typical values for the UE parameters were not determined, the formulation from the current specification is being used.

Table 9.6.1.2.2-1: Requirements for in-band emissions in TS 38.101-1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

##### 9.6.1.2.3 UE co-channel Rx model

For UE co-channel Rx model, currently there is no corresponding RF requirement for this model. In the feasibility of UE co-channel Rx model, the definition of in-channel adjacent subband selectivity is introduced for SBFD feasibility study purpose:

- In-channel adjacent subband selectivity is a measure of a receiver’s ability to receive an NR signal on its assigned downlink subband in the presence of an interference power on the adjacent uplink subband. The value of in-channel adjacent subband selectivity is the ratio of the receiver attenuation on the assigned downlink subband to the receiver attenuation on the adjacent uplink subband.

For legacy UE, no sub-band filtering is assumed in the feasibility study, and accordingly no rejection/attenuation due to RF/BB filtering is assumed and only the selectivity performance of the FFT operation is studied. 33 dB was agreed for FR1 in RAN4 for in-channel adjacent subband selectivity considering FFT operation.

Thermal self-noise performance

RAN4 decided on using a simplified fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was regarded to be sufficient for the purpose of system studies for SBFD operation. RAN4 decided on a noise figure value of 9 dB for FR1 UE.

Effect of jammer – non-thermal self-noise aspect

There are a few factors to consider in determining the in- channel subband interference in the presence of a co-channel jammer. With an in-channel adjacent-subband interferer the 3rd order distortion, reciprocal mixing, residual sideband, quantization noise, phase noise, ADC distortion, and analog filtering should be considered.

In RAN4 study, measurements were made of a UE receiver for various signal levels, interferer levels, interferer offsets, sub-band bandwidths, and interferer bandwidths. The measurements have included the impact of the entire receiver, which includes everything through the FFT operation, in which 120 various conditions were measured.

FFT leakage and selectivity

In the SBFD system an aggressor UE (UE2) operating in the UL sub-band may interfere with a UE (UE1) receiving in the adjacent DL sub-band. The UL signal from UE2 may arrive at UE1 misaligned in time or frequency, which can potentially cause UE1 DL SINR degradation in the FFT operation.

The simulation involved the conversion of the OFDM waveform to a spectrum by using FFT, along with the introduction of timing and frequency errors. A 5 RB guard band was assumed. The analysis revealed that time-misalignment was the main cause of the spectral leakage, and even minor timing errors resulted in leakage.

It appears reasonable to consider the leakage as a single average value, and the data shows about 33 dB down from the jammer level would be appropriate. The leakage effect should be taken into consideration and compared to the interference caused by the aggressor IBE.  It has been observed that the interference in the victim sub-band can be modelled as 33 dB below the input jammer power level, and the interference is approximately frequency flat across the victim subband.

Figure 9.6.1.2.3-1: FFT leakage with time and frequency misaligned blocker (5 RB guard band)

#### 9.6.1.3 UE-UE adjacent channel CLI modelling

Editor's note: This section captures the CLI modeling.

##### 9.6.1.3.1 Overview

The UE-UE adjacent channel CLI occurs when the UL transmission of the aggressor UE in a channel interferes with the DL reception of the victim UE in an adjacent channel. Unlike the case of co-channel interference, there is no need to consider any FFT selectivity in the adjacent channel scenario. Apart from the selectivity, it is necessary to account for the leakage from the UL sub-band into the DL sub-band caused by the transmitter. It was decided to assume the power-dependent ACLR of the aggressor UE and selectivity of the victim UE when modeling adjacent channel interference.

To model the NF for adjacent channel CLI in a system-level simulation, a fixed value noise figure of 9dB is used. The effect of AGC is not modeled when a fixed noise figure model is used. Additionally, UE ACLR should be modeled as 30 dB at max power, improving 1 dB/dB with back-off up to a maximum of 10 dB of improvement. Therefore, when the back-off is 10 dB, the ACLR is 40 dB. When the victim and aggressor UEs are close, between 1 and 50m, and close to the cell edge, with low desired signal level, and potentially high interference level, it cannot be guaranteed that the UE receiver will operate in the linear region.

##### 9.6.1.3.2 UE adjacent channel Tx model

UE adjacent channel leakage ratio is used in the feasibility study for adjacent channel UE-UE CLI Tx model. Only ACLR in 1st adjacent channel was considered in the study item and ACLR in 2nd adjacent channel was precluded. In the UE Tx model, only power class 3 was assumed.  UE ACLR is modelled as 30 dB at max power that improves 1dB/dB with backoff up to a maximum 10 dB of improvement. This means at 10 dB backoff the ACLR is 40 dB. Partially allocated UL subband was not considered in the system simulation. This ACLR model can be seen as frequency flat model, and the distortion is modelled as a flat power spectral density across the frequency range of the distortion.

##### 9.6.1.3.3 UE adjacent channel Rx model

UE adjacent channel selectivity (33dB for FR1) is used as adjacent channel UE-UE CLI model under the assumption that the blocker from adjacent channel does not exceed the maximum input level (-25 dBm) for UE. If the blocker is higher than -25dBm, it is assumed that it will result large receiver degradation and hence the RX will not correctly decode the data (100% packet loss).

### 9.6.2	Summary

Editor's note: This section captures the conclusion of feasibility.

[

In the UE feasibility study for FR1 UE, existing UE RF requirements in TS 38.101-1 can be applied as default assumption for study phase conclusion if no issues identified by co-existence study. Detailed UE RF requirements if any should be discussed during WID phase.

For co-channel interference case, RAN4 concluded that the RF effect could be dominant, and the frequency and time tracking errors are not significant factors influencing UE-UE interference. Furthermore, the leakage can be modelled by using the in-band emission (IBE) requirement based model.

As for the adjacent channel case, RAN4 concluded to assume power-dependent ACLR of the aggressor UE and adjacent channel selectivity of the victim UE when modeling adjacent channel interference.

For legacy UE, no sub-band filtering is considered.

A fixed value noise figure of 9 dB has been used to model the AGC and NF modeling for co-channel and adjacent channel CLI in a system level simulation.

]

9.7 FR2-1 Feasibility of UE aspects

9.7.1 Interference analysis

9.7.1.1  UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

##### 9.7.1.1.1 Receiver aspects

Existing co-channel UE RX performance requirements

For legacy UEs, the current UE RF architecture can be assumed without any RF architecture modification. Currently there are no RF requirements for UE co-channel Rx performance.

Sub-band filtering and legacy UEs

For legacy UEs, no sub-band filtering is implemented, and therefore RAN4 has not assumed any subband filtering.

Thermal self-noise aspects (both adjacent channel and co-channel)

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered sufficient for the purpose of system studies for SBFD, therefore AGC effect on self-noise is not modelled. RAN4 decided on a NF of 10dB.

Subband in-channel selectivity

It is worth noting that the RF degradations can cause inter-subband interference. An analysis of the FR2-1 receiver’s design was conducted. Various factors, such as residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions, were considered. The effect of all these distortions is lumped into a single parameter referred to as selectivity. Based on the discussion and analysis from the meeting, contributions suggested possible the sub-band selectivity values from 20 dB to 34 dB. The receiver performance is simply modelled as being 23 dB below the jammer power level. The definition of Subband in-channel selectivity is introduced for clarity in the SBFD feasibility study

- In-channel adjacent subband selectivity is a measure of a receiver’s ability to receive an NR signal on its assigned downlink subband in the presence of an interference power on the adjacent uplink subband. The value of in-channel adjacent subband selectivity is the ratio of the receiver attenuation on the assigned downlink subband to the receiver attenuation on the adjacent uplink subband.

FFT leakage and selectivity

In an ideal scenario, the UL transmission of the aggressor UE should not impact the DL reception of the victim UE due to the OFDM wave orthogonality. However, non-ideal FFT suppression can cause interference to the victim UE, particularly when the UL sub-band has frequency errors and is not time-synchronized with the DL sub-band. The analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity, and frequency and time offset are not significant factors influencing UE-UE interference. It is worth noting that the RF degradation can cause inter-subband interference as well and the impact will depend on the targeted Rx IM and EVM performance. Nonetheless, this interference will not be any worse than the selectivity value. For this reason, the 23 dB was agreed upon for modeling the inter-sub-band selectivity.

##### 9.7.1.1.2 Transmitter aspects

Inband emissions (co-channel)

RAN4 has decided to use the IBE requirements from TS 38.101-2 clause 6.4.2.3.4 (power class 3 UE). It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters, however, no final conclusion has been made regarding typical values. Consequently, the formulation from the current specification is being utilized.

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

Analysis indicates that the IBE interference is higher and dominates the sub-band in-channel selectivity,

Apart from the selectivity, the degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration.

For UE co-channel Tx model, UE IBE in TS 38.101-2 can be used in the feasibility study as shown in Table 9.7.1.1.2-1. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation.

Table 9.7.1.1.2-1: Requirements for in-band emissions in TS 38.101-2 (For Power class 3)

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB | max−25 −10.log10NRBLCRB, 20.log10EVM− 5.¦∆RB−1LCRB, −55.1dBm−PRB |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 10 dBm | > 8.1 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 10 dBm | ≤ 8.1 dBm |  |
| Carrier leakage | dBc | -25 | > 0 dBm | > -1.9dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm | -14.9 dBm ≤ Output power ≤ -1.9 dBm |  |

9.7.1.2  UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

##### 9.7.1.2.1 Receiver aspects

Effect of adjacent channel aggressor UE jammer

An analysis of the FR2-1 receiver’s design was conducted. Various factors, such as residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions, were considered. The receiver’s performance is simply represented as 34 dB lower than the jammer power level.

##### 9.7.1.2.2 Transmitter aspects

ACLR (adjacent channel)

ACLR is one aspect modelled as an interference aspect from a nearby aggressor UE transmitting in an adjacent UL subband. When the victim and aggressor UEs are close, between 1 and 50m, and close to the cell edge, with low desired signal level, and high interference level, it might be the case that the victim UE will not always operate in the linear region, resulting in dominance of the ACLR from the aggressor UE. UE ACLR is modeled as 24 dB at max power, improving 1 dB/dB with backoff up to a maximum of 10 dB of improvement. Therefore, when the backoff is 10 dB, the ACLR is 34 dB. This model is an approximation of the performance of a typical UE.

9.7.2	Summary

Editor's note: This section captures the conclusion of feasibility.

9.8	Summary

10		Impact on RF requirements

10.1	Impact on BS RF requirements

Editor's note: This section captures the impact on BS RF requirements

10.1.1 General

[Editor's note: This section captures the general analysis for BS RF requirements, and also some general assumption which shall be based for the detailed analysis for BS RF requirements. ]

During Rel-18 SI, for SBFD-capable BS, RF requirement shall only be studied based on the semi-static configuration of subband time and frequency location, which is supported by SBFD-capable BS. RAN4 mainly focus on the following multi-carrier configuration for SBFD-capable BS:

- SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier.

During Rel-18 SI, RAN4 didn’t study the following multi-carrier configuration for SBFD-capable BS:

- SBFD operates in more than one BS carriers, and legacy TDD operates in the other intra-band BS carrier(s) (if any), which is contiguous or non-contiguous to the SBFD carriers.

Regarding the baseline assumption for the maximum number of UL sub-bands of SBFD capable BS, RAN4 agreed to restrict the maximum number to be one in an SBFD symbol/slot within a TDD carrier. In addition, RAN4 agreed that RF requirement impacts for SBFD operation in symbols/slots configured as UL in TDD-UL-DL-ConfigCommon is treated as 2nd priority.

For SBFD-capable BS, the existing RF requirements shall be applied in the OFDM symbols/slots others than SBFD symbols/slots and RF requirement impacts in the SBFD symbols/slots will be further clarified in details in the following sections.

10.1.2 Impact on BS TX requirements

10.1.2.1 Base Station output power and radiated transmit power

Since configuration (e.g. antenna, power configuration etc) between SBFD and non-SBFD symbols/slots might be different, RAN4 reached the following consensus for the BS RF requirement of BS output power for both conducted and OTA output power:

- It is allowed to have the different conducted declaration for normal DL symbols/slots and SBFD DL symbols/slots.

- It is allowed to have different EIRP/TRP declaration (for level and direction) for normal DL symbols/slots and SBFD DL symbols/slots.

- Accuracy requirement for TRP/EIRP and conducted power shall be the same for normal DL symbols/slots and SBFD DL symbols/slots.

10.1.2.2 Output power dynamics

Regarding the output power dynamic requirement, which mainly consists of RE power control dynamic range requirement and total dynamic range requirement, RAN4 reached the following consensus:

- To reuse the existing RE power control dynamic range requirement for SBFD BS;

- The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots, the requirement limit and conformance testing will be further discussed in the normative stage.

10.1.2.3 Transmit ON/OFF power

Regarding the transmitter ON/OFF power requirement, RAN4 mainly focus on the ON-OFF time mask and concluded that transmit ON/OFF power requirement is not applicable within SBFD time slot.

10.1.2.4 Transmitted signal quality

Regarding the transmitter signal quality, RAN4 agreed that all the existing requirement for frequency error, modulation quality (EVM) and time alignment error (TAE) shall also be applied to BS in SBFD symbols/slots.

- Further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots during WI phase.

10.1.2.5 Unwanted emissions

Regarding the unwanted emission requirement, it mainly consist of OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement and co-location and coexistence requirement, RAN4 reached the following consensus for SBFD-capable BS respectively:

- For BS OBW requirement, the existing OBW requirement shall be applied for the whole BS channel bandwidth in SBFD symbols/slots instead of DL sub-band.

- For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

- For OBUE requirement, the RF bandwidth edge from which OBUE is defined is the edge of the carrier (same for both SBFD and non-SBFD symbols/slots).

- For transmitter spurious emission requirement, all the existing requirements shall also be applied to SBFD-capable BS in SBFD symbols/slots. The requirement of protection of the BS receiver of own or different BS is not applicable for TDD operation.

10.1.2.6 Transmitter intermodulation

[]

10.1.3 Impact on BS RX requirements

Editor's note: This section captures the RF requirement impact analysis for all existing BS RX requirements, which has been specified in TS38.104 already.

10.1.3.1 Reference sensitivity level and OTA sensitivity

Regarding Reference sensitivity requirement for SBFD-capable BS, due to the self interference caused internally to receiver side, RAN4 reached the following consensus:

- For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed.

- Otherwise, OTA sensitivity requirement could be derived based on the following equation:

PREFSENSdBm=−174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G

    - The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase.

    - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot

    - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

10.1.3.2 Dynamic range

Regarding the dynamic range requirement, this requirement is still applicable for SBFD-capable BS. The IoT level and wanted signal power level could be further discussed in the WI phase.

10.1.3.3 In-band selectivity and blocking

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement:

    - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level.

    - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

- In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement:

    - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level.

    - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

10.1.3.4 Out-of-band blocking

Regarding Out-of-band blocking requirement, the existing OOBB requirement is still applicable for SBFD-capable BS except for OTA sensitivity degradation with the power level of wanted signal taken into account.

10.1.3.5 Receiver spurious emissions

Regarding the receiver spurious emission requirement, apart from existing requirements for normal reception on UL symbols/slots, it’s not necessary to specify additional receiver spurious emissions requirement for SBFD operation in SBFD symbols/slots.

10.1.3.6 Receiver intermodulation

Regarding the receiver intermodulation requirement, in general, RX intermodulation requirement and the interference levels shall be determined by RAN4 co-existence study, and for the definition of RX intermodulation requirement RAN4 reached the following consensus:

- Conducted RX intermodulation: Adopt the existing wanted signal of RX intermodulation requirement by using the existing reference sensitivity level.

- OTA RX intermodulation: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

10.1.3.7 In-channel selectivity

Regarding the receiver in-channel selectivity requirement, the requirement shall be studied based on that the wanted signal and UL interfering signal shall be located in the configured UL subband, and the wanted signal and interfering signal levels could be further studied in the WI phase.

10.1.4 Potentially new requirements for SBFD operation

Editor's note: This section captures the analysis for potentially new requirements for SBFD operation, which has not been specified in TS38.104 for non-SBFD operation.

10.1.4.1 Transmitter transient period

For transmitter transient period between SBFD and non-SBFD or SBFD reconfigurations if needed, the requirement shall be introduced to BS in SBFD symbols/slots, by defining the transient period as the time period which the transmitter is changing from the SBFD operation to non-SBFD operation or vice versa, or during SBFD reconfigurations.

Regarding the transition period requirement, RAN4 mainly focus on the transition period related with SBFD operation and RAN4 agree to further study the following cases in which whether transition period is needed or not:

- [The switch between normal slot and SBFD slots]

- SBFD reconfiguration with antenna array and/or sub-band filtering reconfigured

- Other candidate conditions not precluded

Based on the RAN4 study, between the non-SBFD slot and SBFD slot and vice versa, a transition period is needed. If the SBFD configuration between adjacent SBFD slots is the same, then no transition period is needed.

10.1.4.2 In-channel adjacent subband leakage ratio

10.1.4.3 In-channel adjacent subband Blocking and adjacent subband selectivity

10.2	Impact on UE RF requirements

Editor's note: This section captures the impact on UE RF requirements

RAN4 has analyzed inter-UE co-channel inter-subband CLI modeling and adjacent channel CLI modelling. In Table 10.2-1, in-channel RF assumptions (e.g., IBE and subband selectivity) and adjacent channel RF assumptions (e.g., ACLR and ACS) for the coexistence study are presented for UE Tx/Rx interference modelling.

Table 10.2-1: UE RF assumptions for interference modelling

| In-channel RF assumptions | FR1 | FR2-1 |
| --- | --- | --- |
| Tx Model: IBE | Existing UE IBE requirement in TS38.101-1 | Existing UE IBE requirement in TS38.101-1 |
| Rx Model: Subband Selectivity | 33dB for simulation usage purpose | 23 dB for simulation usage purpose |
| Adjacent channel RF assumptions | FR1 | FR2-1 |
| Tx Model: ACLR | 30 dB at max power that improves 1dB/dB with backoff up to a maximum 10 dB of improvement | 24 dB at max power that improves 1dB/dB with backoff up to a maximum 10 dB of improvement |
| Rx Model: ACS | 33 dB under the assumption that the blocker from adjacent channel does not exceed the maximum input level (-25 dBm) | 23 dB under the assumption that the blocker from adjacent channel does not exceed the maximum input level (-25 dBm) |

For UE in-channel RF assumptions, existing UE IBE requirements in TS 38.101-1/2 were assumed for the Tx model; for the Rx model, RAN4 defined subband selectivity for co-existence simulation purpose, and the value of 33dB for FR1 and 23dB for FR2-1 are derived from the performance of typical user. In the existing 3GPP specification, subband selectivity is not specified as UE RF requirement and it is proposed for the co-existence study usage only in this study item.

For UE adjacent channel RF assumptions, the values of ACLR/ACS for FR1/FR2 are based on the existing UE RF requirements. The existing ACLR requirement was defined with the assumption of the maximum output power. In order to model typical UE performance, this model was revised considering the cases in which UE transmit power is less than the maximum power.

Based on the study, existing UE RF requirements has been applied as default assumptions for study phase conclusion, since no issues related to existing UE RF requirements has been identified in the co-existence study.

11	Adjacent channel co-existence evaluation results.

  - 11.1	Introduction

Editor's note: This section will capture adjacent channel co-existence simulation scenarios and cases. Besides, some brief introduction of what RAN4 have done in R18 for SBFD.

The adjacent channel co-existence studies were performed to the deployment scenarios described in Table 11-1 below. The co-existence cases are described in the Table 11-2 below, and they were performed for each scenarios listed in Table 11-1. The detailed assumptions associated with these scenarios and cases can be found in Annex E.

Table 11-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | SBFD Urban Macro | Legacy Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Table 11-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  |
| 2 | SBFD | TDD UL |  |
| 3 | TDD DL | SBFD |  |
| 4 | TDD UL | SBFD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |

The Urban Hotspot reuses most parameter assumption as Urban Macro, except that Urban Macro adopts random dropping method for UE while Urban Hotspot adopts cluster-based dropping method for UE. Other differences are described in Table E.2.1-1, Table E.2.1-2, and Table E.2.1-3.

The coexistence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in an adjacent channel as well as to understand the impact of legacy TDD network on SBFD network, as described in Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration in terms of performance (based on the agreed RAN4 models and parameters related to leakage).

Additionally, it should be noted that the RAN4 co-existence studies have special assumptions due to the adopted simulation methodologies such as:

- Power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target SINR value according to the power control scheme in 38.213.

- It is assumed that all the slots configurations are the same with the time-invariant ACLR modelling assumption. Compared with the average throughput over all time slots with different configuration, this is the worst case with largest degradation value.

Moreover, in the following sections, all the throughput degradation data were given in a range, where these data are defined as follows:

- The {positive number} means the co-ex study shows throughput loss;

- The {negative number} means the co-ex study shows throughput gain;

- The {n/a} means the co-ex study finds the performance basis have no throughput, thus throughput degradation percentage cannot be mathematically calculated from such basis.

  - 11.2	Summary of all simulation results

Editor's note: This section will capture the excel files and word file that moderator summarized based on all companies’ input with some description of these files.

[

One example:

We list three files.

First excel file is the collection of all simulation results using the template format that companies use to provide simulation results.

Word file is the summary of all simulation results categorized in terms of scenario, case, victim and parameters values of four simulation assumptions, i.e. antenna configuration 1 or 2, gNB Tx power value, grid shift value and enhanced NF or not. In addition to, we list the min value, max value and median for relative ACIR, ACIR enhanced by 2, 4, 6, and 8dB. We also list the numbers of companies that show simulation results for each scenario, each case and each victim band.

Second excel file is the excel file of word file which is much straightforward compared with the word.

[placeholder for clarifying text on ACIR enhancement definition for simulation and results interpretation]

]

  - 11.3	Conclusion

Editor's note: This section will capture adjacent channel co-existence simulation results only categorized by cases and scenarios. Besides, for each case if the conclusions are much similar among some/all scenarios. Final conclusion would be merged together among these scenarios.

11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL

Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The following can be summarized:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different Tx powers (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid shifts (5% to 100%), and different SBFD antenna configuration. |
| Urban Hotspot -> Urban Hotspot | FR1 | DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid shifts (5% to 100% ) and gNB Tx powers (49 dBm to 53 dBm). |
| Urban Macro -> Urban Micro | FR1 | No DL throughput degradation for both average throughput and cell edge throughput is observed. |
| Indoor -> Indoor | FR1 and FR2-1 |  |
| Urban Micro/Dense -> Urban Micro/Dense | FR1 and FR2-1 |  |

11.3.2 Case 2: aggressor SBFD DU victim NR TDD UL

Case 2 considers legacy TDD in UL slot as a victim while SBFD is operating in the UL slot in the adjacent channel for both FR1 and FR2-1. The following can be summarized:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 | The TDD UL has significant throughput degradation for different SBFD gNB antenna configuration and different gNB Tx powers. - The cell edge throughput degradation is worse than the average throughput degradation. - The throughput degradation is due to the inter-gNB ACI introduced by SBFD, which increases as grid shift (gNB-to-gNB distance) decreases except in the UMa-to-UMi scenario. - The throughput degradation increases with SBFD gNB Tx power. - SBFD gNB antenna configuration slightly impacts the throughput degradation. |
| Urban Hotspot -> Urban Hotspot |  |  |
| Urban Macro ->Urban Micro |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Indoor -> Indoor | FR1 & FR2-1 | No TDD UL throughput degradation is observed. |
| Urban Macro -> Urban Macro | FR2-1 | TDD UL throughput degradation is observed at cell edge, no strong degradation is observed for the average throughput. |
| Urban Dense -> Urban Dense |  |  |

11.3.3 Case 3: aggressor NR TDD DL victim SBFD DU

Case 3 considers SBFD as a victim while NR TDD is operating DL in the adjacent channel for both FR1 and FR2-1. The following can be summarized:

Impact on SBFD DL:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different gNB Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and different SBFD antenna configuration. |
| Urban Hotspot -> Urban Hotspot (N/A for FR2-1) |  |  |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |

Impact on SBFD UL:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 | Under baseline assumptions, observed SBFD UL throughput degradation is observed only for cell edge throughput and [no degradation is observed for average throughput]. With higher gNB Tx power and lower grid shifts, the degradation is increased for cell edge throughput and [average throughput]. |
|  | FR2-1 | Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher gNB Tx power and lower grid shifts. |
| Urban Hotspot -> Urban Hotspot | FR1 | Under baseline assumptions, observed throughput degradation is observed at cell edge throughput and average throughput. With higher gNB Tx power and lower grid shifts, the degradation is increased. |
| Indoor -> Indoor | FR1 and FR2-1 | No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed. |
| Urban Micro/Dense -> Urban Micro/Dense | FR1 | Under FR1 Urban micro 38dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher gNB Tx power (46dBm) and lower grid shifts. |
|  | FR2-1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and [no degradation is observed for average throughput]. |

11.3.4 Case 4: aggressor NR TDD UL victim SBFD DU

Case 4 considers SBFD as a victim while NR TDD is operating UL in the adjacent channel for both FR1 and FR2-1.

Impact on SBFD DL can be summarized:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL and UL for both average throughput and cell edge throughput for different gNB Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and antenna configuration (single and double panels for SBFD operation). |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Urban Hotspot -> Urban Hotspot | FR1 | Some companies’ results show DL throughput degradation is observed only for cell edge throughput due to inter-UE CLI for different grid shifts (5% to 100%), gNB Tx powers (46dBm to 53 dBm) and for all antenna configurations. the less grid shift, the larger degradation due to shorter UE-to-UE distance. However more companies show that there is no observed interference for cell edge throughput and cell average throughput for 100% grid shift, 49dBm gNB Tx power and antenna configuration 2. |

Impact on SBFD UL can be summarized:

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL and UL for both average throughput and cell edge throughput for different gNB Tx powers, ranging (46dBm to 53 dBm for FR1 and 30 dBm for FR2-1), Grid shifts (5% to 100%), and antenna configuration (single and double panels for SBFD operation). |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Urban Hotspot -> Urban Hotspot (N/A for FR2-1) |  |  |

11.3.5 General remarks on coexistence findings

[For the above cases where no throughput degradation has been observed, no additional coexistence measures are required for SBFD deployment. However, for other cases where throughput degradation has been observed, interference mitigation techniques might need to be considered.]

12	Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum

Editor's note: This section captures the summary of the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

## 12.1	ITU Region 1

12.1.1	Europe

The European Conference of Postal and Telecommunications Administrations (CEPT) made coexistence studies with adjacent services assuming a certain DL/UL ratio for International Mobile Telecommunications (IMT) TDD bands, e.g., 3.4 – 3.8 GHz band in Europe [51]. The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation and consequently may affect the outcomes of the coexistence studies and, consequently, the regulated license conditions.

To address the cross-border issue and facilitate coordination, the Electronics Communications Committee (ECC) recommended the usage of two frame structures in the 3.4 – 3.8 GHz frequency band [52].

However, enabling operation with various TDD patterns and removing the need of synchronized networks, CEPT has specified additional baselines for unsynchronized or semi-unsynchronized networks. Nevertheless, those baselines are more stringent, making the BS design more challenging, impacting final cost and possibly product’s volume and weight. As an example, for the 3.4 – 3.8 GHz band, inside the band, ECC specified below and above the block edge a restricted baseline of -34 dBm/5MHz EIRP for non AAS BS or -43 dBm/MHz TRP for AAS BS [53] operators, the situation may be reported to the competent authority for resolution.

## 12.2	ITU Region 2

12.2.1	North America

In the United States, TDD network operators operating in proximal geographic areas in adjacent bands are encouraged and sometimes required to synchronize their networks and coordinate their TDD configurations to avoid mutual interference. Unsynchronized operation is allowed, more stringent regulation parameters have not been specified for such case but, again, operators would have to work their differences to avoid any claim to Federal Communications Commission (FCC) and Innovation, Science and Economic Development Canada (ISED). FCC requires 3450 – 3550 MHz service (AMBIT band) licensees to negotiate with 3550 – 3700 MHz (CBRS band) licensees to enable TDD synchronization across these services [54]-[55]. Notice that the term TDD synchronization refers to aligning TDD uplink and downlink slots. FCC recognizes the potential for harmful interference from a high-power AMBIT band downlink transmission to a CBRS band uplink. Licensees in the 3700 – 3980 MHz band (C-Band) are encouraged to explore synchronization of TDD operations to minimize interference between adjacent band services [56].

The shared band 48/n48 (3550 – 3700 MHz), also known as the CBRS band, requires spectrum sharing among three tiers of users controlled by one or multiple spectrum access systems (SASs) [57]. Coexistence, including TDD synchronization, among cellular users within the band is supported by OnGo Alliance coexistence requirements set forth in OnGo-TS-2001 [58].

The ISED Canada is reallocating portions of the 3500 to 4200 MHz band as TDD bands for cellular use. The ISED is considering TDD synchronization as a means of facilitating sharing and co-existence with adjacent band services [59].

Currently there are no specific regulatory requirements for SBFD operation in North America. Some SBFD operations result in similar interference scenarios as found in unsynchronized TDD systems. The potential coexistence risk introduced by SBFD may break the standard body agreement on TDD synchronization by OnGo Alliance coexistence requirements set forth in OnGo-TS-2001 [58].

## 12.3	ITU Region 3

12.3.1	Australia

In Australia there are frame structure requirements which only apply when interference occurs between licences and there is no agreement between licensees on how to resolve it. Operators can use different frame structures if there are no issues.

12.3.2	China

In China, spectrum is allocated with clearly stating it for TDD or FDD operation. Besides, spectrum is allocated to operators with specified RF requirements.

For the same TDD operation band, now only synchronization operation is allowed between operators owning adjacent carriers among one TDD operation band. The Ministry of Industry and Information Technology (MIIT) has specified RF requirements assuming the synchronization operation. There are no regulatory requirements about the TDD pattern choice, instead, operators will determine final TDD pattern provided adjacent channel network are synchronized operation.

For adjacent TDD operation band and FDD operation band, inter-operator gNB-gNB CLI occurs. To avoid such interference, MIIT specify some interference mitigation scheme, e.g., frequency guard band and minimum spatial isolation requirement. One example is the interference between band 39 and band 1/band 3. To avoid severe gNB-to-gNB interference, a 5MHz frequency guard band is reserved between two adjacent operation bands. Besides, 50dB MCL is required between different operators’ gNB.

There are no SBFD regulatory requirements in China until now. MIIT mainly cares interference between different operators. Necessary interference coordination mechanism and solutions may be proposed by MIIT to avoid interference before any SBFD deployment.

12.3.3	India

In India no frame structure is mandated. In case operators have incompatible frame structures resulting in interference then the responsibility of mitigating interference falls amongst the operators.

12.3.4	Japan

No TDD pattern has been mandated in Japan, but operators are required to coordinate their TDD patterns. Operators are allowed to use unsynchronized operation if operators can get necessary agreements with the stakeholders.

12.3.5	Korea

In Korea, there is no frame structure that is mandated. The principle is that the operators should confer among themselves to solve any coexistence problems, if such problems exist, and implement an interference protection and avoidance plan in the “Frequency Use Plan”.

According to “Ministry of Science and ICT Announcement No. 2018-235,” frequency allocation notice for mobile communications (3.5GHz), it was an official requirement in Korea that operators have to present a “Frequency Use Plan”, in which the solution or network construction plan to avoid interference must be officially listed [60].

Based on Article 11 of Enforcement Decree Of The Radio Waves Act in Korea, “Matters related to frequency use and technology methods,” a new regulatory requirement has been adopted whenever a new spectrum is introduced regardless the band is licensed or unlicensed. Additionally, the regulatory requirement clearly describes what multiplexing technology, modulation technology, frequency band, occupied bandwidth, etc. are used. MSIT in Korea could review the SBFD as a new multiplexing technology, in which new regulatory requirement might be introduced [61].

12.3.6	New Zealand

In New Zealand a TDD pattern has been mandated and in addition the networks must be time synchronised. Operator deployments that do not conform to the synchronisation requirement must not interfere with deployments that are conforming with the described synchronisation requirements, and therefore cannot claim protection from interference [62]. Therefore, it will be difficult to introduce SBFD without a regulatory rule change.

12.4	Summary

At present, many bands are issued by regulators with clearly defined duplex modes, i.e., FDD or TDD, and probably SDL or SUL. The evolution of NR duplex operation, as a new technology, may require regulators to consider the new regulatory requirements and/or update the ruling.

At least for regions studied so far, there is no regulation rule directly related to SBFD operation. The evolution of NR duplex operation would bring changes to the frame structures of legacy TDD operation, which has been assumed in many regions for coexistence. As a result, rules related to TDD synchronization and interference to incumbent services may be impacted.

Regulators try to harmonize spectrum usage and pay attention to the use of new technologies that might create interference to incumbent services operating in or adjacent to the considered spectrum. When allocating spectrum to IMT TDD operation, many regulators made coexistence studies with incumbent services assuming a certain TDD configuration. Based on the conclusions of those studies, regulators have then specified the corresponding parameters to enable such deployment. Existing 3GPP specifications e.g., TS 38.104, assume the TDD base stations deployed in the same geographical area and using the same or adjacent operating bands are synchronized. Unsynchronized operations have not been considered and so, no specific RF requirements are defined for the unsynchronized operations.

Some regulators and regional organizations (e.g., CEPT ECC in Europe) have recommended specific TDD frame structure usage to facilitate coordination, addressing the cross-border issues between countries. In most studied regions, to avoid cross-link interference situations, regulatory conditions at the national/regional level define the common TDD frame structures for multiple operators’ operations in or adjacent to the considered spectrum, or administrations ask MNOs to agree on a common frame structure for Macro cellular deployments.

To enable unsynchronized TDD deployments without creating interference in the network(s) deployed in the same geographic areas, some regulators have specified more stringent parameters (e.g., CEPT specified below and above the block edge a restricted baseline of -34dBm/5 MHz EIRP for non-AAS BS or -43dBm/MHz TRP for AAS BS), which usually increases BS design’s complexity. For adjacent TDD operation band and FDD operation band, some regulators (e.g., MIIT in China) specify interference mitigation scheme such as guard band and minimum spatial isolation requirement.

Nevertheless, when deployed in environments that guarantee and prevent any interference in the adjacent spectrum (e.g., isolated indoor deployment), neither specific condition nor recommendation has been specified by the Regulators, allowing any TDD deployment in such environments as long as no interference disturbs adjacent services. For example, in a single operator’s TDD network, there may be no limitation on the frame structure and it is up to the operator’s choice. It is already possible today to use different TDD frame structures for isolated deployment, e.g., isolated indoor factory, as long as the obligation to avoid interference is guaranteed. For such types of deployments, existing regulation rules should not be impacted when operating SBFD.

SBFD operation would allow simultaneous transmission and reception in different sub-bands within the same carrier. New regulatory requirements or changes to current regulations may be needed to allow SBFD operation for multiple operators’ deployment.

13	Conclusions and recommendations

13.1	SBFD

13.1.1	RAN1

13.1.1.1	UPT performance

All conclusions are based on SLS evaluation results with no less than 3 sources for single operator scenarios (SBFD deployment case 1) and no less than 2 sources for 2-Layer scenarios (SBFD deployment case 3-2) and two operator scenarios (SBFD deployment case 4), respectively.

RAN1 did not draw any conclusion on the performance of SBFD operation with “same total number of antenna elements and half the total number of TxRUs” and “same total number of antenna elements and same total number of TxRUs” with respect to semi-static TDD due to fewer than 3 sources for SBFD deployment case 1 and 2 sources for SBFD deployment case 3-2 and SBFD deployment case 4. The summary of observations for the above cases are included in Section 7.3.1.

All conclusions are drawn with assumption of 1dB desense for self-interference suppression and “twice the total number of antenna elements and same total number of TxRUs”.

In the Urban Macro or Dense Urban Macro layer, the co-site inter-sector spatial isolation value is categorized into three cases.

- Less than 93dB for FR1 (98dB for FR2-1) includes spatial isolation values of no lower than 75dB (88dB for FR2-1), which is a typical spatial isolation value from RAN4.

- Equal to 93dB for FR1 (98dB for FR2-1) is the best spatial isolation value from RAN4.

- No less than 93dB for FR1 (98dB for FR2-1) includes spatial isolation values of up to 110dB (115dB for FR2-1) including 10dB digital cancelation value.

In this section, {X%,Y%,Z%} notation represents X% UPT gain or loss for low load level, Y% UPT gain or loss for medium load level, and Z% UPT gain or loss for high load level, respectively. The values X, Y, Z are median values of all evaluation result for a given evaluation assumption.

SBFD deployment case 1 (Non-coexistence case with same SBFD configuration amongst gNBs)

For SBFD deployment case 1, SBFD with XXXXX slot format (X is SBFD slot with UL/DL subband) are assumed as compared to semi-static TDD (DDDSU)

- For FR1 indoor scenario, semi-static SBFD provides performance improvement for both DL and UL for all load levels and small/large packet size except for limited 5% DL UPT loss at medium load level and large packet sizes

- In case of small packet size,

  - {9.56%, 9.35%, 8.58%} / {10.50%, 12.71%, 8.79%} for mean/5% DL UPT gain

  - {101.83%, 97.42%, 93.85%} / {107.58%, 105.44%, 106.52%} for mean/5% UL UPT gain

- In case of large packet size,

  - {1.86%, 2.21%, 2.73%} / {1.73%, -1.19%, 0.54%} for mean/5% DL UPT gain/loss

  - {10.78%, 13.38%, 13.75%} / {14.13%, 19.91%, 17.70%} for mean/5% UL UPT gain

- For FR2-1 indoor scenario, semi-static SBFD provides performance improvement for both DL and UL for all load levels and both small/large packet sizes, except 5% DL UPT loss at high load level and small packet sizes

- In case of small packet size,

  - {4.84%, 7.57%, 5.95%} / {5.18%, 4.45%, -10.25%} for mean/5% DL UPT gain/loss

  - {55.30%, 54.71%, 72.66%} / {50.71%, 46.45%, 59.26%} for mean/5% UL UPT gain

- In case of large packet size,

  - {3.63%, 3.36%, 3.60%} / {6.03%, 6.67%, 3.35%} for mean/5% DL UPT gain

  - {16.60%, 22.22%, 20.61%} / {38.79%, 71.05%, 86.18%} for mean/5% UL UPT gain

- For Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB,

  - In case of small packet size, semi-static SBFD provides performance improvement for DL for low load level and for UL for all load levels (for at least one of mean and 5% UPT)

  - {6.53%, -1.96%, -19.87%} / {-0.51%, -68.33%, -85.70%} for mean/5% DL UPT gain/loss

  - {97.24%, 69.31%, 62.22%} / {67.19%, 38.66%, -0.67%} for mean/5% UL UPT gain/loss

  - In case of large packet size, semi-static SBFD provides performance improvement for UL for low load level

  - {-8.17%, -30.29%, -33.03%} / {-9.59%, -73.35%, -89.16%} for mean/5% DL UPT loss

  - {24.00%, -2.88%, -17.64%} / {164.97%, -52.87%, -69.03%} for mean/5% UL UPT gain/loss

- For Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB,

  - In case of small packet size, semi-static SBFD provides performance improvement for UL for low load level and medium load levels (for at least one of mean and 5% UPT)

  - {-2.59%, -6.88%, -17.55%} / {-37.98%, -48.89%, -78.27%} for mean/5% DL UPT loss

  - {34.65%, 18.85%, -1.33%} / {18.12%, -13.03%, -38.49%} for mean/5% UL UPT gain/loss

  - In case of large packet size, semi-static SBFD provides performance improvement for UL for low load level

  - {-8.47%, -34.75%, -32.86%} / {-4.26%, -88.67%, -84.60%} for mean/5% DL UPT loss

  - {8.54%, -23.92%, -78.23%} / {187.62%, -45.51%, -69.03%} for mean/5% UL UPT gain/loss

- For Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is less than 93 dB,

- In case of small packet size, semi-static SBFD provides no performance improvement

  - {-14.55%, -15.11%, -20.87%} / {-78.16%, -98.06%, -99.62%} for mean/5% DL UPT loss

  - {-6.50%, -27.59%, -43.73%} / {-98.32%, -100%, -100%} for mean/5% UL UPT loss

- In case of large packet size, semi-static SBFD provides no performance improvement

  - {-25.70%, -40.50%, -45.29%} / {-75.30%, -94.59%, -97.64%} for mean/5% DL UPT loss

  - {-31.00%, -74.48%, -86.16%} / {-100%, -100%, -100%} for mean/5% UL UPT loss

- For Dense Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB,

- In case of small packet size, semi-static SBFD provides performance improvement for UL for all load levels (for at least one of mean and 5% UPT)

  - {1.90%, -5.76%, -12.57%} / {0.01%, -24.09%, -67.74%} for mean/5% DL UPT gain/loss,

  - {94.73%, 63.40%, 32.12%} / {36.17%, -37.56%, -88.59%} for mean/5% UL UPT gain/loss

- In case of large packet size, semi-static SBFD provides performance improvement for DL and UL for low load level (for at least one of mean and 5% UPT)

  - {1.18%, -5.57%, -11.35%} / {-1.49%, -27.51%, -58.09%} for mean/5% DL UPT gain/loss

  - {20.54%, -12.87%, -49.27%} / {15.51%, -59.62%, -96.97%} for mean/5% UL UPT gain/loss

- For Dense Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB,

- In case of large packet size, semi-static SBFD provides performance improvement for UL for all load levels

  - {1.18%, -1.58%, -6.07%} / {-1.49%, -5.34%, -10.59%} for mean/5% DL UPT gain/loss,

  - {28.96%, 21.52%, 17.32%} / {52.17%, 55.17%, 25.98%} for mean/5% UL UPT gain

- For Dense Urban Macro layer (FR2-1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 98 dB,

- In case of small packet size, semi-static SBFD provides performance improvement for both DL and UL for all load levels except 5% DL UPT loss at high load level

  - {4.69%, 2.57%, 0.90%} / {4.20%, 1.27%, -7.41%} for mean/5% DL UPT gain/loss

  - {57.78%, 49.40%, 41.22%} / {65.51%, 63.03%, 96.07%} for mean/5% UL UPT gain

- In case of large packet size, semi-static SBFD provides performance improvement for DL for low load level and performance improvement for UL for all load levels

  - {2.45%, 2.18%, 1.48%} / {2.30%, 1.17%, -1.37%} for mean/5% DL UPT gain/loss

  - {37.31%, 30.11%, 3.11%} / {19.44%, 30.93%, 11.76%} for mean/5% UL UPT gain

- For Dense Urban Macro layer (FR2-1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is less than 98 dB,

- In case of large packet size, semi-static SBFD provides limited performance improvement for DL for low and medium load levels and significant performance improvement for 5% UL UPT for low and medium load levels (for at least one of mean and 5% UPT)

  - {1.48%, 1.65%, 1.23%} / {2.17%, 0.70%, -2.28%} for mean/5% DL UPT gain/loss

  - {8.88%, -31.13%, -74.61%} / {159.27%, 127.28%, -52.69%} for mean/5% UL UPT gain/loss

- For Dense Urban Macro layer (FR2-1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 98 dB,

- In case of large packet size, semi-static SBFD provides limited performance improvement for DL for low and medium load levels (for at least one of mean and 5% UPT) and performance improvement for UL for all load levels (for at least one of mean and 5% UPT)

  - {1.33%, 1.60%, 1.36%} / {1.71%, -1.26%, -5.34%} for mean/5% DL UPT gain/loss,

  - {36.23%, 18.49%, -17.88%} / {18.52%, 28.87%, 10.78%} for mean/5% UL UPT gain/loss

For SBFD deployment case 1, SBFD with XXXXU slot format is assumed,

- For FR1 indoor scenario, semi-static SBFD provides performance improvement for UL but may suffer from degradation for DL

- In case of small packet size,

  - {-0.52%, -1.19%, -6.26%} / {-0.35%, -1.66%, -17.59%} for mean/5% DL UPT loss

  - {99.50%, 98.83%, 104.00%} / {116.24%, 110.00%, 127.81%} for mean/5% UL UPT gain

- In case of large packet size,

  - {-20.38%, -26.30%, -33.95%} / {-22.88%, -29.57%, -53.83%} for mean/5% DL UPT loss

  - {78.53%, 93.92%, 113.75%} / {81.03%, 106.39%, 150.17%} for mean/5% UL UPT gain

- For FR2-1 indoor scenario, semi-static SBFD provides performance improvement for UL but may suffer from degradation for DL

- In case of small packet size,

  - {-4.40%, -6.90%, -22.06%} / {-22.33%, -30.02%, -40.36%} for mean/5% DL UPT loss

  - {78.57%, 91.25%, 149.67%} / {84.98%, 102.86%, 201.41%} for mean/5% UL UPT gain

- For Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB,

- In case of small packet, semi-static SBFD provides significant performance improvement for UL but suffers from degradation for DL

  - {-2.92%, -10.59%, -20.61%} / {-7.18%, -18.66%, -57.30%} for mean/5% DL UPT loss

  - {126.08%, 120.22%, 107.06%} / {199.31%, 206.18%, 205.37%} for mean/5% UL UPT gain

- In case of large packet, semi-static SBFD provides performance improvement for UL but suffers from degradation for DL

  - {-25.62%, -28.92%, -47.44%} / {-25.50%, -57.92%, -85.67%} for mean/5% DL UPT loss

  - {66.15%, 53.69%, 40.66%} / {170.00%, 82.67%, 50.34%} for mean/5% UL UPT gain

- For Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is less than 93 dB,

- In case of large packet, semi-static SBFD provides no performance improvement, except limited mean UL UPT gain for low load level and 5% UL UPT gain for high load

  - {-32.72%, -50.26%, -52.78%} / {-73.00%, -96.20%, -99.73%} for mean/5% DL UPT loss

  - {1.27%, -37.96%, -35.17%} / {-31.70%, -26.68%, 14.21%} for mean/5% UL UPT gain/loss

- For Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB,

- In case of small packet, semi-static SBFD provides significant performance improvement for UL but suffers from degradation for DL

  - {-0.04%, -2.62%, -12.55%} / {-0.30%, -5.88%, -23.21%} for mean/5% DL UPT loss

  - {71.55%, 89.49%, 102.27%} / {273.08%, 238.46%, 198.00%} for mean/5% UL UPT gain

- In case of large packet, semi-static SBFD provides performance improvement for UL but suffers from degradation for DL

  - {-25.79%, -35.29%, -51.62%} / {-25.50%, -57.92%, -81.73%} for mean/5% DL UPT loss

  - {63.70%, 44.06%, 40.66%} / {203.20%, 55.93%, 42.37%} for mean/5% UL UPT gain

- For Dense Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB,

- In case of large packet size, semi-static SBFD provides significant performance improvement for UL for all load levels.

  - {-26.93%, -31.81%, -38.12%} / {-27.97%, -42.92%, -64.49%} for mean/5% DL UPT loss

  - {90.67%, 97.71%, 68.10%} / {102.57%, 103.45%, 147.37%} for mean/5% UL UPT gain

- For Dense Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB,

- In case of large packet, semi-static SBFD provides significant performance improvement for UL but suffers from degradation for DL

  - {-21.92%, -21.92%, -28.36%} / {-33.05%, -32.32%, -51.08%} for mean/5% DL UPT loss

  - {123.47%, 112.72%, 123.71%} / {104.36%, 139.28%, 147.37%} for mean/5% UL UPT gain

- For Dense Urban Macro layer (FR2-1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 98 dB,

- In case of large packet size, semi-static SBFD provides performance improvement for UL but suffers from degradation for DL

  - {-20.50%, -28.49%, -37.91%} / {-22.48%, -47.88%, -49.87%} for mean/5% DL UPT loss

  - {79.47%, 96.75%, 123.96%} / {75.86%, 139.31%, 100.01%} for mean/5% UL UPT gain

For SBFD deployment case 1, SBFD with DXXXU slot format is assumed,

- For Urban Macro (FR1), if the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB,

- In case of large packet size, semi-static SBFD provides performance improvement for UL but suffers from degradation for DL

  - {-23.21%, -35.27%, -50.23%} / {-46.49%, -78.32%, -79.73%} for mean/5% DL UPT loss

  - {72.55%, 47.69%, 33.76%} / {126.00%, 90.91%, 55.81%} for mean/5% UL UPT gain

SBFD deployment case 3-2 (Co-channel co-existence case)

For the indoor layer of 2-layer scenario (FR1) in SBFD deployment case 3-2, when SBFD with XXXXX slot format for indoor layer and TDD with DDDSU for Macro layer are assumed,

- In case of small packet, semi-static SBFD provides performance improvement for both DL and UL for all load levels (for at least one of mean and 5% UPT)

- {5.69%, 5.29%, -2.27%} / {6.87%, 3.42%, 50.93%} for mean/5% DL UPT gain/loss

- {91.80%, 89.00%, 78.10%} / {93.70%, 26.42%, -37.25%} for mean/5% UL UPT gain/loss

- In case of large packet, semi-static SBFD provides performance improvement for UL but suffers from degradation for DL

- {-5.38%, -7.29%, -9.20%} / {-10.49%, -11.87%, -12.30%} for mean/5% DL UPT loss

- {3.90%, 3.38%, 14.78%} / {17.83%, 68.34%, 71.07%} for mean/5% UL UPT gain

For the indoor layer of 2-layer scenario (FR1) in SBFD deployment case 3-2, when SBFD with XXXXU slot format for indoor layer and DDDSU slot format for Macro layer are assumed,

- In case of small packet size, semi-static SBFD provides significant performance improvement for UL but may suffer from degradation for DL for all load levels (for at least one of mean and 5% UPT for DL at low load)

- {0.43%, -1.58%, -13.98%} / {-0.01%, -7.82%, -80.99%} for mean/5% DL UPT gain/loss

- {99.80%, 102.60%, 110.12%} / {104.37%, 91.69%, 218.36%} for mean/5% UL UPT gain

- In case of large packet, semi-static SBFD provides significant performance improvement for UL but may suffer from degradation for DL for all load levels

- {-29.99%, -45.57%, -66.40%} / {-34.28%, -53.32%, -82.87%} for mean/5% DL UPT loss

- {106.57%, 136.02%,214.26%} / {107.83%, 121.56%, 595.02%} for mean/5% UL UPT gain

SBFD deployment case 4 (Adjacent channel co-existence)

For SBFD deployment case 4 (FR1) with 0% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB, spatial isolation for co-site adjacent-channel CLI is no less than 93dB, and SBFD with XXXXX slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for UL at low and medium load levels but suffers from degradation for DL for all load levels

  - {-11.54%, -13.46%, -13.37%} / {-39.64%, -50.44%, -68.58%} for mean/5% DL UPT loss of SBFD operator

  - {21.09%, 18.52%, -27.10%} / {47.60%, 0%, -19.36%} for mean/5% UL UPT gain/loss of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance

  - {-6.46%, -6.73%, -5.22%} / {-29.43%, -39.73%, -53.81%} for mean/5% DL UPT loss of legacy TDD operator

  - {-16.16%, -24.42%, -27.10%} / {-16.18%, 0%, 0%} for mean/5% UL UPT loss of legacy TDD operator

For SBFD deployment case 4 (FR1) with 0% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB, spatial isolation for co-site adjacent-channel CLI is no less than 93dB, and SBFD with XXXXU slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for UL but suffer from degradation for DL for all load levels

  - {-22.97%, -21.22%, -26.20%} / {-27.07%, -52.53%, -65.36%} for mean/5% DL UPT loss of SBFD operator

  - {59.89%, 26.32%, 23.29%} / {168.31%, 37.37%, 24.69%} for mean/5% UL UPT gain of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited improvement and degradation for UL and DL performance

  - {-0.45%, -2.12%, -3.39%} / {-1.52%, -2.25%, -4.86%} for mean/5% DL UPT loss of legacy TDD operator

  - {-0.01%, 0.04%, 0.13%} / {0.07%, 0.10%, 2.08%} for mean/5% UL UPT gain/loss of legacy TDD operator

For SBFD deployment case 4 (FR1) with 0% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB, spatial isolation for co-site adjacent-channel CLI is equal to 93 dB, and SBFD with XXXXX slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for UL at low load level but suffers from degradation for DL for all load levels

  - {-0.6%, -5.70%, -12.29%} / {3.34%, -10.72%, -23.48%} for mean/5% DL UPT gain/loss of SBFD operator

  - {3.50%, -36.04%, -55.59%} / {114.57%, -18.46%, -69.36%} for mean/5% UL UPT gain/loss of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance

  - {-0.44, -3.39%, -4.45%} / {-1.25%, -6.93%, -7.97%} for mean/5% DL UPT loss of legacy TDD operator

  - {-7.43%, -30.66%, -39.94%} / {-16.18%, -46.23%, -49.99%} for mean/5% UL UPT loss of legacy TDD operator

For SBFD deployment case 4 (FR1) with 0% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB, spatial isolation for co-site adjacent-channel CLI is equal to 93dB, and SBFD with XXXXU slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for UL but suffers from degradation for DL for all load levels

  - {-23.27%, -29.59%, -40.53%} / {-23.08%, -38.52%, -49.36%} for mean/5% DL UPT loss of SBFD operator

  - {88.87%, 68.41%, 34.44%} / {168.31%, 37.37%, 24.69%} for mean/5% UL UPT gain of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited improvement or degradation for UL and DL performance

  - {-0.45%, -2.12%, -3.39%} / {-1.52%, -2.25%, -4.86%} for mean/5% DL UPT loss of legacy TDD operator

  - {-0.01%, 0.04%, 0.13%} / {0.07%, 0.10%, 2.08%} for mean/5% UL UPT gain/loss of legacy TDD operator

For SBFD deployment case 4 (FR1) with 100% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB, and SBFD with XXXXX slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for UL for low and medium load levels but suffers from degradation for DL for all load levels

  - {-0.85%, -5.76%, -10.65%} / {-3.79%, -13.28%, 22.06%} for mean/5% DL UPT loss of SBFD operator

  - {21.64%, 13.37%, -11.43%} / {32.42%, 10.67%, -3.28%} for mean/5% UL UPT gain/loss of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance

  - {-0.35%, -3.31%, -5.38%} / {-2.64%, -9.41%, -7.68%} for mean/5% DL UPT loss of legacy TDD operator

  - {-13.50%, -21.26, -16.74%} / {-24.39%, -0.53%, -0.90%} for mean/5% UL UPT loss of legacy TDD operator

For SBFD deployment case 4 (FR1) with 100% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is no less than 93 dB, and SBFD with XXXXU slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for UL but suffers from degradation for DL for all load levels

  - {-22.30%, -24.57%, -25.84%} / {-21.49%, -31.46%, -51.80%} for mean/5% DL UPT loss of SBFD operator

  - {90.01%, 94.07%, 36.70%} / {94.35%, 58.67%, 38.16%} for mean/5% UL UPT gain of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited improvements or degradation for UL and DL performance

  - {-0.30%, -1.61%, -3.21%} / {-0.16%, -3.59%, -3.92%} for mean/5% DL UPT loss of legacy TDD operator

  - {0%, 0%, 0%} / {2.39%, 1.56%, 0.03%} for mean/5% UL UPT gain of legacy TDD operator

For SBFD deployment case 4 (FR1) with 100% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB, and SBFD with XXXXX slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for both UL and DL for low load levels but suffers from degradation for both UL and DL for medium and high load levels

  - {3.11%, -5.76%, -10.65%} / {2.27%, -13.28%, -22.06%} for mean/5% DL UPT gain/loss of SBFD operator

  - {9.77%, -30.95%, -65.59%} / {89.73%, -17.62%, -53.26%} for mean/5% UL UPT gain/loss of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited or large degradation for UL and DL performance

  - {-0.35%, -2.94%, -4.37%} / {-1.40%, -7.02%, -6.72%} for mean/5% DL UPT loss of legacy TDD operator

  - {-6.75%, -26.88%, -37.96%} / {-12.59%, -44.22%, -50.22%} for mean/5% UL UPT loss of legacy TDD operator

For SBFD deployment case 4 (FR1) with 100% grid shift, and the total capability of spatial isolation and digital isolation for co-site inter-sector CLI is equal to 93 dB, and SBFD with XXXXU slot format and large packet size are assumed,

- For the SBFD operator, semi-static SBFD provides performance improvement for UL for all load levels but suffers from degradation for DL for all load levels

  - {-24.13%, -24.72%, -25.84%} / {-15.39%, -17.56%, -33.07%} for mean/5% DL UPT loss of SBFD operator

  - {101.42%, 95.42%, 36.70%} / {120.78, 58.67%, 38.16%} for mean/5% UL UPT gain of SBFD operator

- For the legacy TDD operator, regarding the performance impact of semi-static SBFD to legacy TDD of another operator, there may be limited degradation for DL performance and limited improvement or degradation for UL performance

  - {-0.30%, -1.61%, -3.21%} / {-0.16%. -3.59%, -3.92%} for mean/5% DL UPT loss of legacy TDD operator

  - {0%, 0%, 0%} / {2.39%, 1.56%, -3.00%} for mean/5% UL UPT gain/loss of legacy TDD operator

RAN1 concluded that DL/UL UPT gain and loss at least come from the following reasons

- In case of using SBFD with XXXXX slot format, the UL UPT gain at least comes from more UL transmission opportunities for semi-static SBFD compared to legacy TDD, and the DL UPT gain at least comes from the more DL transmission opportunities for semi-static SBFD compared to legacy TDD.

- In case of using SBFD with XXXXU or DXXXU slot format, the UL UPT gain at least comes from more UL resources and more UL transmission opportunities for semi-static SBFD compared to legacy TDD, and the DL UPT loss for SBFD at least comes from less DL resources for semi-static SBFD compared to legacy TDD.

- The UL UPT loss at least comes from inter-site gNB-gNB CLI and co-site inter-sector gNB-gNB CLI for Urban Macro and Dense Urban Macro layer. The impact of co-site inter-sector gNB-gNB CLI depends on co-site inter-sector CLI suppression capability. Also, the UL UPT loss at least comes from noise figure increase due to higher blocker power.

- For SBFD deployment case 4, for the SBFD operator, the UL UPT loss at least comes from inter-site adjacent-channel gNB-gNB CLI and co-site adjacent-channel gNB-gNB CLI for Urban Macro and Dense Urban Macro layer. The impact of co-site adjacent-channel gNB-gNB CLI depends on co-site adjacent-channel CLI suppression capability. Also, the UL UPT loss at least comes from noise figure increase due to higher blocker power by adjacent-channel CLI.

- For SBFD deployment case 4, for the legacy TDD operator in the case of XXXXX slot format, adjacent channel gNB-gNB CLI causes loss.

- The DL UPT loss at least comes from UE-to-UE CLI.

13.1.1.2	Coverage performance

Based on link level simulation, comparing SBFD with XXXXU slot format and legacy TDD with DDDSU slot format, RAN1 observed, with assumption of 1dB desense for self-interference suppression and different co-site inter-sector isolation values,

- semi-static SBFD with PUSCH repetition type A without/with joint channel estimation provides the UL coverage gain in range of {0.00~6.75}dB and median value of 5.41dB from 13 sources in FR1 UMa and in range of {5.86~8.76}dB and median value of 6.92dB from 4 sources in FR2-1 Dense UMa, respectively.

- semi-static SBFD with TBoMS with/without joint channel estimation provides the UL coverage gain in range of {2.83~6.88}dB and median value of 5.09dB from 4 sources in FR1 UMa and in range of {4.49~7.82}dB and median value of 5.72dB from 2 sources in FR2-1 Dense UMa, respectively.

13.1.1.3	SBFD operation scheme

SBFD operation at gNB for UEs was studied under the following assumptions,

  - SBFD operation within a TDD carrier,

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies, and

  - Up to one UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier.

RAN1 concluded SBFD operation Option 4 is feasible for RRC_CONNECTED state from the RAN1 specification perspective, where SBFD operation Option 4 assumes

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side from RAN1 specification point of view.

To support SBFD operation Option 4 for RRC_CONNECTED state, RAN1 identified the following potential specification impact for SBFD-aware UE:

  - Indication of time and frequency domain locations of SBFD subbands to UEs

  - UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols

13.1.1.4	CLI handling scheme

For semi-static SBFD, the gNB-to-gNB co-channel CLI handling schemes and UE-to-UE co-channel CLI handling schemes were studied, including performance and specification impact, which are included in Section 7.4.2, Section 7.4.3 and Section 7.4.4. The summary of observations for co-channel CLI handling schemes are included in Section 7.4.

13.1.2	RAN4

13.2	Dynamic/flexible TDD

For dynamic/flexible TDD, the gNB-to-gNB co-channel CLI handling schemes and UE-to-UE co-channel CLI handling schemes, which can be specific for dynamic/flexible TDD and/or common for both SBFD and dynamic/flexible TDD, were studied, including analysis, performance and specification impact, which are included in Section 8.3 and Section 8.4. The summary of observations for gNB-to-gNB CLI handling schemes are included in Section 8.3.

## <<Next Change for TR 38.858>>

Annex <E>:
Adjacent channel co-existence evaluation

E.1	RAN4 co-existence simulation scenarios

The coexistence evaluation is conducted considering the different scenarios listed in Table E.1-1.

Table E.1-1: Network deployment scenarios

| Scenario | FR | Aggressor | Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | High |
| 2 | FR1 | Urban Hotspot | Urban Hotspot | Low |
| 3 | FR1 | Indoor | Indoor | Low |
| 4 | FR1 | Urban Macro | Micro | High |
| 5 | FR1 | Micro | Micro | Low |
| 6 | FR2-1 | Urban Macro | Urban Macro | High |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot | Low |
| 8 | FR2-1 | Urban Dense | Urban Dense | Low |
| 9 | FR2-1 | Indoor | Indoor | Low |
| Note 1: Scenario 7 has been down selected. |

The Urban Hotspot reuses most parameter assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE as described in Table E.2.1-1 and Table E.2.1-2 and Table E.2.1-3.

The coexistence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in a neighbouring channel, also to understand impact on SBFD network due to the legacy TDD network, as described in Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable in terms of performance (based on RAN4 models and parameters) to the {D, U, D} SBFD configuration.

Table E.1-2: Coexistence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim | Priority |
| --- | --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  | High |
| 2 | SBFD | TDD UL |  | Low |
| 3 | TDD DL | SBFD |  | High |
| 4 | TDD UL | SBFD |  | Low |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |

E.2	RAN4 co-existence simulation assumptions

E.2.1	Deployment

Simulation assumptions related to network layout is captured for FR1 in Table E.2.1-1 and for FR2-1 in Table E.2.1-2.

Simulation assumptions related to cluster-based UE dropping are captured in Table E.2.1-3.

Table E.2.1-1: FR1 deployment parameters

| Parameter | Urban Macro (Note 1) | Micro | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Single layer with 19 hexagonal cells with wrap around. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 500 m | 289 m | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 50%, 10% | Baseline: 100% The victim network (UMi) in the center, the aggressor network is moved by the grid offset. 100% is relative to the micro ISD. 2nd priority: Other values | N/A |
| Carrier frequency | 4 GHz | 4 GHz | 4 GHz |
| Path-loss model | BS-to-UE: UMa see TR 38.803 BS-to-BS: 1st priority: Reuse the same model as in TR 38.828 with h_UT equals to 25m. 2nd priority: If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-UE: UMi see TR 38.803 BS-to-BS: for the UMa-to-UMi scenario: UMa (h_UE = 10 m) see TR 38.803. For LoS probability for Macro-to-Micro case: Reuse the same model as in TR 38.828 with h_UT = 10 m for the UMi-to-UMi scenario: UMa (h_UE = 10 m) see TR 38.803. UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803. UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m | 1.5 m |
| UE distribution | Random dropping (Note 1) Baseline: 20% indoor and 80% outdoor Optional: 80% indoor and 20% outdoor | Uniformly UE distribution in the cell 20% indoor and 80% outdoor | 100% indoor |
| O-to-I penetration loss model | 80% low-loss model and 20% high-loss model | 80% low-loss model and 20% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 5 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 6 degrees | 90 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes Cluster-based drop. See Table E.2.1-3 for Cluster-based dropping. |  |  |  |

Table E.2.1-2: FR2-1 deployment parameters

| Parameter | Urban Macro (Note 1) | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Layout | Single layer with 19 hexagonal cells with wrap around. | Fixed cluster circle within a macro cell. Number of micro BSs per macro cell is 3. Radius of UE dropping within a micro cell is < 28.9 m. Minimum distance between micro BSs in different operator is 10 m. Shadowing correlation is 0.5. | Total 12 BSs (Operator A: 6 BSs & Operator B: 6 BSs) 120 x 50 m |
| Inter-BS distance | 200 m | 10 m Minimum distance between micro BSs in different operator | 20 m |
| Grid shift | Baseline: 100% 2nd priority: Other values, e.g., 50%, 10% | N/A | N/A |
| Carrier frequency | 30 GHz | 30 GHz | 30 GHz |
| Path-loss model | BS-to-UE: UMa + penetration loss see TR 38.803 BS-to-BS: 1st priority: Reuse the same model as in TR 38.828 with h_UT equals to 25m. 2nd priority: If the 2D distance between two BS are less than or equal to the ISD, set the LOS probability to X; Otherwise, reuse BS-to-UE LOS probability equation in TR 38.828. X = 0.75. For other cases, reuse BS-to-UE LOS probability equation in TR 38.803. UE-to-UE: UMi (h_BS=1.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-UE: UMi + penetration loss BS-to-BS: UMi UE-to-UE: UMi (h_BS=1.5m~22.5m) + penetration loss, see TR 38.803. UMi model is not applicable when 2D distance is less than 10 m, instead FSPL is applicable. | BS-to-BS: InH-office BS-to-UE: InH-office UE-to-UE: InH-office As described in TR 38.803, subclause 5.2.2.1 |
| Site-interference level | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | Baseline: 6 dB below noise floor (Mco-site = 6 dB) (corresponding to 1 dB desens) Optional: A reported value | N/A |
| BS height | 25 m | 10 m | 3 m |
| UE height | 1.5 m | 1.5 m ≦ hUT ≦ 22.5 m | 1.5 m |
| UE distribution | Indoor UE ratio: 0% Uniform UE distribution | Indoor UE ratio: 80 % Uniform UE distribution | 100% indoor |
| O-to-I penetration loss model | N/A | 50% low-loss model and 50% high-loss model | N/A |
| Minimum BS-UE (2D) distance | 35 m | 3 m | 0 m |
| Minimum UE-UE (2D) distance | 3 m | 3 m | 1 m |
| DL power control | No | No | No |
| UL power control | Yes | Yes | Yes |
| Handover margin | 3 dB | 3 dB | 3 dB |
| BS mechanical down-tilt angle | 6 degrees | 10 degrees | 90 degrees |
| Note 1: The Urban Hotspot reuses the assumptions of Urban Macro, except Urban Hotspot assumes Cluster-based drop. See Table E.2.1-3 for Cluster-based dropping. |  |  |  |

[Table E.2.1-3: Cluster-based UE dropping parameters

| Parameter | Urban Hotspot |
| --- | --- |
| Cluster number per macro cell | 1 |
| Cluster area | Circular area with 25 m radius |
| UE distribution | UEs dropped within a cluster are indoor and UEs dropped outside a cluster are outdoor |
| Indoor and outdoor ratio | 80% indoor and 20% outdoor |
| Minimum (2D) distance | UE-UE: 1 m Cluster center – cluster center: 50 m Macro TRP – cluster center: 60 m (Note 1) |
| UE dropping | Evenly random dropping into the cluster with minimum UE-UE distance limitation |
| Note 1: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro TRP to cluster centre should be respected also for TRPs belonging to the other operator |  |

]

E.2.2	Traffic

Assumptions related to network traffic load configuration for FR1 is captured in Table E.2.2-1 and for FR2-1 in Table E.2.2-2.

Table E.2.2-1: FR1 network traffic parameters

| Parameter | Urban Macro | Micro | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 100 MHz | 100 MHz | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 80 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz | For legacy TDD: 100 MHz For SBFD {DU}: 20 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD | The PSD of SBFD is the same as TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

Table E.2.2-2: FR2-1 network traffic parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Carrier bandwidth | 200 MHz | 200 MHz | 200 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 160 MHz |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For SBFD {DU}: 40 MHz | For legacy TDD: 200 MHz For {DU}: 40 MHz |
| SBFD BS PSD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD | The PSD of SBFD is the same as legacy TDD |
| Traffic load | Full buffer | Full buffer | Full buffer |

E.2.3	BS RF characteristics

The SBFD system is defined for two different antenna configurations:

- SBFD config 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- SBFD config 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

As baseline the array antenna model, for single element configuration, is described in TR 38.803, subclause 5.2.3. As an option the antenna model extension, required for modelling sub-array configurations, is described in TR 38.803, subclause 5.2.3.2.4.

As an option, power boosting of 3 dB can be considered to compensate for fewer transmitter branches in the case of SBFD config 1.

Assumptions related to BS RF characteristics relevant for different deployment scenarios are captured for FR1 in Table E.2.3-1 and for FR2-1 in Table E.2.3-2.

Table E.2.3-1: FR1 BS parameters

| Parameter | Urban Macro (Wide Area) | Micro (Medium Range) | Indoor (Local Area) |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | Option 1: TDD: 49 dBm SBFD config 1: 46 dBm SBFD config 2: 49 dBm Option 2: TDD: 53 dBm SBFD config 1: 50 dBm SBFD config 2: 53 dBm | TDD: 46 dBm SBFD: 46 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) SBFD config 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) SBFD config 2: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | TDD: (Mg,Ng,M,N,P)=(1,1,2,2,2) SBFD: (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.8)λ, (3dB,3dB)=(65,65) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi | TDD: (Mg,Ng,M,N,P) =(1,1,4,4,2) SBFD config 1: (Mg,Ng,M,N,P) =(1,1,2,4,2) SBFD config 2: (Mg,Ng,M,N,P) =(1,1,4,4,2) (dH,dV)=(0.5,0.5)λ, (3dB,3dB)=(90,90) o SLAv=25 dB, Am=25 dB, GE,max=5 dBi |
| Receiver noise figure model parameters | Baseline: P1=-43 dBm P2=-25 dBm F1=5 dB F2=14 dB Optional Enhanced NF model): P1=-33 dBm P2=-15 dBm F1=5 dB F2=14 dB See Figure E.2.3-1 | P1=-38 dBm P2=-20 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-35 dBm P2=-17 dBm F1=13 dB F2=22 dB See Figure E.2.3-1 |
| ACLR | 45 dBc | 45 dBc | 45 dBc |
| ACS | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc | Baseline: 50 dBc Optional: A reported value within the range 46 to 62 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

Table E.2.3-2: FR2-1 BS parameters

| Parameter | Urban Macro | Urban Dense | Indoor |
| --- | --- | --- | --- |
| Transmitter power (Total conducted power) | 1st priority: TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm 2nd priority: TDD: 40 dBm SBFD config 1: 37 dBm SBFD config 2: 40 dBm | TDD: 30 dBm SBFD config 1: 27 dBm SBFD config 2: 30 dBm | TDD: 24 dBm SBFD config 1: 21 dBm SBFD config 2: 24 dBm |
| Antenna configuration | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi Optional: (3dB, 3dB)=(90,90) o, SLAv=30 dB, Am=30 dB, GE,max=5.5 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,8,16,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,4,16,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(65,65) o, SLAv=30 dB, Am=30 dB, GE,max=3 dBi | TDD: (Mg,Ng,M,N,P)=(1,1,4,8,2) SBFD config 1: (Mg,Ng,M,N,P)=(1,1,2,8,2) SBFD config 2: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(90,90) o, SLAv=30 dB Am=30 dB, GE,max=3 dBi |
| Receiver noise figure parameters | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 | P1=-58 dBm P2=-40 dBm F1=10 dB F2=19 dB See Figure E.2.3-1 |
| ACLR | 28 dBc | 28 dBc | 28 dBc |
| ACS | 24 dBc | 24 dBc | 24 dBc |
| BS self-interference receiver sensitivity degradation | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) | 1 dB (Corresponds to Mself=6 dB) |

To capture receiver blocking aspects the receiver noise figure (Fblock) will be characterised as a piece-wise linear function of input receiver power (Pin) as shown in Figure E.2.3-1.

Figure E.2.3-1: Receiver blocking model

Relevant values for F, F2, P1 and P2 for different BS classes are listed in Table E.2.3-1 and Table E.2.3-2. If the total received power is larger than P2 the receiver will be blocked, which correspond to zero throughput.

The input power to the blocking model is defined as average total power in dBm and the output is the noise figure in dB including noise rise due to receiver blocking aspects, such as e.g., receiver intermodulation.

The receiver input total power can be expressed as a sum of wanted signal and all interferer signals in linear scale as:

Pin=Pw+Iintra−SB, inter−UE+Iinter−SB,inter−gNB+IACI+Iself+Ico−site,inter−sector

, where the signal and interferer sources are defined as:

Pw is the wanted signal received from scheduled UE. The wanted signal level is calculated as:
Pw = PTX,UECLUE→serving gNB

Iintra-SB,inter-UE is the interference from UEs transmitting within the UL sub-band in the victim network. The interference level is calculated as:

Iintra−SB,inter−UE = PTX,UECLUE→serving BS

Iinter-SB,inter-BS is the interference from BS transmitting on the adjacent DL sub-band in the victim network. The interference level is calculated as:

Iinter−SB,inter−BS=PTX,BSCLown network→serving BS

IACI is the adjacent channel interference from BS transmitting on the DL channel in the aggressor network. The interference level is calculated as:

IACI=PTX,BSCLadjacent network gNB→serving BS

Iself is the victim self-interference power. The interference level is calculated as:

Iself=kTBSB FMself

Ico-site,inter-sector is the interference from neighboring sectors in a 3-sector configuration. The interference level is calculated as:

Ico−site,inter−sector=kTBSB FMco−site

, where

PTX,UE is the configured UE transmitter power (see transmission power control in Annex E.3.5).

PTX,BS is the BS transmitter power.

BSB is the allocated SBFD UL receiver slot bandwidth.

Mself is the noise-floor-to-self interference ratio.

Mco-site is the noise-floor-to-aggregated co-site inter sector interference ratio.

kT is the thermal noise (equal to -174 dBm/Hz in logarithmical scale).

F is the receiver noise figure.

The total interference power can be expressed in linear scale as:

Itot=Iintra−SB,inter−UE+Iinter−SB,inter−BSASBIRinter−SB,inter−BS+IACIACIR+Iself+Ico−site,inter−sector

, where additional variables are defined as:

ASBIRinter-SB,inter-BS is the in-channel Adjacent Sub-Band Interference Ratio, which is derived by applicable BS ACLR and BS ACS.

ACIR is derived by applicable BS ACLR and BS ACS.

The SINR at the victim receiver including total noise and total interference can be expressed in linear scale as:

SINR=PwItot+kTBSBFblock

The receiver noise figure for the receiver exposed for blocking Fblock is produced by the blocking model described in Figure E.2.3-1.

In simulation, the UE power control scheme is only used to compensate path loss and doesn’t consider noise figure increase due to BS receiver blocking. Therefore, the final SINR for UL is less than target SINR.

Note that additional, power scaling of all interference contributions should be considered with respect to aggressor bandwidth and victim bandwidth when ACIR is applied.

E.2.4	UE RF characteristics

Assumptions relevant for modelling the UE RF characteristics are captured in Table E.2.4-1.

Table E.2.4-1: UE parameters

| Parameter | FR1 | FR2-1 |
| --- | --- | --- |
| Maximum transmitter power | 23 dBm | 22.4 dBm (EIRP) |
| Minimum transmitter power | -33 dBm (TS 38.101-1, Table 6.3.1-1) | -40 dBm |
| Antenna configuration | 0 dBi | (Mg,Ng,M,N,P) = (1,1,2,2,2) (dH,dV)=(0.5,0.5)λ (3dB, 3dB)=(90,90) o, SLAv=25 dB Am=25 dB, GE,max=5.5 dBi |
| Receiver noise figure | 9 dB | 10 dB |
| ACLR | 30 dBc The ACLR is modelled as 30 dB at UE maximum TX power and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 40 dB. | 24 dBc The ACLR is modelled as 24 dB at UE maximum TX power and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 34 dB. |
| ACS | 33 dBc | 23 dBc |

E.3	RAN4 co-existence simulation methodology

E.3.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

- Aggressor and victim network are generated. UEs are distributed as described by parameter assumptions.

- UEs are associated to BS based on coupling loss.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS and UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering Adjacent Channel Interference (ACI) as:

ThputNO ACI[bit/s/Hz]=f(SINRICI)=f(S/(N+IICI)), where IICI is the inter-cell interference.

- Throughput is computed considering ACI as:

ThputACI[bit/s/Hz]=f(SINRICI+ACI)=f(S/(N+IICI+IACI)), where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation caused by ACI as:

LossACI=1-ThputACI/ThputSINGLE

The simulation results should be in the form of the throughput with adjacent network  and the relative difference between the two can be compared at 50% and 5% points.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput (Tp) in bps/Hz over a channel with a given SNIR, when using link adaptation:

Tp=0∝SSNIR∝SSNIRMAXfor SNIR≤SNIRMIN for SNIRMIN≤SNIR<SNIRMAXfor SNIR≥SNIRMAX

Where:

| S(SNIR) | Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz |
| --- | --- |
|  | Attenuation factor, representing implementation losses |
| SNIRMIN | Minimum SNIR of the code set, dB |
| SNIRMAX | Maximum SNIR of the code set, dB |

The parameters can be chosen to represent different modem implementations and link conditions. A parameter set relevant for eMBB is listed in Table E.3.1-1.

Table E.3.1-1: Parameters describing baseline Link Level performance for NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| , attenuation | 0.6 | 0.4 | Represent implementation losses |
| SNIRMIN | -10 | -10 | Based on QPSK, 1/8 rate (DL) and 1/5 rate (UL) |
| SNIRMAX | 30 | 22 | Based on 256QAM, 0.93 (DL) and 64QAM 0.93 (UL) |

E.3.2	Received signal power model

The following model is applied:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

, where

RX_PWR is the received power.

TX_PWR is the transmitted power.

G_TX is the transmitter antenna gain (directional array gain).

G_RX is the receiver antenna gain (directional array gain).

E.3.3	Network grid shift

In cases with grid shift >0%, the victim network base stations should be placed as shown in Figure E.3.3-1. The victim network grid is shifted along the line between BS and its closest 100% grid-shift BS, where the distance from any BS in victim network to its second and third closest BS in the aggressor network is the same.

Figure E.3.3-1: Network grid shift definition

E.3.4	Coupling-loss

The Coupling Loss (CL) is defined as the loss in signal between BS-to-UE, UE-to-UE and BS-to-BS. CL is defined as the loss including propagation loss and antenna gains with BF weights applied measured between antenna connectors.

E.3.5	Transmission power control

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2 are 15 dB.

In simulation, power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target value according to power control scheme in TS 38.213.

# E.4	Differences in simulation assumptions of RAN1 and RAN4

## E.4.1 Simulation objectives in RAN1 and RAN4

RAN4 simulations are focused on coexistence analysis between two networks operating on adjacent channels. Specifically, RAN4 evaluated the impact of a SBFD network towards a (legacy) TDD network and vice versa. The RAN4 coexistence evaluation is conducted considering the different scenarios and cases listed in Annex E.1 in Table E.1-1 and Table E.1-2, respectively.

RAN1 conducted the SBFD performance assessment with the main purpose of understanding SBFD performance benefits over legacy static TDD, as well as to investigate the potential of different techniques to combat the cross-link interference present in SBFD.  Four deployment cases are considered for evaluation in RAN1, as described in Clause 7.1.

## E.4.2 Simulation methodology in RAN1 and RAN4

Typically, RAN4 simulations consists of multiple drops (with different UE positions, etc.) where each drop represents a ‘snapshot’ of the network at a particular time. For each drop, RAN4 simulation methodology is outlined in Section E.3.1. Note that RAN4 simulations are fully static (i.e., no time-variant processes, such as fast fading or UE movement, are modeled) so the obtained SINR and throughput statistics intend to capture long-term averaged SINR and throughput performance of the corresponding user. The reported throughput represents the upper bound based on the Shannon capacity. Additionally, RAN4 considers full buffer assumptions with no HARQ or dynamic link adaptation as typically RAN4 is studying coexistence from RF emissions point of view.

On the other hand, RAN1 system level simulations are generally time-driven simulations which try to capture most of the system dynamics. This includes dynamic traffic models resulting in time-varying load and interference conditions, as well as several radio resource management (RRM) functionalities such as CSI acquisition, HARQ retransmissions, packet scheduler, MIMO (including beamforming and precoding techniques) and link adaptation. For each simulated link, a stochastic 3D channel model is used to generate fast-fading components taking into account the angular and delay spreads of the channel, as specified in TR 38.901.  Given the dynamicity of the underlying system model, throughput and SINR vary over time for each simulated user.

E4.3 System level differences between RAN1 and RAN4

This section highlights some differences between the SBFD system level simulations performed in RAN1 and RAN4. A table is presented in Table E.4.3-1 lists a highlight of such differences, while using FR1 Urban Macro scenario as an example. It’s worth noting that there are other differences in simulation methodologies and assumptions due to the different objectives and mechanisms considered by two separated groups as highlighted in the above. For the interested reader, the detailed assumptions along with the different methodologies can be found in Annex E for RAN4 and in Annex A and B for RAN1 simulations.

Table E.4.3-1: An exemplary table of comparison between RAN1 and RAN4 for FR1 UMa

| Parameter | RAN4 assumption (All details provided in Annex E) | RAN1 assumption (All details provided in Annex A and B) |
| --- | --- | --- |
| Network layout | Single layer with 19 sites, each with three hexagonal sectors (57 cells in total); Wrap around | Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around (see Figure A.1.1-2) Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. |
| UE deployment | 1 UE per cell per subband, 20% indoor probability as baseline | Baseline: UE clustering distribution. Optional: Uniform UE distribution (10 users per macro TRP per direction, and all users are randomly and uniformly dropped within the macro cell) Details in Annex A.1.2, Table A.1.2-1 |
| BS self-interference isolation | UL receiver sensitivity degradation due to self-interference is 1dB. | UL receiver sensitivity degradation due to self-interference is 1dB. Details in Annex A.2.1. |
| BS Inter-sector interference isolation | UL receiver sensitivity degradation due sum of all inter-sector gNB-gNB CLI per site is 1 dB (i.e., corresponds to Noise floor - 6 dB). Note: for FR1 wide area scenario, this means the overall inter-sector isolation is dependent on the above as well as the gNB Tx power | Total inter-sector isolation is the sum of spatial isolation + BS-BS ACIR. Agreed values for spatial isolation are: - Option 1: 75dB (spatial isolation), 0dB digital isolation - Option 2: 93dB (spatial isolation), 0dB digital isolation - Option 3: 100dB (spatial isolation), 0dB digital isolation - Option 4: 100dB (spatial isolation), 10dB digital isolation (recommended by Moderator) Details in Annex A.2.3. |
| Propagation model | BS-to-UE: UMa see TR 38.803 BS-to-BS: UMa see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.828. - X = [0.75] - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.803. - Option 2: Reuse the same model as in TR 38.828 with h_UT equals to 25m; - Option 2 as 1st priority and option 1 as 2nd priority UE-to-UE: UMi see TR 36.828, subclause 5.2.2.1. Model is not applicable when 2D distance is less than 10m, instead free space model is applicable. Optional: TR 38.901 model | BS-to-UE: UMa in TR 38.901 BS-to-BS: UMa in TR 38.901 (hUE =25m). If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 75%; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. UE-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m). For UE-UE penetration loss, Table A.2.1-12 in TR 38.802 is used with modification on the criterion used to determine whether two indoor UEs are in the same building. Details in Annex A.3 for BS-UE, BS-BS, and UE-UE channel modelling. |
| BS transmission power | 1st priority: 49 dBm for 100 MHz 2nd priority: 53dBm for 100MHz | Baseline: 53 dBm for 100 MHz; Optional: 49 dBm for 100 MHz Details in Annex B.1. |
| Traffic | Full buffer (100% resource load) | FTP3 traffic model consisting of Poisson packet arrival process and packet size: - Option 1 (higher priority): 4Kbytes for DL and 1Kbyte for UL - Option 2 (higher priority): 0.5Mbyte for DL and 0.125 Mbytes for UL Loads: Low (5-10% RB utilization), medium (20-40%) and high (>50%) Details in Annex A.6. |
| BS Antenna configuration | Baseline: Reuse TR 38.828 antenna model as in 2.2.1.5 For legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ Optional: Extended AAS model in TR 38.803, subclause 5.2.3.2.4. | For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption. For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception. The separate-Tx/Rx antenna array for description of evaluation assumption can be modelled by two panel groups as in Figure A.5-1. Legacy parameters M,N,P,Mg,Ng, dH,dV and dg,H,dg,V are used for description of each panel group. Details in Annex A.5. |
| BS antenna element gain | 5 dBi (assuming 1.8dB loss) | 8 dBi |
| UL scheduling and power control | Entire bandwidth scheduled to the UL user (for legacy TDD: 100 MHz, for SBFD {DUD} and {DU}: 20MHz). Transmit power selected to reach a SNR target of 15 dB | One or more UEs scheduled per slot. Open-loop power control where both the number of DL RBs and transmit power per RB is dynamically adjusted according to P0 and alpha parameters. Details in Annex B.1, Table B.1-1. |

E.4.4 Modeling of adjacent-channel interference in RAN1 and RAN4

In RAN4, the SINR is calculated based on the power of wanted and interference signals, and noise. The signal power of each link is based on the transmission power and the corresponding coupling loss (CL) of the link. The CL is defined in Annex E.3.4 as the loss including propagation loss and antenna gains with BF weights applied measured between antenna connectors. As baseline assumption, the beamforming model in TR 38.803 clause 5.2.3.1 is used which assumes that there is one beam formed to the scheduled user using all the antenna elements.

In contrast, for inter-site gNB-gNB adjacent channel interference modeling in RAN1 (Annex A.2.6), the CL is measured on each individual Tx-Rx antenna port using equation (A-16) and then averaged across all Tx-Rx antenna port combinations.

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | RAN1#109-e | R1-2205399 |  |  |  | Skeleton | 0.0.1 |
| 2022-05 | RAN1#109-e | R1-2205692 |  |  |  | Endorsed skeleton | 0.0.2 |
| 2022-11 | RAN1#111 | R1-2212374 |  |  |  | Capture evaluation methodology agreements until RAN1#110-b meeting | 0.1.0 |
| 2023-02 | RAN1#112 | R1-2300997 |  |  |  | Capture evaluation methodology agreements until RAN1#111 meeting | 0.2.0 |
| 2023-04 | RAN1#112b-e | R1-2303230 |  |  |  | Version not formally endorsed by RAN1. Capture evaluation methodology agreements until RAN1#112 meeting; Update TR skeleton of RAN4 part | 0.3.0 |
| 2023-05 | RAN1#113 | R1-2305091 |  |  |  | Capture agreements and TP until RAN1#112b-e meeting | 0.4.0 |
| 2023-06 | RAN1#113 | R1-2306344 |  |  |  | MCC clean-up - no changes compare to v0.4.0 | 0.4.1 |
| 2023-08 | RAN1#114 | R1-2308675 |  |  |  | Capture endorsed TPs made in RAN1#114 meeting | 0.5.0 |
| 2023-09 | RAN#101 | RP-232421 |  |  |  | Submitted to RAN for information | 1.0.0 |

## <<End of Change for TR 38.858>>