# 3GPP TSG-RAN WG4 Meeting # 108bis													R4-2316274

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | TBD | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for RF specification update for eRedcap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding RF requirements for eRedCap UE in Rel-18 |  |  |  |  |  |  |  |  |
| Summary of change: |  | REFSENS and FRC of eRedCap UE is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No RF requirement for eRedcap UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3I.2, A.3.2.2, A.3.2.3, A.3.3.2, A.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- START CHANGE ------------------------------------------------------

### 7.3I.2	Reference sensitivity power level

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE with capability [IE, eRedCap-r18] equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3I.2-1a and Table 7.3I.2-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-1c and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE with capability [IE, eRedCap-r18] equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-1a and Table 7.3I.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-1c and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Table 7.3I.2-1a: Two antenna port reference sensitivity for eRedcap UE for FDD band

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |
| --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) |
| n1 | 15 | 5,10,15, 20 | -100 |
|  | 30 | 10,15, 20 | -97.1+10log(12/24)= -100.1 |
| n2 | 15 | 5,10,15,20 | -98 |
|  | 30 | 10,15,20 | -98.1 |
| n3 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
| n5 | 15 | 5,10,15,20 | -98.0 |
|  | 30 | 10,15,20 | -98.1 |
| n7 | 15 | 5,10,15,20 | -98.0 |
|  | 30 | 10,15,20 | -98.1 |
| n8 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
| n12 | 15 | 5,10,15 | -97.0 |
|  | 30 | 10,15 | -97.1 |
| n13 | 15 | 5,10 | -97.0 |
|  | 30 | 10 | -97.1 |
| n14 | 15 | 5,10 | -97.0 |
|  | 30 | 10 | -97.1 |
| n18 | 15 | 5,10,15 | -100.0 |
|  | 30 | 10,15 | -100.1 |
| n20 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
| n24 | 15 | 5,10 | -100.0 |
|  | 30 | 10 | -100.5 |
| n25 | 15 | 5,10,15,20 | -96.5 |
|  | 30 | 10,15,20 | -96.6 |
| n26 | 15 | 5,10,15,20 | -97.56 |
|  | 30 | 10,15,20 | -97.86 |
| n28 | 15 | 5,10,15,20 | -98.5 |
|  | 30 | 10,15,20 | -98.6 |
| n30 | 15 | 5,10 | -99.0 |
|  | 30 | 10 | -99.1 |
| n65 | 15 | 5,10,15,20 | -99.5 |
|  | 30 | 10,15,20 | -99.6 |
| n66 | 15 | 5,10,15,20 | -99.5 |
|  | 30 | 10,15,20 | -99.6 |
| n70 | 15 | 5,10,15,20 | -100.0 |
|  | 30 | 10,15,20 | -100.1 |
| n71 | 15 | 5,10,15,20 | -97.2 |
|  | 30 | 10,15,20 | -97.3 |
| n74 | 15 | 5,10,15,20 | -99.53 |
|  | 30 | 10,15,20 | -99.63 |
| n85 | 15 | 5,10,15 | -97.0 |
|  | 30 | 10,15 | -97.1 |
| n91 | 15 | 5 | -100 |
| n92 | 15 | 5,10,15,20 | -100 |
|  | 30 | 10,15,20 | -100.1 |
| n93 | 15 | 5 | -100 |
| n94 | 15 | 5,10,15,20 | -100 |
|  | 30 | 10,15,20 | -100.1 |
| n100 | 15 | 5 | -100 |
| n105 | 15 | 5,10,15,20 | -97.28 |
|  | 30 | 10,15,20 | -97.3 |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |

Table 7.3I.2-1b: Two antenna port reference sensitivity for eRedcap UE for TDD band and FDD band with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 | TDD |
|  | 30 | 10, 15 | -97.1 +10log10(12/24)=-100.1 |  |
| n381 | 15 | 5, 10, 15, 20, | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
| n39 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
| n40 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -1001 |  |
| n411, n901 | 15 | 5, 10, 15, 20 | -94.8 | TDD |
|  | 30 | 10, 15, 20 | -98.1 |  |
| n481 | 15 | 5, 10, 15, 20 | -99 | TDD |
|  | 30 | 10, 15, 20 | -99.1 |  |
| n50 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 | TDD |
|  | 30 | 10 | -100.1 |  |
| n771,4 | 15 | 10, 15, 20 |  | TDD |
|  | 30 | 10, 15, 20 | -98.6 |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(25/52)=-99 | TDD |
|  | 30 | 10, 15, 20 | -99.1 |  |
| n791 | 15 | 10, 20 | -95.8 + 10log10(25/52)=-99 | TDD |
|  | 30 | 10, 20 | -99.1 |  |
| n101 | 15 | 5, 10 | -100 | TDD |
|  | 30 | 10 | -100.1 |  |
| n1041,10 | 15 | 20 | -90.7 + 10log10(25/106)=-97 | TDD |
|  | 30 | 20 | -90.8 +10log10(12/51)=-97.1 |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. |  |  |  |  |

Table 7.3I.2-1c: Uplink configuration for reference sensitivity for eRedcap UE

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
| n1 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n2 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 | 101 | 121 | 121 | 121 |  |
| n3 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n5 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n7 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n8 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n12 | 15 | 201 | 201 | 201 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n13 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n14 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n18 | 15 | 25 | 251 | 251 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n20 | 15 | 25 | 201 | 202 | 202 | FDD |
|  | 30 |  | 101 | 102 | 102 |  |
| n24 | 15 | 25 | 251 |  |  | FDD |
|  | 30 |  | 121 |  |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 | FDD |
|  | 30 |  | 24 | 241 | 241 |  |
| n26 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n28 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
| n30 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n34 | 15 | 25 | 251 | 251 |  | TDD |
|  | 30 |  | 121 | 121 |  |  |
| n38 | 15 | 25 | 50 | 75 | 100 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n39 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n40 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n41, n90 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n48 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n50 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n51 | 15 | 25 |  |  |  | TDD |
| n53 | 15 | 25 | 251 |  |  | TDD |
|  | 30 |  | 121 |  |  |  |
| n65 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n66 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n70 | 15 | 25 | 251 | 251 | Note 3 | FDD |
|  | 30 |  | 121 | 121 | Note 3 |  |
| n71 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n74 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
| n77 | 15 |  | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n78 | 15 |  | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n79 | 15 |  | 251 |  | 251 | TDD |
|  | 30 |  | 121 |  | 121 |  |
| n85 | 15 | 201 | 201 | 201 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n91 | 15 | 254 | 201,4 |  |  | FDD |
| n92 | 15 | 25 | 201 | 201 | 201 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
| n93 | 15 | 254 | 251,4 |  |  | FDD |
| n94 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n100 | 15 | 25 |  |  |  | FDD |
| n101 | 15 | 25 | 251 |  |  | TDD |
|  | 30 |  | 121 |  |  |  |
| n104 | 15 |  |  |  | 251 | TDD |
|  | 30 |  |  |  | 121 |  |
| Note 1: UL resource blocks shall be allocated contiguously at the middle of the configured channel bandwidth but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: Void Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: Void |  |  |  |  |  |  |

For a RedCap UE equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-4.

For a RedCap UE with capability [IE, eRedCap-r18] equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-2a shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-5.

Table 7.3I.2-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n2 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n3 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
|  | 60 |  | -95.3 | -93.2 | -92.0 |
| n5 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n8 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n12 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n13 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n14 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.2 | -95.2 |  |
| n20 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n24 | 15 | -100.0 | -96.8 |  |  |
|  | 30 |  | -97.2 |  |  |
|  | 60 |  | -97.5 |  |  |
| n25 | 15 | -97.3 | -94.1 | -92.3 | -91.0 |
|  | 30 |  | -94.5 | -92.5 | -91.2 |
|  | 60 |  | -94.8 | -92.7 | -91.5 |
| n26 | 15 | -98.3 | -95.1 | -93.3 | -92.0 |
|  | 30 |  | -95.5 | -93.5 | -92.2 |
| n28 | 15 | -99.3 | -96.1 | -94.3 | -93.0 |
|  | 30 |  | -96.5 | -94.5 | -93.2 |
| n30 | 15 | -99.5 | -96.3 |  |  |
|  | 30 |  | -96.7 |  |  |
| n65 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n66 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| n74 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n85 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |

Table 7.3I.2-2a: HD-FDD eRedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -97.2+ 10log(12/24)= -100.2 | -100.2 | -100.2 |
| n2 | 15 | -98.8 | -98.8 | -98.8 | -98.8 |
|  | 30 |  | -99.0 | -99.0 | -99.0 |
| n3 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
| n5 | 15 | -98.8 | -98.8 | -98.8 | -98.8 |
|  | 30 |  | -99.0 | -99.0 | -99.0 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n8 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
| n12 | 15 | -97.8 | -97.8 | -97.8 |  |
|  | 30 |  | -98.0 | -98.0 |  |
| n13 | 15 | -97.8 | -97.8 |  |  |
|  | 30 |  | -98.0 |  |  |
| n14 | 15 | -97.8 | -97.8 |  |  |
|  | 30 |  | -98.0 |  |  |
| n18 | 15 | -100.0 | -100.0 | -100.0 |  |
|  | 30 |  | -100.2 | -100.2 |  |
| n20 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
| n24 | 15 | -100.0 | -100.0 |  |  |
|  | 30 |  | -100.2 |  |  |
| n25 | 15 | -97.3 | -97.3 | -97.3 | -97.3 |
|  | 30 |  | -97.5 | -97.5 | -97.5 |
| n26 | 15 | -98.3 | -98.3 | -98.3 | -98.3 |
|  | 30 |  | -98.5 | -98.5 | -98.5 |
| n28 | 15 | -99.3 | -99.3 | -99.3 | -99.3 |
|  | 30 |  | -99.5 | -99.5 | -99.5 |
| n30 | 15 | -99.5 | -99.5 |  |  |
|  | 30 |  | -99.7 |  |  |
| n65 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n66 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n70 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n71 | 15 | -98.0 | -98.0 | -98.0 | -98.0 |
|  | 30 |  | -98.2 | -98.2 | -98.2 |
| n74 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n85 | 15 | -97.8 | -97.8 | -97.8 |  |
|  | 30 |  | -98.0 | -98.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n100 | 15 | -100 | -100 | -100 | -100 |
| n105 | 15 | -97.2 | -97.2 | -97.2 | -97.2 |
|  | 30 |  | -97.3 | -97.3 | -97.3 |

For a RedCap UE equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2-3 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-4.

For a RedCap UE with capability [IE, eRedCap-r18] equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2-3a shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-5.

Table 7.3I.2-3: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
| n7 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n8 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n12 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n13 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n14 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n18 | 15 | -97.5 | -94.3 | -92.5 |  |
|  | 30 |  | -94.7 | -92.7 |  |
| n20 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n24 | 15 | -97.5 | -94.3 |  |  |
|  | 30 |  | -94.7 |  |  |
|  | 60 |  | -95.0 |  |  |
| n25 | 15 | -94.8 | -91.6 | -89.8 | -88.5 |
|  | 30 |  | -92.0 | -90.0 | -88.7 |
|  | 60 |  | -92.3 | -90.2 | -89.0 |
| n26 | 15 | -95.8 | -92.6 | -90.8 | -89.5 |
|  | 30 |  | -93.0 | -91.0 | -89.7 |
| n28 | 15 | -96.8 | -93.6 | -91.8 | -90.5 |
|  | 30 |  | -94.0 | -92.0 | -90.7 |
| n30 | 15 | -97.0 | -93.8 |  |  |
|  | 30 |  | -94.2 |  |  |
| n65 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n66 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n70 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n71 | 15 | -95.5 | -92.3 | -90.5 | -89.2 |
|  | 30 |  | -92.7 | -90.7 | -89.4 |
| n74 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n85 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n91 | 15 | -97.5 |  |  |  |
| n92 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
| n93 | 15 | -97.5 |  |  |  |
| n94 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |

Table 7.3I.2-3a: HD-FDD eRedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n2 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
| n3 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
| n5 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
| n7 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
| n8 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
| n12 | 15 | -95.3 | -95.3 | -95.3 |  |
|  | 30 |  | -95.5 | -95.5 |  |
| n13 | 15 | -95.3 | -95.3 |  |  |
|  | 30 |  | -95.5 |  |  |
| n14 | 15 | -95.3 | -95.3 |  |  |
|  | 30 |  | -95.5 |  |  |
| n18 | 15 | -97.5 | -97.5 | -97.5 |  |
|  | 30 |  | -97.7 | -97.7 |  |
| n20 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
| n24 | 15 | -97.5 | -97.5 |  |  |
|  | 30 |  | -97.7 |  |  |
| n25 | 15 | -94.8 | -94.8 | -94.8 | -94.8 |
|  | 30 |  | -95.0 | -95.0 | -95.0 |
| n26 | 15 | -95.8 | -95.8 | -95.8 | -95.8 |
|  | 30 |  | -96.0 | -96.0 | -96.0 |
| n28 | 15 | -96.8 | -96.8 | -96.8 | -96.8 |
|  | 30 |  | -97.0 | -97.0 | -97.0 |
| n30 | 15 | -97.0 | -97.0 |  |  |
|  | 30 |  | -97.2 |  |  |
| n65 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n66 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n70 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n71 | 15 | -95.5 | -95.5 | -95.5 | -95.5 |
|  | 30 |  | -95.7 | -95.7 | -95.7 |
| n74 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n85 | 15 | -95.3 | -95.3 | -95.3 |  |
|  | 30 |  | -95.5 | -95.5 |  |
| n91 | 15 | -97.5 |  |  |  |
| n92 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n93 | 15 | -97.5 |  |  |  |
| n94 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |

Table 7.3I.2-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n18 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
|  | 60 |  | 10 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 50 | 75 | NOTE 1 |
|  | 30 |  | 24 | 36 | NOTE 1 |
|  | 60 |  | 10 | 18 | NOTE 1 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n85 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |

Table 7.3I.2-5: Uplink configuration for HD-FDD reference sensitivity for eRedCap

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n2 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n3 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n5 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n7 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n8 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n12 | 15 | 25 | 25 | 25 |  |
|  | 30 |  | 12 | 12 |  |
| n13 | 15 | 25 | 25 |  |  |
|  | 30 |  | 12 |  |  |
| n14 | 15 | 25 | 25 |  |  |
|  | 30 |  | 12 |  |  |
| n18 | 15 | 25 | 25 | 25 |  |
|  | 30 |  | 12 | 12 |  |
| n20 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n24 | 15 | 25 | 25 |  |  |
|  | 30 |  | 12 |  |  |
| n25 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n26 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n28 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n30 | 15 | 25 | 25 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n66 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n70 | 15 | 25 | 25 | 25 | NOTE 1 |
|  | 30 |  | 12 | 12 | NOTE 1 |
| n71 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 50 |
| n74 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 50 |
| n85 | 15 | 25 | 25 | 25 |  |
|  | 30 |  | 12 | 12 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |

------------------------------------------------------------- NEXT  CHANGES ------------------------------------------------------

### A.3.2.2	FRC for receiver requirements for QPSK

Table A.3.2.2-1 Fixed reference channel for receiver requirements (SCS 15 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 5,10,15,20 (NOTE 5) |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | 25 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 | 1672 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 | 16 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 | 5400 |
| Max. Throughput averaged over 1 frame | Mbps | 1.338 | 2.694 | 4.096 | 5.530 | 6.970 | 8.403 | 11.270 | 13.9392 | 1.338 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: FRC applies to a RedCap UE with capability [IE, eRedCap-r18]. The allocated RBs are located continuously in the middle of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |

### A.3.2.3	FRC for maximum input level for 64QAM

Table A.3.2.3-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 5,10,15,20 (NOTE 5) |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | 25 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 12296 | 25608 | 38936 | 52224 | 64552 | 77896 | 106576 | 131176 | 9736 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slot 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 2 | 4 | 5 | 7 | 8 | 10 | 13 | 16 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slot 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 16200 | 33696 | 51192 | 68688 | 86184 | 103680 | 139968 | 174960 | 12960 |
| Max. Throughput averaged over 1 frame | Mbps | 9.837 | 20.486 | 31.149 | 41.779 | 51.642 | 62.317 | 85.261 | 104.941 | 7.789 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: FRC applies to a RedCap UE with capability [IE, eRedCap-r18] and capability [IE, eRedCapNotReducedBB-BW-r18]. For a RedCap UE with capability [IE, eRedCap-r18], The allocated RBs are located continuously in the middle of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |

### A.3.3.2	FRC for receiver requirements for QPSK

Table A.3.3.2-1 Fixed reference channel for receiver requirements (SCS 15 kHz, TDD, QPSK 1/3)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 5, 10,15,20 (NOTE 5) |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | 25 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 | 1672 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 | 16 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | CBs | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 | 5400 |
| Max. Throughput averaged over 1 frame | Mbps | 0.669 | 1.347 | 2.048 | 2.765 | 3.485 | 4.202 | 5.635 | 6.970 | 0.669 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: FRC applies to a RedCap UE with capability [IE, eRedCap-r18]. The allocated RBs are located continuously in the middle of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |

| Parameter | Unit |  | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 | 10, 15,20 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 162 | 217 | 273 | 12 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 13 | 11 | 11 | 11 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 736 | 1608 | 2472 | 3368 | 4224 | 4992 | 6912 | 8712 | 10504 | 12296 | 14088 | 17928 | 808 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 16 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | CBs | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 2376 | 5184 | 8208 | 11016 | 14040 | 16848 | 22896 | 28728 | 34992 | 40824 | 46872 | 58968 | 2592 |
| Max. Throughput averaged over 1 frame | Mbps | 0.810 | 2.1.769 | 2.719 | 3.705 | 4.646 | 5.491 | 7.603 | 9.583 | 11.554 | 13.526 | 15.497 | 19.721 | 0.889 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: FRC applies to a RedCap UE with capability [IE eRedCap-r18]. The allocated RBs are located continuously in the middle of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.2-2 Fixed reference channel for receiver requirements (SCS 30 kHz, TDD, QPSK 1/3)

### A.3.3.3	FRC for maximum input level for 64QAM

Table A.3.3.3-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, TDD, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 5,10,15,20 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | 20 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 8 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,5,6,7 | Bits | 12296 | 25608 | 38936 | 52224 | 64552 | 77896 | 106576 | 131176 | 9736 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Slots 2,5,6,7 | CBs | 2 | 4 | 5 | 7 | 8 | 10 | 13 | 16 | 2 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Slots 2,5,6,7 | Bits | 16200 | 33696 | 51192 | 68688 | 86184 | 103680 | 139968 | 174960 | 12960 |
| Max. Throughput averaged over 1 frame | Mbps | 4.918 | 10.243 | 15.574 | 20.890 | 20.890 | 31.158 | 42.630 | 52.470 | 3.894 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: FRC applies to a RedCap UE with capability [IE, eRedCap-r18] and capability [IE, eRedCapNotReducedBB-BW-r18]. For a RedCap UE with capability [IE, eRedCap-r18], The allocated RBs are located continuously in the middle of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |

Table A.3.3.3-2 Fixed reference channel for maximum input level receiver requirements (SCS 30 kHz, TDD, 64QAM)

| Parameter | Unit |  | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 100 | 5,10,15,20 (NOTE 5) |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 273 | 10 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 11 | 13 | 11 | 11 | 17 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM | 64 QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 5376 | 11784 | 18432 | 25104 | 31752 | 37896 | 52224 | 64552 | 79896 | 92200 | 106576 | 135296 | 4864 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | CBs | 1 | 2 | 3 | 3 | 4 | 5 | 7 | 8 | 10 | 11 | 13 | 17 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,2 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,19} | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {3,…,19} | Bits | 7128 | 15552 | 24624 | 33048 | 42120 | 50544 | 68688 | 86184 | 104976 | 122472 | 140616 | 176904 | 6480 |
| Max. Throughput averaged over 1 frame | Mbps | 5.914 | 12.962 | 20.275 | 27.614 | 34.927 | 41.686 | 57.446 | 71.007 | 87.886 | 101.42 | 117.234 | 148.826 | 5.350 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: FRC applies to a RedCap UE with capability [IE, eRedCap-r18] and capability [IE, eRedCapNotReducedBB-BW-r18]. For a RedCap UE with capability [IE, eRedCap-r18], The allocated RBs are located continuously in the middle of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------