3GPP TSG-RAN WG4 Meeting #109	R4-2319319

# Chicago, USA, 13th – 17th November 2023

Source:	Ericsson

Title:	Simulation results for FR2 PUSCH 256QAM demodulation requirements

Agenda item:	8.6.4.1

Document for:	Information

# Introduction

According to the WF in RAN4#108bis [1], following simulation assumptions are agreed.

Table 1-1 Simulation parameters

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
| RV sequence |  | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
| DM-RS duration |  | single-symbol DM-RS |
| Additional DM-RS symbols |  | pos0, pos1 |
| Number of DM-RS CDM group(s) without data |  | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
| DM-RS port(s) |  | {0} |
| DM-RS sequence generation |  | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
| Allocation length |  | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | 2, Disabled |
|  | Time density (LPT-RS) | 1, Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

In this contribution, simulation results are provided.

# Simulation results

Table 2-1 Simulation results for FR2 PUSCH 256QAM MCS20

| PT-RS configuration | SCS and CBW | Channel model | DM-RS configuration | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal | Impairment |
| PT-RS Enabled | 60kHz, 50MHz | TDLD30-35 | DMRS 1+1 | 17.09 | 19.79 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.33 | 19.83 |
|  | 120kHz, 50MHz | TDLD30-35 | DMRS 1+1 | 17.12 | 19.86 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.55 | 20.05 |
|  | 120kHz, 100MHz | TDLD30-35 | DMRS 1+1 | 17.22 | 19.83 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.48 | 19.98 |
|  | 120kHz, 200MHz | TDLD30-35 | DMRS 1+1 | 17.21 | 19.79 |
|  |  | TDLD30-35 | DMRS 1+0 | 17.49 | 19.99 |
| PT-RS disabled | 60kHz, 50MHz | TDLD30-35 | DMRS 1+1 | 16.45 | 19.36 |
|  | 120kHz, 50MHz | TDLD30-35 | DMRS 1+1 | 16.5 | 19.18 |
|  | 120kHz, 100MHz | TDLD30-35 | DMRS 1+1 | 16.58 | 19.14 |
|  | 120kHz, 200MHz | TDLD30-35 | DMRS 1+1 | 16.58 | 19.56 |

# Conclusions

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1]	R4-2316903, Way Forward for 256 QAM BS Demod, Nokia, Nokia Shanghai Bell