3GPP TSG-RAN4 Meeting  # 109	R4-2319360

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TCI state switching requirements for FR2 multi-Rx chain DL reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Core |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following agreement shall be reflect in the DCI based TCI state switching for sDCI case: - There is no TCI switching delay when UE is configured with GBBR and is NOT configured with non-GBBR Add clarification for the case when when the time span between PDCCHs and scheduled PDSCHs cross TRPs are smaller than timeDurationForQCL for mDCI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Draft CR is based on the agreed Big CR R4-2317431 in RAN4#108bis. - Add the clarificaiton that no TCI TCI switching delay when UE is configured with GBBR and is NOT configured with non-GBBR. - UE is not expected to be scheduled simultaneous PDSCH reception with different QCL type-D when the time span between PDCCHs and scheduled PDSCHs cross TRPs are smaller than timeDurationForQCL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10D.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

### 8.10D.4	DCI based TCI state switch delay

For sDCI, if the dual target TCI states is known, when a UE is configured with the higher layer parameter tci-PresentInDCI from one TRP which is set as 'enabled' for the CORESET scheduling two PDSCHs at slot n, UE shall be able to receive PDSCHs with target TCI states of the serving cell on which TCI state switch occurs at the first slot that is after slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and apply spatial QCL information received in DCI for PDSCHs processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.331 [2]. If TCI state switching is from dual TCI states to single TCI state and the target TCI state is one of the source TCI states, there is no TCI state switching delay provided that UE is configured with group-based RSRP report (groupBasedBeamReporting-r17) and is not configured with non-group-based RSRP report.

For mDCI, if the dual target TCI states is known, when a UE is configured with the higher layer parameter tci-PresentInDCI from two TRPs which is set as 'enabled' for the DCIs scheduling two PDSCHs at slot n1 and n2 respectively, UE shall be able to receive PDSCHs with target TCI states of the serving cell on which TCI state switch occurs at the first slot that is after slot n1 + timeDurationForQCL and slot n2 + timeDurationForQCL respecitvely, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and apply spatial QCL information received in DCI for PDSCHs processing as described in TS 38.214 [26], the value of timeDurationForQCL is defined in TS 38.331 [2]. The requirements apply provided that the offset betweeen the reception of the DCI received in slot n1 and PDSCH scheduled by DCI received in slot n2 is equal to or greater than timeDurationForQCL, and the offset betweeen the reception of the DCI received in slot n2 and PDSCH scheduled by DCI received in slot n1 is equal to or greater than timeDurationForQCL. UE is not expected to be scheduled simultaneous PDSCH reception with different QCL type-D when the time span between PDCCHs and scheduled PDSCHs cross TRPs are smaller than timeDurationForQCL.

The known condition for dual TCI states defined in clause 8.10D.2 is applied.

# <End of Change #1>