# 3GPP TSG-RAN WG4 Meeting # 109		 R4-2319463

# Chicago, US, November 13 – 17, 2023

Agenda Item:	8.7.2.1

Source: 	OPPO

Title: 	On general aspects for FR2_multiRX_DL

Document for:	Approval

1	Introduction

A WF on Rel-18 FR2_multiRX_DL was approved in RAN4#108bis meeting [1].

In this contribution, we further discuss the remaining issues related to scope and scenarios for FR2-1 multi-Rx reception.

2	Discussion

Applicability and conditions

#### Issue 1-3-5: UE behaviour when a condition becomes violated during a measurement - FFS - Option A: When the side conditions are violated, the corresponding requirement is not applicable, and/or no UE behavior needs to be defined. - Alternative wording for option A: When the side conditions are changed with a transition between multi-Rx operation and no multi-Rx operation, the corresponding requirement is not applicable, and/or no UE behavior needs to be defined. - The side conditions are discussed separately for each topic. - Option B: UE beviour needs to be speficied. - Option B1: - When the condition of multi-Rx becomes violated during measurement, UE can continue the on-going L1 measurement but with the relaxed measurement period requirement instead of the requirement for simultaneous reception. - Option B2: - UE in multi-rx operation may need to stop and restart (in new conditions) L1 measurements or evaluations, when at least the following conditions for multi-rx operation become violated: - A completely new dual TCI configuration (e.g., dual TCI swapped to another dual TCI where both active states are new). - UE in multi-rx operation may need to continue L1 measurements or evaluations (using a single panel), but more relaxed requirements will apply for such measurements/evaluations, when any of the following conditions for multi-rx operation become violated: - Partially new dual TCI configuration (e.g., one active TCI state remains the same), - Different QCL type D condition is not valid, etc. - RAN4 to discuss whether there are scenarios when a UE in multi-rx operation may need to complete L1 measurements or evaluations in multi-rx operation prior to switching to single-rx operation, e.g.: - UE becomes configured with CA operation or with some other operation suggesting single-rx operation.

The set of conditions for the L1 measurement requirements have been discussed for several meetings. We don’t see it is necessary to have general agreements on the set of conditions to be considered in the L1 measurement requirements. Different applicable conditions may need to be specified for different requirements. They can be discussed in the corresponding requirements and drafting CR. When side conditions for multi-Rx operation are violated, UE is not expected to meet the requirements of multi-Rx operation.

Proposal 1: When the side conditions are violated, the corresponding requirement is not applicable, and/or no UE behavior needs to be defined.

UE capabilities

#### Issue 1-4-3: UE capability of simultaneousRxDataSSB-DiffNumerology <Online agreement> - It is RAN4 common understanding that there is no scenario of simultaneous reception of mix numerologies for: SSB (TRP1) + data (TRP2), and for SSB (TRP1) + CSI-RS (TRP2) in Rel-18. - No new UE capability for mix numerologies is needed in Rel-18 #### Issue 1-4-4: UE capability for simultaneous reception with different QCL typeD for L1 measurements - FFS - Option 1: - A new UE capability of supporting simultaneous reception of RS and data from different directions with different QCL type D RSs for enhanced L1 measurements for multi-Rx is introduced. - A new UE capability of supporting simultaneous reception of RS and RS from different directions with different QCL type D RSs for enhanced L1 measurements for multi-Rx is introduced, if measurement restriction is enhanced for multi-Rx. - Option 2: - To support Rel-18 multi-Rx DL simultaneous reception modes, an additional UE capability needs to be defined. - Option 3: - specify UE capability to indicate whether the UE can support simultaneous reception of data and L1 measurement - Option 4: - No need to define new additional UE capability to support simultaneous reception of RS+data or RS+RS. #### Issue 1-4-5: UE capability for indication of whether additional delay is needed in dual TCI state switching for multi-Rx - FFS - Option 1: - Specify UE capability to indicate whether additional delay is needed in MAC CE based dual TCI state switching. By default, a UE should support MAC CE based dual TCI state switching without the additional delay. - Option 2: - No UE capability is introduced for additional delay in dual TCI state switch. #### Issue 1-4-6: UE feature list NR_FR2_multiRX_DL Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. X. NR_FR2_multiRX_DL; X-1; Simultaneous reception of NR PDCCH/PDSCH overlapping with layer 1 RS with different QCL Type-D; Supports simultaneous reception of PDCCH/PDSCH with different QCL Type-D layer 1 RS for measurement on overlapping OFDM symbols.; 16-2c; Yes; N/A; Per band; N/A; FR2 only; Optional with capability signalling. X. NR_FR2_multiRX_DL; X-2; Simultaneous measurement of layer 1 RS overlapping with another layer 1 RS with different QCL Type-D; Supports Simultaneous measurement of layer 1 RS overlapping with another layer 1 RS with different QCL Type-D on overlapping OFDM symbol(s).; 16-2c; Yes; N/A; Per band; N/A; FR2 only; Optional with capability signalling. X. NR_FR2_multiRX_DL; X-3; Fast beam sweeping; Supports beam sweeping factor reduction for SSB-based layer 1 measurement.; Yes; N/A; Per band; N/A; FR2 only; Candidate values for Component 2: {2,4,6} for FR2-1; Optional with capability signalling.

Since Rel-16 UE capability simultaneousReceptionDiffTypeD-r16 is applicable only for PDSCH reception, we slightly prefer to introduce a new capability in R18 to make the UE capability clearer. We slightly prefer the support of simultaneous reception of RS+data or RS+RS on overlapping OFDM symbols based on the main feature of multi-Rx DL simultaneous reception, but not separate or additional UE capabilities. They be counted as components for this big feature group.

Proposal 2: Define the support of simultaneous reception of RS+data or RS+RS on overlapping OFDM symbols as component of the feature group of multi-Rx DL simultaneous reception, without separate or additional UE capabilities.

3	Conclusion

Proposal 1: When the side conditions are violated, the corresponding requirement is not applicable, and/or no UE behavior needs to be defined.

Proposal 2: Define the support of simultaneous reception of RS+data or RS+RS on overlapping OFDM symbols as component of the feature group of multi-Rx DL simultaneous reception, without separate or additional UE capabilities.

4	Reference

[1] R4-2317332, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo