# 3GPP TSG-RAN WG4 Meeting # 109	R4-2318804

# Chicago, US, November 13 – 17, 2023

Source:		Verizon, Samsung, Nokia

Title:			TP for TR 38.880: 3Tx inter-band CA_n2-n77

Agenda item:		x.x.x

Document for:		Discussion/Approval

# Introduction

This is a TP to TR 38.880 to add CA_n2_n77.

# Reference

[1] WI description, RP-232674: "New WID: Rel-18 new basket WI for 3Tx inter-band UL CA and EN-DC"

[2] TS 38.101-1 V18.3.0

# Text proposal

====================== <Start of Text Proposal> ======================

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band nX+nY

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n2-n77 | n2, n77 |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. |  |  |  |  |  |  |

Table 5.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

Editor's note: The targeted max Tx power configuration shall be within the scope of basket WI.

### 5.x.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n2-n77.

Table 5.x.3-1: MSD scenarios for CA_n2-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | 1st order of n77 UL | 2nd order of n2 DL | Yes |
| Cross band leakage | n77 UL | n2 DL | Yes |
| IMD | IMD2 (1*f_n77-1*f_n2) | n2 DL | Yes |
|  | IMD4 (3*f_n2-1*f_n77) | n2 DL | Yes |
|  | IMD5 (2*f_n77-3*f_n2) | n2 DL | Yes |

Editor's note: only list the MSD scenario that exists for this band combination.

Table 5.x.3-2: MSD scenarios for CA_n2-n77 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | 1st order of n77 UL | 2nd order of n2 DL | Yes |
| Cross band leakage | n77 UL | n2 DL | Yes |
| IMD | IMD2 (1*f_n77-1*f_n2) | n2 DL | No |
|  | IMD4 (3*f_n2-1*f_n77) | n2 DL | No |
|  | IMD5 (2*f_n77-3*f_n2) | n2 DL | No |

Editor's note: only list the MSD scenario that exists for this band combination.

### 5.x.4	REFSENS requirements

Editor's note1: Make it clear whether the existing 2Tx requirements are reused or newly analysed due to for example missing of the 2Tx requirements.

Editor's note2: If reused, just say to reuse the existing 2Tx MSD requirements for MSD type harmonic mixing, or cross band leakage or IMD.

#### 5.x.4.1	REFSENS requirements for total power class 2

In previous meeting, RAN4 agreed that the following rules will be as the baseline to define MSD requirement for PC2 inter-band CA or EN-DC:

- Same MSD requirements can be applied for the band combination with either 1Tx or 2Tx in TDD aggressor band UL with same power class for harmonic mixing and cross band isolation, and

- the requirements specified based on 1Tx-1Tx UL configuration are applicable for 1Tx-2Tx UL configuration

Based on the agreement, the analysis MSD results are summarized in Table 5.x.4.2-1. Mainly, they are the same as PC2 2Tx configuration for this 3Tx requirement to the current RAN4 specifications [2].

According to the current RAN4 specifications [2], CA_n2-n77 is subject to IMD5 impact to n2 DL too. The value of PC2 IMD5 MSD is resulted in the Table 5.x.4.1-1, and it is derived based on linear extrapolation and referred the MSD for PC3 configuration.

Table 5.x.4.1-1: Derived MSD values for PC2 UL configurations

| CA Combination | Power Class | NR Band | PUL (dBm) | IMD Source | IMD Order | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2-n77 | 3 | n2 | 20 | IMD2 | 1 | 26.0 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n2 | 23 |  | 1 | 32.1 |
|  |  | n77 | 23 |  | 1 |  |
|  | 3 | n2 | 20 | IMD4 | 3 | 8.0 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n2 | 23 |  | 3 | 19.1 |
|  |  | n77 | 23 |  | 1 |  |
|  | 3 | n2 | 20 | IMD5 | 3 | 5.0 |
|  |  | n77 | 20 |  | 2 |  |
|  | 2 | n2 | 23 |  | 3 | 20.0 |
|  |  | n77 | 23 |  | 2 |  |

For the proposed MSD requirements, all required results are summarized in the following tables, except a new MSD value for IMD5 in the Table 5.x.4.1-3 as discussed above.

Table 5.x.4.1-2 (same as Table 7.3A.4-4a of 38.101-1): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 9.1 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 6.7 | NOTE 4 | UL1/DL2 |

Table 5.x.4.1-3: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 32.1 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 19.1 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 20.0 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |

Table 5.x.4.1-4 (same as Table 7.3A.6-1a of 38.101-1): Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n2 | 3305 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.0 | >ACLR2 |

#### 5.x.4.2	REFSENS requirements for total power class 1.5

According to the current RAN4 specifications [2], CA_n2-n77 is subject to IMD2, IMD4, and IMD5 impact to n2 DL. For IMD2, the IMD composition is 1st order in n2 UL and 1st order in n77 UL. For IMD4, the IMD composition is 3rd order in n2 UL and 1st order in n77 UL, and for IMD5, the IMD composition is 3rd order in n2 UL and 2nd order in n77 UL.

Table 5.x.4.2-1 summarizes the MSD analysis results where the MSD for PC3 and PC2 UL configurations are referenced to the current RAN4 specifications [2]. And, the MSD for PC1.5 UL configuration is derived based on linear extrapolation.

Table 5.x.4.2-1: Derived MSD values for PC2 UL configurations

| CA Combination | Power Class | NR Band | PUL (dBm) | IMD Source | IMD Order | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2-n77 | 3 | n2 | 20 | IMD2 | 1 | 26.0 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n2 | 23 |  | 1 | 32.1 |
|  |  | n77 | 23 |  | 1 |  |
|  | 1.5 | n2 | 23 |  | 1 | 35.2 |
|  |  | n77 | 27.8 |  | 1 |  |
|  | 3 | n2 | 20 | IMD4 | 3 | 8.0 |
|  |  | n77 | 20 |  | 1 |  |
|  | 2 | n2 | 23 |  | 3 | 19.1 |
|  |  | n77 | 23 |  | 1 |  |
|  | 1.5 | n2 | 23 |  | 3 | 26.4 |
|  |  | n77 | 27.8 |  | 1 |  |
|  | 3 | n2 | 20 | IMD5 | 3 | 5.0 |
|  |  | n77 | 20 |  | 2 |  |
|  | 2 | n2 | 23 |  | 3 | 20.0 |
|  |  | n77 | 23 |  | 2 |  |
|  | 1.5 | n2 | 23 |  | 3 | 28.0 |
|  |  | n77 | 27.8 |  | 2 |  |

For PC1.5 UL CA_n2-n5 inter-band combination, the reference sensitivity values are proposed in Table 5.x.4.1-3.

Table 5.x.4.1-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 35.2 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 26.4 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 28.0 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |

As both harmonic mixing and cross band isolation for this PC1.5 UL CA_n2-n77 have been specified in 38.101-1, we copy/paste the same requirements into the following two tables

Table 5.x.4.2-3 (same as Table 7.3A.4-4b of 38.101-1): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 11.8 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 9.2 | NOTE 4 | UL1/DL2 |

Table 5.x.4.2-4 (same as Table 7.3A.6-1b of 38.101-1): Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n2 | 3305 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 1.8 | >ACLR2 |

====================== <End of Text Proposal> ======================