3GPP TSG-RAN WG4 Meeting #109	R4-2319944

Chicago, US, November 13 – 17, 2023

Title: 	[TEI16] Discussion on MRTD/MTTD requirements for inter-band EN-DC with overlapping DL bands

Source: 	Huawei, HiSilicon

Agenda item:	4.7

Document for:	Discussion

Introduction

In this contribution, we provide discussion on the maintaining issues for MRTD/MTTD requirements for inter-band EN-DC with overlapping DL bands.

Discussion

In NR R15, RAN4 achieved the following agreements on synchronous/asynchronous operations for EN-DC, which were captured in [1]

Table 1. Clarification on EN-DC deployment scenarios and UE support/capability in Rel-15 [1]

|  | FDD E-UTRA-FDD NR | TDD E-UTRA-TDD NR | TDD E-UTRA-FDD NR and FDD E-UTRA-TDD NR |
| --- | --- | --- | --- |
| Intra- band | Sync. (Mandatory supported by all UEs), Async. (Optional supported by some UEs, depends on UE capability IE asyncIntraBandENDC) Note-1: only support collocation for Rel-15 | Sync. (UE only support Sync operation for Rel-15) Note-1: only support collocation for Rel-15 | N.A. |
| Inter- band | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) (Additionally, Simultaneous TX and RX depends on UE capability RRC IE simultaneousRxTxInterBandENDC) | Sync. (Mandatory support by all UEs), Async. (Mandatory support by all UEs) (Additionally, Simultaneous TX and RX depends on UE capability RRC IE simultaneousRxTxInterBandENDC) |

For inter-band EN-DC, all UEs are mandatory to support synchronous operation and asynchronous operation.

For intra-band EN-DC, it can be observed that only co-located deployment is supported. For TDD-TDD intra-band EN-DC, UE only supports synchronous operation. For FDD-FDD intra-band EN-DC, all UEs are mandatory to support synchronous operation and whether to support asynchronous operation depends on UE capability IE asyncIntraBandENDC.

According to TS38.101-3, the requirements for TDD-TDD intra-band EN-DC needs to be applied for the combinations DC_42_n77/DC_42_n78 (TDD-TDD inter-band EN-DC with overlapping DL bands), i.e. UE only supports synchronous operation and co-located deployment for the combination DC_42_n77/DC_42_n78. For the combinations DC_20_n28/ DC_28_n20 (FDD-FDD inter-band EN-DC with partially overlapping DL bands), it requires that DL carriers are synchronized and MRTD=3us, which means UE only supports synchronous operation and co-located deployment for combination DC_42_n77/DC_42_n78.

The clarification and MRTD requirements for EN-DC in R15 can be summarized as Table 2.

Table 2. Clarification on EN-DC deployment, sync and UE support/capability in Rel-15

| EN-DC combination |  | Deployment | Sync/async operation | MRTD requirements |
| --- | --- | --- | --- | --- |
| Intra-band | TDD-TDD | Limited to co-located | Only limited to sync operation | 3us |
|  | FDD-FDD | Limited to co-located | UE capability: only support sync operation | 3us |
|  |  |  | UE capability: support both sync and async | 0.5slot |
| Inter-band | TDD-TDD inter-band EN-DC with overlapping DL bands (DC_42_n77/DC_42_n78) | Limited to co-located | Only limited to sync operation | 3us |
|  | FDD-FDD inter-band EN-DC with partially overlapping DL bands (DC_20_n28/DC_28_n20) | Limited to co-located | Only limited to sync operation | 3us |
|  | inter-band EN-DC without overlapping DL bands | No limitation | Mandatory support both sync and async | 0.5 slot |

When E-UTRA BS and NR BS are co-located deployed, it may require that E-UTRA BS and NR BS are synchronous for intra-band EN-DC or inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_42_n77/DC_42_n78/ DC_20_n28/DC_28_n20). When E-UTRA BS and NR BS are non-co-located deployed, only inter-band EN-DC without overlapping DL bands are supported and there is no limitation on synchronous/asynchronous operation.

Observation 1: In R15, for inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_42_n77/DC_42_n78/ DC_20_n28/DC_28_n20), the MRTD requirements for synchronous intra-band EN-DC are applied with the limitation on co-located deployment.

Observation 2: In R15, the limitation on supporting only synchronous operation can be applied only under co-located deployment for intra-band EN-DC or inter-band EN-DC with overlapping or partially overlapping DL bands.

In TS 38.306, the UE capability IE asyncIntraBandENDC is defined as follows:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| asyncIntraBandENDC Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC. This capability applies to: - Intra-band (NG)EN-DC combination without additional inter-band NR and LTE CA component; - Intra-band (NG)EN-DC combination supporting both UL and DL intra-band (NG)EN-DC parts with additional inter-band NR/LTE CA component; - Intra-band (NG)EN-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC UL part; - Inter-band (NG)EN-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]). If this capability is included in an "Intra-band (NG)EN-DC combination supporting both UL and DL intra-band (NG)EN-DC parts with additional inter-band NR/LTE CA component" or in an "Intra-band (NG)EN-DC combination without supporting UL in both the bands of the intra-band (NG)EN-DC UL part", this capability applies to the intra-band (NG)EN-DC BC part. | BC | No | FDD only | FR1 only |

According to the Table 5.5B.4.1-1 specified in TS 38.101-3, it can be observed that only DC_42_n77 and DC_42_n78 are the inter-band EN-DC combination where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band. So, the UE capability asyncIntraBandENDC applies to FDD-FDD intra-band EN-DC combination and some certain TDD-TDD inter-band EN-DC combinations (i.e. DC_42_n77/DC_42_n78). If the IE asyncIntraBandENDC is not indicated, then it means that the UE supports only synchronous operation for this EN-DC combination.

Observation 3: UE capability IE asyncIntraBandENDC only apply to FDD-FDD intra-band EN-DC combinations and TDD-TDD inter-band EN-DC combinations DC_42_n77 and DC_42_n78. For these EN-DC combinations, UE supports only synchronous operation by default when IE asyncIntraBandENDC is not indicated.

In R16, the UE capability of supporting interBandMRDC-WithOverlapDL-Bands-r16 (i.e. Type 2 UE) is introduced for the inter-band combinations with overlapping or partially overlapping DL bands (i.e. DC_42_n77/DC_42_n78/DC_20_n28/DC_28_n20). When UE indicates the capability interBandMRDC-WithOverlapDL-Bands-r16 for the inter-band EN-DC combination with overlapping or partially overlapping DL bands, UE is assumed to support non-co-located deployment and the MRTD requirements for legacy inter-band EN-DC (i.e. without overlapping DL bands) are applied for this inter-band EN-DC combination. MRTD requirements for legacy inter-band EN-DC are defined as 0.5slot without deployment limitation and without limitation on synchronous only operation.

For TDD-TDD inter-band EN-DC with overlapping DL bands (i.e. DC_42_n77 and DC_42_n78), whether to support asynchronous operation is up to UE capability. When UE indicates interBandMRDC-WithOverlapDL-Bands-r16 but not indicates asyncIntraBandENDC, then the UE only needs to apply MRTD requirements for synchronous inter-band EN-DC (i.e. MRTD=33us). When UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and asyncIntraBandENDC, then the UE only needs to apply MRTD requirements for legacy inter-band EN-DC (i.e. MRTD=0.5slot).

Proposal 1: For TDD-TDD inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_42_n77 and DC_42_n78), UE indicating interBandMRDC-WithOverlapDL-Bands-r16 applies

- MRTD=33us (i.e. synchronous inter-band EN-DC MRTD value) when asyncIntraBandENDC is not indicated

- MRTD=0.5slot (i.e. asynchronous inter-band EN-DC MRTD value) when asyncIntraBandENDC is indicated

For FDD-FDD inter-band EN-DC with partially overlapping DL bands (i.e. DC_20_n28 and DC_28_n20), UE capability asyncIntraBandENDC are not applied. In last RAN4 meeting, RAN4 considered to apply UE capability asyncIntraBandENDC for FDD-FDD inter-band EN-DC with partially overlapping DL bands. In R15, for FDD-FDD inter-band EN-DC with partially overlapping DL bands, the limitation on supporting only synchronous operation is only for co-located deployment rather than non-co-located deployment. So, there is no synchronous limitation for FDD-FDD inter-band EN-DC in non-co-located deployment. It is common understanding that FDD is asynchronous network and FDD-FDD inter-band EN-DC in non-co-located deployment are typically asynchronous. If UE capability asyncIntraBandENDC is applied for FDD-FDD inter-band EN-DC with partially overlapping DL bands, then UE not indicating asyncIntraBandENDC is not backward-compatible to the current asynchronous operation for FDD-FDD inter-band EN-DC in non-co-located deployment.

Observation 4: If UE capability IE asyncIntraBandENDC was reused to be applied for FDD-FDD inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_20_n28/DC_28_n20), the UE not indicating asyncIntraBandENDC would have backward compatibility issue.

So, there is no need to apply UE capability asyncIntraBandENDC for FDD-FDD inter-band EN-DC with partially overlapping DL bands. UE indicating interBandMRDC-WithOverlapDL-Bands-r16 needs to support FDD-FDD inter-band EN-DC under asynchronous operation. When UE indicating interBandMRDC-WithOverlapDL-Bands-r16 is assumed to support non-co-located deployment, the UE needs to apply MRTD requirements for asynchronous inter-band EN-DC (i.e. MRTD=0.5slot).

Proposal 2: For FDD-FDD inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_20_n28/DC_28_n20), there is no need to introduce UE capability asyncIntraBandENDC, and UE indicating interBandMRDC-WithOverlapDL-Bands-r16 applies MRTD=0.5slot (i.e. asynchronous inter-band EN-DC MRTD value).

Conclusions

This contribution provides the discussion on maintaining issues for MRTD/MTTD requirements for inter-band EN-DC with overlapping DL bands. The following are provided:

Observation 1: In R15, for inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_42_n77/DC_42_n78/ DC_20_n28/DC_28_n20), the MRTD requirements for synchronous intra-band EN-DC are applied with the limitation on co-located deployment.

Observation 2: In R15, the limitation on supporting only synchronous operation can be applied only under co-located deployment for intra-band EN-DC or inter-band EN-DC with overlapping or partially overlapping DL bands.

Observation 3: UE capability IE asyncIntraBandENDC only apply to FDD-FDD intra-band EN-DC combinations and TDD-TDD inter-band EN-DC combinations DC_42_n77 and DC_42_n78. For these EN-DC combinations, UE supports only synchronous operation by default when IE asyncIntraBandENDC is not indicated.

Proposal 1: For TDD-TDD inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_42_n77 and DC_42_n78), UE indicating interBandMRDC-WithOverlapDL-Bands-r16 applies

- MRTD=33us (i.e. MRTD value for synchronous inter-band EN-DC) when asyncIntraBandENDC is not indicated

- MRTD=0.5slot (i.e. MRTD value for asynchronous inter-band EN-DC) when asyncIntraBandENDC is indicated

Observation 4: If UE capability IE asyncIntraBandENDC was reused to be applied for FDD-FDD inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_20_n28/DC_28_n20), the UE not indicating asyncIntraBandENDC would have backward compatibility issue.

Proposal 2: For FDD-FDD inter-band EN-DC with overlapping or partially overlapping DL bands (i.e. DC_20_n28/DC_28_n20), there is no need to introduce UE capability asyncIntraBandENDC, and UE indicating interBandMRDC-WithOverlapDL-Bands-r16 applies MRTD=0.5slot (i.e. MRTD value for asynchronous inter-band EN-DC).

Reference

- R4-1809935, “Clarification on Sync/Async EN-DC scenarios”, Samsung, NTT DOCOMO, INC.