3GPP TSG-RAN WG4 Meeting # 109	R4-2320594
Chicago, US, November 13 – 17, 2023

Agenda item:	8.16.3

Source:	CAICT

Title:	On FR1 MIMO OTA test time reduction

Document for:	Approval

1	Introduction

In Rel-16, we adopted small Number of Slots per Stream for throughput as 10k, but only for bands >1GHz. For band<1GHz, the slot number is tentatively defined as [20k], in TS 38.151:

Table E.1-1: FR1 Common test parameters Parameter; Unit; Value. PDSCH transmission scheme; Transmission scheme 1. Carrier configuration; Offset between Point A and the lowest usable subcarrier on this carrier (Note 2); RBs; 0. Subcarrier spacing; kHz; 15 or 30. <Skip unnecessary part in the table…> Minimum Number of Slots per Stream; 20000 for 15kHz SCS 40000 for 30kHz SCS (Note 3). Transmit Power Control; dBm; 13. DL power level (RS EPRE of SSS); dBm / SCS; Set at gNodeB simulator with correction from calibration. EPRE ratio of PDSCH to SSS; dB; 0. Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 for tested channel bandwidth and subcarrier spacing. Note 3: For FR1 MIMO OTA test lab alignments and FR1 MIMO OTA UE performance requirements, the following values can be used: For FR1 bands >1GHz: 20k for 30kHz SCS, 10k for 15kHz SCS; For FR1 bands <1GHz: [20k] for 15kHz SCS;.

At the last meeting, Xiaomi submitted some measurement results on this issue [1], it was agreed to consider to reduce Minimum Number of Slots per Stream to 10k for bands < 1GHz. [2]

Issue 1-1-1: Whether Minimum Number of Slots per Stream for bands <1GHz can be reduced to 10k Agreement: - Proposal 1: Consider to reduce Minimum Number of Slots per Stream to 10k for bands < 1GHz, to reduce test time. - Proposal 2: Encourage companies to provide more measurement data to check the impact of the slot number on MIMO OTA performance. - Proposal 3: Encourage to conclude this issue no later than the RAN4#109 (Nov.), to complete the core part WI.

In this contribution, we present our measurement results at band n5 for confirming this issue.

2	Discussion

The measurement results of two smartphones are presented in Table 1 and Table 2. It can be observed that the difference of TRMS between 20k and 10k is within 0.2dB.

Table 1. Measurement results of DUT 1 with different number of slots at band n5 (881.5MHz)

| Number of Slots | DMP | DML | DMSU | TRMS |
| --- | --- | --- | --- | --- |
| 20k | -90.40 | -90.36 | -86.36 | -89.41 |
| 10k | -90.64 | -90.25 | -86.47 | -89.48 |
| Delta | 0.23 | -0.11 | 0.11 | 0.07 |

Table 1. Measurement results of DUT 2 with different number of slots at band n5 (881.5MHz)

| Number of Slots | DMP | DML | DMSU | TRMS |
| --- | --- | --- | --- | --- |
| 20k | -89.38 | -89.49 | -83.88 | -88.24 |
| 10k | -89.38 | -89.79 | -83.99 | -88.39 |
| Delta | 0.00 | 0.30 | 0.11 | 0.15 |

Observation 1: Based on our measurement results, the TRMS difference between 10k and 20k is small, the maximum difference is 0.15 dB.

Observation 2: The actual test time could be reduced obviously if Minimum Number of Slots per Stream is decreased from 20k to 10k.

Proposal 1: For FR1 MIMO OTA test lab alignments and FR1 MIMO OTA UE performance requirements: the Minimum Number of Slots per Stream can be reduced to 10k for 15kHz SCS.

3	Conclusion

Observation 1: Based on our measurement results, the TRMS difference between 10k and 20k is small, the maximum difference is 0.15 dB.

Observation 2: The actual test time could be reduced obviously if Minimum Number of Slots per Stream is decreased from 20k to 10k.

Proposal 1: For FR1 MIMO OTA test lab alignments and FR1 MIMO OTA UE performance requirements: the Minimum Number of Slots per Stream can be reduced to 10k for 15kHz SCS.

References

- R4-2315433, Minimum Number of Slots per Stream for below 1GHz MIMO OTA test, Xiaomi, 3GPP RAN4#108-bis, Oct. 2023.

- R4-2316945, WF on NR_MIMO_OTA_enh, CAICT, 3GPP RAN4#108-bis, Oct. 2023.