3GPP TSG-RAN WG4 Meeting # 109   																										R4-2321391

Chicago, US, November 13 - 17, 2023

Title: 	WF on RRM requirements for NR ATG

Agenda Item:	8.13.9

Source: 	CMCC

Document for:	Approval

- Core Maintenance

Issue 1-1: ATG UE feature

Agreement:

- L1 features:

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink Time and Frequency pre-compensation and timing relationship enhancements | Support of UE specific TA calculation based on its GNSS-acquired position and the indicated BS location. For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation) and closed (i.e., received TA commands) control loops. Support of pre-compensation of the calculated TA in its uplink transmissions Support of frequency pre-compensation to counter shift the Doppler experienced on the service link. Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated. Support of UE receiving cell-specific K_offset in system information. |  | Yes | N/A | If UE does not support this feature, the performance of ATG UE cannot be guaranteed due to the large propagation delay. | Per UE | No | FR1 only | N/A |  | Mandatory with capability signaling for UE supports NR communication via ATG |
| UE reporting of TA information | Support UE reporting of TA information | [Uplink Time and Frequency pre-compensation and timing relationship enhancements] | Yes | N/A | If UE does not support this feature, UE cannot report the TA information to network. | Per UE | No | FR1 only | N/A |  | Optional with capability siganling |
| Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | N/A | If UE does not support this feature, the HARQ process is number is limited. | Per UE | No | FR1 only | N/A |  | Optional with capability signalling |
| K1 range extension | Support of extended K1 value range of (0..31) for unpaired spectrum |  | Yes | N/A | If UE does not support this feature, K1 value is limited. | Per UE | TDD only | FR1 only | N/A |  | Optional with capability signalling |

- L2/3 features

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location based CHO | Indicates whether the UE supports location based CHO with event A3, A4, A5 |  | Yes | N/A | If UE does not support this feature, location based CHO cannot be configured. | Per UE | No | FR1 only | N/A |  | Optional with capability siganling |

- RRM features

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Indicate the support of enhanced inter-frequency cell re-selection requirements for ATG (as specific in TS 38.133 Table 4.2D.2.4-2) |  | no | N/A | If UE does not support the capability, legacy measurement requirements (as specified in TS 38.133, Table 4.2D.2.4-1) are applied. | Per UE | No | FR1 only | N/A |  | Optional without capability signaling |

FFS:

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location-based measurement report trigger | Indicates whether the UE supports location-based triggered measurement reporting (i.e., event D1) |  | Yes | N/A | If UE does not support this feature, location-based triggered measurement reporting cannot be configured. | [Per Band] | No | FR1 only | N/A |  | Optional with capability siganling |
| SR triggered by a TA report | Indicates whether the UE supports triggering of SR when a TA report is triggered and there are no available UL-SCH resources. |  | Yes | N/A | If UE does not support this feature, SR cannot be triggered by a TA report. | [Per Band] | No | FR1 only | N/A |  | Optional with capability siganling |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 as specified in TS 38.321 [10]. |  | Yes | N/A | If UE does not support this feature, TA reporting during initial access cannot be configured. | [Per Band] | No | FR1 only | N/A |  | Optional with capability siganling |

Issue 1-3:  UE specific koffset

- Agreement:

  - The koffset mechanism (i.e. cell-specific koffset ) should be supported in R18 ATG.

  - The maximum value of cell-specific koffset (unit: slot of 15kHz SCS) should cover 3ms (3 slots) in ATG scenario

Topic #2: ATG RRM performance requirements

Issue 2-1: TDD pattern

- Agreement:

  - The legacy TDD pattern are used as baseline TDD pattern for all ATG test cases.

  - In Section A.3.1.4, define a new TDD configuration with the pattern of  ‘30D4S6U’ for ATG UE.

  - Introduce the new TDD pattern ‘30D4S6U’ in one or more ATG test cases. FFS which test cases.

  - In the test, the new TDD pattern configuration only applies to UE supporting increasing the number of HARQ processes and K1 range extension.

  - If UE pass the test cases with new TDD pattern, the same test cases with legacy TDD pattern can be skipped.

Issue 2-2: Channel model

- Agreement:

Use the AWGN with residual doppler channel model for RRM test cases

Issue 2-3: Whether to define test cases for TCI switching delay requirements

- Agreement:

  - Do not define test case for TCI state switching for ATG UE

Issue 2-4: GNSS setup

- Agreement:

Positioning is viable via AT command for all test cases

Issue 2-5: UE mobility assumption

- Agreement:

  - GNSS changed during the test for location-based CHO.

  - FFS on the UL transmit timing test

    - Option 1: Constant GNSS

    - Option 2: Changed GNSS

  - FFS how to set the GNSS change,

    - Option 1: 1200km/h.

    - Option 2: modeled using the Doppler shift

    - Option 3: The GNSS change should be set with the consideration of two distance threshold istanceThreshFromReference1 and distanceThreshFromReference2, e.g. max{distanceThreshFromReference1,distanceThreshFromReference2}+50m.

Issue 2-6: Test method for UE with antenna array

- Proposals

  - Option 1: The approach of only to introduce the scaling factor in the RRM core requirement and not to have the scaling factor in the tests is more simpler and cheaper. (CATT)

  - Option 2: Conducted test should be reused for ATG UE with antenna array, the scaling factor need to be considered in the test requirements. (CMCC, LGE, HW)

  - Option 3: RAN4 to consult RAN5 feedback on whether it is feasibility to define test cases for ATG UEs in FR1 with beam sweeping capability. (Ericsson)

  - Option 4: Two alternatives for the test method with antenna array assumption: (ZTE)

    - Alternative 1: Define OTA test to verify beam sweeping for ATG UE with phase antenna array capability. (CATT open to discuss the feasibility)

    - Alternative 2: Not to distinguish the test between conductive test and OTA test, only focus on the requirements of delay. Leave the test details to actual implementation.

Issue 2-7: Neighbour cell configuration

- Proposals

  - Option 1: For the TDD cell re-selection test and and intra/inter-frequency measurement test for UE with antenna array, choose some test cases to configure 2 neighbour cells.

Issue 2-8: RMC

- Agreement:

  - The legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases

  - New RMCs are needed for new TDD configuration

Issue 2-9: OCNG

- Agreement:

  - Following legacy NR OCNGs are reused for ATG test cases:

    - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133.

    - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133.

    - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

Issue 2-10: Test scope

- Proposals

  - Option 1: All the tests listed in last meeting’s agreement should be applied for ATG UE. (CMCC, ZTE)

  - Option 2: Test case list presented in R4-2317341 is adopted for ATG performance requirements with following changes: (Ericsson)

    - Only non-DRX mode tests are defined in CONNECTED mode.

    - For HO: only inter-frequency handover with unknown target cell is defined.

    - For RRC re-establishment, following tests are defined:

      - Intra-frequency RRC Re-establishment with known target cell

      - Inter-frequency RRC Re-establishment with unknown target cell

- The tests listed in last meeting’s agreement:

| Test scope RRC_IDLE state mobility | Test number |
| --- | --- |
| Cell reselection to intra-frequency neighbour cell | ATG 1-1 |
| Location based cell reselection to intra-frequency neighbour cell | ATG 1-2 |
| Cell reselection to inter-frequency neighbour cell | ATG 1-3 |
| Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements] | ATG 1-4 |
| Location based cell reselection to inter-frequency neighbour cell | ATG 1-5 |

| Test scope RRC_CONNECTED state mobility | Test number |
| --- | --- |
| Intra-frequency HO with known target cell | ATG 2-1 |
| Intra-frequency HO with unknown target cell | ATG 2-2 |
| Inter-frequency HO with known target cell | ATG 2-3 |
| Inter-frequency HO with unknown target cell | ATG 2-4 |
| Intra-frequency location based CHO | ATG 2-5 |
| Inter-frequency location based CHO | ATG 2-6 |
| Intra-frequency RRC Re-establishment with known target cell | ATG 2-7 |
| Intra-frequency RRC Re-establishment with unknown target cell | ATG 2-8 |
| Inter-frequency RRC Re-establishment with known target cell | ATG 2-9 |
| Inter-frequency RRC Re-establishment with unknown target cell | ATG 2-10 |
| Inter-frequency RRC Re-establishment in FR1 without serving cell timing | ATG 2-11 |
| 4-step RA type contention based random access test | ATG 2-12 |
| 4-step RA type Non-Contention based random access test | ATG 2-13 |
| 2-step RA type contention based random access test | ATG 2-14 |
| 2-step RA type Non-Contention based random access test | ATG 2-15 |
| RRC Connection Release with Redirection | ATG 2-16 |

| Test scope Timing | Test number |
| --- | --- |
| UE transmit timing test | ATG 3-1 |
| Timing advance adjustment delay and accuracy | ATG 3-2 |

| Test scope Signalling characteristics | Test number |
| --- | --- |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-1 |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-2 |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-3 |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-4 |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | ATG 4-5 |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | ATG 4-6 |
| DCI-based and Timer-based Active BWP Switch | ATG 4-7 |
| RRC-based Active BWP Switch | ATG 4-8 |
| UE specific CBW change | ATG 4-9 |
| Pathloss reference signal switching delay | ATG 4-10 |

| Test scope Measurement procedure | Test number |
| --- | --- |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX | ATG 5-1 |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX | ATG 5-2 |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading | ATG 5-3 |
| Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | ATG 5-4 |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | ATG 5-5 |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | ATG 5-6 |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used | ATG 5-7 |
| SSB based L1-RSRP measurement when DRX is not used | ATG 5-8 |
| CSI-RS based L1-RSRP measurement when DRX is not used | ATG 5-9 |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used | ATG 5-10 |
| L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used | ATG 5-11 |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used | ATG 5-12 |
| SA intra-frequency CGI identification of NR neighbor cell in FR1 | ATG 5-13 |

| Test scope Measurement Performance requirements | Test number |
| --- | --- |
| SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-1 |
| SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-2 |
| SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-3 |
| SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-4 |
| SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-5 |
| SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-6 |
| L1-RSRP measurement SSB based L1-RSRP measurement | ATG 6-7 |
| L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off | ATG 6-8 |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off | ATG 6-9 |
| L1-SINR measurement with SSB based CMR and dedicated IMR | ATG 6-10 |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR | ATG 6-11 |
| CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-12 |
| CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-13 |
| CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-14 |
| CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-15 |
| CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-16 |
| CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-17 |