3GPP TSG-RAN WG4 Meeting #109	R4-2321896

Chicago, USA, 13th November – 17th November 2023

Source:	ZTE Corporation

Title:	TP to TR37.718-21-11 to add DC_8A-39A_n79A and DC_8A-39A_n79C

Agenda Item:	7.4.2

Document for:	Approval

- Introduction

The configurations of DC_8A-39A_n79A and DC_8A-39A_n79C were requested in RAN4#108 meeting. So this contribution provides a text proposal to introduce DC_8A-39A_n79A and DC_8A-39A_n79C in TR37.718-21-11.

- Reference

- [1] TR37.718-21-11, v0.7.0

- Text Proposal

----- Start of TP -----

  - 5.x			DC_8-39_n79

    - 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-39A_n79A DC_8A-39A_n79C | DC_8A_n79A DC_39A_n79A |

    - 5.x.2	Co-existence studies

In terms of the co-existence studies of the fallbacks of DC_8_n79 and DC_39_n79, which are captured in the TR 37.863-01-01, it can be found that:

- IMD4 products produced  by Band 8 and Band n79 that impact the reference sensitivity of LTE band 39.

- IMD3, IMD4 products produced  by Band 39 and Band n79 that impact the reference sensitivity of LTE band 8.

    - 5.x.3	∆TIB and ∆RIB values

For DC_8-39_n79, the TIB,c and RIB,c values are reused from DC_8_n39-n79 and given in the tables below.

Table 5.x.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8-39_n79 | 0.3 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.x.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8-39_n79 | - | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

    - 5.x.4	Reference sensitivity exceptions

According to the co-existence study, the MSD values are defined as below, in which the corresponding MSD values are reused from the NR 3DL/2UL CA_n8A-n39A-n79A

Table 5.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8-39_n79 | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 39 | N/A | 5 | N/A | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 15.1 | IMD3 |
|  | 39 | 1900 | 10 | 50 | 1900 | N/A | N/A |
|  | n79 | 4740 | 40 | 216 | 4740 | N/A | N/A |
|  | 8 | N/A | 5 | N/A | 940 | 7.1 | IMD4 |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n79 | 4760 | 40 | 216 | 4760 | N/A | N/A |

----- End of TP -----