3GPP TSG-RAN WG4 Meeting #110	R4-2401713

# Athens, 26th Feb - 1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | Draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on FRC for NCR-MT (TS38.106, Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce FRC for NCR-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For introducing FRC for NCR-MT, update clause 2, add new clause B, change original clause B to clause D. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, B (New clause), B (original) -> D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.115-1, TS 38.115-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause: B Clause number change: B (original) -> D |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TR 25.942: "RF system scenarios".

[4]	Recommendation ITU-R SM.328: "Spectra and bandwidth of emissions".

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[6]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”.

[7]	3GPP TS 38.115-1: “NR; Repeater conformance testing - Part 1: Conducted conformance testing”.

[8]	3GPP TS 38.115-2: “NR; Repeater conformance testing - Part 2: Radiated conformance testing”.

[9]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain".

[10]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[11]	Void

[12]	Void

[13]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[14]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”.

[15]	Void

[16]	Void

[17]	Void

[18]	Void

[19]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[20]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[21]	3GPP TS 38.211: "NR; Physical channels and modulation".

[22]	3GPP TS 38.174: "NR; Integrated Access and Backhaul (IAB) radio transmission and reception".

[23]	3GPP TS 38.331: “NR; Radio Resource Control (RRC); Protocol specification”.

[24]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

[25]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”.

[26]	3GPP TS 38.211: “NR;  Physical channels and modulation”.

[27]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

[28]	3GPP TS 38.214: "NR; Physical layer procedures for data".

<END OF THE CHANGE 1>

<START OF THE CHANGE 2>

Annex B (normative):
NCR-MT Reference measurement channels

## B.1	IAB-MT Fixed Reference Channels

### B.1.1	Fixed Reference Channels for PDSCH performance requirements (QPSK)

The parameters for the reference measurement channels are specified in table B.1.1-1 for FR1 PDSCH performance requirements.

The parameters for the reference measurement channels are specified in table B.1.1-2 for FR2-1 PDSCH performance requirements.

Table B.1.1-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layer, QPSK

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | M-FR1-B.1.1-1 | M-FR1-B.1.1-2 |
| Channel bandwidth | MHz | 10 | 40 |
| Subcarrier spacing | kHz | 15 | 30 |
| Allocated resource blocks | PRBs | 52 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  | 4096 | 8456 |
| Transport block CRC per Slot |  | 24 | 24 |
| Number of Code Blocks per Slot |  | 1 | 2 |
| Binary Channel Bits Per Slot |  | 13728 | 27984 |

Table B.1.1-2: FRC parameters for FR2-1 PDSCH performance requirements, 1 transmission layer, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR2-B.1.1-1 |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

### B.1.2	Fixed Reference Channels for PDSCH performance requirements (16QAM)

The parameters for the reference measurement channels are specified in table B.1.2-1 for FR1 PDSCH performance requirements.

Table B.1.2-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layer, 16QAM

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | M-FR1-B.1.2-1 | M-FR1-B.1.2-2 |
| Channel bandwidth | MHz | 10 | 40 |
| Subcarrier spacing | kHz | 15 | 30 |
| Allocated resource blocks | PRBs | 52 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  | 13064 | 26632 |
| Transport block CRC per Slot |  | 24 | 24 |
| Number of Code Blocks per Slot |  | 2 | 4 |
| Binary Channel Bits Per Slot |  | 27456 | 55968 |

### B.1.3	Fixed Reference Channels for PDCCH performance requirements

The parameters for the reference measurement channels are specified in table B.1.3-1 for FR1 PDCCH performance requirements.

The parameters for the reference measurement channels are specified in table B.1.3-2 for FR2-1 PDCCH performance requirements.

Table B.1.3-1: FR1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | M-FR1-B.1.3-1 | M-FR1-B.1.3-2 | M-FR1-B.1.3-3 | M-FR1-B.1.3-4 | M-FR1-B.1.3-5 | M-FR1-B.1.3-6 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 24 | 48 | 48 | 102 | 102 | 90 |
| CORESET time domain allocation |  | 2 | 2 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 | 52 | 41 | 53 | 53 |

Table B.1.3-2: FR2-1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | M-FR2-B.1.3-1 | M-FR2-B.1.3-2 | M-FR2-B.1.3-3 |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 90 |
| CORESET time domain allocation |  | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 41 | 53 | 53 |

### B.1.4	Fixed Reference Channels for CSI reporting performance

This clause defines the DL signal applicable to the reporting of channel status information.

Tables in this clause specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [11] and with MCS definition specified in clause 5.1.3 of TS 38.214 [11].

Table B.1.4-1: Fixed Reference Channels for FR1 CSI reporting with CQI table 2 and MCS table 2

| Reference channel |  |  |  | M-FR1-B.1.4-1 | M-FR1-B.1.4-2 |
| --- | --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18960 |
| 5 | 1.9141 | 7 |  | 12040 | 24576 |
| 6 | 2.4063 | 9 |  | 15112 | 30728 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 34816 |
| 8 | 3.3223 | 13 |  | 20496 | 42016 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 |
| 10 | 4.5234 | 17 |  | 28168 | 57376 |
| 11 | 5.1152 | 19 |  | 31752 | 65576 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 |
| 13 | 6.2266 | 23 |  | 38936 | 79896 |
| 14 | 6.9141 | 25 |  | 43032 | 88064 |
| 15 | 7.4063 | 27 |  | 46104 | 94248 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |  |

Table B.1.4-2: Fixed Reference Channels for FR2-1 CSI reporting with CQI table 1 and MCS table 1

| Reference channel |  |  |  | M-FR2-B.1.4-1 |
| --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 66 |
| Number of consecutive PDSCH symbols |  |  |  | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 |
| Overhead for TBS determination |  |  |  | 6 |
| Available RE-s for PDSCH |  |  |  | 7590 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |
| 0 | OOR | OOR | OOR | N/A |
| 1 | 0.1523 | 0 | QPSK | 1800 |
| 2 | 0.2344 | 0 |  | 1800 |
| 3 | 0.3770 | 2 |  | 2856 |
| 4 | 0.6016 | 4 |  | 4480 |
| 5 | 0.8770 | 6 |  | 6528 |
| 6 | 1.1758 | 8 |  | 8712 |
| 7 | 1.4766 | 11 | 16QAM | 11016 |
| 8 | 1.9141 | 13 |  | 14343 |
| 9 | 2.4063 | 15 |  | 17928 |
| 10 | 2.7305 | 18 | 64QAM | 20496 |
| 11 | 3.3223 | 20 |  | 25104 |
| 12 | 3.9023 | 22 |  | 29192 |
| 13 | 4.5234 | 24 |  | 33816 |
| 14 | 5.1152 | 26 |  | 38936 |
| 15 | 5.5547 | 28 |  | 42016 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |

<END OF THE CHANGE 2>

<START OF THE CHANGE 3>

Annex D (informative):
Change history

<END OF THE CHANGE 3>