3GPP TSG-RAN WG4 Meeting #110bis              	R4-2405869

Changsha, China, 15th-19th April, 2024

Agenda item:	6.16.8.1

Source:	Samsung

Title:	Discussion and simulation results for NTN enhancement

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope and test setup of demodulation requirements for NTN enhancement was discussed.  The related agreement was captured in the WF as following [1]

- General - Channel model - Doppler for DL - Option 1: 600Hz - Option 2: 1200Hz - Option 3: 2000Hz - Doppler for UL - 3000Hz as baseline - SAN demodulation requirement - General issues for above 10GHz bands - Antenna configuration - Do not consider 2Tx - Phase Noise - Do not model the phase noise for PUSCH SAN demodulation requirements in FR2-NTN in the ideal simulation assumption. The impact of phase noise can be considered in the impairment results - Test setup for normal PUSCH with CP-OFDM for above 10GHz bands - MCS - MCS 2/16/20 in Table 1 with down selection based on SNR operating point - SNR operating point should be lower than the link budget - Additional DM-RS position - Pos 1 - RB assignment - Full applicable test bandwidth - PTRS configuration - Not configure PT-RS for all test cases to be defines - FFS the configuration of PT-RS for test cases other than MCS2 - Other parameters - Test setup for normal PUSCH with DFT-s-OFDM for above 10GHz - MCS - MCS 2 in Table 1 - Additional DM-RS position - Pos 1 - RB assignment - 30 PRBs in the middle of the test bandwidth - PTRS configuration - Do not configure - Test setup for PUSCH repetition type A for above 10GHz bands - MCS - MCS 5 in Table 3 - Test setup for PUCCH for above 10 GHz bands - UCI info - FFS on the UCI information on PUCCH demodulation requirement - Test setup for PUCCH with DM-RS bunding for FR1 UL coverage enhancement - Timing drift - Do not model timing drift/frequency drift in the ideal simulation. The impact of timing drift/frequency drift can be considered in the impairment results. - Antenna configuration - Consider both 1Tx1Rx and 1Tx2Rx for PUSCH with DMRS bundling demodulation requirements. Apply the same test applicability for 1T1Rx and 1Tx2Rx performance test defined in TS 38.181. - Channel Model - NTN-TDLA 100-200 - MCS - MCS 4 in Table 1 - Additional DM-RS position - Position - RB assignment - Option 1: Full applicable test bandwidth - Option 2: 6RBs for both 15KHz and 30KHz - PUSCH mapping type - A and B, Apply the same test applicability for 1Tx1Rx and 1Tx2Rx performance test defined in TS 38.101 - Allocation length - 14 - PTRS configuration - Do not configure - RV sequence - 0,0,0,0 - PUSCH aggregation factor - Option 1: n8 - Option 2: n4 - PUSCH-TimeDomainWindowlength - Option 1: 8 - Option 2: 4

In this contribution, the view on the remaining issue of test setup for demodulation requirement for NTN enhancement.

# 2	General

Channel Model

As for Maximum Doppler spread, up to 1000km/h can be supported based on VSIT UE. How to derive the doppler, based on the residual frequency error after compensation prior to the baseband processing, another is based on UE speed.

Based on frequency range for above 10GHz in Ka-band as following, up to 30GHz is supported in UL and 20GHz is supported in DL

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

Assuming the 0.1 ppm for residual Frequency error after pre-compensation, the related frequency offset is around 3000Hz and 2000Hz, respectively. While basing on the UE speed with 120km/h, the maximum doppler should be 3333Hz, and if taking UE speed with 1000km/h, the maximum doppler will be extremally high, which is not practical in the FR2 considering the beam forming.

Based on the discussion in the last meeting, for UE side, three options with 600Hz, 1200Hz and 2000Hz, where the value of 2000Hz is the worst cases of residual frequency error with assuming 0.1ppm. Also, 3000Hz was considered baseline as the residual frequency error in SAN side.  Based on our results for SAN PUSCH in the last meeting [2], it is feasible to achieve the maximum throughput with worse case of residual frequency 3000Hz.

Meanwhile, from channel perspective, NTN-TDLC5 was considered for specifying the related demodulation requirement for above 10GHz, it is almost LOS channel. Therefore, we do not think there is any issue to handle the residual doppler with considering 0.1ppm based on existing RS configuration,

Considering the doppler value was derived based on 0.1ppm requirement in S band, the same approach can be applied for NTN scenario with above 10GHz.

Proposal 1: The doppler value with 2000Hz can be considered to define the PDSCH requirement

# 3	Test setup of BS demodulation requirement

In this part, the remaining issue of test setup for SAN demodulation requirement is discussed based on the test scope agreed in the previous meeting.

## 3.1	Test setup of PUSCH requirement with DMRS bundling

In Rel-17, DMRS bundling was introduced for coverage enhancement. To enable DMRS bunding feature, RAN4 has introduced the related phase difference, timing error and frequency error limitation requirement.

Actual TDW and nominal TDW

For Rel-17 CE with DMRS bundling, the value of nominal TDW is configured for 8 for FDD and 2 for TDD, and the values are same the configured PUSCH aggregation factor. Different with TN scenario, both UE and SAN are moving, the timing and frequency drifting model should be considered in the NTN scenario in the practical scenario, similar as IoT NTN scenario, the timing and frequency drifting model were considered to define requirement with multiple repetition transmission, considering UE can do the pre-compression for the first transmission.

Meanwhile, the channel model for NTN scenario is NTN TDLA100-200, which has large value of doppler, it is difficult to guarantee the phase continuous with large TDW and PUSCH aggregation, compared with TDLA30-10 for specifying the PUSCH requirement with DMRS bundling.

Although it was agreed that there is no modeling timing drift/frequency drift in the ideal simulation, to leave some margin, our preference is to consider the small value for nominal TDW for specifying PUSCH requirement with DMRS bundling

Proposal 2: The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- Actual TDW and PUSCH-TimeDomainWindowLength = 4 for FDD

- PUSCH aggregation factor = n4 for FDD

BW& SCS

Regarding the channel bandwidth, two options are considered with full applicable test bandwidth, or 6RBs for both 15KHz and 30KHz SCS.

In Rel-17 TN scenario, CE with DMRS bundling, the minimum CBW for each SCS was considered for requirement definition.  As observed in RAN1, phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier. Therefore, large CBWs or larger PRB may be not applied for NTN-specific UE with DMRS bundling, considering for satisfying the phase difference limit.

Proposal 3: The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- PRB: 6RBs for both 15KHz and 30KHz

In summary, the following test parameters can be considered for PUSCH requirement with DMRS bundling.

Table 1: Simulation assumption for PUSCH requirement with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern [Note 1] |  | 15 kHz SCS: FDD 30 kHz SCS: FDD |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 2] | 0, 0, 0, 0 for FDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A and B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | [n4] for FDD |
| pusch-TimeDomainWindowLength |  | [4] slots for FDD |
| Frequency domain resource assignment | RB assignment | 6RB for both 15KHz and 30KHz |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 0, 0, 0} with slot aggregation. |  |  |

## 3.2	Test setup of SAN demodulation requirement above 10GHz

In this subsection, the remaining issue of test setup for SAN demodulation requirements above 10GHz, including PUCCH, and PUSCH

PUSCH

Regarding the PTRS configuration, since the carrier frequency of NTN above 10GHz is up to 30GHz, the phase noise impact should not be omitted, especially for high MCS. Similar as TN scenario in FR2-1, both with PTRS configured or without PTRS configured shall be considered.

Proposal 4:  Both with PTRS configured or without PTRS configured shall be considered for PUSCH requirement for MCS 16 and MCS 20

PUCCH

In the last meeting, regarding which UCI information considered for defining PUCCH requirement above 10GHz are under discussion. For Rel-17 NTN WI, the PUCCH requirements with all the formats with including both ACK/NACK, and CSI payload, where the UCI information does not contain CSI part 2.

For Rel-18 NTN scenario, since there is no limitation of UE to report CSI information for above 10GHz, therefore, the same PUCCH formats can be considered for PUCCH requirement. The same test parameters can be reused for requirement definition, excepting for channel model.

Proposal 5:  Both ACK/NACK and CSI payload can be considered for specifying PUCCH requirement above 10GHz.

Proposal 6: The following test parameters could be considered for specifying PUCCH requirement for above 10GHz

Table 2:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 3:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 4:   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 5:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 6:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 7:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |
| DMRS | With and without additional DMRS |

# 4	Simulation Results

In this section, the initial simulation results based agreed test parameters for SAN demodulation requirement were provided

## 4.1	PUSCH requirement

Table 8: Initial simulation results for PUSCH requirement with CP-OFDM

| Test case | waveform | SCS& BW | MCS | Channel | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@ 70 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CP-OFDM | 120KH 50MHz | 2 | NTN TDLC5-3000 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -5.33 |
| 2 | CP-OFDM | 120KH 50MHz | 16 | NTN TDLC5-3000 | B | 10 | 1T2R | Disable | 1+1 (0,8) | 7.09 |
| 3 | CP-OFDM | 120KH 50MHz | 20 | NTN TDLC5-3000 | B | 10 | 1T2R | Disable | 1+1 (0,8) | 10.6 |
| 4 | CP-OFDM | 120KH 50MHz | 16 | NTN TDLC5-3000 | B | 10 | 1T2R | K_ptrs=2 L_ptrs=1 | 1+1 (0,8) | 7.47 |
| 5 | CP-OFDM | 120KH 50MHz | 20 | NTN TDLC5-3000 | B | 10 | 1T2R | K_ptrs=2 L_ptrs=1 | 1+1 (0,8) | 11.03 |
| 6 | CP-OFDM | 120KH 50MHz | 2 | NTN TDLC5-3000 | B | 10 | 1T1R | Disable | 1+1 (0,8) | -1.83 |
| 7 | CP-OFDM | 120KH 50MHz | 16 | NTN TDLC5-3000 | B | 10 | 1T1R | Disable | 1+1 (0,8) | 10.55 |
| 8 | CP-OFDM | 120KH 50MHz | 20 | NTN TDLC5-3000 | B | 10 | 1T1R | Disable | 1+1 (0,8) | 14.16 |
| 9 | CP-OFDM | 120KH 50MHz | 16 | NTN TDLC5-3000 | B | 10 | 1T1R | Disable | 1+1 (0,8) | 10.92 |
| 10 | CP-OFDM | 120KH 50MHz | 20 | NTN TDLC5-3000 | B | 10 | 1T1R | K_ptrs=2 L_ptrs=1 | 1+1 (0,8) | 14.57 |

Table 9: Initial simulation results for PUSCH requirement with DFT-s-OFDM

| Test case | waveform | SCS& BW | RB | MCS | Channel | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@ 70 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM | 120KH 50MHz | 30 PRBs in the middle of test bandwidth | 2 | NTN TDLC5-3000 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -5.19 |
| 2 | DFT-s-OFDM | 120KH 50MHz | 30 PRBs in the middle of test bandwidth | 2 | NTN TDLC5-3000 | B | 10 | 1T1R | Disable | 1+1 (0,8) | 1.71 |

## 4.2	PUCCH requirement

Table 10: Initial simulation results for Format 0

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 0 | 120KH 50MHz | Disabled | 1 | NTN-TDLC5-3000 | 9 | 1T2R | 1 | 1.93 |
| 1 | Format 0 | 120KH 50MHz | Enabled | 1 | NTN-TDLC5-3000 | 9 | 1T2R | 2 | -0.43 |
| 1 | Format 0 | 120KH 50MHz | Disabled | 1 | NTN-TDLC5-3000 | 9 | 1T1R | 1 | 6.76 |
| 1 | Format 0 | 120KH 50MHz | Enabled | 1 | NTN-TDLC5-3000 | 9 | 1T1R | 2 | 4.37 |

Table 11: Initial simulation results for Format 1

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) | NACK to ACK %0.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-3000 | 1 | 1T2R | 14 | -9.22 | -7.56 |
| 2 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-3000 | 1 | 1T1R | 14 | -5.30 | -3.52 |

Table 12Initial simulation results for Format 2

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 2 | 120KH 50MHz | Disabled | 4 | NTN-TDLC5-3000 | 4 | 1T2R | 1 | -1.31 |
| 2 | Format 2 | 120KH 50MHz | Disabled | 4 | NTN-TDLC5-3000 | 4 | 1T1R | 1 | 3.73 |
| 3 | Format 2 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-3000 | 9 | 1T2R | 2 | -3.38 |
| 4 | Format 2 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-3000 | 9 | 1T1R | 2 | 1.82 |

Table 13: Initial simulation results for Format 3

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | DMRS | Number of Symbol | 1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-3000 | 1 | 1T2R | No additional DMRS | 14 | -5.51 |
| 2 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-3000 | 1 | 1T2R | With additional DMRS | 14 | -5.80 |
| 3 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-3000 | 1 | 1T1R | No additional DMRS | 14 | -1.73 |
| 4 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-3000 | 1 | 1T1R | With additional DMRS | 14 | -1.86 |
| 5 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-3000 | 3 | 1T2R | No additional DMRS | 4 | -2.87 |
| 6 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-3000 | 3 | 1T2R | No additional DMRS | 4 | 2.33 |

Table 14: Initial simulation results for Format 4

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | DMRS | Number of Symbol | 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-3000 | 1 | 1T2R | No additional DMRS | 14 | -3.31 |
| 2 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-3000 | 1 | 1T2R | With additional DMRS | 14 | -3.51 |
| 3 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-3000 | 1 | 1T1R | No additional DMRS | 14 | 0.98 |
| 4 | Format 4 | 120KHz 50MHz | Enabled | 22 | NTN-TDLC5-3000 | 1 | 1T1R | With additional DMRS | 14 | 1.00 |

## 4.3	PRACH requirement

Table 15: Initial simulation results for Format 4

| Test case | Format | SCS& BW | PRACH sequence length | Ncs | Logical sequence index | v | Number of Tx and Rx | Channel | Frequency offset | Time estimation tolerance | SNR@1 % Miss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C2 | 120KH | 139 | 69 | 0 | 0 | 1T2R | NTN-TDLC5-3000 | 3000 | 0.13us | -5.77 |
| 2 | B4 | 120KH | 139 | 69 | 0- | 0 | 1T2R | NTN-TDLC5-3000 | 3000 | 0.13us | -10.92 |
| 3 | C2 | 120KH | 139 | 69 | 0 | 0 | 1T1R | NTN-TDLC5-3000 | 3000 | 0.13us | -11.25 |
| 4 | B4 | 120KHz | 139 | 69 | 0 | 0 | 1T1R | NTN-TDLC5-3000 | 3000 | 0.13us | -15.83 |

# 5	Conclusion

In this contribution, the view on remaining issue of BS demodulation requirement for NTN enhancement was provided

Proposal 1: The doppler value with 2000Hz can be considered to define the PDSCH requirement

Proposal 2: The following value for actual TDW and nominal TDW can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- Actual TDW and PUSCH-TimeDomainWindowLength = 4 for FDD

- PUSCH aggregation factor = n4 for FDD

Proposal 3: The following PRB allocation can be considered for PUSCH requirement with DMRS bundling in NTN scenario

- PRB: 6RBs for both 15KHz and 30KHz

Proposal 4:  Both with PTRS configured or without PTRS configured shall be considered for PUSCH requirement for MCS 16 and MCS 20

Proposal 5:  Both ACK/NACK and CSI payload can be considered for specifying PUCCH requirement above 10GHz.

Proposal 6: The following test parameters could be considered for specifying PUCCH requirement for above 10GHz

Table 2:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 3:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 4:   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 5:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 6:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 7:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |
| DMRS | With and without additional DMRS |

# References

R4-2402865, WF on NR_NTN_enh_SAN_UE_demod