3GPP TSG-RAN WG4 Meeting #111	R4-2407007

Fukuoka City, Fukuoka, Japan, 20 – 24 May, 2024

3GPP TSG RAN1#116bis Meeting	R1-2403752

# Changsha, China, Apr. 15th – 19th, 2024

Title:	LS on two TAs for intra-cell multi-DCI based Multi-TRP operation

Response to:

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Person:

Name:	Siva Muruganathan

E-mail Address:	Siva (dot) Muruganathan (at) ericsson (dot) com

Attachments:

1. Overall Description:

Related to the enhancement of two TAs for intra-cell multi-DCI based Multi-TRP operation, RAN1 made the following agreement regarding DL reference timing determination for PRACH transmission triggered by PDCCH order.

Agreement

For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the following for determining the PRACH timing:

Alt 4:

- For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 0:

  - If “PRACH association indicator” is 0, DL reference timing associated with coresetPoolIndex value 0 is used.

  - If “PRACH association indicator” is 1, DL reference timing associated with coresetPoolIndex value 1 is used.

- For PRACH transmission triggered by PDCCH order associated with coresetPoolIndex value 1:

  - If “PRACH association indicator” is 0, DL reference timing associated with coresetPoolIndex value 1 is used.

  - If “PRACH association indicator” is 1, DL reference timing associated with coresetPoolIndex value 0 is used.

2. Actions:

To RAN4.

ACTION: 	RAN1 would like respectfully ask RAN4 to capture the above agreement in RAN4 specifications.

4. Date of Next TSG-RAN1 Meetings:

TSG RAN WG1 Meeting #117		May 20 – May 24, 2024				Fukuoka, JP

TSG RAN WG1 Meeting #118		August 19 – August 23, 2024			Maastricht, NL