3GPP RAN TSG-RAN4 Meeting # 111		R4-2407072

Fukuoka City, Fukuoka, Japan, May 20 – 24, 2024

Agenda item:	6.1.1.1

Source:	Apple

Title:	MSD Analysis for CA_n40A-n41C

WI/SI:	NR_CADC_R18_2BDL_xBUL-Core

Release:	Rel-18

Document for:	Discussion

# 1	Introduction and Scope

Due to simultaneous Rx/Tx operation supported between n40 and n41, the inter-modulation products  (IMDs) caused by UL CA_n41C would fall into band 40 DL Rx. This implies that, to prevent n40 receiver desensitization due to IMDs,  MSD values should be defined. In the previous RAN4 meeting (#110Bis, Changsha, China, April 15 – 19, 2024), discussions were held on defining the MSD test points for the band combination CA_n40A-n41C. The WF for those discussions is captured in the summary highlighted in the Table below (from the final meeting report):

Sub-topic 1-2 CA_n40A-n41C Agreement: - Assign “WF on MSD requirements with intra-band contiguous ULCA” to Apple to further discuss if TR 38.862 guidelines on TDD band 1RB+1RB UL configuration should be revisited. - All MSD discussion papers to be noted.

It can be seen that n40DL will be subjected to intereferences caused by the 3rd order,  5th order, and 7th order intermodulation products (IMDs) due two carriers from UL CA_41C during simultaneous operation of n40 and n41 [1].  In this new contribution paper, we provide an update on our MSD analysis using the correct test point that was recommended in the previous meeting. We will assume PC3 for the UL transmission.

# Discussion

As depicted in the diagram below, with UL CA_n41C, due to the non-linearities in the n41C UL, two CC’s can create an IMD3 that could land in the n40 DL.

Figure 2.1: IMD3 created by two CCs in n41C UL could land in n40 DL

For CA_n40A-n41C, the bandwitch class is as shown in Table 2-1 below:

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n41C | CA_n41C CA_n40A-n41A CA_n40A-n41C | n40 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |

Figure 2.2: CA_n40A-n41C BCS

## MSD analysis for CA_n40A-n41C

For the MSD analysis, we will consider the architecture shown in Figure 2.1-1 below for the case of UL CA_41C PC3 with 1TX. The  worst-case assumptions for the RF front-end component isolation and linearity performance are shown in Table 2.1-1.

Figure 2.1-1: Front-end architecture used for CA_n40A-n41C MSD Analysis

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| Front-end loss | 4 | dB |
| Triplexer isolation | 20 | dB |
| n41filter rejection on n40 DL | 40 | dB |
| n40 filter TX to antenna rejection on to COM port on n41 DL | 35 | dB |
| sPnT switch IP3 | 70 | dBm |
| LNA IP3 | 0 | dBm |
| CBW | 5 | MHz |

Table 2.1-1: RF front-end rejection and linearity assumptions for CA_n8A1-n41A

With PC3 power level on n41, the simulated n41 2UL IMD3 level within n40 is 5.5dBm at the n41 PA output, which is 1.5dBm at the antenna port when 4dB of post-PA loss is assumed as indicated in the Table above.

Given the assumptions highlighted above, the MSD analysis is summarized in Table 2.1-2.

| Parameter | Unit | Main Path | Div. Path |
| --- | --- | --- | --- |
| Total n41 PA output noise | dBm/CBW | 5.5 |  |
| Total n41 PA output noise at antenna port | dBm/CBW | -34.5 | -44.5 |
| Thermal Noise | dBm/CBW | -96 | -96 |
| Composite noise | dBm/CBW | -34.5 | -44.5 |
| Composite noise after MRC | dBm | -43 |  |
| MSD | dB | 55 |  |

Table 2.1-2: MSD Analysis for CA_n40A_41C

# 3	Conclusion

In this contribution, we shared the result of our CA_n40A-n41C MSD analysis due to IMD3.

Proposal: Adopt CA_n25A-n77A MSD Levels proposed in Table 3-1 shown below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2358.5 | 55 | TDD | IMD3 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |

Table 3.1: IMD3 MSD proposal for CA_n40A_41C

# 4	References

RP-23401272 “TP for TR38.718-02-01_CA_n40A-n41C”, ZTE Corporation, Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, February 26th – March 1st, 2024