3GPP TSG-RAN WG4 Meeting #111	R4-2407140

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	Simulations for Coverage Enhancement BS Demodulation

Agenda item:	7.17.2

Document for:	Information

Introduction

This document introduces to RAN4 the features associated with coverage enhancement under the Work Item “Further NR coverage enhancements” and the impact of the newly introduced features on Base Station demodulation within RAN4.

Specifically, within this paper we provide simulation results for the PRACH repetition feature introduced in RAN1 and its impact for BS demodulation performance requirements.

Discussion

PRACH Repetitions

FR-1

The following section presents the simulation results of Nokia’s simulation campaign on PRACH repetitions in FR1

Table 1 : 99% Missed Detection for Different PRACH formats with Coverage Enhancements (FR1, ,1T2R)

| Preamble Format | BW | Propagation condition | Frequency offset (Hz) | SNR Ideal (dB) | SNR Impairment (dB) |
| --- | --- | --- | --- | --- | --- |
| Format A2 | 15kHz | AWGN | 0 | -18.8 | -16.3 |
|  |  | TDLC300-100 Low | 400 | -11.3 | -8.8 |
|  | 30kHz | AWGN | 0 | -18.8 | -16.3 |
|  |  | TDLC300-100 Low | 400 | -11.4 | -8.8 |
| Format B4 | 15kHz | AWGN | 0 | -23.5 | -21 |
|  |  | TDLC300-100 Low | 400 | -11.9 | -9.4 |
|  | 30kHz | AWGN | 0 | -23.5 | -21 |
|  |  | TDLC300-100 Low | 400 | -12.1 | -9.6 |
| Format C2 | 15kHz | AWGN | 0 | -18.8 | -16.3 |
|  |  | TDLC300-100 Low | 400 | -10.9 | -8.4 |
|  | 30kHz | AWGN | 0 | -18.8 | -16.3 |
|  |  | TDLC300-100 Low | 400 | -11.5 | -9 |

FR2-1

The following section presents the simulation results of Nokia’s simulation campaign on PRACH repetitions in FR2-1.

Table 1 : 99% Missed Detection for Different PRACH formats with Coverage Enhancements (FR2, 1T2R)

| Preamble Format | BW | Propagation condition | Frequency offset (Hz) | SNR Ideal (dB) | SNR Impairment (dB) |
| --- | --- | --- | --- | --- | --- |
| Format A2 | 60kHz | AWGN | 0 | -18.7 | -16 |
|  |  | TDLA30-300 Low | 4000 | -11.1 | -8.4 |
|  | 120kHz | AWGN | 0 | -18.8 | -16.1 |
|  |  | TDLA30-300 Low | 4000 | -12.1 | -9.4 |
| Format B4 | 60kHz | AWGN | 0 | -23.5 | -20.8 |
|  |  | TDLA30-300 Low | 4000 | -4.8 | -2.1 |
|  | 120kHz | AWGN | 0 | -23.5 | -20.8 |
|  |  | TDLA30-300 Low | 4000 | -14.8 | -12.1 |
| Format C2 | 60kHz | AWGN | 0 | -18.7 | -16 |
|  |  | TDLA30-300 Low | 4000 | -11.4 | -8.7 |
|  | 120kHz | AWGN | 0 | -18.8 | -16.1 |
|  |  | TDLA30-300 Low | 4000 | -12.3 | -9.6 |

Conclusion

This contribution provides initial results on our simulation campaign for PRACH repetitions on Coverage Enhancement and its impact on BS demodulation. No observations were made:

References

- RP-221858 “Further NR coverage enhancements” RAN #96 Budapest, Hungary, June 2022

PRACH configuration parameters

|  |  | Parameter name | Description | Value |
| --- | --- | --- | --- | --- |
| General |  | Carrier frequency |  | 28 GHz |
|  |  | Channel bandwidth | - | 50 MHz |
|  |  | Subcarrier spacing | - | 60 ¦ 120 kHz |
|  |  | Antenna configuration | - | 1T2R |
|  |  | Antenna type | - | Omni |
|  |  | Antenna gain | - | 0 dBi |
| PRACH | Preamble | preambleGeneration | The preambles can be generated randomly per TTI, which it is expected to result in a better detection performance or based on a fixed root sequence (mainly RAN4 assumption). | fixed-preamble-mapping |
|  |  | number-of-sequence-repetitions | This parameter defines the number of times a sequence is repeated, to enable sequence combining over multiple slots. | 2 ¦ 4 |
|  |  | nbr-preambles | This parameter defines the number of preambles used within the cell. | 64 |
|  |  | L | The parameter L defines the type of sequence. | 139 |
|  |  | format | NR supports several preamble formats, defined according to tables 6.3.3.1-1 and 6.3.3.1-2 in 38211-g00. | A2 ¦ B4 ¦ C2 |
|  |  | PRACH duration | Parameter defined according to tables 6.3.3.2-2, 6.3.3.2-3, and 6.3.3.2-4 | A2: 4 symbols ¦ B4: 12 symbols ¦ C2: 6 symbols |
|  |  | Number of PRACH occasions within a slot | Parameter defined according to tables 6.3.3.2-2, 6.3.3.2-3, and 6.3.3.2-4 | A2: 3 ¦ B4: 1 ¦ C2: 2 |
|  | Detection | Idft_size | This parameter defines IFFT at receiver side to increase detection resolution | 2048 |
|  |  | Sequence combining method | This parameter defines how sequences from different repetitions are combined together. | Mean |