3GPP TSG-RAN WG4 Meeting # 111									          R4-2407157

# Fukuoka, Japan, 20 – 24 May 2024

# Title:	PC2 CA_n71B MSD

# Agenda Item:	6.19.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# 1	Introduction

This contribution proposes intra-band contiguous downlink (DL) CA_n71B secondary component carrier (SCC) MSD analysis for power class 3 (PC3) and power class 2 (PC2) operation based on the test points discussed in [1,2] and agreed in WF [3]. A companion draftCR is proposed to complete this band combination at this meeting.

# Discussion

## 2.1 One UL Measurement Results

## 2.1.1 Power Amplifier Calibration & Front-end assumptions

For power amplifier (PA) measurements, we assume:

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB, C-IM3 -60 dBc, C-IM5 -70 dBc.

- PC3 operation and 4 dB post PA insertion losses.

- The PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR and:

  - ACLR -30 dBc for PC3 and transmit diversity (TxD) operation for each PA.

  - ACLR -31 dBc for PC2 operation.

For MSD analysis, we assume:

- Tx-Rx duplexer isolation: 55 dB;

- Tx-antenna duplex rejection in Rx range: 50 dB;

- Primary antenna to diversity antenna isolation: 10 dB;

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR); and

- Based on the agreed WF [4], the PC2 ΔRIBC is calculated relative to PC3 SCC CBW REFSENS requirement as specified in Table 7.3.2-1a.

## 2.1.2 One UL PC3, PC2 Noise Levels

Table 1 summarizes the measured interference levels affecting the DL SCC from [1,2].

Table 1: 1Tx PC3, PC2 single Tx and PC2 dual Tx (PC3+PC3) measured noise levels.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation | PC3 [1] [dBm/MBW] | PC2 single Tx [1] [dBm/MBW] | PC2 dual Tx PA1 [dBm/MBW] | PC2 dual Tx PA2 [dBm/MBW] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30 MHz + 5 MHz | 20 (RBstart = 0) | -49.5 | -46.6 | -49.1 | -48.9 |

Figure 1 shows the measured PA1 output spectrum for the 30 MHz+5 MHz configuration at 27 dBm output power.

Figure 1: PA1 PC3 output spectrum for CA_n71B 30 MHz+5 MHz 20RBstart=0.

## 2.1.3 One UL PC3 MSD

The CA_n71B PC3 one uplink carrier MSD for SCC 5MHz is evaluated at 0.7dB. The MSD test point and text proposal based on [5] is captured in Proposal 1.

Proposal 1: Consider adopting the text proposal highlighted in blue and MSD test point of Table 2 to introduce the PC3 CA_n71B SCC 5MHz CBW REFSENS for one uplink carrier.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. For power class 3, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 2 (new table “Table 7.3A.2.1-2”). The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 2: Power class 3 intra-band contiguous CA reference sensitivity with one uplink carrier (new Table 7.3A.2.1-2)

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B1 | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | [0.7] | FDD |
| NOTE 1: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |

## 2.1.3 One UL PC2 MSD

Proposal 2 is re-submission from [2].

Proposal 2: Consider adopting the text proposal highlighted in blue and MSD test point of Table 2 to introduce the PC2 CA_n71B SCC 5MHz CBW REFSENS for one uplink carrier.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. For power class 2, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 3 (new table “Table 7.3A.2.1-3”). The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 3: Power class 2 intra-band contiguous CA reference sensitivity with one uplink carrier (new Table 7.3A.2.1-3).

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30 MHz + 5 MHz | 20 (RBstart = 0) | [1.1] 3 | [1.6] 3 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with dual Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

# Conclusion

In this contribution, we propose the one uplink carrier MSD requirements for the 5MHz CBW SCC of CA_n71B for PC3 and PC2 operation for 1Tx and 2Tx architectures. A companion draftCR is presented at this meeting to help capture RAN4 agreements.

Proposal 1: Consider adopting the text proposal highlighted in blue and MSD test point of Table 2 to introduce the PC3 CA_n71B SCC 5MHz CBW REFSENS for one uplink carrier.

For power class 3, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 2 (new table “Table 7.3A.2.1-2”). The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 2: Power class 3 intra-band contiguous CA reference sensitivity with one uplink carrier (new Table 7.3A.2.1-2)

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B1 | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | [0.7] | FDD |
| NOTE 1: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |

Proposal 2: Consider adopting the text proposal highlighted in blue and MSD test point of Table 2 to introduce the PC2 CA_n71B SCC 5MHz CBW REFSENS for one uplink carrier.

For power class 2, the reference sensitivity power level is increased by ΔRIBC for specific uplink and downlink test points which are specified in Table 3 (new table “Table 7.3A.2.1-3”). The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 3: Power class 2 intra-band contiguous CA reference sensitivity with one uplink carrier (new Table 7.3A.2.1-3).

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30 MHz + 5 MHz | 20 (RBstart = 0) | [1.1] 3 | [1.6] 3 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with dual Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

# References

- R4-2400366 PC2 CA_n71B BCS4-5 MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions, Inc.

- R4-2405957 PC2 CA_n71B MSD, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Skyworks Solutions, Inc.

- R4-2406683 WF on MSD test point for CA_n71B MSD evaluation, 3GPP TSG-RAN WG4 Meeting #110-bis, Changsha, China, Qualcomm, Skyworks Solutions, Inc., Murata Manufacturing Co. Ltd.

- R4-2403628 WF on PC2 HPUE MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions, Inc., Huawei, Qualcomm Inc.

- R4-2405449 Missing MSD for PC3 CA_n71B BCS4_5, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Qualcomm Inc.