3GPP TSG-RAN WG4 Meeting # 111									          R4-2407165

# Fukuoka, Japan, 20 – 24 May, 2024

# Title:	PC2 CA_n41-n66 MSD

# Agenda Item:	6.18.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# 1	Introduction

This contribution proposes power class 2 (PC2) cross-band MSD requirements for CA_n41A-n66A.

# Discussion

Table 1 summarizes the source of cross-band interference affecting DL band n41 for CA_n41A-n66A BCS4/5.

Table 1: CA_n41-n66 source of cross-band interference for BCS4/5.

| Band combination | UL band | DL band | max UL BW | UL RB Allocation | min DL BW | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | LCRB | (MHz) |  |
| CA_n41-n66 | n66 | n41 | 45 | 240 | 10 | ACLR ~ 16 |
|  | n41 | n66 | 100 | 273 | 5 | ACLR ~ 3 |

Cross-band isolation test points are specified for Band n66. For Band n41, no test points are specified for Band n66 PC3 operation. Table 1 indicates that the DL Band n41 may be affected by ACLR ~16, i.e. it can be assumed that this DL band is affected by the transmitter chain “flat-band” noise. We propose to evaluate the Band n41 MSD based on this assumption.

## 2.1 MSD analysis

## 2.1.1 Front-end assumptions

For MSD analysis, we assume the front-end architecture of [1]:

- Band n66 duplexer Tx-Ant rejection at band n41: 30dB

- Diplexer rejection at band n41: 15dB

- Primary antenna to diversity antenna isolation: 10dB

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR)

- Flat-band noise power spectral density (PSD) at PA output due to transceiver input noise: -130dBm/Hz.

Table 2 summarizes the interference levels affecting DL Band n41.

Table 2:  PC3, PC2 single-Tx and PC2 dual-Tx (PC3+PC3) measured noise levels.

| UL band: UL CBW / UL carrier (MHz) | DL Band: DL CBW / DL carrier (MHz) | UL RB allocation | PC3 [dBm/MBW] | PC2 single Tx [dBm/MBW] | PC2 dual Tx PA1 [dBm/MBW] | PC2 dual Tx PA2 [dBm/MBW] |
| --- | --- | --- | --- | --- | --- | --- |
| n66: 45 / 1757.5 | n41: 10 / 2498.5 | 240 (RBstart=2) | -60.2 | -57.2 | -59.8 | -59.7 |

## 2.1.2 PC3, PC2 MSD

The detailed MSD analysis is summarized Table 3. With less than 0.4dB desense, the PC2 Band n41 MSD may be ignored.

Table 3: Band n41 10MHz PC2 MSD results.

# Conclusion

This contribution evaluates the PC2 MSD requirements for CA_n41A-n66A with the following proposal.

Proposal: Band n41 is not affected by cross-band isolation interference from band n66 PC2 operation.

# References

[1]	R4-1907461, TP for TR 37.716-11-11: DC_66A_n41A, Ericsson, T-Mobile US, MediaTek Inc, 3GPP TSG-RAN WG4 Meeting #91.