| 3GPP TR 37.718-11-21 V0.12.0 (2024-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	30

1	Scope	31

2	References	31

3	Definitions of terms, symbols and abbreviations	31

3.1	Terms	31

3.2	Symbols	32

3.3	Abbreviations	32

4	Background	32

4.1	TR Maintenance	32

5	Rel-18 DC of x bands (x = 1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL): General Part	32

5.1	UE RF architectures	32

5.2	General treatment of ∆TIB and ∆RIB values	32

6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	33

6.1	DC_38_n28-n78	33

6.1.1	Operating bands for DC	33

6.1.2	Configuration for DC	33

6.1.3	Co-existence studies	33

6.1.4	∆TIB and ∆RIB values	39

6.1.5	MSD requirements	39

6.2	DC_(n)3-n8	40

6.2.1	Operating bands for DC	40

6.2.2	Channel bandwidths per operating band for DC	41

6.2.3	Co-existence studies	41

6.2.4	∆TIB and ∆RIB values	41

6.3	DC_(n)3-n77	42

6.3.1	Operating bands for DC	42

6.3.2	Channel bandwidths per operating band for DC	42

6.3.3	Co-existence studies	42

6.3.4	∆TIB and ∆RIB values	43

6.3.5	MSD	43

6.4	DC_(n)3-n257	43

6.4.1	Operating bands for DC	43

6.4.2	Configuration for DC	43

6.4.3	Co-existence studies	44

6.4.4	∆TIB and ∆RIB values	44

6.4.5	MSD	44

6.5	DC_1_n8-n257	44

6.5.1	Operating bands for DC	44

6.5.2	Configuration for DC	45

6.5.3	Co-existence studies	45

6.5.4	∆TIB and ∆RIB values	45

6.5.5	MSD	45

6.6	DC_1_n77-n257	46

6.6.1	Operating bands for DC	46

6.6.2	Configuration for DC	46

6.6.3	Co-existence studies	46

6.6.4	∆TIB and ∆RIB values	46

6.6.5	MSD	47

6.7	DC_3_n77-n257	47

6.7.1	Operating bands for DC	47

6.7.2	Configuration for DC	47

6.7.3	Co-existence studies	47

6.7.4	∆TIB and ∆RIB values	47

6.7.5	MSD	48

6.8	DC_8_n3-n257	48

6.8.1	Operating bands for DC	48

6.8.2	Configuration for DC	48

6.8.3	Co-existence studies	48

6.8.4	∆TIB and ∆RIB values	48

6.8.5	MSD	49

6.9	DC_3-n26-n78	49

6.9.1	Operating bands for DC	49

6.9.2	Channel bandwidths per operating band for DC	49

6.9.3	Co-existence studies	50

6.9.4	∆TIB and ∆RIB values	51

6.9.5	MSD requirements	52

6.10	DC_7-n26-n78	52

6.10.1	Operating bands for DC	52

6.10.2	Channel bandwidths per operating band for DC	53

6.10.3	Co-existence studies	53

6.10.4	∆TIB and ∆RIB values	55

6.10.5	MSD requirements	56

6.11	DC_20_n1-n75	56

6.11.1	Operating bands for DC	56

6.11.2	Channel bandwidths per operating band for DC	57

6.11.3	UE co-existence studies	57

6.11.4	∆TIB and ∆RIB values	58

6.11.5	MSD	58

6.12	DC_7_n1-n75	58

6.12.1	Operating bands for DC	58

6.12.2	Channel bandwidths per operating band for DC	59

6.12.3	UE co-existence studies	59

6.12.4	∆TIB and ∆RIB values	60

6.12.5	MSD	60

6.13	DC_1-n26-n78	60

6.13.1	Operating bands for DC	60

6.13.2	Channel bandwidths per operating band for DC	61

6.13.3	Co-existence studies	61

6.13.4	∆TIB and ∆RIB values	63

6.13.5	MSD requirements	64

6.14	DC_3_n38-n40	64

6.14.1	Operating bands for DC	64

6.14.2	Configuration for DC	65

6.14.3	Co-existence studies	65

6.14.4	∆TIB and ∆RIB values	67

6.14.5	MSD requirements	68

6.15	DC_41_n1-n78	68

6.15.1	Configuration for DC	68

6.15.2	Channel bandwidths per operating band for DC	68

6.15.3	Co-existence studies	69

6.15.4	 ∆TIB and ∆RIB values	70

6.15.5	MSD requirements	71

6.16	 DC_8_n38-n40	71

6.16.1	Operating bands for DC	71

6.16.2	Configuration for DC	71

6.16.3	Co-existence studies	72

6.16.4	∆TIB and ∆RIB values	74

6.16.5	 MSD requirements	75

6.17	DC_3_n1-n75	75

6.17.1	Operating bands for DC	75

6.17.2	Channel bandwidths per operating band for DC	75

6.17.3	UE co-existence studies	75

6.17.4	∆TIB and ∆RIB values	76

6.17.5	MSD	76

6.18	DC_1_n40-n77	76

6.18.1	Operating bands for DC	76

6.18.2	Channel bandwidths per operating band for DC	77

6.18.3	Co-existence studies	77

6.18.4	∆TIB and ∆RIB values	79

6.18.5	MSD requirements	79

6.19	DC_3_n40-n77	80

6.19.1	Operating bands for DC	80

6.19.2	Channel bandwidths per operating band for DC	80

6.19.3	Co-existence studies	80

6.19.4	∆TIB and ∆RIB values	82

6.19.5	MSD requirements	82

6.20	DC_5_n40-n77	83

6.20.1	Operating bands for DC	83

6.20.2	Channel bandwidths per operating band for DC	83

6.20.3	Co-existence studies	84

6.20.4	∆TIB and ∆RIB values	86

6.20.5	MSD requirements	86

6.21	DC_7_n40-n77	86

6.21.1	Operating bands for DC	86

6.21.2	Channel bandwidths per operating band for DC	87

6.21.3	Co-existence studies	87

6.21.4	∆TIB and ∆RIB values	89

6.21.5	MSD requirements	89

6.22	DC_5_n40-n78	90

6.22.1	Operating bands for DC	90

6.22.2	Channel bandwidths per operating band for DC	90

6.22.3	Co-existence studies	90

6.22.4	∆TIB and ∆RIB values	92

6.22.5	MSD requirements	92

6.23	DC_7_n1-n28	93

6.23.1	Operating bands for DC	93

6.23.2	Channel bandwidths per operating band for DC	93

6.23.3	Co-existence studies	94

6.23.4	∆TIB and ∆RIB values	94

6.23.5	MSD	94

6.24	DC_66_n71-n77	94

6.24.1	Configuration for DC	94

6.24.2 Channel bandwidths per operating band for DC	95

6.24.3	Co-existence studies	96

6.24.4	∆TIB and ∆RIB values	97

6.24.5	MSD requirements	98

6.25	DC_1_n5-n40	98

6.25.1	Operating bands for DC	98

6.25.2 Channel bandwidths per operating band for DC	98

6.25.3	Co-existence studies	99

6.25.4	∆TIB and ∆RIB values	100

6.25.5	MSD requirements	101

6.26	DC_7_n5-n40	101

6.26.1	Operating bands for DC	101

6.26.2 Channel bandwidths per operating band for DC	102

6.26.3	Co-existence studies	102

6.26.4	∆TIB and ∆RIB values	103

6.26.5	MSD requirements	104

6.27	DC_28_n5-n40	104

6.27.1	Operating bands for DC	104

6.27.2 Channel bandwidths per operating band for DC	104

6.27.3	Co-existence studies	105

6.27.4	∆TIB and ∆RIB values	106

6.27.5	MSD requirements	107

6.28	DC_(n)3-n78	107

6.28.1	Operating bands for DC	107

6.28.2	Channel bandwidths per operating band for DC	108

6.28.3	Co-existence studies	108

6.28.4	∆TIB and ∆RIB values	108

6.28.5	MSD	109

6.29	DC_(n)3-n7	109

6.29.1	Configuration for DC	109

6.29.2 	Channel bandwidths per operating band for DC	109

6.29.3	Co-existence studies	110

6.29.4	 ∆TIB and ∆RIB values	110

6.29.5	MSD requirements	110

6.30	DC_(n)3-n28	110

6.30.1	Configuration for DC	110

6.30.2 	Channel bandwidths per operating band for DC	111

6.30.3	Co-existence studies	111

6.30.4	 ∆TIB and ∆RIB values	111

6.30.5	MSD requirements	111

6.31	DC_(n)3-n67	112

6.31.1	Configuration for DC	112

6.31.2 	Channel bandwidths per operating band for DC	112

6.31.3	Co-existence studies	112

6.31.4	 ∆TIB and ∆RIB values	112

6.31.5	MSD requirements	113

6.32	 DC_3_n8-n41	113

6.32.1	 Operating bands for DC	113

6.32.2 	Configuration for DC	113

6.32.3	 Co-existence studies	113

6.32.4	 ∆TIB and ∆RIB values	115

6.32.5	 MSD requirements	116

6.33	DC_2_n71-n77	116

6.33.1	Configuration for DC	116

6.33.2 	Channel bandwidths per operating band for DC	116

6.33.3	Co-existence studies	118

6.33.4	 ∆TIB and ∆RIB values	119

6.33.5	MSD requirements	120

6.34	DC_3_n3-n7	120

6.34.1	Configuration for DC	120

6.34.2 	Channel bandwidths per operating band for DC	120

6.34.3	Co-existence studies	120

6.34.4	 ∆TIB and ∆RIB values	121

6.34.5	MSD requirements	121

6.35	DC_3_n3-n28	121

6.35.1	Configuration for DC	121

6.35.2 	Channel bandwidths per operating band for DC	121

6.35.3	Co-existence studies	122

6.35.4	 ∆TIB and ∆RIB values	122

6.35.5	MSD requirements	122

6.36	DC_3_n3-n67	122

6.36.1	Configuration for DC	122

6.36.2 	Channel bandwidths per operating band for DC	123

6.36.3	Co-existence studies	123

6.36.4	 ∆TIB and ∆RIB values	123

6.36.5	MSD requirements	123

6.37	DC_7_n12-n77	124

6.37.1	Operating bands for DC	124

6.37.2	Channel bandwidths per operating band for DC	124

6.37.3	Co-existence studies	124

6.37.4	∆TIB and ∆RIB values	124

6.37.5	MSD requirements	125

6.38	DC_66_n12-n77	125

6.38.1	Operating bands for DC	125

6.38.2	Channel bandwidths per operating band for DC	126

6.38.3	Co-existence studies	126

6.38.4	∆TIB and ∆RIB values	126

6.38.5	MSD requirements	126

6.39	DC_71_n2-n77	127

6.39.1	Operating bands for DC	127

6.39.2	Channel bandwidths per operating band for DC	127

6.39.3	Co-existence studies	128

6.39.4	∆TIB and ∆RIB values	128

6.39.5	MSD requirements	128

6.40	DC_71_n66-n77	129

6.40.1	Operating bands for DC	129

6.40.2	Channel bandwidths per operating band for DC	129

6.40.3	Co-existence studies	130

6.40.4	∆TIB and ∆RIB values	130

6.40.5	MSD requirements	130

6.41	DC_2_n12-n77	131

6.41.1	Operating bands for DC	131

6.41.2	Channel bandwidths per operating band for DC	131

6.41.3	Co-existence studies	131

6.41.4	∆TIB and ∆RIB values	132

6.41.5	MSD requirements	132

6.42	DC_1_n78-n105	132

6.42.1	Operating bands for DC	132

6.42.2	Channel bandwidths per operating band for DC	133

6.42.3	  Co-existence studies	133

6.42.4	 ∆TIB and ∆RIB values	135

6.42.5	REFSENS requirements	135

6.43	DC_3_n78-n105	135

6.43.1	Operating bands for DC	135

6.43.2	Channel bandwidths per operating band for DC	136

6.43.3	  Co-existence studies	136

6.43.4	 ∆TIB and ∆RIB values	137

6.43.5	REFSENS requirements	138

6.44	DC_7_n78-n105	138

6.44.1	Operating bands for DC	138

6.44.2	Channel bandwidths per operating band for DC	139

6.44.3	  Co-existence studies	139

6.44.4	 ∆TIB and ∆RIB values	140

6.44.5	REFSENS requirements	141

6.45	DC_7_n71-n77	141

6.45.1	Operating bands for DC	141

6.45.2	Channel bandwidths per operating band for DC	142

6.45.3	Co-existence studies	142

6.45.4	∆TIB and ∆RIB values	142

6.45.5	MSD requirements	142

6.46	DC_5_n2-n41	143

6.46.1	Operating bands for DC	143

6.46.2	Channel bandwidths per operating band for DC	143

6.46.3	Co-existence studies	143

6.46.4	∆TIB and ∆RIB values	143

6.46.5	MSD requirements	144

6.47	DC_7_n2-n77	144

6.47.1	Operating bands for DC	144

6.47.2	Channel bandwidths per operating band for DC	145

6.47.3	Co-existence studies	145

6.47.4	∆TIB and ∆RIB values	145

6.47.5	MSD requirements	145

6.48	DC_66_n2-n41	146

6.48.1	Operating bands for DC	146

6.48.2	Channel bandwidths per operating band for DC	146

6.48.3	Co-existence studies	146

6.48.4	∆TIB and ∆RIB values	147

6.48.5	MSD requirements	147

6.49	DC_66_n12-n78	147

6.49.1	Operating bands for DC	147

6.49.2	Channel bandwidths per operating band for DC	148

6.49.3	Co-existence studies	148

6.49.4	∆TIB and ∆RIB values	148

6.49.5	MSD requirements	148

6.50	DC_71_n41-n66	149

6.50.1	Operating bands for DC	149

6.50.2	Channel bandwidths per operating band for DC	149

6.50.3	Co-existence studies	149

6.50.4	∆TIB and ∆RIB values	149

6.50.5	MSD requirements	150

6.51	DC_7_n2-n77	150

6.51.1	Operating bands for DC	150

6.51.2	Channel bandwidths per operating band for DC	150

6.51.3	Co-existence studies	151

6.51.4	∆TIB and ∆RIB values	153

6.51.5	MSD requirements	153

6.52	 DC_5_n28-n77	154

6.52.1	 Operating bands for DC	154

6.52.2 	Channel bandwidths per operating band for DC	154

6.52.3	 Co-existence studies	155

6.52.4	 ∆TIB and ∆RIB values	155

6.52.5	 MSD requirements	155

6.53	DC_12_n2-n66	155

6.53.1	Operating bands for DC	155

6.53.2	Channel bandwidths per operating band for DC	156

6.53.3	Co-existence studies	156

6.53.4	∆TIB and ∆RIB values	159

6.53.5	MSD requirements	159

6.54	DC_12_n2-n77	159

6.54.1	Operating bands for DC	159

6.54.2	Channel bandwidths per operating band for DC	160

6.54.3	Co-existence studies	160

6.54.4	∆TIB and ∆RIB values	162

6.54.5	MSD requirements	163

6.55	DC_12_n66-n77	163

6.55.1	Operating bands for DC	163

6.55.2	Channel bandwidths per operating band for DC	163

6.55.3	Co-existence studies	164

6.55.4	∆TIB and ∆RIB values	166

6.55.5	MSD requirements	166

6.56	DC_5_n2-n66	167

6.56.1	Operating bands for DC	167

6.56.2	Channel bandwidths per operating band for DC	167

6.56.3	Co-existence studies	168

6.56.4	∆TIB and ∆RIB values	168

6.56.5	MSD requirements	168

6.57	DC_7_n12-n78	168

6.57.1	Operating bands for DC	168

6.57.2	Channel bandwidths per operating band for DC	169

6.57.3	Co-existence studies	169

6.57.4	∆TIB and ∆RIB values	169

6.57.5	MSD requirements	170

6.58	DC_12_n41-n66	170

6.58.1	Operating bands for DC	170

6.58.2	Channel bandwidths per operating band for DC	170

6.58.3	Co-existence studies	171

6.58.4	∆TIB and ∆RIB values	171

6.58.5	MSD requirements	171

6.59	 DC_5_n28-n78	171

6.59.1	 Operating bands for DC	171

6.59.2 	Channel bandwidths per operating band for DC	172

6.59.3	 Co-existence studies	172

6.59.4	 ∆TIB and ∆RIB values	172

6.59.5	 MSD requirements	173

6.60	DC_2_n12-n78	173

6.60.1	Operating bands for DC	173

6.60.2	Channel bandwidths per operating band for DC	173

6.60.3	Co-existence studies	174

6.60.4	∆TIB and ∆RIB values	174

6.60.5	MSD requirements	174

6.61	DC_5_n41-n66	174

6.61.1	Operating bands for DC	174

6.61.2	Channel bandwidths per operating band for DC	175

6.61.3	Co-existence studies	175

6.61.4	∆TIB and ∆RIB values	175

6.61.5	MSD requirements	176

6.62	DC_66_n41-n66	176

6.62.1	Operating bands for DC	176

6.62.2	Channel bandwidths per operating band for DC	176

6.62.3	Co-existence studies	177

6.62.4	∆TIB and ∆RIB values	177

6.62.5	MSD requirements	177

6.63	 DC_7_n75-n78	177

6.63.1	Operating bands for DC	177

6.63.2	Channel bandwidths per operating band for DC	178

6.63.3	UE co-existence studies	180

6.63.4	∆TIB and ∆RIB values	180

6.63.5	MSD	181

6.64	DC_1_n40-n105	181

6.64.1	Operating bands for DC	181

6.64.2	Channel bandwidths per operating band for DC	181

6.64.3	  Co-existence studies	182

6.64.4	 ∆TIB and ∆RIB values	184

6.64.5	REFSENS requirements	185

6.65	DC_7_n40-n105	185

6.65.1	Operating bands for DC	185

6.65.2	Channel bandwidths per operating band for DC	185

6.65.3	  Co-existence studies	186

6.65.4	 ∆TIB and ∆RIB values	188

6.65.5	REFSENS requirements	189

6.66	DC_3_n40-n105	189

6.66.1	Operating bands for DC	189

6.66.2	Channel bandwidths per operating band for DC	189

6.66.3	  Co-existence studies	190

6.66.4	 ∆TIB and ∆RIB values	192

6.66.5	REFSENS requirements	193

6.67	 DC_8_n1-n79	193

6.67.1	 Operating bands for DC	193

6.67.2	 configuration for DC	193

6.67.3	 Co-existence studies	193

6.67.4	 ∆TIB and ∆RIB values	193

6.67.5	MSD	194

6.68	DC_5_n1-n28	194

6.68.1	Operating bands for DC	194

6.68.2	Channel bandwidths per operating band for DC	195

6.68.3	Co-existence studies	195

6.68.4	∆TIB and ∆RIB values	195

6.68.5	MSD requirements	195

6.69	DC_5_n3-n28	196

6.69.1	Operating bands for DC	196

6.69.2	Channel bandwidths per operating band for DC	196

6.69.3	Co-existence studies	196

6.69.4	∆TIB and ∆RIB values	197

6.69.5	MSD requirements	197

6.70	DC_5_n28-n79	197

6.70.1	Operating bands for DC	197

6.70.2	Channel bandwidths per operating band for DC	198

6.70.3	Co-existence studies	198

6.70.4	∆TIB and ∆RIB values	198

6.70.5	MSD requirements	198

6.71	DC_7_n25-n71	199

6.71.1	Operating bands for DC	199

6.71.2	Channel bandwidths per operating band for DC	199

6.71.3	Co-existence studies	199

6.71.4	∆TIB and ∆RIB values	203

6.71.5	MSD requirements	203

6.72	DC_12_n25-n77	204

6.72.1	Operating bands for DC	204

6.72.2	Channel bandwidths per operating band for DC	204

6.72.3	Co-existence studies	204

6.72.4	∆TIB and ∆RIB values	207

6.72.5	MSD requirements	208

6.73	DC_71_n25-n77	209

6.73.1	Operating bands for DC	209

6.73.2	Channel bandwidths per operating band for DC	209

6.73.3	Co-existence studies	209

6.73.4	∆TIB and ∆RIB values	212

6.73.5	MSD requirements	213

6.74	DC_12_n25-n41	213

6.74.1	Operating bands for DC	213

6.74.2	Channel bandwidths per operating band for DC	214

6.74.3	Co-existence studies	214

6.74.4	∆TIB and ∆RIB values	218

6.74.5	MSD requirements	218

6.75	DC_71_n25-n41	219

6.75.1	Operating bands for DC	219

6.75.2	Channel bandwidths per operating band for DC	219

6.75.3	Co-existence studies	219

6.75.4	∆TIB and ∆RIB values	222

6.75.5	MSD requirements	223

6.76	DC_12_n25-n66	223

6.76.1	Operating bands for DC	223

6.76.2	Channel bandwidths per operating band for DC	224

6.76.3	Co-existence studies	224

6.76.4	∆TIB and ∆RIB values	226

6.76.5	MSD requirements	227

6.77	DC_71_n25-n66	227

6.77.1	Operating bands for DC	227

6.77.2	Channel bandwidths per operating band for DC	227

6.77.3	Co-existence studies	228

6.77.4	∆TIB and ∆RIB values	232

6.77.5	MSD requirements	232

6.71	DC_7_n25-n71	232

6.71.1	Operating bands for DC	232

6.71.2	Channel bandwidths per operating band for DC	232

6.71.3	Co-existence studies	233

6.71.4	∆TIB and ∆RIB values	237

6.71.5	MSD requirements	237

6.72	DC_12_n25-n77	238

6.72.1	Operating bands for DC	238

6.72.2	Channel bandwidths per operating band for DC	238

6.72.3	Co-existence studies	238

6.72.4	∆TIB and ∆RIB values	241

6.72.5	MSD requirements	242

6.73	DC_71_n25-n77	243

6.73.1	Operating bands for DC	243

6.73.2	Channel bandwidths per operating band for DC	243

6.73.3	Co-existence studies	243

6.73.4	∆TIB and ∆RIB values	246

6.73.5	MSD requirements	247

6.74	DC_12_n25-n41	247

6.74.1	Operating bands for DC	247

6.74.2	Channel bandwidths per operating band for DC	248

6.74.3	Co-existence studies	248

6.74.4	∆TIB and ∆RIB values	252

6.74.5	MSD requirements	252

6.75	DC_71_n25-n41	253

6.75.1	Operating bands for DC	253

6.75.2	Channel bandwidths per operating band for DC	253

6.75.3	Co-existence studies	253

6.75.4	∆TIB and ∆RIB values	256

6.75.5	MSD requirements	257

6.76	DC_12_n25-n66	257

6.76.1	Operating bands for DC	257

6.76.2	Channel bandwidths per operating band for DC	258

6.76.3	Co-existence studies	258

6.76.4	∆TIB and ∆RIB values	260

6.76.5	MSD requirements	261

6.77	DC_71_n25-n66	261

6.77.1	Operating bands for DC	261

6.77.2	Channel bandwidths per operating band for DC	261

6.77.3	Co-existence studies	262

6.77.4	∆TIB and ∆RIB values	266

6.77.5	MSD requirements	266

6.78	DC_3(n)_n77	266

6.78.1	Operating bands for DC	266

6.78.2	Channel bandwidths per operating band for DC	266

6.78.3	Co-existence studies	267

6.78.4	∆TIB and ∆RIB values	267

6.78.5	MSD	267

6.79	DC_3(n)_n8	268

6.79.1	Operating bands for DC	268

6.79.2	Channel bandwidths per operating band for DC	268

6.79.3	Co-existence studies	268

6.79.4	∆TIB and ∆RIB values	268

6.79.5	MSD	268

6.80	DC_8_n1-n78	269

6.80.1	Configurations for DC	269

6.80.2	Co-existence studies	269

6.80.3	∆TIB and ∆RIB values	270

6.80.4	Reference sensitivity exceptions	270

6.81	DC_3_n8-n77	270

6.81.1	Operating bands for DC	270

6.81.2	Configuration for DC	270

6.81.3	Co-existence studies	270

6.81.4	∆TIB and ∆RIB values	270

6.81.5	MSD	271

6.82	DC_8_n7-n78	271

6.82.1	Operating bands for DC	271

6.82.2	Channel bandwidths per operating band for DC	273

6.82.3	Co-existence studies	274

6.82.4	∆TIB and ∆RIB values	274

6.82.5	MSD requirements	274

6.83	DC_28_n5-n78	276

6.83.1	Operating bands for DC	276

6.83.2	Channel bandwidths per operating band for DC	276

6.83.3	Co-existence studies	277

6.83.4	∆TIB and ∆RIB values	281

6.83.5	MSD requirements	281

6.84	DC_28_n40-n77	281

6.84.1	Operating bands for DC	281

6.84.2	Channel bandwidths per operating band for DC	282

6.84.3	Co-existence studies	282

6.84.4	∆TIB and ∆RIB values	282

6.84.5	MSD requirements	283

6.85	DC_28_n1-n5	283

6.85.1	Operating bands for DC	283

6.85.2	Channel bandwidths per operating band for DC	283

6.85.3	Co-existence studies	284

6.85.4	∆TIB and ∆RIB values	288

6.85.5	MSD requirements	288

6.86	DC_28_n5-n105	288

6.86.1	Operating bands for DC	288

6.86.2	Channel bandwidths per operating band for DC	289

6.86.3	Co-existence studies	289

6.86.4	∆TIB and ∆RIB values	291

6.86.5	MSD requirements	291

6.87	DC_28_n1-n105	291

6.87.1	Operating bands for DC	291

6.87.2	Channel bandwidths per operating band for DC	292

6.87.3	Co-existence studies	292

6.87.4	∆TIB and ∆RIB values	294

6.87.5	MSD requirements	294

6.88	DC_28_n78-n105	294

6.88.1	Operating bands for DC	294

6.88.2	Channel bandwidths per operating band for DC	295

6.88.3	Co-existence studies	295

6.88.4	∆TIB and ∆RIB values	297

6.88.5	MSD requirements	297

Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	297

7.1	DC_1-38_n28-n78	297

7.1.1	Operating bands for DC	297

7.1.2	Configuration for DC	298

7.1.3	∆TIB and ∆RIB values	298

7.2	DC_3-38_n28-n78	299

7.2.1	Operating bands for DC	299

7.2.2	Configuration for DC	299

7.2.3	∆TIB and ∆RIB values	300

7.3	DC_1_(n)3-n257	300

7.3.1	Operating bands for DC	300

7.3.2	Configuration for DC	301

7.3.3	Co-existence studies	301

7.3.4	∆TIB and ∆RIB values	301

7.3.5	MSD	301

7.4	DC_1-3_n26-n78	302

7.4.1	Operating bands for DC	302

7.4.2	Channel bandwidths per operating band for DC	302

7.4.3	∆TIB and ∆RIB values	302

7.5	DC_1-7_n26-n78	303

7.5.1	Operating bands for DC	303

7.5.2	Channel bandwidths per operating band for DC	304

7.5.3	∆TIB and ∆RIB values	304

7.6	 DC_3-7_n26-n78	304

7.6.1	Operating bands for DC	304

7.6.2	Channel bandwidths per operating band for DC	306

7.6.3	∆TIB and ∆RIB values	306

7.7	DC_1-38_n7-n78	307

7.7.1	Operating bands for DC	307

7.7.2	Configuration for DC	307

7.7.3	Co-existence studies	307

7.7.4	∆TIB and ∆RIB values	307

7.7.5	MSD	308

7.8	DC_7-20_n1-n75	308

7.8.1	Operating bands for DC	308

7.8.2	Channel bandwidths per operating band for DC	308

7.8.3	UE co-existence studies	308

7.8.4	∆TIB and ∆RIB values	308

7.8.5	MSD	309

7.9	DC_3-20_n1-n75	309

7.9.1	Operating bands for DC	309

7.9.2	Channel bandwidths per operating band for DC	310

7.9.3	UE co-existence studies	310

7.9.4	∆TIB and ∆RIB values	310

7.9.5	MSD	310

7.10	 DC_3-7_n1-n75	311

7.10.1	Operating bands for DC	311

7.10.2	Configuration for DC	311

7.10.3	UE co-existence studies	311

7.10.4	∆TIB and ∆RIB values	311

7.10.5	MSD	312

7.11	DC_1-7_n75-n78	312

7.11.1	Operating bands for DC	312

7.11.2	Configuration for DC	312

7.11.3	UE co-existence studies	312

7.11.4	∆TIB and ∆RIB values	312

7.11.5	MSD	313

7.12	DC_3-41_n1-n78	313

7.12.1	Operating bands for DC	313

7.12.2	Channel bandwidths per operating band for DC	314

7.12.3	UE co-existence studies	314

7.12.4	∆TIB and ∆RIB values	314

7.12.5	MSD	314

7.13	DC_8-41_n1-n78	315

7.13.1	Operating bands for DC	315

7.13.2	Channel bandwidths per operating band for DC	315

7.13.3	UE co-existence studies	316

7.13.4	∆TIB and ∆RIB values	316

7.13.5	MSD	316

7.14	DC_20-41_n1-n78	316

7.14.1	Operating bands for DC	316

7.14.2	Channel bandwidths per operating band for DC	317

7.14.3	UE co-existence studies	317

7.14.4	∆TIB and ∆RIB values	317

7.14.5	MSD	318

7.15	DC_3-7_n75-n78	318

7.15.1	Operating bands for DC	318

7.15.2	Configuration for DC	318

7.15.3	UE co-existence studies	318

7.15.4	∆TIB and ∆RIB values	318

7.15.5	MSD	319

7.16	DC_1-3_n75-n78	319

7.16.1	Operating bands for DC	319

7.16.2	Configuration for DC	319

7.16.3	UE co-existence studies	320

7.16.4	∆TIB and ∆RIB values	320

7.16.5	MSD	320

7.17	DC_1-3_n40-n77	320

7.17.1	Operating bands for DC	320

7.17.2	Channel bandwidths per operating band for DC	321

7.17.3	UE co-existence studies	321

7.17.4	∆TIB and ∆RIB values	321

7.17.5	MSD	322

7.18	DC_1-5_n40-n77	322

7.18.1	Operating bands for DC	322

7.18.2	Channel bandwidths per operating band for DC	322

7.18.3	UE co-existence studies	323

7.18.4	∆TIB and ∆RIB values	323

7.18.5	MSD	323

7.19	DC_3-5_n40-n77	324

7.19.1	Operating bands for DC	324

7.19.2	Channel bandwidths per operating band for DC	324

7.19.3	UE co-existence studies	324

7.19.4	∆TIB and ∆RIB values	325

7.19.5	MSD	325

7.20	DC_3-7_n40-n77	325

7.20.1	Operating bands for DC	325

7.20.2	Channel bandwidths per operating band for DC	326

7.20.3	UE co-existence studies	326

7.20.4	∆TIB and ∆RIB values	326

7.20.5	MSD	326

7.21	DC_5-7_n40-n77	327

7.21.1	Operating bands for DC	327

7.21.2	Channel bandwidths per operating band for DC	327

7.21.3	UE co-existence studies	327

7.21.4	∆TIB and ∆RIB values	328

7.21.5	MSD	328

7.22	DC_1-5_n40-n78	328

7.22.1	Operating bands for DC	328

7.22.2	Channel bandwidths per operating band for DC	329

7.22.3	UE co-existence studies	329

7.22.4	∆TIB and ∆RIB values	329

7.22.5	MSD	329

7.23	DC_3-5_n40-n78	330

7.23.1	Operating bands for DC	330

7.23.2	Channel bandwidths per operating band for DC	330

7.23.3	UE co-existence studies	330

7.23.4	∆TIB and ∆RIB values	331

7.23.5	MSD	331

7.24	DC_5-7_n40-n78	331

7.24.1	Operating bands for DC	331

7.24.2	Channel bandwidths per operating band for DC	332

7.24.3	UE co-existence studies	332

7.24.4	∆TIB and ∆RIB values	332

7.24.5	MSD	332

7.25	DC_1-7_n40-n77	333

7.25.1	Operating bands for DC	333

7.25.2	Channel bandwidths per operating band for DC	333

7.25.3	UE co-existence studies	333

7.25.4	∆TIB and ∆RIB values	334

7.25.5	MSD	334

7.26	DC_3-38_n7-n78	334

7.26.1	Operating bands for DC	334

7.26.2	Configuration for DC	334

7.26.3	Co-existence studies	335

7.26.4	∆TIB and ∆RIB values	335

7.26.5	MSD	335

7.27	DC_8-(n)3-n77	335

7.27.1	Operating bands for DC	335

7.27.2	Channel bandwidths per operating band for DC	336

7.27.3	Co-existence studies	336

7.27.4	∆TIB and ∆RIB values	336

7.27.5	MSD	337

7.28	DC_1-(n)3-n8	337

7.28.1	Operating bands for DC	337

7.28.2	Channel bandwidths per operating band for DC	337

7.28.3	Co-existence studies	338

7.28.4	∆TIB and ∆RIB values	338

7.28.5	MSD	338

7.29	DC_5-7_n1-n78	338

7.29.1	Operating bands for DC	338

7.29.2	Channel bandwidths per operating band for DC	339

7.29.3	UE co-existence studies	341

7.29.4	∆TIB and ∆RIB values	341

7.29.5	MSD	341

7.30	DC_1-8_n7-n78	341

7.30.1	Operating bands for DC	341

7.30.2	Configuration for DC	342

7.30.3	Co-existence studies	342

7.30.4	∆TIB and ∆RIB values	342

7.30.5	MSD	342

7.31	DC_3-8_n7-n78	343

7.31.1	Operating bands for DC	343

7.31.2	Configuration for DC	343

7.31.3	Co-existence studies	343

7.31.4	∆TIB and ∆RIB values	343

7.31.5	MSD	344

7.32	DC_7-8_n7-n78	344

7.32.1	Operating bands for DC	344

7.32.2	Configuration for DC	345

7.32.3	Co-existence studies	345

7.32.4	∆TIB and ∆RIB values	345

7.32.5	MSD	345

8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	346

8.1	DC_1-3-38_n28-n78	346

8.1.1	Operating bands for DC	346

8.1.2	Configuration for DC	347

8.1.3	∆TIB and ∆RIB values	347

8.2	DC_1-3-7_n26-n78	348

8.2.1	Operating bands for DC	348

8.2.2	Channel bandwidths per operating band for DC	349

8.2.3	∆TIB and ∆RIB values	349

8.3	DC_3-8-41_n1-n78	350

8.3.1	Operating bands for DC	350

8.3.2	Channel bandwidths per operating band for DC	351

8.3.3	UE co-existence studies	351

8.3.4	∆TIB and ∆RIB values	351

8.3.5	MSD	352

8.4	DC_3-20-41_n1-n78	352

8.4.1	Operating bands for DC	352

8.4.2	Channel bandwidths per operating band for DC	353

8.4.3	UE co-existence studies	353

8.4.4	∆TIB and ∆RIB values	353

8.4.5	MSD	354

8.5	DC_3-7-20_n1-n75	354

8.5.1	Operating bands for DC	354

8.5.2	Channel bandwidths per operating band for DC	354

8.5.3	UE co-existence studies	354

8.5.4	∆TIB and ∆RIB values	354

8.5.5	MSD	355

8.6	DC_1-3-7_n75-n78	355

8.6.1	Operating bands for DC	355

8.6.2	Channel bandwidths per operating band for DC	356

8.6.3	UE co-existence studies	356

8.6.4	∆TIB and ∆RIB values	356

8.6.5	MSD	356

8.7	DC_1-3-8_n7-n78	357

8.7.1	Operating bands for DC	357

8.7.2	Configuration for DC	357

8.7.3	Co-existence studies	357

8.7.4	∆TIB and ∆RIB values	357

8.7.5	MSD	358

8.8	DC_1-7-8_n7-n78	358

8.8.1	Operating bands for DC	358

8.8.2	Configuration for DC	359

8.8.3	Co-existence studies	359

8.8.4	∆TIB and ∆RIB values	359

8.8.5	MSD	359

8.9	DC_3-7-8_n7-n78	360

8.9.1	Operating bands for DC	360

8.9.2	Configuration for DC	360

8.9.3	Co-existence studies	360

8.9.4	∆TIB and ∆RIB values	360

8.9.5	MSD	361

9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	361

9.1	DC_UA-VA-WA-XA_nYA-nZ	361

9.1.1	Operating bands for DC	361

9.1.2	Channel bandwidths per operating band for DC	361

9.1.3	Co-existence studies	361

9.1.4	∆TIB and ∆RIB values	361

9.1.5	MSD	361

Annex A: Change history	362

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) under Rel-18 time frame. The purpose is to gather the relevant background information and studies to address DC band combinations of inter-band CA for LTE xDL/1UL (x=1,2,3,4) and  inter-band CA for NR 2DL/1UL in Rel-18 as shown in Table 1-1.

Table 1-1: Release 18 Dual Connectivity band combination of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL

| Combination |  |  |
| --- | --- | --- |
| LTE X Bands (x=1,2,3,4) Combinations |  | NR 2 Bands Combinations |
| LTE 1 Band | Band x | CA_nY-nZ |
| LTE 2 Bands | CA_w-x | CA_nY-nZ |
| LTE 3 Band | CA_v-w-x | CA_nY-nZ |
| LTE 4 Bands | CA_u-v-w-x | CA_nY-nZ |

This TR contains a general part and band-specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

The present document is a technical report for DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) under Rel-18 time frame. It covers both the UE and BS side. The document is divided in two different parts:

-	General part: This part covers BS and UE specific requirements which are independent of band combinations.

-	Specific band combination part:  This part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	Rel-18 DC of x bands (x = 1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL): General Part

5.1	UE RF architectures

RAN4 considers the same example RF architecture which is described in section 5.1 in TR37.863-01-01.  One different point is that LTE and NR can be integrated into one chip solution from Rel-16.

To support multiple component carriers in LTE and NR, RAN4 considers specific RF components such as Diplexer, Triplexer, Quadplexer, and Hexaplexer to derive self-interference issues according to the band combinations of x bands(x = 1,2,3,4) LTE inter-band CA(xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL).

5.2	General treatment of ∆TIB and ∆RIB values

For the x bands(x = 1,2,3,4) LTE inter-band CA(xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), RAN4 should consider reusing the agreed additional insertion losses for all DC band combinations in Rel-16 when new RF components are not introduced to support this basket WI. If the new RF components are introduced, then more detailed description will be captured according to the specific DC band combinations.

6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

6.1	DC_38_n28-n78

6.1.1	Operating bands for DC

Table 6.1.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_38_n28-n78 | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.1.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_38A_n28A-n78A | DC_38A_n28A DC_38A_n78A |

6.1.2	Configuration for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_38A_n28A-n78A |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.1.3	Co-existence studies

The harmonic and IMD products caused by UL DC_38_n28 are shown below.

Table 6.1.3-1: The harmonic and IMD products caused by UL DC_38_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10280 | 10480 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12850 | 13100 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1917 | 1822 | 3273 | 3368 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4392 | 4537 | 1214 | 1074 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5843 | 5988 | 3976 | 4116 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6962 | 7157 | 511 | 326 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8413 | 8608 | 4679 | 4864 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3644 | 3834 | 6546 | 6736 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 422 | 192 | 9777 | 9532 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2896 | 3131 | 6454 | 6214 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5382 | 5612 | 10983 | 11228 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7249 | 7484 | 9116 | 9356 |

The harmonic and IMD products caused by UL DC_38_n78 are shown below.

Table 6.1.3-2: The harmonic and IMD products caused by UL DC_38_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 680 | 1230 | 5870 | 6420 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1340 | 1940 | 3980 | 5030 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8440 | 9040 | 9170 | 10220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3910 | 4560 | 7280 | 8830 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11010 | 11660 | 12470 | 14020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2460 | 1360 | 11740 | 12840 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12630 | 10580 | 7180 | 6480 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 4660 | 1260 | 110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15770 | 17820 | 13580 | 14280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15040 | 16640 | 14310 | 15460 |

The IMD2 and IMD4 interference of UL DC_38_n28 may have an impact on the Rx of band n78.

The IMD2 and IMD5 interference of UL DC_38_n78 may have an impact on the Rx of band n28.

6.1.4	∆TIB and ∆RIB values

Referring to DC_41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.1.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_38_n28-n78 | 0.3 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.1.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_38-n28-n78 | - | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.1.5	MSD requirements

Since DC_38_n28-n78 share the similar RF architecture of DC_41_n28-n78, the MSD requirements can be specified as below, referring to DC_41A_n28A-n78A.

Table 6.1.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 798 | N/A | N/A |
| DC_38A_n28A-n78A | n78 | 3360 | 10 | 25 | 3360 | 28.2 | IMD29 |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | 30.8 | IMD24 |
|  | n78 | 3400 | 10 | 25 | 3400 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.2	DC_(n)3-n8

6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3AA-n8A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

Table 6.2.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)3AA-n8A | DC_(n)3AA1 DC_3A_n8A |
| NOTE1: Only single switched UL is supported. |  |

6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n8A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 70 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.3	Co-existence studies

The studies are already performed for DC_(n)3AA and DC_3A_n8A, in TR 37.717-11-11 and TR 37.716-11-11, respectively.

6.2.4	∆TIB and ∆RIB values

For DC_(n)3-n8, the TIB,c and RIB,c values are given in the tables below.

Table 6.2.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n8 | 0.3 | - | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.2.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n8 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.2.5	MSD

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_(n)3AA-n8A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1837.5 | [6.4] | IMD5 |
|  | n3 | 1747.5 | 5 | 25 | 1842.5 | [6.4] | IMD5 |

6.3	DC_(n)3-n77

6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3AA-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.3.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)3AA-n77A | DC_(n)3AA1 DC_3A_n77A |
| DC_(n)3AA-n77(2A) | DC_(n)3AA1 DC_3A_n77A |
| NOTE1: Only single switched UL is supported. |  |

6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_(n)3AA-n77(2A) |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.3	Co-existence studies

The studies are already performed for DC_(n)3AA and DC_3A_n77A, in TR 37.717-11-11 and TR 37.863-01-01, respectively.

6.3.4	∆TIB and ∆RIB values

For DC_(n)3_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n77 | - | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.3.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n77 | - | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.3.5	MSD

MSD analysis is already available for DC_(n)3AA and DC_3A_n77A, in TR 37.717-11-11 and TR 37.863-01-01, respectively.

6.4	DC_(n)3-n257

6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.4.2	Configuration for DC

Table 6.4.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_(n)3AA-n257A DC_(n)3AA-n257G DC_(n)3AA-n257H DC_(n)3AA-n257I DC_(n)3AA-n257J DC_(n)3AA-n257K DC_(n)3AA-n257L DC_(n)3AA-n257M | DC_(n)3AA1 DC_3A_n257A |
| NOTE1: Only single switched UL is supported. |  |

6.4.3	Co-existence studies

The studies are already performed for DC_(n)3AA in TR 37.717-11-11.

No study is needed for DC_3A_n257A, due to large frequency separation.

6.4.4	∆TIB and ∆RIB values

For DC_(n)3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n257 | - | 0.6 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.4.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n257 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.4.5	MSD

MSD analysis is already available for DC_(n)3AA in TR 37.717-11-11.

No MSD analysis is needed for DC_3A_n257A, as there is no coexistence issue.

6.5	DC_1_n8-n257

6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n8-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.5.2	Configuration for DC

Table 6.5.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n8A-n257A DC_1A_n8A-n257G DC_1A_n8A-n257H DC_1A_n8A-n257I DC_1A_n8A-n257J DC_1A_n8A-n257K DC_1A_n8A-n257L DC_1A_n8A-n257M | DC_1A_n8A DC_1A_n257A |

6.5.3	Co-existence studies

For DC_1A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_1A_n8A is already given in TR 37.716-11-11.

6.5.4	∆TIB and ∆RIB values

For DC_1_n8-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n8-n257 | 0.6 | 0.3 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.5.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n8-n257 | 0.2 | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.5.5	MSD

MSD analysis is already available for DC_1A_n8A is already given in TR 37.716-11-11.

No MSD analysis is needed for DC_1A_n257A, as there is no coexistence issue.

6.6	DC_1_n77-n257

6.6.1	Operating bands for DC

Table 6.6.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.6.2	Configuration for DC

Table 6.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n77A-n257J DC_1A_n77A-n257K DC_1A_n77A-n257L DC_1A_n77A-n257M DC_1A_n77A-n257A | DC_1A_n77A DC_1A_n257A |

6.6.3	Co-existence studies

For DC_1A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_1A_n77A is already given in TR 37.863-01-01.

6.6.4	∆TIB and ∆RIB values

For DC_1_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.6.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n77-n257 | 0.6 | 0.8 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.6.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n77-n257 | 0.2 | 0.5 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.6.5	MSD

MSD analysis is already available for DC_1A_n77A is already given in TR 37.863-01-01.

No MSD analysis is needed for DC_1A_n257A, as there is no coexistence issue.

6.7	DC_3_n77-n257

6.7.1	Operating bands for DC

Table 6.7.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.7.2	Configuration for DC

Table 6.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_3A_n77A-n257J DC_3A_n77A-n257K DC_3A_n77A-n257L DC_3A_n77A-n257M DC_3A_n77(2A)-n257A DC_3A_n77(2A)-n257G DC_3A_n77(2A)-n257H DC_3A_n77(2A)-n257I DC_3A_n77(2A)-n257J DC_3A_n77(2A)-n257K DC_3A_n77(2A)-n257L DC_3A_n77(2A)-n257M | DC_3A_n77A DC_3A_n257A |

6.7.3	Co-existence studies

For DC_3A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_3A_n77A is already given in TR 37.863-01-01.

6.7.4	∆TIB and ∆RIB values

For DC_3_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n77-n257 | 0.6 | 0.8 | 0 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.7.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n77-n257 | 0.2 | 0.5 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.7.5	MSD

MSD analysis is already available for DC_3A_n77A is already given in TR 37.863-01-01.

No MSD analysis is needed for DC_3A_n257A, as there is no coexistence issue.

6.8	DC_8_n3-n257

6.8.1	Operating bands for DC

Table 6.8.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n257 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.8.2	Configuration for DC

Table 6.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n3A-n257A DC_8A_n3A-n257G DC_8A_n3A-n257H DC_8A_n3A-n257I DC_8A_n3A-n257J DC_8A_n3A-n257K DC_8A_n3A-n257L DC_8A_n3A-n257M | DC_8A_n3A DC_8A_n257A |

6.8.3	Co-existence studies

For DC_8A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_8A_n3A is already given in TR 37.716-11-11.

6.8.4	∆TIB and ∆RIB values

For DC_1_n8-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8_n3-n257 | 0.3 | 0.3 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.8.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8_n3-n257 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.8.5	MSD

MSD analysis is already available for DC_8A_n3A is already given in TR 37.716-11-11.

No MSD analysis is needed for DC_8A_n257A, as there is no coexistence issue.

6.9	DC_3-n26-n78

6.9.1	Operating bands for DC

Table 6.9.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-n26-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.9.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-n26A-n78A | DC_3A_n26A DC_3A_n78A |
| DC_3C_n26A-n78A | DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |

6.9.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-n26A-n78A |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3C_n26A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.9.3	Co-existence studies

The harmonic and IMD products caused by UL DC_3_n26 are shown below.

Table 6.9.3-1: The harmonic and IMD products caused by UL DC_3_n26

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5130 | 5355 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 6840 | 7140 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 971 | 861 | 2524 | 2634 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1942 | 1722 | 5048 | 5268 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3727 | 3432 | 873 | 1128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

The harmonic and IMD products caused by UL DC_3_n78 are shown below.

6.9.4	∆TIB and ∆RIB values

Referring to DC_3-5_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.9.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-n26-n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.9.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-n26-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.9.5	MSD requirements

Referring to CA_n3-n5-n78, the MSD requirements can be specified as below:

Table 6.9.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n26A-n78A DC_3C_n26A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n26 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n78 | 3408 | 10 | 50 | 3408 | 16.1 | IMD3 |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n26 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n78 | 3512 | 10 | 50 | 3512 | 4.5 | IMD5 |

6.10	DC_7-n26-n78

6.10.1	Operating bands for DC

Table 6.10.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-n26-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.10.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-n26A-n78A | DC_7A_n26A DC_7A_n78A |
| DC_7C_n26A-n78A | DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |

6.10.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n26A-n78A |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_7C_n26A-n78A |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.10.3	Co-existence studies

The harmonic and IMD products caused by UL DC_7_n26 are shown below.

Table 6.10.3-1: The harmonic and IMD products caused by UL DC_7_n26

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1756 | 1651 | 3314 | 3419 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 942 | 802 | 4151 | 4326 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

The harmonic and IMD products caused by UL DC_7_n78 are shown below.

Table 6.10.3-2: The harmonic and IMD products caused by UL DC_7_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 15200 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1300 | 730 | 5800 | 6370 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

The IMD2 and IMD4 interference of UL DC_7_n26 may have an impact on the Rx of band n78.

The IMD2 and IMD5 interference of UL DC_7_n78 may have an impact on the Rx of band n26.

6.10.4	∆TIB and ∆RIB values

Referring to DC_5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.10.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n26-n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.10.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n26-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.10.5	MSD requirements

Referring to CA_n7-n7-n78, the MSD requirements can be specified as below:

Table 6.10.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n26A-n78A DC_7C_n26A-n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n26 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n26 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n26 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n26 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | 9.7 | IMD4 |

6.11	DC_20_n1-n75

6.11.1	Operating bands for DC

Table 6.11.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n1-n75 | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

6.11.2	Channel bandwidths per operating band for DC

Table 6.11.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_20A_n1A-n75A | DC_20A_n1A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.11.3	UE co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_20_n1-n75 with dual uplink on DC_20_n1 are listed in the table below.

Table 6.11.3-1: The harmonic and IMD products caused by UL DC_20_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1058 | 1148 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 316 | 196 | 2978 | 3128 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3584 | 3704 | 4672 | 4822 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 516 | 606 | 4898 | 5108 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4416 | 4566 | 6592 | 6802 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7088 | 6818 | 1528 | 1348 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8512 | 8782 | 5248 | 5428 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4276 | 4036 | 1254 | 1464 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 | 7664 | 6336 | 6546 |

Based on co-existence studies, IMD5 from UL 20_n1 may impact DL n75.

6.11.4	∆TIB and ∆RIB values

For DC_20_n1-n75, the TIB,c and RIB,c values can reuse the values of DC_20-32_n1 as given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20_n1-n75 | 0.3 | 0.5 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.11.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_20_n1-n75 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.11.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_20A-32A_n1A.

Table 6.11.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-n1A_n75A | n1 | 1950.5 | 5 | 50 | 2140.5 | N/A | N/A |
|  | 20 | 852.5 | 5 | 25 | 811.5 | N/A | N/A |
|  | n75 | N/A | 5 | N/A | 1459.5 | 4.0 | IMD5 |

6.12	DC_7_n1-n75

6.12.1	Operating bands for DC

Table 6.12.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n75 | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

6.12.2	Channel bandwidths per operating band for DC

Table 6.12.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A_n1A-n75A | DC_7A_n1A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.12.3	UE co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_7A_n1A-n75A with dual uplink on DC_7A_n1A are listed in the table below.

Table 6.12.3-1: The harmonic and IMD products caused by UL DC_7_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3370 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1040 | 1300 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

Based on co-existence studies, IMD3 from UL 7_n1 may impact DL n75.

6.12.4	∆TIB and ∆RIB values

For DC_7A_n1A-n75A, the TIB,c and RIB,c values can reuse the values of DC_7A-32A_n1A as given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7A_n1A-n75A | 0.6 | 0.5 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.12.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7A_n1A-n75A | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.12.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_7A-32A_n1A.

Table 6.12.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n1A-n75A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 75 | N/A | 5 | N/A | 1454.5 | 15.2 | IMD3 |

6.13	DC_1-n26-n78

6.13.1	Operating bands for DC

Table 6.13.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.13.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-n26A-n78A | DC_1A_n26A DC_1A_n78A |

6.13.2	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.13.3	Co-existence studies

The harmonic and IMD products caused by UL DC_1_n26 are shown below.

Table 6.13.3-1: The harmonic and IMD products caused by UL DC_1_n26

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1166 | 1071 | 2734 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 352 | 222 | 2991 | 3146 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3548 | 3678 | 4654 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2332 | 2142 | 5468 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – *fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 462 | 627 | 4911 | 5126 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4527 | 6574 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7106 | 6831 | 1476 | 1276 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + *fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8494 | 8769 | 5176 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – *fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – *fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4312 | 4062 | 1293 | 1518 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7388 | 7638 | 6282 | 6507 |

The harmonic and IMD products caused by UL DC_1_n78 are shown below.

Table 6.13.3-2: The harmonic and IMD products caused by UL DC_1_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 3840 | 3960 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 11400 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 15200 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1880 | 1320 | 5220 | 5780 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 | 660 | 4620 | 5680 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7760 | 8520 | 9580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3760 | 2640 | 10440 | 11560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 9740 | 11820 | 13380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 | 11220 | 4620 | 3880 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 | 5940 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 15360 | 12360 | 13540 |

The IMD3 interference of UL DC_1_n26 may have an impact on the Rx of band n78.

The IMD5 interference of UL DC_1_n78 may have an impact on the Rx of band n26.

6.13.4	∆TIB and ∆RIB values

Referring to DC_1-5_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.13.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1-n26-n78 | 0.3 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.13.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1-n26-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.13.5	MSD requirements

Referring to CA_n1-n5-n78, the MSD requirements can be specified as below:

Table 6.13.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n26A-n78A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n26 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | IMD3 |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n26 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |

6.14	DC_3_n38-n40

6.14.1	Operating bands for DC

Table 6.14.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n38-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.14.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n38A-n40A | DC_3A_n38A DC_3A_n40A |

6.14.2	Configuration for DC

Table 6.14.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n38A-n40A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.14.3	Co-existence studies

The harmonic and IMD products caused by UL DC_3_n38 are shown below.

Table 6.14.3-1: The harmonic and IMD products caused by UL DC_3_n38

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 12850 | 13100 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10710 | 15420 | 15720 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12495 | 17990 | 18340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 785 | 910 | 4280 | 4405 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1000 | 3355 | 3530 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5990 | 6190 | 6850 | 7025 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2510 | 2785 | 5925 | 6150 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1820 | 1570 | 8560 | 8810 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7700 | 7975 | 9420 | 9645 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8770 | 8495 | 4570 | 4220 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4440 | 4140 | 215 | 110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11990 | 12265 | 9410 | 9760 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11130 | 11430 | 10270 | 10595 |

The harmonic and IMD products caused by UL DC_3_n40 are shown below.

Table 6.14.3-2: The harmonic and IMD products caused by UL DC_3_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10710 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12495 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 515 | 690 | 4010 | 4185 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1020 | 1270 | 2815 | 3090 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5720 | 5970 | 6310 | 6585 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2730 | 3055 | 5115 | 5490 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1380 | 1030 | 8020 | 8370 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7430 | 7755 | 8610 | 8985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7890 | 7415 | 4840 | 4440 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3780 | 3330 | 755 | 330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10910 | 11385 | 9140 | 9540 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10320 | 10770 | 9730 | 10155 |

There are no intermodulation products of DC_3_n38 falling inside the RX band of n40.

There are no intermodulation products of DC_3_n40 falling inside the RX band of n38

6.14.4	∆TIB and ∆RIB values

Referring to CA_n3A-n38A-n40A, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.14.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n38-n40 | 0.5 | 0.53 | 0.5 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.14.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n38-n40 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.14.5	MSD requirements

All MSD requirements for DC_3_n38-n40 are already covered by constituent fallback modes. No additional MSD requirement is needed.

6.15	DC_41_n1-n78

6.15.1	Configuration for DC

Table 6.15.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n1-n78 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.15.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_41A_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41C_n1A-n78A | DC_41A_n1A DC_41A_n78A |

6.15.2	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A_n1A-n78A |  |  | 41 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | n1 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_41C_n1A-n78A |  |  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | n1 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.15.3	Co-existence studies

Table 6.15.3-1: Band 41 and Band n78 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 3800 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1304 | 610 | 5796 | 6490 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 | 2080 | 3910 | 5104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9180 | 9096 | 10290 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2608 | 1220 | 11592 | 12980 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 | 4770 | 7210 | 8904 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 11870 | 12396 | 14090 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 | 10510 | 7460 | 6184 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 17890 | 13284 | 14560 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 | 4520 | 1470 | 112 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 16780 | 14088 | 15670 |

Table 6.15.3-2: Band 41 and Band n1 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 1920 | 1980 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3012 | 3460 | 1150 | 1464 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6912 | 7360 | 6336 | 6650 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1032 | 1540 | 8832 | 9340 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5508 | 6150 | 3070 | 3444 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9408 | 10050 | 8256 | 8630 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5424 | 4990 | 8840 | 8004 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10176 | 10610 | 11904 | 12740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 948 | 380 | 4230 | 3528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10752 | 11320 | 11328 | 12030 |

As can be seen in Table 6.15.3-1 there are IMD4 impact from UL 41_n78 affecting DL band 1.

As can be seen in Table 6.15.3-2 there are IMD3 impact from UL 41_n1 affecting DL band n78.

6.15.4		∆TIB and ∆RIB values

For DC_41_n1-n78, the TIB,c and RIB,c values are reused from DC_1-41_n78 and are given in the tables below.

Table 6.15.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_41_n1-n78 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.15.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_41_n1-n78 | - | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.15.5	MSD requirements

MSD value band n1 are reused from DC_1A-41A_n78A.

MSD value band n78 is reused from DC_1A_n41A-n78A.

Table 6.15.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_41A_n1A-n78A | n1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_41C_n1A-n78A | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 19.6 | IMD3 |

6.16		DC_8_n38-n40

6.16.1	Operating bands for DC

Table 6.16.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n38-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.16.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n38A-n40A | DC_8A_n38A DC_8A_n40A |

6.16.2	Configuration for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n38A-n40A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.16.3	Co-existence studies

The harmonic and IMD products caused by UL DC_8_n38 are shown below.

Table 6.16.3-1: The harmonic and IMD products caused by UL DC_8_n38

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 880 | 915 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 12850 | 13100 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 | 5490 | 15420 | 15720 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 6160 | 6405 | 17990 | 18340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1655 | 1740 | 3450 | 3535 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 860 | 740 | 4225 | 4360 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4330 | 4450 | 6020 | 6155 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20 | 175 | 6795 | 6980 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3480 | 3310 | 6900 | 7070 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5210 | 5365 | 8590 | 8775 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9600 | 9365 | 1090 | 900 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6100 | 5880 | 2395 | 2600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11160 | 11395 | 6090 | 6280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9470 | 9690 | 7780 | 7985 |

The harmonic and IMD products caused by UL DC_8_n40 are shown below.

Table 6.16.3-2: The harmonic and IMD products caused by UL DC_8_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 880 | 915 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 | 5490 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 6160 | 6405 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385 | 1520 | 3180 | 3315 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 640 | 470 | 3685 | 3920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4060 | 4230 | 5480 | 5715 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 240 | 445 | 5985 | 6320 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3040 | 2770 | 6360 | 6630 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4940 | 5145 | 7780 | 8115 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8720 | 8285 | 1360 | 1120 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5440 | 5070 | 1855 | 2160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10080 | 10515 | 5820 | 6060 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8660 | 9030 | 7240 | 7545 |

The 5th order IMD product 2*2572MHz – 3*912MHz = 2408MHz will have a BW of the interference of 25MHz, which falls inside n40 upper channel.

There are no intermodulation products of DC_8_n40 falling inside the RX band of n38.

6.16.4	∆TIB and ∆RIB values

Referring to CA_n8A-n38A-n40A, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.16.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8_n38-n40 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.16.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8_n38-n40 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.16.5	 MSD requirements

Referring to CA_n8-n38-n40, the MSD requirement that addresses the 5th order product has been omitted. The reason being that CA_ n38-n40 has Note 9 applied meaning it is only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. Following same reasoning no MSD have been defined for the IMD5.

6.17	DC_3_n1-n75

6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n75A | DC_3A_n1A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C_n1A-n75A | DC_3C_n1A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.17.3	UE co-existence studies

Based on co-existence studies, IMD3 from UL 3_n1 may impact DL n1.

Based on co-existence studies, IMD3 and IMD5 from UL 3_n1 may impact DL n75

6.17.4	∆TIB and ∆RIB values

For DC_3_n1-n75, the TIB,c and RIB,c values can reuse the values of DC_1_n3-n75 as given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n1-n75 | 0.5 | 0.5 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.17.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n1-n75 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.17.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_1_n3-n75.

Table 6.17.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n75A | n75 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
| DC_3C_n1A-n75A | n75 | N/A | 5 | 25 | 1495 | [FFS] | IMD3 |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 3 | 1730 1749.8 | 20 20 | 1(RBSTART=20) 1(RBSTART=79) | 1825 1844.8 | N/A N/A | N/A N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.18	DC_1_n40-n77

6.18.1	Operating bands for DC

Table 6.18.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n40-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.18.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A_n40A-n77A | DC_1A_n40A DC_1A_n77A |

6.18.2	Channel bandwidths per operating band for DC

Table 6.18.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n40A-n77A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n40 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.18.3	Co-existence studies

For UE coexistence study of Band 1 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.18.3-1.

Based on the calculation, we identify the following interference impact:

- The 2nd order harmonic of UL Band 1 may have an impact on DL Band n77.

- The IMD4 and IMD5 interference of UL DC_1_n40 may have an impact on DL Band n77.

Table 6.18.3-1: The harmonic and IMD products caused by UL DC_1_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 11500 | 12000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 | 480 | 4220 | 4380 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1660 | 2620 | 2880 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 960 | 640 | 8440 | 8760 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

For UE coexistence study of Band 1 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.18.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD4 and IMD5 interference of UL DC_1_n77 may have an impact on DL Band n40.

Table 6.18.3-2: The harmonic and IMD products caused by UL DC_1_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 21000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 2280 | 5220 | 6180 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 660 | 4620 | 6480 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 8160 | 8520 | 10380 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1560 | 2640 | 7920 | 10680 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4560 | 2640 | 10440 | 12360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 10140 | 11820 | 14580 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14880 | 11220 | 4620 | 3480 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8760 | 5940 | 660 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 18780 | 10980 | 12120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 16560 | 12360 | 14340 |

6.18.4	∆TIB and ∆RIB values

For DC_1_n40-n77, TIB,c and RIB,c values are reused from DC_1_n40-n78, which is very similar.

Table 6.18.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n40-n77 | 0.3 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.18.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n40-n77 | - | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.18.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 1 on DL Band n77 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

The MSD values for IMD interference are reused from DC_1_n40-n78, which is very similar.

Table 6.18.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n40A-n77A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n77 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 |
|  | n77 | 3520 | 10 | 50 | 3520 | N/A | N/A |

6.19	DC_3_n40-n77

6.19.1	Operating bands for DC

Table 6.19.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n40-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.19.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n40A-n77A | DC_3A_n40A DC_3A_n77A |

6.19.2	Channel bandwidths per operating band for DC

Table 6.19.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n40A-n77A |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n40 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.19.3	Co-existence studies

For UE coexistence study of Band 3 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.19.3-1.

Based on the calculation, we identify the following interference impact:

- The 2nd order harmonic of UL Band 3 may have an impact on DL Band n77.

- The IMD2 and IMD5 interference of UL DC_3_n40 may have an impact on DL Band n77.

Table 6.19.3-1: The harmonic and IMD products caused by UL DC_3_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 11500 | 12000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 515 | 690 | 4010 | 4185 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1020 | 1270 | 2815 | 3090 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5720 | 5970 | 6310 | 6585 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2730 | 3055 | 5115 | 5490 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1380 | 1030 | 8020 | 8370 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7430 | 7755 | 8610 | 8985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7890 | 7415 | 4840 | 4440 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3780 | 3330 | 755 | 330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10910 | 11385 | 9140 | 9540 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10320 | 10770 | 9730 | 10155 |

For UE coexistence study of Band 3 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.19.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD2 and IMD5 interference of UL DC_3_n77 may have an impact on DL Band n40.

Table 6.19.3-2: The harmonic and IMD products caused by UL DC_3_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 16500 | 21000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2490 | 5010 | 5985 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 | 270 | 4815 | 6690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7770 | 8310 | 10185 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 | 2055 | 8115 | 10890 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4980 | 3030 | 10020 | 11970 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9555 | 11610 | 14385 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11415 | 3840 | 2640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6330 | 1245 | 3270 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18585 | 10140 | 11340 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16170 | 11730 | 13755 |

6.19.4	∆TIB and ∆RIB values

For DC_3_n40-n77, ΔTIB,c and ΔRIB,c values are reused from DC_3_n40-n78, which is very similar.

Table 6.19.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n40-n77 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.19.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n40-n77 | 0.2 | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.19.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 3 on DL Band n77 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

The MSD values for IMD interference are obtained as follows.

- The MSD value for the IMD2 interference of UL DC_3_n40 on DL Band n77 is reused from DC_3A_n41A-n79A, which is very similar, with the modification by -0.5 dB due to the difference on REFSENS of n77 and n79, where the modification is described in Table 7.3.2-1b (NOTE 4) in TS 38.101-1.

- The MSD value for the IMD5 interference of UL DC_3_n40 on DL Band n77 is reused from DC_3A_n40A-n78A, which is very similar.

- The MSD value for the IMD2 interference of UL DC_3_n77 on DL Band n40 is reused from DC_30A-66A_n77A, which is very similar.

- The MSD value for the IMD5 interference of UL DC_3_n77 on DL Band n40 is reused from DC_3A_n40A-n78A, which is very similar.

Table 6.19.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n40A-n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | N/A |
|  | n77 | 4070 | 10 | 50 | 4070 | 30.3 | IMD2 |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 4.8 | IMD5 |
|  | 3 | 1745 | 5 | 25 | 1840 | N/A | N/A |
|  | n40 | 2355 | 5 | 25 | 2355 | 29,2 | IMD2 |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n77 | 3760 | 10 | 50 | 3760 | N/A | N/A |

6.20	DC_5_n40-n77

6.20.1	Operating bands for DC

Table 6.20.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n40-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.20.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n40A-n77A | DC_5A_n40A DC_5A_n77A |

6.20.2	Channel bandwidths per operating band for DC

Table 6.20.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n40A-n77A |  | 5 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | n40 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.20.3	Co-existence studies

For UE coexistence study of Band 5 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.20.3-1.

Based on the calculation, we identify the following interference impact:

- The 4th and 5th order harmonics of UL Band 5 may have an impact on DL Band n77.

- The IMD3 interference of UL DC_5_n40 may have an impact on DL Band n77.

Table 6.20.3-1: The harmonic and IMD products caused by UL DC_5_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 11500 | 12000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1451 | 1576 | 3124 | 3249 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 752 | 602 | 3751 | 3976 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948 | 4098 | 5424 | 5649 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 72 | 247 | 6051 | 6376 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3152 | 2902 | 6248 | 6498 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4772 | 4947 | 7724 | 8049 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8776 | 8351 | 1096 | 896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5552 | 5202 | 2053 | 2328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10024 | 10449 | 5596 | 5796 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8548 | 8898 | 7072 | 7347 |

For UE coexistence study of Band 5 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.20.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD3 interference of UL DC_5_n77 may have an impact on DL Band n40.

Table 6.20.3-2: The harmonic and IMD products caused by UL DC_5_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 16500 | 21000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 3376 | 4124 | 5049 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2552 | 1602 | 5751 | 7576 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948 | 5898 | 7424 | 9249 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1728 | 753 | 9051 | 11776 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 6752 | 4902 | 8248 | 10098 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5772 | 6747 | 10724 | 13449 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15976 | 12351 | 96 | 904 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10952 | 8202 | 4053 | 5928 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 | 17649 | 6596 | 7596 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 | 14298 | 9072 | 10947 |

6.20.4	∆TIB and ∆RIB values

For DC_5_n40-n77, TIB,c and RIB,c values are reused from CA_n5-n30-n77, which is very similar.

Table 6.20.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n40-n77 | 0.6 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.20.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n40-n77 | 0.2 | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.20.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 5 on DL Band n77 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

The MSD values for IMD interference are reused from CA_n5-n30-n77, which is very similar.

Table 6.20.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n40A-n77A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 16.1 | IMD3 |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n40 | 2355 | 5 | 25 | 2355 | 13.2 | IMD3 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | N/A |

6.21	DC_7_n40-n77

6.21.1	Operating bands for DC

Table 6.21.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n40-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.21.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n40A-n77A | DC_7A_n40A DC_7A_n77A |

6.21.2	Channel bandwidths per operating band for DC

Table 6.21.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n40A-n77A |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n40 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.21.3	Co-existence studies

For UE coexistence study of Band 7 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.21.3-1.

Based on the calculation, we find that there is no impact of harmonics and inter-modulation products from UL DC_7_n40 affecting DL Band 77.

Table 6.21.3-1: The harmonic and IMD products caused by UL DC_7_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 11500 | 12000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 100 | 4800 | 4970 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2600 | 2840 | 2030 | 2300 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7300 | 7540 | 7100 | 7370 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5100 | 5410 | 4330 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 200 | 540 | 9600 | 9940 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9800 | 10110 | 9400 | 9770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7100 | 6630 | 7980 | 7600 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2200 | 1760 | 3110 | 2700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11700 | 12170 | 12300 | 12680 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12340 | 12100 | 12510 |

For UE coexistence study of Band 7 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.21.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD4 interference of UL DC_7_n77 may have an impact on DL Band n40.

Table 6.21.3-2: The harmonic and IMD products caused by UL DC_7_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 21000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1700 | 5800 | 6770 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1840 | 4030 | 5900 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 9340 | 9100 | 10970 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3300 | 4410 | 7330 | 10100 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3400 | 1460 | 11600 | 13540 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11910 | 12400 | 15170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14300 | 10630 | 6980 | 5800 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7600 | 4760 | 1110 | 900 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 19370 | 13300 | 14480 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 17740 | 14100 | 16110 |

6.21.4	∆TIB and ∆RIB values

For DC_7_n40-n77, TIB,c and RIB,c values are reused from DC_7_n40-n78, which is very similar.

Table 6.21.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n40-n77 | 0.5 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.21.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n40-n77 | - | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.21.5	MSD requirements

The MSD values for IMD interference are reused from DC_7_n40-n78, which is very similar.

Table 6.21.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n40A-n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 9.2 | IMD4 |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | N/A |

6.22	DC_5_n40-n78

6.22.1	Operating bands for DC

Table 6.22.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n40-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.22.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n40A-n78A | DC_5A_n40A DC_5A_n78A |

6.22.2	Channel bandwidths per operating band for DC

Table 6.22.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n40A-n78A |  | 5 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | n40 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.22.3	Co-existence studies

For UE coexistence study of Band 5 + Band n40, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.22.3-1.

Based on the calculation, we identify the following interference impact:

- The 4th order harmonic of UL Band 5 may have an impact on DL Band n78.

- The IMD3 interference of UL DC_5_n40 may have an impact on DL Band n78.

Table 6.22.3-1: The harmonic and IMD products caused by UL DC_5_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 11500 | 12000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1451 | 1576 | 3124 | 3249 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 752 | 602 | 3751 | 3976 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3948 | 4098 | 5424 | 5649 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 72 | 247 | 6051 | 6376 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3152 | 2902 | 6248 | 6498 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4772 | 4947 | 7724 | 8049 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8776 | 8351 | 1096 | 896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5552 | 5202 | 2053 | 2328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10024 | 10449 | 5596 | 5796 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8548 | 8898 | 7072 | 7347 |

For UE coexistence study of Band 5 + Band n78, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.22.3-2.

Based on the calculation, we find that there is no impact of harmonics and inter-modulation products from UL DC_5_n78 affecting DL Band n40.

Table 6.22.3-2: The harmonic and IMD products caused by UL DC_5_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 16500 | 19000 |
| Two-tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 2976 | 4124 | 4649 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2152 | 1602 | 5751 | 6776 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948 | 5498 | 7424 | 8449 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1328 | 753 | 9051 | 10576 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5952 | 4902 | 8248 | 9298 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5772 | 6347 | 10724 | 12249 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14376 | 12351 | 96 | 504 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9752 | 8202 | 4053 | 5128 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 | 16049 | 6596 | 7196 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 | 13098 | 9072 | 10147 |

6.22.4	∆TIB and ∆RIB values

For DC_5_n40-n78, TIB,c and RIB,c values are reused from CA_n5-n40-n78.

Table 6.22.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n40-n78 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.22.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n40-n78 | 0.2 | 0.4 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.22.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 5 on DL Band n78 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

The MSD values for IMD interference are reused from CA_n5-n40-n78.

Table 6.22.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n40A-n78A | 5 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | 16.1 | IMD3 |

6.23	DC_7_n1-n28

6.23.1	Operating bands for DC

Table 6.23.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n28 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

Table 6.23.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n1A-n28A | DC_7A_n1A DC_7A_n28A |
| DC_7C_n1A-n28A | DC_7A_n1A DC_7A_n28A DC_7C_n1A DC_7C_n28A |

6.23.2	Channel bandwidths per operating band for DC

Table 6.23.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n1A-n28A |  |  | 7 | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  |  | n1 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n28 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7C_n1A-n28A |  |  | 7 |  | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 |
|  |  |  | n1 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n28 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.23.3	Co-existence studies

UL DC_7_n1 gives IMD5 into DL n28.

6.23.4	∆TIB and ∆RIB values

For DC_7_n1-n28, the TIB,c and RIB,c values can reuse the values of DC_7-28_n1 as given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n1-n28 | 0.6 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.23.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n1-n28 | - | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.23.5	MSD

Based on co-existence studies MSD needs to be defined. MSD values are reused from DC_7A-28A_n1A.

Table 6.23.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n1A-n28A DC_7C_n1A-n28A | 7 | 2535 | 5 | 25 | 2655 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 725 | 5 | N/A | 780 | 4.3 | IMD5 |

6.24	DC_66_n71-n77

6.24.1	Configuration for DC

Table 6.24.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n71-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.24.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_66A_n71A-n77A | DC_66A_n71A DC_66A_n77A |

6.24.2 Channel bandwidths per operating band for DC

Table 6.24.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n71A-n77A |  |  | 66 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n71 |  | 15 | 5 | 10 | 15 | 20 | 253 | 30 | 35 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 30 | 35 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.24.3	Co-existence studies

Table 6.24.3-1: Band 66 and Band n71 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 663 | 698 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1012 | 1117 | 2373 | 2478 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 | 2897 | 454 | 314 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4258 | 3036 | 3176 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2024 | 2234 | 4746 | 4956 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 | 4677 | 209 | 384 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6038 | 3699 | 3874 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 | 872 | 6457 | 6142 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4572 | 7503 | 7818 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 | 1571 | 4014 | 3734 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5654 | 6456 | 6736 |

Table 6.24.3-2: Band 66 and Band n77 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 4200 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2490 | 1520 | 5010 | 5980 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 | 260 | 4820 | 6690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7760 | 8310 | 10180 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4980 | 3040 | 10020 | 11960 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 | 2040 | 8120 | 10890 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9540 | 11610 | 14380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11420 | 3820 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18580 | 10140 | 11320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6340 | 1260 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16160 | 11730 | 13740 |

As can be seen in Table 6.24.3-1 there are IMD3, IMD4 and IMD5 impact from UL 66_n71 affecting DL band n77.

As can be seen in Table 6.24.3-2 there are IMD3 impact from UL 66_n77 affecting DL band n71.

6.24.4	∆TIB and ∆RIB values

For DC_66_n71-n77, the TIB,c and RIB,c values are reused from DC_66_n71-n78 and are given in the tables below.

Table 6.24.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66_n71-n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.24.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66_n71-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.24.5	MSD requirements

MSD value band n71 and testpoints are reused from CA_n66-n71-n77.

MSD value band n77 and testpoints are reused from CA_n66-n71-n77A.

Table 6.24.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n71A-n77A | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | IMD34,9,11 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | IMD311 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

6.25	DC_1_n5-n40

6.25.1	Operating bands for DC

Table 6.25.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n5-n40 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.25.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n5A-n40A | DC_1A_n5A DC_1A_n40A |

6.25.2 Channel bandwidths per operating band for DC

Table 6.25.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n5A-n40A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n5 | 15 | 5 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | 55 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.25.3	Co-existence studies

The IMD products caused by UL DC_1_n5 are shown below.

Table 6.25.3-1: The IMD products caused by UL DC_1_n5

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1156 | 1071 | 2829 | 2744 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 3136 | 2991 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3678 | 3568 | 4809 | 4664 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2142 | 2312 | 5488 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – *fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6381 | 7096 | 1316 | 1476 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low –3 *fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4062 | 4292 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

The IMD products caused by UL DC_1_n40 are shown below.

Table 6.25.3-2: The IMD products caused by UL DC_1_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 480 | 320 | 4380 | 4220 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1660 | 1440 | 2880 | 2620 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6360 | 6140 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 640 | 960 | 8440 | 8760 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 640 | 960 | 8440 | 8760 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7220 | 7680 | 5280 | 5620 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 111580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2940 | 3360 | 960 | 1340 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

The IMD4 interference of UL DC_1_n5 may have an impact on the Rx of band n40.

The IMD4 interference of UL DC_1_n40 may have an impact on the Rx of band n5.

6.25.4	∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are specified as below.

Table 6.25.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n5-n40 | 0.6 | 0.6 | 0.9 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.25.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n5-n40 | - | 0.2 | 0.8 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.25.5	MSD requirements

Referring to DC_1-5_n40 and CA_1-5-40, the MSD requirements can be specified as below:

Table 6.25.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n5A-n40A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n40 | 2305 | 10 | 50 | 2305 | 9.0 | IMD4 |
|  | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 8.5 | IMD4 |
|  | n40 | 2385 | 5 | 25 | 2385 | N/A | N/A |

6.26	DC_7_n5-n40

6.26.1	Operating bands for DC

Table 6.26.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n5-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.26.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n5A-n40A | DC_7A_n5A DC_7A_n40A |

6.26.2 Channel bandwidths per operating band for DC

Table 6.26.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n5A-n40A |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n5 | 15 | 5 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | 55 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.26.3	Co-existence studies

The IMD products caused by UL DC_7_n5 are shown below.

Table 6.26.3-1: The IMD products caused by UL DC_7_n5

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1746 | 1651 | 3419 | 3324 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4316 | 4151 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4268 | 4148 | 5989 | 5824 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3302 | 3492 | 6648 | 6838 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – *fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 47 | 98 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9151 | 9456 | 726 | 896 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low –3 *fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5802 | 6062 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

The IMD products caused by UL DC_7_n40 are shown below.

Table 6.26.3-2: The IMD products caused by UL DC_7_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 100 | 4970 | 4800 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2840 | 2600 | 2300 | 2030 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7540 | 7300 | 7370 | 7100 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 200 | 540 | 9600 | 9940 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5100 | 5410 | 4330 | 4700 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9800 | 10110 | 9400 | 9770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6630 | 7100 | 7600 | 7980 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11700 | 12170 | 12300 | 12680 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1760 | 2200 | 2700 | 3110 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12340 | 12100 | 12510 |

No IMD of UL DC_7_n5 falls into band n40 Rx.

No IMD of UL DC_7_n40 falls into band n5 Rx.

6.26.4	∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are specified as below.

Table 6.26.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n5-n40 | 0.8 | 0.6 | 0.9 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.26.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n5-n40 | 0.3 | 0.2 | 0.8 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.26.5	MSD requirements

According to 6.26.3, no MSD need to be defined.

6.27	DC_28_n5-n40

6.27.1	Operating bands for DC

Table 6.27.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n5-n40 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.27.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_28A_n5A-n40A | DC_28A_n5A DC_28A_n40A |

6.27.2 Channel bandwidths per operating band for DC

Table 6.27.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n5A-n40A |  | 28 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n5 | 15 | 5 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | 55 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.27.3	Co-existence studies

The IMD products caused by UL DC_28_n5 are shown below.

Table 6.27.3-1: The IMD products caused by UL DC_28_n5

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 146 | 76 | 1597 | 1527 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 672 | 557 | 995 | 900 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2345 | 2230 | 2446 | 2351 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 152 | 292 | 3054 | 3194 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – *fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1420 | 1724 | 1844 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3093 | 3175 | 3295 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2548 | 2693 | 1963 | 2168 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4144 | 3636 | 3841 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low –3 *fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 976 | 1141 | 411 | 596 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4043 | 3757 | 3942 |

The IMD products caused by UL DC_28_n40 are shown below.

Table 6.27.3-2: The IMD products caused by UL DC_28_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1496 | 1406 | 4800 | 4600 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 994 | 804 | 4097 | 3852 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3896 | 3706 | 5548 | 5303 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3104 | 3394 | 6006 | 6296 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 56 | 291 | 6152 | 6497 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4409 | 4644 | 7603 | 7948 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8452 | 8897 | 412 | 692 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9903 | 10348 | 5112 | 5392 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5404 | 5794 | 2356 | 2691 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8306 | 8696 | 6709 | 7044 |

The IMD3 interference of UL DC_28_n5 may have an impact on the Rx of band n40.

The IMD3 interference of UL DC_28_n40 may have an impact on the Rx of band n5.

6.27.4	∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are specified as below.

Table 6.27.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n5-n40 | 0.6 | 0.6 | 0.9 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.27.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n5-n40 | 0.2 | 0.2 | 0.8 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.27.5	MSD requirements

Referring to DC_12-30_n5, the MSD requirements can be specified as below:

Table 6.27.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n5A-n40A | 28 | 712 | 5 | 25 | 767 | N/A | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n40 | 2365 | 5 | 25 | 2365 | 18.8 | IMD3 |
|  | 28 | 720 | 5 | 25 | 775 | N/A | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 17.0 | IMD3 |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | N/A |

6.28	DC_(n)3-n78

6.28.1	Operating bands for DC

Table 6.28.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.28.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)3AA-n78A | DC_(n)3AA1 DC_3A_n78A |
| DC_(n)3AA-n78(2A) | DC_(n)3AA1 DC_3A_n78A |
| NOTE1: Only single switched UL is supported. |  |

6.28.2	Channel bandwidths per operating band for DC

Table 6.28.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n78A | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  | n3 | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_(n)3AA-n78(2A) | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 250 |
|  | n3 | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.28.3	Co-existence studies

UL 3-n78 gives IMD2 and IMD5 into DL n3.

6.28.4	∆TIB and ∆RIB values

For DC_(n)3_n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.28.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n78 | - | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.28.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n78 | - | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.28.5	MSD

Based on the co-existence studies there is a need to define MSD values. MSD value of n3 from DC_(n)3-n78 is reused from CA_n3-n78.

Table 6.28.2.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_(n)3AA-n78A DC_(n)3AA-n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD24 |
|  | n3 | N/A | 5 | N/A | 1840 | [28.9] | IMD24 |
|  | n78 | 3575 | 5 | 25 | 3575 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.29	DC_(n)3-n7

6.29.1	Configuration for DC

Table 6.29.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3-n7 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

Table 6.29.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_(n)3AA-n7A | DC_(n)3AA2 DC_3A_n7A |
| NOTE 2: Only single switched UL is supported |  |

6.29.2 	Channel bandwidths per operating band for DC

Table 6.29.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n7A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  |  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |

6.29.3	Co-existence studies

UL DC_3_n7 does not give IMD into DL band n3.

6.29.4		∆TIB and ∆RIB values

For DC_(n)3-n7, the TIB,c and RIB,c values are reused from DC_3_n7 and are given in the tables below.

Table 6.29.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n7 | 0.5 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.29.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n7 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.29.5	MSD requirements

MSD value does not need to be defined.

6.30	DC_(n)3-n28

6.30.1	Configuration for DC

Table 6.30.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

Table 6.30.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_(n)3AA-n28A | DC_(n)3AA2 DC_3A_n28A |
| NOTE 2: Only single switched UL is supported |  |

6.30.2 	Channel bandwidths per operating band for DC

Table 6.30.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n28A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  |  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.30.3	Co-existence studies

UL DC_3_n28 does not give IMD into DL band n3.

6.30.4		∆TIB and ∆RIB values

For DC_(n)3-n28, the TIB,c and RIB,c values are reused from DC_3_n28 and are given in the tables below.

Table 6.30.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n28 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.30.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n28 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.30.5	MSD requirements

MSD value does not need to be defined.

6.31	DC_(n)3-n67

6.31.1	Configuration for DC

Table 6.31.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3-n67 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n67 | N/A |  |  | 738 MHz | – | 758 MHz | FDD |

Table 6.31.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_(n)3AA-n67A | DC_(n)3AA2 |
| NOTE 2: Only single switched UL is supported |  |

6.31.2 	Channel bandwidths per operating band for DC

Table 6.31.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n67A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  |  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n67 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.31.3	Co-existence studies

UL DC_3_n67 is not a possible UL configuration since n67 is SDL.

6.31.4		∆TIB and ∆RIB values

For DC_(n)3-n67, the TIB,c and RIB,c values are reused from DC_3_n67 and are given in the tables below.

Table 6.31.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n67 | - | - | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.31.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n67 | 0.3 | 0.3 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.31.5	MSD requirements

MSD value does not need to be defined.

6.32		DC_3_n8-n41

6.32.1	 Operating bands for DC

Table 6.32.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n8-n41 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.32.2 	Configuration for DC

Table 6.32.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n8A-n41A | DC_3A_n41A DC_3A_n8A |

6.32.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_3_n8 are shown below.

Table 6.32.3-1: The harmonic and IMD products caused by UL DC_3_n8

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 880 | 915 | 1710 | 1785 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 860 | 935 | 1675 | 1820 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1810 | 1590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5180 | 5400 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6235 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

The harmonic and IMD products caused by UL DC_3_n41 are shown below.

Table 6.32.3-2: The harmonic and IMD products caused by UL DC_3_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 2496 | 2690 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1960 | 1422 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8412 | 8950 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

In light of the above co-existence table, we can observe:

IMD 2 and IMD3 caused by 3+n8 may fall into the its own band n41 Rx

IMD 2 and IMD3 caused by 3+n41 may fall into the its own band n8 Rx

6.32.4	 ∆TIB and ∆RIB values

Referring to CA_n3A-n8A-n41A, ΔTIB,c and ΔRIB,c for DC_3A_n8A-n41A can be specified as below.

Table 6.32.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n8-n41 | 0.5 | 0.3 | 0.33/0.84 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.32.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n8-n41 | - | - | 03/0.54 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.32.5	 MSD requirements

Referring to DC_3A-n8A-n41A, the MSD requirement are defined in table 6.32.5-1:

Table 6.32.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) |  |
| DC_3A_n8A-n41A | 3 | 1722.5 | 5 | 25 | 1817.5 | N/A | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | 28.0 | IMD2x |
|  | 3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 26.0 | IMD2x |
|  | n41 | 2516 | 10 | 50 | 2516 | N/A | N/A |
| NOTE x: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

6.33	DC_2_n71-n77

6.33.1	Configuration for DC

Table 6.33.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n71-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.33.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A_n71A-n77A | DC_2A_n71A DC_2A_n77A |

6.33.2 	Channel bandwidths per operating band for DC

Table 6.33.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A_n71A-n77A |  |  | 2 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n71 |  | 15 | 5 | 10 | 15 | 20 | 253 | 30 | 35 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 30 | 35 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.33.3	Co-existence studies

Table 6.33.3-1: Band 2 and Band n71 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 663 | 698 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1152 | 1247 | 2513 | 2608 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3002 | 3157 | 584 | 454 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4363 | 4518 | 3176 | 3306 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2304 | 2494 | 5026 | 5216 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4852 | 5067 | 79 | 244 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6213 | 6428 | 3839 | 4004 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 942 | 742 | 6977 | 6702 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4502 | 4702 | 8063 | 8338 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1606 | 1831 | 4404 | 4154 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5689 | 5914 | 6876 | 7126 |

Table 6.33.3-2: Band 2 and Band n77 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2350 | 1390 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 500 | 520 | 4690 | 6550 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4700 | 2780 | 10300 | 12220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14950 | 11290 | 4340 | 3200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8900 | 6080 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

As can be seen in Table 6.33.3-1 there are IMD3, IMD4 and IMD5 impact from UL 2_n71 affecting DL band n77.

As can be seen in Table 6.33.3-2 there are no impact from UL 2_n77 affecting DL band n71.

6.33.4		∆TIB and ∆RIB values

For DC_2_n71-n77, the TIB,c and RIB,c values are reused from DC_2_n71-n78 and are given in the tables below.

Table 6.33.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2_n71-n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.33.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2_n71-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.33.5	MSD requirements

MSD value band n77 and testpoints are reused from DC_2_n71-n78.

Table 6.33.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n71A-n77A | 2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8 | IMD3 |

6.34	DC_3_n3-n7

6.34.1	Configuration for DC

Table 6.34.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n3-n7. | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

Table 6.34.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n3A-n7A | DC_3A_n3A2 DC_3A_n7A |
| NOTE 2: Only single switched UL is supported |  |

6.34.2 	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n3A-n7A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  |  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  | 50 |  |  |  |  |  |  |

6.34.3	Co-existence studies

UL DC_3_n7 does not give IMD into DL band n3.

6.34.4		∆TIB and ∆RIB values

For DC_3_n3-n7, the TIB,c and RIB,c values are reused from DC_3_n7 and are given in the tables below.

Table 6.34.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n3-n7 | 0.5 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.34.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n3-n7 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.34.5	MSD requirements

MSD value does not need to be defined.

6.35	DC_3_n3-n28

6.35.1	Configuration for DC

Table 6.35.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n3-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

Table 6.35.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n3A-n28A | DC_3A_n3A2 DC_3A_n28A |
| NOTE 2: Only single switched UL is supported |  |

6.35.2 	Channel bandwidths per operating band for DC

Table 6.35.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n3A-n28A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  |  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |

6.35.3	Co-existence studies

UL DC_3_n28 does not give IMD into DL band n3.

6.35.4		∆TIB and ∆RIB values

For DC_3_n3-n28, the TIB,c and RIB,c values are reused from DC_3_n28 and are given in the tables below.

Table 6.35.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n3-n28 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.35.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n3-n28 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.35.5	MSD requirements

MSD value does not need to be defined.

6.36	DC_3_n3-n67

6.36.1	Configuration for DC

Table 6.36.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n3-n67 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n67 | N/A | – | N/A | 738 MHz | – | 758 MHz | SDL |

Table 6.36.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n3A-n67A | DC_3A_n3A2 |
| NOTE 2: Only single switched UL is supported |  |

6.36.2 	Channel bandwidths per operating band for DC

Table 6.36.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n3A-n67A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  |  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n67 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |

6.36.3	Co-existence studies

There are no co-existence issues.

6.36.4		∆TIB and ∆RIB values

For DC_3_n3-n67, the TIB,c and RIB,c values are reused from CA_n3-n67 and are given in the tables below.

Table 6.36.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n3-n67 | - | - | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.36.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n3-n67 | 0.3 | 0.3 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.36.5	MSD requirements

MSD value does not need to be defined.

6.37	DC_7_n12-n77

6.37.1	Operating bands for DC

Table 6.37.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n12-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.37.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n12A-n77A | DC_7A_n12A DC_7A_n77A |

6.37.2	Channel bandwidths per operating band for DC

Table 6.37.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n12A-n77A |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 135 |
|  |  | n12 | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.37.3	Co-existence studies

Based on co-existence studies of DC_7_n77 and DC_7_n12, own Rx impact of the 3rd band is the followings.

-    3rd order IMD products generated by DC_7_n12 uplink may fall into own Rx of band n77.

-    2nd order IMD products generated by DC_7_n77 uplink may fall into own Rx of band n12.

6.37.4	∆TIB and ∆RIB values

For DC_7_n12-n77, the TIB,c and RIB,c values are reused from DC_7-12_n78 and are given in the tables below.

Table 6.37.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n12-n77 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.37.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n12-n77 | 0.2 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.37.5	MSD requirements

Table 6.37.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A-12A_n78A and CA_n7-n71-n77 are reused.

Table 6.37.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n12A-n77A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 30.8 | IMD2 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |
|  | n12 | 702 | 5 | 25 | 732 | N/A | N/A |
|  | n77 | 3909 | 10 | 50 | 3909 | 16.0 | IMD3 |

6.38	DC_66_n12-n77

6.38.1	Operating bands for DC

Table 6.38.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n12-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.38.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n12A-n77A | DC_66A_n77A DC_66A_n12A |

6.38.2	Channel bandwidths per operating band for DC

Table 6.38.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n12A-n77A |  | 66 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 135 |
|  |  | n12 | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.38.3	Co-existence studies

Based on co-existence studies of DC_66_n77 and DC_66_n12, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_66_n77 uplink may fall into own Rx of band n12.

-    3rd,4th and 5th order IMD products generated by DC_66_n12 uplink may fall into own Rx of band n77.

6.38.4	∆TIB and ∆RIB values

For DC_66_n12-n77, the TIB,c and RIB,c values are reused from DC_12-66_n77 and are given in the tables below.

Table 6.38.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66_n12-n77 | 0.6 | 0.8 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.38.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66_n12-n77 | 0.5 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.38.5	MSD requirements

Table 6.38.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n12-n66-n77 are reused.

Table 6.38.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n12A-n77A | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 15.2 | IMD311 |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | N/A |
|  | 66 | 1723 | 5 | 25 | 2123 | N/A | N/A |
|  | n12 | 704 | 5 | 25 | 734 | N/A | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | 16.0 | IMD34,9,11 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

6.39	DC_71_n2-n77

6.39.1	Operating bands for DC

Table 6.39.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n2-n77 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.39.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n2A-n77A | DC_71A_n77A DC_71A_n2A |

6.39.2	Channel bandwidths per operating band for DC

Table 6.39.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n2A-n77A |  |  | 71 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  | n2 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 354 | 40 |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.39.3	Co-existence studies

Based on co-existence studies of DC_71_n77 and DC_71_n2, own Rx impact of the 3rd band is the followings.

-    3rd and 4th order IMD products generated by DC_71_n77 uplink may fall into own Rx of band n2.

-    3rd,4th and 5th order IMD products generated by DC_71_n2 uplink may fall into own Rx of band n77.

6.39.4	∆TIB and ∆RIB values

For DC_71_n2-n77, the TIB,c and RIB,c values are reused from DC_2_n71-n77 and  are given in the tables below.

Table 6.39.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n2-n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.39.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n2-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.39.5	MSD requirements

Table 6.39.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n25-n71-n77 are reused.

Table 6.39.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_71A_n2A-n77A | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8.0 | IMD34,9,11 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD39,11 |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

6.40	DC_71_n66-n77

6.40.1	Operating bands for DC

Table 6.40.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n66-n77 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.40.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n66A-n77A | DC_71A_n66A DC_71A_n77A |

6.40.2	Channel bandwidths per operating band for DC

Table 6.40.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n66A-n77A |  |  | 71 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | n66 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 454 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 454 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 354 | 40 | 454 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.40.3	Co-existence studies

Based on co-existence studies of DC_71_n77 and DC_71_n66, own Rx impact of the 3rd band is the followings.

-    3rd and 4th order IMD products generated by DC_71_n77 uplink may fall into own Rx of band n66.

-    3rd,4th and 5th order IMD products generated by DC_71_n66 uplink may fall into own Rx of band n77.

6.40.4	∆TIB and ∆RIB values

For DC_71_n66-n77, the TIB,c and RIB,c values are reused from DC_66_n71-n77 and are given in the tables below.

Table 6.40.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n66-n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.40.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n66-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.40.5	MSD requirements

Table 6.40.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n66-n71-n77 are reused.

Table 6.40.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_71A_n66A-n77A | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | IMD34,9,11 |
|  | 71 | 690 | 5 | 25 | 644 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 15.5 | IMD39,11 |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

6.41	DC_2_n12-n77

6.41.1	Operating bands for DC

Table 6.41.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n12-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.41.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n12A-n77A | DC_2A_n77A DC_2A_n12A |

6.41.2	Channel bandwidths per operating band for DC

Table 6.41.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A_n12A-n77A |  | 2 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 135 |
|  |  | n12 | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.41.3	Co-existence studies

Based on co-existence studies of DC_2_n77 and DC_2_n12, own Rx impact of the 3rd band is the followings.

-    3rd,4th and 5th order IMD products generated by DC_2_n12 uplink may fall into own Rx of band n77.

6.41.4	∆TIB and ∆RIB values

For DC_2_n12-n77, the TIB,c and RIB,c values are reused from DC_2-12_n77 and are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2_n12-n77 | 0.6 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.41.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2_n12-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.41.5	MSD requirements

Table 6.41.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n2-n12-n77 are reused.

Table 6.41.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n12A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | IMD34,9,11 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. NOTE 11: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

6.42	DC_1_n78-n105

6.42.1	Operating bands for DC

Table 6.42.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n78-n105 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.42.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n78A-n105A | DC_1A_n78A DC_1A_n105A |

6.42.2	Channel bandwidths per operating band for DC

Table 6.42.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n78A-n105A |  | 1 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n105 |  | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.42.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.42.3-1 for DC_1A_n78A into band n105 downlink.

Table 6.42.3-1: IMD analysis for DC_1A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 3300 | 3800 |
| DL Frequency [MHz] | 2110 | 2170 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 1880 | 5220 | 5780 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 | 660 | 4620 | 5680 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7760 | 8520 | 9580 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3760 | 2640 | 10440 | 11560 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 9740 | 11820 | 13380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 | 11220 | 4620 | 3880 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 | 5940 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 15360 | 12360 | 13540 |

The analysis shows the 3rd order IMD product falls inside band n105 downlink

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.42.3-2 for DC_1A_n105A into band n78 downlink.

Table 6.42.3-2: IMD analysis for DC_1A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 663 | 703 |
| DL Frequency [MHz] | 2110 | 2170 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 | 1217 | 2583 | 2683 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3137 | 3297 | 654 | 514 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 | 4663 | 3246 | 3386 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5057 | 5277 | 9 | 189 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2434 | 2634 | 5166 | 5366 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 | 6643 | 3909 | 4089 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 892 | 672 | 7257 | 6977 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1731 | 1971 | 4614 | 4354 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 | 4792 | 8343 | 8623 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 | 6069 | 7086 | 7346 |

The analysis shows the 3rd order IMD product falls inside band n78 downlink.

Based on Table 6.42.3-1 and Table 6.42.3-2:

- The 3rd order IMD product of DC_1_n78 falls inside band n105 downlink

- The 3rd order IMD product of DC_1_n105 falls inside band n78 downlink

6.42.4		∆TIB and ∆RIB values

For DC_1_n78-n105, the TIB,c and RIB,c values are taken from fallbacks.

Table 6.42.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n78-n105 | 0.3 | 0.8 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.42.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n78-n105 | - | 0.5 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.42.5	REFSENS requirements

MSD values are reused from DC_1A_n28A-n78A and DC_1A-28A_n79A.

Table 6.42.5-1: : MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n78-n105 | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 15.2 | IMD3 |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3342 | 10 | 50 | 3342 | 15.7 | IMD3 |
|  | n105 | 686 | 5 | 25 | 635 | N/A | N/A |

6.43	DC_3_n78-n105

6.43.1	Operating bands for DC

Table 6.43.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n105 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.43.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n78A-n105A | DC_3A_n78A DC_3A_n105A |

6.43.2	Channel bandwidths per operating band for DC

Table 6.43.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n78A-n105A |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n105 |  | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.43.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.43.3-1 for DC_3A_n78A into band n105 downlink.

Table 6.43.3-1: IMD analysis for DC_3A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 3300 | 3800 |
| DL Frequency [MHz] | 1805 | 1880 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 270 | 4815 | 5890 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3030 | 10020 | 11170 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11415 | 3840 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6330 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

The analysis shows that there are no IMD product falling inside band n105 downlink

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.43.3-2 for DC_3A_n105A into band n78 downlink.

Table 6.43.3-2: IMD analysis for DC_3A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 663 | 703 |
| DL Frequency [MHz] | 1805 | 1880 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1007 | 2373 | 2488 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2717 | 2907 | 459 | 304 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4273 | 3036 | 3191 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4427 | 4692 | 204 | 399 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2014 | 2244 | 4746 | 4976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6058 | 3699 | 3894 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1102 | 867 | 6477 | 6137 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1311 | 1581 | 4029 | 3724 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4597 | 7503 | 7843 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5679 | 6456 | 6761 |

The analysis shows the 4th and 5th order IMD product falls inside band n78 downlink.

Based on Table 6.43.3-1 and Table 6.43.3-2:

- The 4th and 5th order IMD product of DC_3_n105 falls inside band n78 downlink

6.43.4		∆TIB and ∆RIB values

For DC_3_n78-n105, the TIB,c and RIB,c values are taken from fallbacks.

Table 6.43.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n78-n105 | 0.3 | 0.8 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.43.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n78-n105 | - | 0.5 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.43.5	REFSENS requirements

MSD values are reused from DC_66A_n71A-n78A.

Table 6.43.5-1: : MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3_n78-n105 | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | 13 | IMD44 |
|  | n105 | 670 | 5 | 25 | 619 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.44	DC_7_n78-n105

6.44.1	Operating bands for DC

Table 6.44.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n78-n105 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.44.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n78A-n105A | DC_7A_n78A DC_7A_n105A |

6.44.2	Channel bandwidths per operating band for DC

Table 6.44.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n78A-n105A |  | 7 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n105 |  | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.44.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.44.3-1 for DC_7A_n78A into band n105 downlink.

Table 6.44.3-1: IMD analysis for DC_7A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 3300 | 3800 |
| DL Frequency [MHz] | 2620 | 2690 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

The analysis shows the 5th order IMD product falls inside band n105 downlink.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.44.3-2 for DC_7A_n105A into band n78 downlink.

Table 6.44.3-2: IMD analysis for DC_7A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 663 | 703 |
| DL Frequency [MHz] | 2620 | 2690 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1797 | 3163 | 3273 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4297 | 4477 | 1244 | 1094 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5843 | 3826 | 3976 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6797 | 7047 | 581 | 391 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3594 | 3814 | 6326 | 6546 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8413 | 4489 | 4679 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 312 | 82 | 9617 | 9297 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 3151 | 6384 | 6094 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5382 | 10663 | 10983 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7249 | 8826 | 9116 |

The analysis shows the 4th order IMD product falls inside band n78 downlink.

Based on Table 6.44.3-1 and Table 6.44.3-2:

- The 5th order IMD product of DC_7_n78 falls inside band n105 downlink

- The 4th order IMD product of DC_7_n105 falls inside band n78 downlink

6.44.4		∆TIB and ∆RIB values

For DC_7_n78-n105, the TIB,c and RIB,c values are taken from fallbacks.

Table 6.44.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n78-n105 | 0.5 | 0.8 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.44.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n78-n105 | - | 0.5 | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.44.5	REFSENS requirements

MSD values are reused from DC_7A_n71A-n78A.

Table 6.44.5-1: : MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7_n78-n105 | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 9.7 | IMD4 |
|  | n105 | 670 | 5 | 25 | 619 | N/A | N/A |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | n105 | 676 | 5 | 25 | 625 | 3.9 | IMD5 |

6.45	DC_7_n71-n77

6.45.1	Operating bands for DC

Table 6.45.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n71-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.45.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n71A-n77A | DC_7A_n71A DC_7A_n77A |

6.45.2	Channel bandwidths per operating band for DC

Table 6.45.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n71A-n77A |  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n71 | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 | 5 | 10 | 15 | 20 | 253 | 303 | 353,4 |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | 10 | 15 | 20 | 253 | 303 | 353,4 | 40 | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.45.3	Co-existence studies

Based on co-existence studies of DC_7_n77 and DC_7_n71, own Rx impact of the 3rd band is the followings.

-    3rd order IMD products generated by DC_7_n71 uplink may fall into own Rx of band n77.

6.45.4	∆TIB and ∆RIB values

For DC_7_n71-n77, the TIB,c and RIB,c values are reused from CA_n7-n71-n77 and are given in the tables below.

Table 6.45.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n71-n77 | 0.5 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.45.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n71-n77 | - | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.45.5	MSD requirements

Table 6.45.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n7-n71-n77 are reused.

Table 6.45.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n71A-n77A | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |
|  | n71 | 666 | 5 | 25 | 620 | N/A | N/A |
|  | n77 | 3837 | 10 | 50 | 3837 | 16.0 | IMD3 |

6.46	DC_5_n2-n41

6.46.1	Operating bands for DC

Table 6.46.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n2-n41 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.46.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n2A-n41A | DC_5A_n2A DC_5A_n41A |

6.46.2	Channel bandwidths per operating band for DC

Table 6.46.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n2A-n41A |  |  | 5 |  | 15 | 5 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  |  | n2 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 15 | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.46.3	Co-existence studies

Based on co-existence studies of DC_5_n41 and DC_5_n2, own Rx impact of the 3rd band is the followings.

-    2nd order IMD products generated by DC_5_n2 uplink may fall into own Rx of band n41.

6.46.4	∆TIB and ∆RIB values

For DC_5_n2-n41, the TIB,c and RIB,c values are reused from CA_n2-n5-n41 and  are given in the tables below.

Table 6.46.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n2-n41 | 0.6 | 0.5 | 0.41 / 0.92 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.46.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n2-n41 | 0.2 | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.46.5	MSD requirements

Table 6.46.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n2-n5-n41 are reused.

Table 6.46.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n2A-n41A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n2 | 1855 | 10 | 50 | 1935 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | 30.0 | IMD2 |

6.47	DC_7_n2-n77

6.47.1	Operating bands for DC

Table 6.47.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.47.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n2A-n77A | DC_7A_n2A DC_7A_n77A |

6.47.2	Channel bandwidths per operating band for DC

Table 6.47.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n2A-n77A |  |  | 7 |  | 15 | 5 | 10 | 151 | 201,3 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  | n2 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.47.3	Co-existence studies

Based on co-existence studies of DC_7_n77 and DC_7_n2, own Rx impact of the 3rd band is the followings.

-    4th order IMD products generated by DC_7_n77 uplink may fall into own Rx of band n2.

-    5th order IMD products generated by DC_7_n2 uplink may fall into own Rx of band n77.

6.47.4	∆TIB and ∆RIB values

For DC_7_n2-n77, the TIB,c and RIB,c values are reused from DC_7_n2-n78 and  are given in the tables below.

Table 6.47.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n2-n77 | 0.5 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.47.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n2-n77 | 0.5 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.47.5	MSD requirements

Table 6.47.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A_n2A-n78A are reused.

Table 6.47.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n2A-n77A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 |

6.48	DC_66_n2-n41

6.48.1	Operating bands for DC

Table 6.48.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n41 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.48.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n2A-n41A | DC_66A_n2A DC_66A_n41A |

6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n2A-n41A |  |  | 66 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  | n2 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 15 | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.48.3	Co-existence studies

Based on co-existence studies of DC_66_n41 and DC_66_n2, own Rx impact of the 3rd band is the followings.

-    4th order IMD products generated by DC_66_n41 uplink may fall into own Rx of band n2.

6.48.4	∆TIB and ∆RIB values

For DC_66_n2-n41, the TIB,c and RIB,c values are reused from DC_2-66_n41 and  are given in the tables below.

Table 6.48.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66_n2-n41 | 0.5 | 0.5 | 0.81 / 1.32 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.48.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66_n2-n41 | 0.3 | 0.5 | 0.51 / 12 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.48.5	MSD requirements

Table 6.48.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_2A-66A_n41A are reused.

Table 6.48.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n2A-n41A | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |

6.49	DC_66_n12-n78

6.49.1	Operating bands for DC

Table 6.49.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n12-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110MHz | – | 2200 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.49.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n12A-n78A | DC_66A_n12A DC_66A_n78A |

6.49.2	Channel bandwidths per operating band for DC

Table 6.49.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n12A-n78A |  |  | 66 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 135 |
|  |  |  | n12 |  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.49.3	Co-existence studies

Based on co-existence studies of DC_66_n78 and DC_66_n12, own Rx impact of the 3rd band is the followings.

-   5th order IMD products generated by DC_66_n12 uplink may fall into own Rx of band n78.

6.49.4	∆TIB and ∆RIB values

For DC_66_n12-n78, the TIB,c and RIB,c values are reused from DC_12_n66-n78 and  are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66_n12-n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.49.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66_n12-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.49.5	MSD requirements

Table 6.49.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_12A_n66A-n78A are reused.

Table 6.49.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n12A-n78A | 66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n12 | 703 | 5 | 25 | 733 | N/A | N/A |
|  | n78 | 3754 | 10 | 50 | 3754 | 4.1 | IMD5 |

6.50	DC_71_n41-n66

6.50.1	Operating bands for DC

Table 6.50.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n41-n66 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110MHz | – | 2200 MHz | FDD |

Table 6.50.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n41A-n66A | DC_71A_n41A DC_71A_n66A |

6.50.2	Channel bandwidths per operating band for DC

Table 6.50.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n41A-n66A |  |  | 71 |  | 15 | 5 | 101 | 151 | 201,6 |  |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | n41 |  | 15 | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n66 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.50.3	Co-existence studies

Based on co-existence studies of DC_71_n66 and DC_71_n41, there is no own Rx impact of the 3rd band for DC_71_n41-n66.

6.50.4	∆TIB and ∆RIB values

For DC_71_n41-n66, the TIB,c and RIB,c values are reused from DC_66-71_n41 and  are given in the tables below.

Table 6.50.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n41-n66 | 0.5 | 0.81 / 1.32 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |

Table 6.50.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n41-n66 | 0.5 | 0.51 / 12 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |

6.50.5	MSD requirements

Based on the co-existence study in section 6.50.3, there is no need to define MSD requirements.

6.51	DC_7_n2-n77

6.51.1	Operating bands for DC

Table 6.51.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.51.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n2A-n77A | DC_7A_n2A DC_7A_n77A |

6.51.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n2A-n77A | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  | n2 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.51.3	Co-existence studies

The harmonic and IMD products caused by UL DC_7_n2 are shown below.

Table 6.1.3-1: The harmonic and IMD products caused by UL DC_7_n2

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 590 | 720 | 4350 | 4480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1130 | 1320 | 3090 | 3290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6200 | 6390 | 6850 | 7050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2980 | 3230 | 5590 | 5860 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8050 | 8300 | 9350 | 9620 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1180 | 8700 | 8960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8430 | 8090 | 5140 | 4830 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4010 | 3680 | 730 | 410 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11850 | 12190 | 9900 | 10210 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11200 | 11530 | 10550 | 10870 |

Based on the above table, the 2nd harmonics of band n2 may have an impact on the band n77 (3700-3820MHz) , and the IMD5 interference of UL DC_7_n2 may have an impact on the band n77 （3680-4010M）.

The harmonic and IMD products caused by UL DC_7_n77 are shown below.

Table 6.1.3-2: The harmonic and IMD products caused by UL DC_7_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1700 | 5800 | 6770 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1840 | 4030 | 5900 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 9340 | 9100 | 10970 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3300 | 4410 | 7330 | 10100 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11910 | 12400 | 15170 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3400 | 1460 | 11600 | 13540 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14300 | 10630 | 6980 | 5800 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7600 | 4760 | 1110 | 900 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 19370 | 13300 | 14480 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 17740 | 14100 | 16110 |

The IMD4 interference of UL DC_7_n77 may have an impact on the Rx of band n2 （1930-1990M）and band 7 (2620-2690MHz).

6.51.4	∆TIB and ∆RIB values

For DC_7_n2-n77, the TIB,c and RIB,c values can reuse the values of DC_2-7_n77 as given in the tables below.

Table 6.1.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n2-n77 | 0.5 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.1.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n2-n77 | 0.5 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.51.5	MSD requirements

Based on co-existence studies MSD needs to be defined. MSD values are reused from DC_2A-7A_n77A and CA_n7-n25-n77.

Table 6.51.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n2A-n77A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | n77 | 3750 | 10 | 50 | 3750 | 4.5 | IMD5 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.52		DC_5_n28-n77

6.52.1	 Operating bands for DC

Table 6.52.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n28-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.52.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | DC_5A_n77A |

6.52.2 	Channel bandwidths per operating band for DC

Table 6.52.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n28A-n77A |  | 5 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 135 |
|  |  | n28 | 15 | 5 | 10 | 15 | 207 | 257 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 207 | 257 |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.52.3	 Co-existence studies

Based on co-existence studies of DC_5_n77, own Rx impact of the 3rd band is the following.

-  4th and 5th IMD products caused by UL DC_5_n77 falls into the third band n28.

6.52.4	 ∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are specified as below.

Table 6.52.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n28-n77 | 0.6 | 0.5 | 0.9 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.52.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n28-n77 | 0.2 | 0.2 | 0.8 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.52.5	 MSD requirements

Referring to CA_n5-n14-n77 and CA_n5-n12-n77, the MSD requirements can be specified as below:

Table 6.52.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | 11.6 | IMD4 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 4.4 | IMD5 |
|  | n77 | 4105 | 10 | 50 | 4105 | N/A | N/A |

6.53	DC_12_n2-n66

6.53.1	Operating bands for DC

Table 6.53.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n66 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

Table 6.53.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A_n2A-n66A | DC_12A_n2A DC_12A_n66A |

6.53.2	Channel bandwidths per operating band for DC

Table 6.53.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n2A-n66A | 12 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 |
|  | n2 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.53.3	Co-existence studies

The harmonic and IMD products caused by UL DC_12_n2 are shown below.

Table 6.53.3-1: The harmonic and IMD products caused by UL DC_12_n2

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 699 | 716 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1398 | 1432 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2097 | 2148 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 2796 | 2864 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 3495 | 3580 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1211 | 1134 | 2549 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2984 | 3121 | 512 | 418 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4399 | 4536 | 3248 | 3342 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4834 | 5031 | 187 | 298 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6249 | 6446 | 3947 | 4058 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2268 | 2422 | 5098 | 5252 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1014 | 886 | 6941 | 6684 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1552 | 1723 | 4332 | 4118 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4646 | 4774 | 8099 | 8356 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5797 | 5968 | 6948 | 7162 |

The 3rd harmonic of band 12 may have an impact on the Rx of band n66.

The harmonic and IMD products caused by UL DC_12_n66 are shown below.

Table 6.53.3-2: The harmonic and IMD products caused by UL DC_12_n66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1081 | 2409 | 2496 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 | 278 | 2704 | 2861 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 | 3928 | 5829 | 6056 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 | 1988 | 4818 | 4992 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 | 6772 | 5517 | 5708 |

The 3rd harmonics of band 12 may have an impact on the DL of band n66.

The IMD4 interference of UL DC_12_n66 may have an impact on the Rx of band n2.

The IMD4 interference of UL DC_12_n66 may have an impact on the Rx of band n66.

6.53.4	∆TIB and ∆RIB values

For DC_12_n2-n66, the TIB,c and RIB,c values are reused from CA_n2-n12-n66 and are given in the tables below.

Table 6.53.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n2-n66 | 0.8 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.53.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n2-n66 | 0.5 | 0.3 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.53.5	MSD requirements

No MSD value need to be defined since there is no test points to allow the hit on n2 DL from UL DC_12_n66.

6.54	DC_12_n2-n77

6.54.1	Operating bands for DC

Table 6.54.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n77 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.54.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A_n2A-n77A | DC_12A_n2A DC_12A_n77A |

6.54.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n2A-n77A | 12 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  | n2 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.54.3	Co-existence studies

The harmonic and IMD products caused by UL DC_12_n2 are shown below.

Table 6.54.3-1: The harmonic and IMD products caused by UL DC_12_n2

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 699 | 716 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1398 | 1432 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2097 | 2148 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 2796 | 2864 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 3495 | 3580 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1211 | 1134 | 2549 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2984 | 3121 | 512 | 418 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4399 | 4536 | 3248 | 3342 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4834 | 5031 | 187 | 298 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6249 | 6446 | 3947 | 4058 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2268 | 2422 | 5098 | 5252 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1014 | 886 | 6941 | 6684 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1552 | 1723 | 4332 | 4118 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4646 | 4774 | 8099 | 8356 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5797 | 5968 | 6948 | 7162 |

No harmonic or IMD effect.

Table 6.54.3-2: The harmonic and IMD products caused by UL DC_12_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2584 | 3501 | 3999 | 4916 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2802 | 1868 | 5884 | 7701 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7002 | 5168 | 7998 | 9832 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16101 | 12484 | 436 | 1404 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11202 | 8468 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

The IMD3 and IMD4 interference of UL DC_12_n77 may have an impact on the Rx of band n2.

The IMD5 interference of UL DC_12_n77 may have an impact on the Rx of band n12.

6.54.4	∆TIB and ∆RIB values

For DC_12_n2-n77, the TIB,c and RIB,c values are reused from CA_n2-n12-n77 and are given in the tables below.

Table 6.54.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n2-n77 | 0.3 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.54.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n2-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.54.5	MSD requirements

Based on co-existence studies MSD need to be defined. MSD values are reused from CA_n2-n12-n77.

Table 6.54.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n2A-n77A | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.5 | IMD39 |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | IMD34,9 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.55	DC_12_n66-n77

6.55.1	Operating bands for DC

Table 6.55.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n66-n77 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.55.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A_n66A-n77A | DC_12A_n66A DC_12A_n77A |

6.55.2	Channel bandwidths per operating band for DC

Table 6.55.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n66A-n77A | 12 | 15 | 51 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  | n77 | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.55.3	Co-existence studies

The harmonic and IMD products caused by UL DC_12_n66 are shown below.

Table 6.55.3-1: The harmonic and IMD products caused by UL DC_12_n66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1081 | 2409 | 2496 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 | 278 | 2704 | 2861 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 | 3928 | 5829 | 6056 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 | 1988 | 4818 | 4992 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 | 6772 | 5517 | 5708 |

Based on the above table, the 3rd, 4th and 5th order IMD products generated by UL DC_12_n66 uplink may have an impact on the band n77.

The harmonic and IMD products caused by UL DC_12_n77 are shown below.

Table 6.55.3-2: The harmonic and IMD products caused by UL DC_12_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2584 | 3501 | 3999 | 4916 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2802 | 1868 | 5884 | 7701 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7002 | 5168 | 7998 | 9832 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16101 | 12484 | 436 | 1404 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11202 | 8468 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

The IMD3 product caused by UL DC_12_n77 may have an impact on the Rx of band n66.

6.55.4	∆TIB and ∆RIB values

For DC_12_n66-n77, the TIB,c and RIB,c values are reused from CA_n12-n66-n77 and are given in the tables below.

Table 6.55.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n66-n77 | 0.8 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.55.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n66-n77 | 0.5 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.55.5	MSD requirements

MSD values are reused from CA_n12-n66-n77 for IMD3. Based on the guideline in TR 38.846, that is, for type 2 UL configurations, if the DL band may be affected by a mix of even and odd order IMD products, then the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications, the IMD4 is added.

Table 6.55.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n66A-n77A | 12 | 707 | 5 | 25 | 737 | N/A | N/A |
|  | n66 | 1726 | 5 | 25 | 2126 | 13.2 | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
|  | 12 | 704 | 5 | 25 | 734 | N/A | N/A |
|  | n66 | 1723 | 5 | 25 | 2123 | N/A | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | 16.0 | IMD32,4, |
|  | 12 | 709 | 5 | 25 | 739 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n77 | 3842 | 10 | 50 | 3842 | 9 | IMD4 |
| NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.56	DC_5_n2-n66

6.56.1	Operating bands for DC

Table 6.56.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n2-n66 | 5 | 824 MHz | – | 849MHz | 869 MHz | – | 894MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110MHz | – | 2200 MHz | FDD |

Table 6.56.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n2A-n66A | DC_5A_n2A DC_5A_n66A |

6.56.2	Channel bandwidths per operating band for DC

Table 6.56.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n2A-n66A |  |  | 5 |  | 15 | 5 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 |
|  |  |  | n2 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  |  |  | n66 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.56.3	Co-existence studies

Based on co-existence studies of DC_5_n66 and DC_5_n2, own Rx impact of the 3rd band is the followings.

-    4th order IMD products generated by DC_5_n2 uplink may fall into own Rx of band n66.

6.56.4	∆TIB and ∆RIB values

For DC_5_n2-n66, the TIB,c and RIB,c values are reused from DC_2-66_n5 and  are given in the tables below.

Table 6.56.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n2-n66 | 0.3 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.56.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n2-n66 | - | 0.3 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.56.5	MSD requirements

Table 6.56.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_2A-66A_n5A are reused.

Table 6.56.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n2A-n66A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |

6.57	DC_7_n12-n78

6.57.1	Operating bands for DC

Table 6.57.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n12-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.57.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n12A-n78A | DC_7A_n12A DC_7A_n78A |

6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n12A-n78A |  |  | 7 |  | 15 | 5 | 10 | 151 | 201,3 |  |  |  |  |  |  |  |  |  |  |  | 135 |
|  |  |  | n12 |  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.57.3	Co-existence studies

Based on co-existence studies of DC_7_n12 and DC_7_n78, own Rx impact of the 3rd band is the followings.

-    4th order IMD products generated by DC_7_n12 uplink may fall into own Rx of band n78.

-    2nd and 5th order IMD products generated by DC_7_n78 uplink may fall into own Rx of band n12.

6.57.4	∆TIB and ∆RIB values

For DC_7_n12-n78, the TIB,c and RIB,c values are reused from DC_7_n12-n77 and  are given in the tables below.

Table 6.57.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n12-n78 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.57.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n12-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.57.5	MSD requirements

Table 6.57.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_7A-12A_n78A and CA_n41-n71-n78 are reused.

Table 6.57.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n12A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 30.8 | IMD24 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 7 | 2505 | 5 | 25 | 2625 | N/A | N/A |
|  | n12 | 673 | 5 | 25 | 732 | N/A | N/A |
|  | n78 | 3664 | 10 | 50 | 3664 | 10.3 | IMD4 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.58	DC_12_n41-n66

6.58.1	Operating bands for DC

Table 6.58.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n41-n66 | 12 | 699 MHz | – | 716MHz | 729 MHz | – | 746 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110MHz | – | 2200 MHz | FDD |

Table 6.58.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A_n41A-n66A | DC_12A_n41A DC_12A_n66A |

6.58.2	Channel bandwidths per operating band for DC

Table 6.58.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n41A-n66A |  |  | 12 |  | 15 | 51 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n41 |  | 15 | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n66 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.58.3	Co-existence studies

Based on co-existence studies of DC_12_n66 and DC_12_n41, own Rx impact of the 3rd band is the followings.

-    2nd order IMD products generated by DC_12_n66 uplink may fall into own Rx of band n41.

6.58.4	∆TIB and ∆RIB values

For DC_12_n41-n66, the TIB,c and RIB,c values are reused from DC_12-66_n41 and  are given in the tables below.

Table 6.58.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n41-n66 | 0.6 | 0.81 / 1.32 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.58.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n41-n66 | 0.5 | 0.51 / 12 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.58.5	MSD requirements

Table 6.58.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5-n41-n66 are reused.

Table 6.58.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n41A-n66A | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |
|  | n41 | 2501 | 10 | 50 | 2501 | 20.0 | IMD218 |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |
| NOTE 18: In the MSD test configuration, the IMD center does not fall into the DL victim Fc |  |  |  |  |  |  |  |

6.59		DC_5_n28-n78

6.59.1	 Operating bands for DC

Table 6.59.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n28-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.59.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A_n28A-n78A DC_5A_n28A-n78C | DC_5A_n78A |

6.59.2 	Channel bandwidths per operating band for DC

Table 6.59.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n28A-n78A |  | 5 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 135 |
|  |  | n28 | 15 | 5 | 10 | 15 | 207 | 257 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 207 | 257 |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.59.3	 Co-existence studies

Based on co-existence studies of DC_5_n78, own Rx impact of the 3rd band is the following.

4th IMD products caused by UL DC_5_n78 falls into the third band n28.

6.59.4	 ∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are specified as below.

Table 6.59.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n28-n78 | 0.6 | 0.5 | 0.9 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.59.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n28-n78 | 0.2 | 0.2 | 0.8 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.59.5	 MSD requirements

Referring to CA_n5-n14-n77 and DC_18-28_n78, the MSD requirements can be specified as below:

Table 6.59.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n28A-n78A DC_5A_n28A-n78C | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | 11.6 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n28 | 707 | 5 | 25 | 762 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3658 | 4.0 | IMD5 |

6.60	DC_2_n12-n78

6.60.1	Operating bands for DC

Table 6.60.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n12-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.60.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A_n12A-n78A | DC_2A_n12A DC_2A_n78A |

6.60.2	Channel bandwidths per operating band for DC

Table 6.60.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A_n12A-n78A |  |  | 2 |  | 15 | 5 | 10 | 151 | 201 |  |  |  |  |  |  |  |  |  |  |  | 135 |
|  |  |  | n12 |  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.60.3	Co-existence studies

Based on co-existence studies of DC_2_n78 and DC_2_n12, own Rx impact of the 3rd band is the followings.

-   3rd order IMD products generated by DC_2_n12 uplink may fall into own Rx of band n78.

6.60.4	∆TIB and ∆RIB values

For DC_2_n12-n78, the TIB,c and RIB,c values are reused from DC_12_n2-n78 and  are given in the tables below.

Table 6.60.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2_n12-n78 | 0.6 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.60.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2_n12-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.60.5	MSD requirements

Table 6.60.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_12A_n2A-n78A are reused.

Table 6.60.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n12A-n78A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n78 | 3315 | 10 | 50 | 3315 | 16.0 | IMD3 |

6.61	DC_5_n41-n66

6.61.1	Operating bands for DC

Table 6.61.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n41-n66 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110MHz | – | 2200 MHz | FDD |

Table 6.61.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n41A-n66A | DC_5A_n41A DC_5A_n66A |

6.61.2	Channel bandwidths per operating band for DC

Table 6.61.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n41A-n66A |  |  | 5 |  | 15 | 5 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n41 |  | 15 | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n66 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.61.3	Co-existence studies

Based on co-existence studies of DC_5_n66 and DC_5_n41, own Rx impact of the 3rd band is the followings.

-    2nd and 3rd order IMD products generated by DC_5_n66 uplink may fall into own Rx of band n41.

6.61.4	∆TIB and ∆RIB values

For DC_5_n41-n66, the TIB,c and RIB,c values are reused from CA_n5-n41-n66 and  are given in the tables below.

Table 6.61.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n41-n66 | 0.6 | 0.81 / 1.32 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |

Table 6.61.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n41-n66 | 0.2 | 0.51 / 12 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |

6.61.5	MSD requirements

Table 6.61.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5A-n41A-n66A and DC_7A-13A_n66A are reused.

Table 6.61.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n41A-n66A | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n41 | 2624 | 10 | 50 | 2624 | 29.0 | IMD2 |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | 18 | IMD3 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | N/A |

6.62	DC_66_n41-n66

6.62.1	Operating bands for DC

Table 6.62.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n41-n66 | 66 | 1710 MHz | – | 1780 MHz | 2110MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110MHz | – | 2200 MHz | FDD |

Table 6.62.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_66A_n41A-n66A | DC_66A_n41A DC_66A_n66A2 |
| NOTE 2: Only single switched UL is supported. |  |

6.62.2	Channel bandwidths per operating band for DC

Table 6.62.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n41A-n66A |  |  | 66 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | n41 |  | 15 | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | n66 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.62.3	Co-existence studies

Based on co-existence studies of DC_66_n66 and DC_66_n41, there is no own Rx impact of the 3rd band for DC_66_n41-n66.

6.62.4	∆TIB and ∆RIB values

For DC_66_n41-n66, the TIB,c and RIB,c values are reused from DC_7-66_n66 and  are given in the tables below.

Table 6.62.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_66_n41-n66 | 0.5 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.62.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_66_n41-n66 | 0.5 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.62.5	MSD requirements

Based on the co-existence study in section 6.62.3, there is no need to define MSD requirements.

6.63	 DC_7_n75-n78

6.63.1	Operating bands for DC

Table 6.63.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n75-n78 | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n75 |  | N/A |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300MHz | – | 3800MHz | 3300MHz | – | 3800MHz | TDD |

6.63.2	Channel bandwidths per operating band for DC

Table 6.63.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A_n75A-n78A | DC_7A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.63.3	UE co-existence studies

Based on co-existence studies, IMD3 and IMD4 from UL 7_n78 may impact DL n75.

6.63.4	∆TIB and ∆RIB values

For DC_7_n75-n78, the TIB,c and RIB,c values can reuse the values of DC_7_32-n78 as given in the tables below.

Table 6.63.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n75-n78 | 0.5 | - | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.63.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n75-n78 | - | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.63.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_7_32-n78.

Table 6.63.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n75A-n78A | n78 | 3560.5 | 10 | 50 | 3560.5 | N/A | N/A |
|  | 7 | 2517.5 | 5 | 25 | 2637.5 | N/A | N/A |
|  | n75 | N/A | 5 | N/A | 1474.5 | 17.6 | IMD3 |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n75 | N/A | 5 | N/A | 1492 | 4.9 | IMD4 |

6.64	DC_1_n40-n105

6.64.1	Operating bands for DC

Table 6.64.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n40-n105 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.64.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n40A-n105A | DC_1A_n40A DC_1A_n105A |

6.64.2	Channel bandwidths per operating band for DC

Table 6.64.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n40A-n105A |  | 1 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n40 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n105 |  | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.64.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.64.3-1 for DC_1A_n40A into band n105 downlink.

Table 6.64.3-1: IMD analysis for DC_1A_n40A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 2300 | 2400 |
| DL Frequency [MHz] | 2110 | 2170 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 | 480 | 4220 | 4380 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1660 | 2620 | 2880 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 960 | 640 | 8440 | 8760 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

The analysis shows the 4th order IMD product falls inside band n105 downlink

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.64.3-2 for DC_1A_n105A into band n40 downlink.

Table 6.64.3-2: IMD analysis for DC_1A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 663 | 703 |
| DL Frequency [MHz] | 2110 | 2170 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 | 1217 | 2583 | 2683 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3137 | 3297 | 654 | 514 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 | 4663 | 3246 | 3386 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5057 | 5277 | 9 | 189 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2434 | 2634 | 5166 | 5366 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 | 6643 | 3909 | 4089 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 892 | 672 | 7257 | 6977 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1731 | 1971 | 4614 | 4354 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 | 4792 | 8343 | 8623 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 | 6069 | 7086 | 7346 |

The analysis shows there is no IMD issue for this combination.

Based on Table 6.64.3-1 and Table 6.64.3-2:

- The 4th order IMD product of DC_1_n40 falls inside band n105 downlink

6.64.4		∆TIB and ∆RIB values

For DC_1_n40-n105, the TIB,c and RIB,c values are reused from CA_n1-n40-n105 and are given in the tables below.

Table 6.64.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_1_n40-n105 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.64.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_1_n40-n105 | - | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.64.5	REFSENS requirements

MSD values are reused from CA_n1A-n40A-n105A.

Table 6.64.5-1: : MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n40-n105 | 1 | 1977 | 5 | 25 | 2167 | N/A | N/A |
|  | n40 | 2305 | 10 | 50 | 2305 | N/A | N/A |
|  | n105 | 700 | 5 | 25 | 649 | 1 | IMD4 |

6.65	DC_7_n40-n105

6.65.1	Operating bands for DC

Table 6.65.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n40-n105 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.65.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n40A-n105A | DC_7A_n40A DC_7A_n105A |

6.65.2	Channel bandwidths per operating band for DC

Table 6.65.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n40A-n105A |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n40 | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n105 | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.65.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.65.3-1 for DC_7A_n40A into band n105 downlink.

Table 6.65.3-1: IMD analysis for DC_7A_n40A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 2300 | 2400 |
| DL Frequency [MHz] | 2620 | 2690 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 100 | 4800 | 4970 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2600 | 2840 | 2030 | 2300 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7300 | 7540 | 7100 | 7370 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5100 | 5410 | 4330 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 200 | 540 | 9600 | 9940 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9800 | 10110 | 9400 | 9770 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7100 | 6630 | 7980 | 7600 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2200 | 1760 | 3110 | 2700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11700 | 12170 | 12300 | 12680 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12340 | 12100 | 12510 |

The analysis shows there are no IMD products falling inside band n105 downlink.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.65.3-2 for DC_7A_n105A into band n40 downlink.

Table 6.65.3-2: IMD analysis for DC_7A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 663 | 703 |
| DL Frequency [MHz] | 2620 | 2690 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1797 | 3163 | 3273 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4297 | 4477 | 1244 | 1094 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5843 | 3826 | 3976 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6797 | 7047 | 581 | 391 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3594 | 3814 | 6326 | 6546 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8413 | 4489 | 4679 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 312 | 82 | 9617 | 9297 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 3151 | 6384 | 6094 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5382 | 10663 | 10983 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7249 | 8826 | 9116 |

The analysis shows there are no IMD products falling inside band n105 downlink.

Based on Table 6.65.3-1 and Table 6.65.3-2 there are no sensitivity requirements to be captured for MSD.

6.65.4		∆TIB and ∆RIB values

For DC_7_n40-n105, the TIB,c and RIB,c values are reused from CA_n7-n40-n105 from same meeting and are given in the tables below.

Table 6.65.4-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n40-n105 | 0.5 | 0.6 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.65.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n40-n105 | - | 0.5 | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.65.5	REFSENS requirements

No sensitivity requirements are required to be captured based on co-existence study.

6.66	DC_3_n40-n105

6.66.1	Operating bands for DC

Table 6.66.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n40-n105 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.66.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n40A-n105A | DC_3A_n40A DC_3A_n105A |

6.66.2	Channel bandwidths per operating band for DC

Table 6.66.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n40A-n105A |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n40 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n105 |  | 15 | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.66.3			Co-existence studies

This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.66.3-1 for DC_3A_n40A into band n105 downlink.

Table 6.66.3-1: IMD analysis for DC_3A_n40A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 2300 | 2400 |
| DL Frequency [MHz] | 1805 | 1880 | 2300 | 2400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 515 | 690 | 4010 | 4185 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1020 | 1270 | 2815 | 3090 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5720 | 5970 | 6310 | 6585 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2730 | 3055 | 5115 | 5490 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1380 | 1030 | 8020 | 8370 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7430 | 7755 | 8610 | 8985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7890 | 7415 | 4840 | 4440 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3780 | 3330 | 755 | 330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10910 | 11385 | 9140 | 9540 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10320 | 10770 | 9730 | 10155 |

The analysis shows the 2nd and 5th order IMD products fall inside band n105 downlink.

A study of the uplink combination affecting the downlink of the third band is shown in Table 6.66.3-2 for DC_3A_n105A into band n40 downlink.

Table 6.66.3-2: IMD analysis for DC_3A_n105A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 663 | 703 |
| DL Frequency [MHz] | 1805 | 1880 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1007 | 2373 | 2488 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2717 | 2907 | 459 | 304 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4273 | 3036 | 3191 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4427 | 4692 | 204 | 399 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2014 | 2244 | 4746 | 4976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6058 | 3699 | 3894 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1102 | 867 | 6477 | 6137 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1311 | 1581 | 4029 | 3724 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4597 | 7503 | 7843 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5679 | 6456 | 6761 |

The analysis shows the 2nd order IMD product falls inside band n105 downlink.

Based on Table 6.66.3-1 and Table 6.66.3-2:

- The 2nd and 5th order IMD products of DC_3_n40 falls inside band n105 downlink.

- The 2nd order IMD product of DC_3_n105 fall inside band n40 downlink.

6.66.4		∆TIB and ∆RIB values

For DC_3_n40-n105, the TIB,c and RIB,c values are reused from CA_n3-n40-n105 and are given in the tables below.

Table 6.66.4-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n40-n105 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.66.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n40-n105 | - | - | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.66.5	REFSENS requirements

MSD values are reused from CA_n3A-n40A-n105A. Since there’s no IMD5 defined for this CA a new value has been found for the DC configuration. The IMD5 is therefore taken from CA_n29-n30-n66, which shares same UL configuration to produce an IMD5 product in low band.

6.67		DC_8_n1-n79

6.67.1	 Operating bands for DC

Table 6.67.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n1A-n79A | 8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.67.2	 configuration for DC

Table 6.32.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n1A-n79A | DC_8A_n79A |

6.67.3	 Co-existence studies

For DC_8_n1-n79, the harmonic and IMD issues are reused from DC_1-8_n79 referring to 5.1.16.3 in TR37.716-21-11[6].

When Uplink EN-DC configuration is DC_8A_n79A, IMD4 of (3*B8-n79) will fall into Rx band of n1.

6.67.4	 ∆TIB and ∆RIB values

For DC_8_n1-n79, the ΔTIB and ΔRIB values are given tables below.

Table 6.67.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8_n1-n79 | 0.3 | 0.3 | 0 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.67.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8_n1-n79 | 0 | 0 | 0 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.67.5	MSD

For DC_8_n1-n79, the MSD value is reused from DC_1-8_n79 referring to 5.1.16.3 in TR37.716-21-11[6].

Table 6.67.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n1A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n1 | 1955 | 5 | 25 | 2145 | 8.2 | IMD4 |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |

6.68	DC_5_n1-n28

6.68.1	Operating bands for DC

Table 6.68.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n1-n28 | 5 | 824 MHz | – | 849MHz | 869 MHz | – | 894MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

Table 6.68.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n1A-n28A | DC_5A_n1A DC_5A_n28A |

6.68.2	Channel bandwidths per operating band for DC

Table 6.68.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n1A-n28A |  |  | 5 |  | 15 |  |  | 5 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 |
|  |  |  | n1 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n28 |  | 15 |  | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.68.3	Co-existence studies

Based on co-existence studies of DC_5_n28 and DC_5_n1, own Rx impact of the 3rd band is the followings.

-    5th order IMD products generated by DC_5_n28 uplink may fall into own Rx of band n1.

6.68.4	∆TIB and ∆RIB values

For DC_5_n1-n28, the TIB,c and RIB,c values are reused from CA_n1-n18-n28 and  are given in the tables below.

Table 6.68.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n1-n28 | 0.5 | 0.3 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.68.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n1-n28 | - | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.68.5	MSD requirements

Table 6.68.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n1-n18-n28 are reused.

Table 6.68.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n1A-n28A | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n1 | N/A | 5 | N/A | 2123 | 4 | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |

6.69	DC_5_n3-n28

6.69.1	Operating bands for DC

Table 6.69.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n3-n28 | 5 | 824 MHz | – | 849MHz | 869 MHz | – | 894MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

Table 6.69.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n3A-n28A | DC_5A_n3A DC_5A_n28A |

6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n3A-n28A |  |  | 5 |  | 15 |  |  | 5 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 |
|  |  |  | n3 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n28 |  | 15 |  | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.69.3	Co-existence studies

Based on co-existence studies of DC_5_n28 and DC_5_n3, own Rx impact of the 3rd band is the followings.

-    4th order IMD products generated by DC_5_n28 uplink may fall into own Rx of band n3.

-    4th order IMD products generated by DC_5_n3 uplink may fall into own Rx of band n28.

6.69.4	∆TIB and ∆RIB values

For DC_5_n3-n28, the TIB,c and RIB,c values are reused from DC_3-28_n5 and  are given in the tables below.

Table 6.69.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n3-n28 | 0.5 | 0.3 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.69.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n3-n28 | 0.1 | - | 0.1 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.69.5	MSD requirements

Table 6.69.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for DC_3A-28A_n5A are reused.

Table 6.69.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n3A-n28A | 5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n3 | N/A | 5 | N/A | 1829.5 | 8.7 | IMD4 |
|  | n28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |
|  | 5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n3 | 1713 | 5 | 25 | 1808 | N/A | N/A |
|  | n28 | N/A | 5 | N/A | 768 | 9.4 | IMD4 |

6.70	DC_5_n28-n79

6.70.1	Operating bands for DC

Table 6.70.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n28-n79 | 5 | 824 MHz | – | 849MHz | 869 MHz | – | 894MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

Table 6.70.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A_n28A-n79A | DC_5A_n28A DC_5A_n79A |

6.70.2	Channel bandwidths per operating band for DC

Table 6.70.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A_n28A-n79A |  |  | 5 |  | 15 |  |  | 5 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  |  | n28 |  | 15 |  | 34 | 5 | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 207 | 257 | 307 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n79 |  | 15 |  |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |  |

6.70.3	Co-existence studies

Based on the co-existence studies, there is no own Rx impact of the 3rd band for this combination.

6.70.4	∆TIB and ∆RIB values

For DC_5_n28-n79, the TIB,c and RIB,c values are reused from DC_18-28_n78 and and CA_n5-n28 and  are given in the tables below.

Table 6.70.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_5_n28-n79 | 0.7 | 0.7 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.70.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_5_n28-n79 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.70.5	MSD requirements

Based on studies in 6.70.3 no IMD interference falls into the own 3rd Rx frequency band, hence no need to define any MSD values.

6.71	DC_7_n25-n71

6.71.1	Operating bands for DC

Table 6.71.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n25-n71 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | TDD |

Table 6.71.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n25A-n71A | DC_7A_n25A DC_7A_n71A |

6.71.2	Channel bandwidths per operating band for DC

Table 6.71.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n25A-n71A |  |  | 7 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n71 |  | 15 |  |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |  |

6.71.3	Co-existence studies

For UE coexistence study of UL DC_7_n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.71.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD2 of UL DC_7_n25 may have an impact on DL Band n71.

- The IMD5 of UL DC_7_n25 may have an impact on DL Band n71.

Table 6.71.3-1: The harmonic and IMD products caused by UL DC_7_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2620 | 2690 | 1930 | 1995 |
| 3rd Band DL | 617 | 652 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 720 | 585 | 4350 | 4485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3085 | 3290 | 1130 | 1330 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6850 | 7055 | 6200 | 6400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5585 | 5860 | 2980 | 3245 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9350 | 9625 | 8050 | 8315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1170 | 1440 | 8700 | 8970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5160 | 4830 | 8430 | 8085 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 745 | 410 | 4010 | 3670 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9900 | 10230 | 11850 | 12195 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10550 | 10885 | 11200 | 11540 |

For UE coexistence study of UL DC_7_n71, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.71.3-2.

Based on the calculation, there is no impact on the 3rd RX band.

Table 6.71.3-2: The harmonic and IMD products caused by UL DC_7_n71

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2620 | 2690 | 617 | 652 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 1989 | 2094 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 2652 | 2792 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 3315 | 3490 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1802 | 3163 | 3268 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4302 | 4477 | 1244 | 1104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5838 | 3826 | 3966 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6802 | 7047 | 581 | 406 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8408 | 4489 | 4664 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3604 | 3814 | 6326 | 6536 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 292 | 82 | 9617 | 9302 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2906 | 3151 | 6384 | 6104 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5362 | 10663 | 10978 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7234 | 8826 | 9106 |

6.71.4	∆TIB and ∆RIB values

For DC_7_n25-n71, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.71.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n25-n71 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.71.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n25-n71 | - | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.71.5	MSD requirements

The MSD values for IMD2 of UL DC_7_n25 impact on DL Band n71 can reuse that of DC_7A_n2A-n71A. Note that the MSD test points of DC_7A_n2A-n71A has an error and a correct CR （R4-2400791）has been submitted this meeting.

The MSD requirements can be specified as follows:

Table 6.71.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n25A-n71A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 28.7 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n25 | 1910 | 5 | 25 | 1990 | N/A | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 5 | IMD5 |

6.72	DC_12_n25-n77

6.72.1	Operating bands for DC

Table 6.72.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n25-n77 | 12 | 699MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.72.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A_n25A-n77A | DC_12A_n25A DC_12A_n77A |

6.72.2	Channel bandwidths per operating band for DC

Table 6.72.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n25A-n71A |  |  | 12 |  | 15 |  |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.72.3	Co-existence studies

For UE coexistence study of Band 12 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.72.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD3, IMD4 and IMD5 interference of UL DC_12_n25 may have an impact on DL Band n77.

Table 6.72.3-1: The harmonic and IMD products caused by UL DC_12_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1995 |
| 3rd Band DL | 3300 | 4200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1216 | 2549 | 2631 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 517 | 418 | 2984 | 3131 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3248 | 3347 | 4399 | 4546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 182 | 298 | 4834 | 5046 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3947 | 4063 | 6249 | 6461 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2432 | 2268 | 5098 | 5262 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6961 | 6684 | 1014 | 881 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4347 | 4118 | 1552 | 1733 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8376 | 4646 | 4779 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7177 | 5797 | 5978 |

For UE coexistence study of Band 12 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.72.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD3 and IMD4 interference of UL DC_12_n77 may have an impact on DL Band n25.

Table 6.72.3-2: The harmonic and IMD products caused by UL DC_12_n77

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 3300 | 4200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2584 | 3501 | 3999 | 4916 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2802 | 1868 | 5884 | 7701 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7002 | 5168 | 7998 | 9832 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16101 | 12484 | 436 | 1404 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11202 | 8468 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

6.72.4	∆TIB and ∆RIB values

For DC_12_n25-n77, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.72.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n25-n77 | 0.5 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.72.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n25-n77 | 0.2 | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.72.5	MSD requirements

The MSD values for IMD interference are obtained as follows.

The MSD value for the IMD3 and IMD4 interference of UL DC_12_n25 on DL Band n77 is reused from DC_12A_n2A-n77A.

Table 6.72.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n25A-n77A | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 16.5 | IMD3 |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1970 | 12.5 | IMD4 |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3315 | 16.0 | IMD3 |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 4000 | 12 | IMD4 |

6.73	DC_71_n25-n77

6.73.1	Operating bands for DC

Table 6.73.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n25-n77 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300MHz | – | 4200 MHz | TDD |

Table 6.73.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n25A-n77A | DC_71A_n25A DC_71A_n77A |

6.73.2	Channel bandwidths per operating band for DC

Table 6.73.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n25A-n77A |  |  | 71 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.73.3	Co-existence studies

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.19.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD3，IMD4 and IMD5 interference of UL DC_71_n25 may have an impact on DL Band n77.

Table 6.73.3-1: The harmonic and IMD products caused by UL DC_71_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 1930 | 1995 |
| 3rd Band DL | 3300 | 4200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1152 | 1252 | 2513 | 2613 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.19.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD3 and IMD4 interference of UL DC_71_n77 may have an impact on DL Band n25.

Table 6.73.3-2: The harmonic and IMD products caused by UL DC_71_n77

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 3300 | 4200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 | 3537 | 3963 | 4898 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 | 1904 | 5902 | 7737 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 | 5596 | 7263 | 9098 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 | 1206 | 9202 | 11937 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 | 6294 | 10563 | 13298 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7074 | 5204 | 7926 | 9796 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 | 12502 | 508 | 1548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 | 8504 | 4506 | 6411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 | 17498 | 5952 | 6992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 | 13996 | 8589 | 10494 |

6.73.4	∆TIB and ∆RIB values

For DC_71_n25-n77, the ΔTIB,c and ΔRIB,c values are reused from CA_n25-n71-n77 and  are given in the tables below.

Table 6.73.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n25-n77 | 0.5 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.73.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n25-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.73.5	MSD requirements

The IMD3 and IMD4 interference of UL DC_71_n25 on DL Band n77 is reused from DC_12A_n2A-n77A with adjustment of carrier frequency.

The IMD3 and IMD4 interference of UL DC_71_n77 on DL Band n25 is reused from DC_12A_n2A-n77A with adjustment of carrier frequency.

Table 6.73.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_71A_n25A-n77A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1954 | 16.5 | IMD3 |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | 71 | 666 | 5 | 25 | 620 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1982 | 12.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3298.5 | 16 | IMD34 |
|  | 71 | 666 | 5 | 25 | 620 | N/A | N/A |
|  | n25 | 1890 | 5 | 25 | 1970 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3888 | 12 | IMD4 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.74	DC_12_n25-n41

6.74.1	Operating bands for DC

Table 6.74.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n25-n41 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.74.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A_n25A-n41A | DC_12A_n25A DC_12A_n41A |

6.74.2	Channel bandwidths per operating band for DC

Table 6.74.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n25A-n41A |  |  | 12 |  | 15 |  |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 15 |  |  | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.74.3	Co-existence studies

For UE coexistence study of Band 12+ Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.74.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD2 interference of UL DC_12_n25 may have an impact on DL Band n41.

Table 6.74.3-1: The harmonic and IMD products caused by UL DC_12_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1995 |
| 3rd Band DL | 2496 | 2690 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1216 | 2549 | 2631 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 517 | 418 | 2984 | 3131 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3248 | 3347 | 4399 | 4546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 182 | 298 | 4834 | 5046 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3947 | 4063 | 6249 | 6461 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2432 | 2268 | 5098 | 5262 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6961 | 6684 | 1014 | 881 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4347 | 4118 | 1552 | 1733 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8376 | 4646 | 4779 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7177 | 5797 | 5978 |

For UE coexistence study of Band 12 + Band n41, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.74.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD2 interference of UL DC_12_n41 may have an impact on DL Band n25.

Table 6.74.3-2: The harmonic and IMD products caused by UL DC_12_n41

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2496 | 2690 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1780 | 1991 | 3195 | 3406 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1292 | 1064 | 4276 | 4681 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3894 | 4122 | 5691 | 6096 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 593 | 348 | 6772 | 7371 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4593 | 4838 | 8187 | 8786 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3982 | 3560 | 6390 | 6812 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10061 | 9268 | 368 | 106 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6672 | 6056 | 2844 | 3283 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10683 | 11476 | 5292 | 5554 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8886 | 9502 | 7089 | 7528 |

6.74.4	∆TIB and ∆RIB values

For DC_12_n25-n41, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.74.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n25-n41 | 0.3 | 0.5 | 0.41/0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.74.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n25-n41 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.74.5	MSD requirements

The MSD value for the IMD2 interference of UL DC_12_n25 on DL Band n41 is reused from DC_12A_n2A-n41A.

The MSD value for the IMD2 interference of UL DC_12_n41 on DL Band n25 is given as follows.

Table 6.74.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n25A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 | N/A | 2608 | 28.7 | IMD2 |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1970 | 26 | IMD2 |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | N/A |

6.75	DC_71_n25-n41

6.75.1	Operating bands for DC

Table 6.75.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n25-n41 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.75.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n25A-n41A | DC_71A_n25A DC_71A_n41A |

6.75.2	Channel bandwidths per operating band for DC

Table 6.75.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n25A-n41A |  |  | 71 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 15 |  |  | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.75.3	Co-existence studies

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.75.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD2 and IMD4 interference of UL DC_71_n25 may have an impact on DL Band n41.

Table 6.75.3-1: The harmonic and IMD products caused by UL DC_71_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 1930 | 1995 |
| 3rd Band DL | 2496 | 2690 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1152 | 1252 | 2513 | 2613 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

For UE coexistence study of Band 3 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.75.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD2 interference of UL DC_71_n41 may have an impact on DL Band n25.

Table 6.75.3-2: The harmonic and IMD products caused by UL DC_71_n41

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 2496 | 2690 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1798 | 2027 | 3159 | 3388 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1364 | 1100 | 4294 | 4717 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3822 | 4086 | 5655 | 6078 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 701 | 402 | 6790 | 7407 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4485 | 4784 | 8151 | 8768 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4054 | 3596 | 6318 | 6776 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10097 | 9286 | 296 | 38 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6744 | 6092 | 2898 | 3391 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10647 | 11458 | 5148 | 5482 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8814 | 9466 | 6981 | 7474 |

6.75.4	∆TIB and ∆RIB values

For DC_71_n25-n41 ΔTIB,c and ΔRIB,c values are as follows.

Table 6.75.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n25-n41 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.75.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n25-n41 | - | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.75.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 71 on DL Band n25 and n41 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

The IMD2 interference of UL DC_71_n25 on DL Band n41 is reused from DC_71A_n2A-n41A.

The IMD2 interference of UL DC_71_n41 on DL Band n25 is reused from DC_71A_n2A-n41A.

Table 6.75.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_71A_n25A-n41A | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 | N/A | 2586 | 29.2 | IMD29 |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1942 | 26 | IMD2 |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.76	DC_12_n25-n66

6.76.1	Operating bands for DC

Table 6.76.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n25-n66 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | TDD |

Table 6.76.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A_n25A-n66A | DC_12A_n25A DC_12A_n66A |

6.76.2	Channel bandwidths per operating band for DC

Table 6.76.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n25A-n66A |  |  | 12 |  | 15 |  |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n66 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.76.3	Co-existence studies

For UE coexistence study of Band 12 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.76.3-1.

Based on the calculation, there is no IMD interference impact on the 3rd  RX band.

Table 6.76.3-1: The harmonic and IMD products caused by UL DC_12_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1995 |
| 3rd Band DL | 2110 | 2200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1216 | 2549 | 2631 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 517 | 418 | 2984 | 3131 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3248 | 3347 | 4399 | 4546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 182 | 298 | 4834 | 5046 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3947 | 4063 | 6249 | 6461 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2432 | 2268 | 5098 | 5262 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6961 | 6684 | 1014 | 881 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4347 | 4118 | 1552 | 1733 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8376 | 4646 | 4779 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7177 | 5797 | 5978 |

For UE coexistence study of Band 12 + Band n66, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.76.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD4 interference of UL DC_12_n66 may have an impact on DL Band n25.

Table 6.76.3-2: The harmonic and IMD products caused by UL DC_12_n66

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2110 | 2200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1081 | 2409 | 2496 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 | 278 | 2704 | 2861 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 | 3928 | 5829 | 6056 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 | 1988 | 4818 | 4992 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 | 6772 | 5517 | 5708 |

6.76.4	∆TIB and ∆RIB values

For DC_12_n25-n66, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.76.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n25-n66 | 0.8 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.76.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n25-n66 | 0.5 | 0.3 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.76.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 12 on DL Band n66 and n25 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

Based on co-existence studies, MSD need to be defined for the IMD4 interference of UL DC_12_n66 on DL Band n25. The MSD requirements can be specified as follows:

Table 6.76.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n25A-n66A | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1992.5 | 6.2 | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |

6.77	DC_71_n25-n66

6.77.1	Operating bands for DC

Table 6.77.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n25-n66 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | TDD |

Table 6.77.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n25A-n66A | DC_71A_n25A DC_71A_n66A |

6.77.2	Channel bandwidths per operating band for DC

Table 6.77.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n25A-n66A |  |  | 71 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n66 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.77.3	Co-existence studies

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.77.3-1.

Based on the calculation, there is no impact on the 3rd RX band.

Table 6.77.3-1: The harmonic and IMD products caused by UL DC_71_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 1930 | 1995 |
| 3rd Band DL | 2110 | 2200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1152 | 1252 | 2513 | 2613 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

For UE coexistence study of Band 71 + Band n66, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.77.3-2.

Based on the calculation, there is no impact on the 3rd RX band.

Table 6.77.3-2: The harmonic and IMD products caused by UL DC_71_n66

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 2110 | 2200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1012 | 1117 | 2373 | 2478 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 454 | 314 | 2722 | 2897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3036 | 3176 | 4083 | 4258 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 209 | 384 | 4432 | 4677 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3699 | 3874 | 5793 | 6038 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2234 | 2024 | 4746 | 4956 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6457 | 6142 | 1082 | 872 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4014 | 3734 | 1326 | 1571 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7503 | 7818 | 4362 | 4572 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6456 | 6736 | 5409 | 5654 |

6.77.4	∆TIB and ∆RIB values

For DC_71_n25-n66, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.77.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n25-n66 | 0.6 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.77.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n25-n66 | 0.3 | 0.3 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.77.5	MSD requirements

No MSD value need to be defined since there is no impact on the 3rd RX band.

6.71	DC_7_n25-n71

6.71.1	Operating bands for DC

Table 6.71.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n25-n71 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | TDD |

Table 6.71.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n25A-n71A | DC_7A_n25A DC_7A_n71A |

6.71.2	Channel bandwidths per operating band for DC

Table 6.71.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n25A-n71A |  |  | 7 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n71 |  | 15 |  |  | 5 | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 2512 | 3012 | 3512 |  |  |  |  |  |  |  |  |  |

6.71.3	Co-existence studies

For UE coexistence study of UL DC_7_n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.71.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD2 of UL DC_7_n25 may have an impact on DL Band n71.

- The IMD5 of UL DC_7_n25 may have an impact on DL Band n71.

Table 6.71.3-1: The harmonic and IMD products caused by UL DC_7_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2620 | 2690 | 1930 | 1995 |
| 3rd Band DL | 617 | 652 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 720 | 585 | 4350 | 4485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3085 | 3290 | 1130 | 1330 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6850 | 7055 | 6200 | 6400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5585 | 5860 | 2980 | 3245 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9350 | 9625 | 8050 | 8315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1170 | 1440 | 8700 | 8970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5160 | 4830 | 8430 | 8085 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 745 | 410 | 4010 | 3670 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9900 | 10230 | 11850 | 12195 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10550 | 10885 | 11200 | 11540 |

For UE coexistence study of UL DC_7_n71, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.71.3-2.

Based on the calculation, there is no impact on the 3rd RX band.

Table 6.71.3-2: The harmonic and IMD products caused by UL DC_7_n71

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2620 | 2690 | 617 | 652 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 1989 | 2094 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 2652 | 2792 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 3315 | 3490 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1802 | 3163 | 3268 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4302 | 4477 | 1244 | 1104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5838 | 3826 | 3966 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6802 | 7047 | 581 | 406 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8408 | 4489 | 4664 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3604 | 3814 | 6326 | 6536 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 292 | 82 | 9617 | 9302 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2906 | 3151 | 6384 | 6104 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5362 | 10663 | 10978 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7234 | 8826 | 9106 |

6.71.4	∆TIB and ∆RIB values

For DC_7_n25-n71, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.71.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n25-n71 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.71.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n25-n71 | - | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.71.5	MSD requirements

The MSD values for IMD2 of UL DC_7_n25 impact on DL Band n71 can reuse that of DC_7A_n2A-n71A. Note that the MSD test points of DC_7A_n2A-n71A has an error and a correct CR （R4-2400791）has been submitted this meeting.

The MSD requirements can be specified as follows:

Table 6.71.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n25A-n71A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 28.7 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n25 | 1910 | 5 | 25 | 1990 | N/A | N/A |
|  | n71 | N/A | 5 | N/A | 630 | 5 | IMD5 |

6.72	DC_12_n25-n77

6.72.1	Operating bands for DC

Table 6.72.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n25-n77 | 12 | 699MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.72.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A_n25A-n77A | DC_12A_n25A DC_12A_n77A |

6.72.2	Channel bandwidths per operating band for DC

Table 6.72.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n25A-n71A |  |  | 12 |  | 15 |  |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.72.3	Co-existence studies

For UE coexistence study of Band 12 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.72.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD3, IMD4 and IMD5 interference of UL DC_12_n25 may have an impact on DL Band n77.

Table 6.72.3-1: The harmonic and IMD products caused by UL DC_12_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1995 |
| 3rd Band DL | 3300 | 4200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1216 | 2549 | 2631 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 517 | 418 | 2984 | 3131 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3248 | 3347 | 4399 | 4546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 182 | 298 | 4834 | 5046 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3947 | 4063 | 6249 | 6461 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2432 | 2268 | 5098 | 5262 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6961 | 6684 | 1014 | 881 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4347 | 4118 | 1552 | 1733 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8376 | 4646 | 4779 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7177 | 5797 | 5978 |

For UE coexistence study of Band 12 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.72.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD3 and IMD4 interference of UL DC_12_n77 may have an impact on DL Band n25.

Table 6.72.3-2: The harmonic and IMD products caused by UL DC_12_n77

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 3300 | 4200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2584 | 3501 | 3999 | 4916 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2802 | 1868 | 5884 | 7701 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4698 | 5632 | 7299 | 9116 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2103 | 1152 | 9184 | 11901 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5397 | 6348 | 10599 | 13316 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7002 | 5168 | 7998 | 9832 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16101 | 12484 | 436 | 1404 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11202 | 8468 | 4452 | 6303 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13899 | 17516 | 6096 | 7064 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11298 | 14032 | 8697 | 10548 |

6.72.4	∆TIB and ∆RIB values

For DC_12_n25-n77, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.72.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n25-n77 | 0.5 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.72.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n25-n77 | 0.2 | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.72.5	MSD requirements

The MSD values for IMD interference are obtained as follows.

The MSD value for the IMD3 and IMD4 interference of UL DC_12_n25 on DL Band n77 is reused from DC_12A_n2A-n77A.

Table 6.72.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n25A-n77A | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 16.5 | IMD3 |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | N/A |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1970 | 12.5 | IMD4 |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3315 | 16.0 | IMD3 |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 4000 | 12 | IMD4 |

6.73	DC_71_n25-n77

6.73.1	Operating bands for DC

Table 6.73.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n25-n77 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300MHz | – | 4200 MHz | TDD |

Table 6.73.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n25A-n77A | DC_71A_n25A DC_71A_n77A |

6.73.2	Channel bandwidths per operating band for DC

Table 6.73.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n25A-n77A |  |  | 71 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.73.3	Co-existence studies

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.19.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD3，IMD4 and IMD5 interference of UL DC_71_n25 may have an impact on DL Band n77.

Table 6.73.3-1: The harmonic and IMD products caused by UL DC_71_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 1930 | 1995 |
| 3rd Band DL | 3300 | 4200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1152 | 1252 | 2513 | 2613 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.19.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD3 and IMD4 interference of UL DC_71_n77 may have an impact on DL Band n25.

Table 6.73.3-2: The harmonic and IMD products caused by UL DC_71_n77

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 3300 | 4200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 | 3537 | 3963 | 4898 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 | 1904 | 5902 | 7737 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 | 5596 | 7263 | 9098 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2211 | 1206 | 9202 | 11937 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 | 6294 | 10563 | 13298 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7074 | 5204 | 7926 | 9796 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 | 12502 | 508 | 1548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 | 8504 | 4506 | 6411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 | 17498 | 5952 | 6992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 | 13996 | 8589 | 10494 |

6.73.4	∆TIB and ∆RIB values

For DC_71_n25-n77, the ΔTIB,c and ΔRIB,c values are reused from CA_n25-n71-n77 and  are given in the tables below.

Table 6.73.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n25-n77 | 0.5 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.73.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n25-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.73.5	MSD requirements

The IMD3 and IMD4 interference of UL DC_71_n25 on DL Band n77 is reused from DC_12A_n2A-n77A with adjustment of carrier frequency.

The IMD3 and IMD4 interference of UL DC_71_n77 on DL Band n25 is reused from DC_12A_n2A-n77A with adjustment of carrier frequency.

Table 6.73.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_71A_n25A-n77A | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1954 | 16.5 | IMD3 |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | 71 | 666 | 5 | 25 | 620 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1982 | 12.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 71 | 695.5 | 5 | 25 | 649.5 | N/A | N/A |
|  | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3298.5 | 16 | IMD34 |
|  | 71 | 666 | 5 | 25 | 620 | N/A | N/A |
|  | n25 | 1890 | 5 | 25 | 1970 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3888 | 12 | IMD4 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.74	DC_12_n25-n41

6.74.1	Operating bands for DC

Table 6.74.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n25-n41 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.74.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A_n25A-n41A | DC_12A_n25A DC_12A_n41A |

6.74.2	Channel bandwidths per operating band for DC

Table 6.74.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n25A-n41A |  |  | 12 |  | 15 |  |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 15 |  |  | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.74.3	Co-existence studies

For UE coexistence study of Band 12+ Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.74.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD2 interference of UL DC_12_n25 may have an impact on DL Band n41.

Table 6.74.3-1: The harmonic and IMD products caused by UL DC_12_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1995 |
| 3rd Band DL | 2496 | 2690 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1216 | 2549 | 2631 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 517 | 418 | 2984 | 3131 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3248 | 3347 | 4399 | 4546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 182 | 298 | 4834 | 5046 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3947 | 4063 | 6249 | 6461 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2432 | 2268 | 5098 | 5262 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6961 | 6684 | 1014 | 881 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4347 | 4118 | 1552 | 1733 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8376 | 4646 | 4779 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7177 | 5797 | 5978 |

For UE coexistence study of Band 12 + Band n41, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.74.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD2 interference of UL DC_12_n41 may have an impact on DL Band n25.

Table 6.74.3-2: The harmonic and IMD products caused by UL DC_12_n41

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2496 | 2690 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1780 | 1991 | 3195 | 3406 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1292 | 1064 | 4276 | 4681 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3894 | 4122 | 5691 | 6096 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 593 | 348 | 6772 | 7371 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4593 | 4838 | 8187 | 8786 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3982 | 3560 | 6390 | 6812 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10061 | 9268 | 368 | 106 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6672 | 6056 | 2844 | 3283 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10683 | 11476 | 5292 | 5554 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8886 | 9502 | 7089 | 7528 |

6.74.4	∆TIB and ∆RIB values

For DC_12_n25-n41, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.74.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n25-n41 | 0.3 | 0.5 | 0.41/0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.74.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n25-n41 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.74.5	MSD requirements

The MSD value for the IMD2 interference of UL DC_12_n25 on DL Band n41 is reused from DC_12A_n2A-n41A.

The MSD value for the IMD2 interference of UL DC_12_n41 on DL Band n25 is given as follows.

Table 6.74.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n25A-n41A | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 | N/A | 2608 | 28.7 | IMD2 |
|  | 12 | 710 | 5 | 25 | 740 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1970 | 26 | IMD2 |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | N/A |

6.75	DC_71_n25-n41

6.75.1	Operating bands for DC

Table 6.75.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n25-n41 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.75.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n25A-n41A | DC_71A_n25A DC_71A_n41A |

6.75.2	Channel bandwidths per operating band for DC

Table 6.75.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n25A-n41A |  |  | 71 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n41 |  | 15 |  |  | 54,11 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.75.3	Co-existence studies

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.75.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD2 and IMD4 interference of UL DC_71_n25 may have an impact on DL Band n41.

Table 6.75.3-1: The harmonic and IMD products caused by UL DC_71_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 1930 | 1995 |
| 3rd Band DL | 2496 | 2690 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1152 | 1252 | 2513 | 2613 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

For UE coexistence study of Band 3 + Band n77, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.75.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD2 interference of UL DC_71_n41 may have an impact on DL Band n25.

Table 6.75.3-2: The harmonic and IMD products caused by UL DC_71_n41

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 2496 | 2690 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1798 | 2027 | 3159 | 3388 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1364 | 1100 | 4294 | 4717 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3822 | 4086 | 5655 | 6078 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 701 | 402 | 6790 | 7407 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4485 | 4784 | 8151 | 8768 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4054 | 3596 | 6318 | 6776 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10097 | 9286 | 296 | 38 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6744 | 6092 | 2898 | 3391 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10647 | 11458 | 5148 | 5482 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8814 | 9466 | 6981 | 7474 |

6.75.4	∆TIB and ∆RIB values

For DC_71_n25-n41 ΔTIB,c and ΔRIB,c values are as follows.

Table 6.75.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n25-n41 | 0.5 | 0.5 | 0.6 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.75.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n25-n41 | - | - | 0.2 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.75.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 71 on DL Band n25 and n41 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

The IMD2 interference of UL DC_71_n25 on DL Band n41 is reused from DC_71A_n2A-n41A.

The IMD2 interference of UL DC_71_n41 on DL Band n25 is reused from DC_71A_n2A-n41A.

Table 6.75.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_71A_n25A-n41A | n25 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | N/A | 5 | N/A | 2586 | 29.2 | IMD29 |
|  | 71 | 686 | 5 | 50 | 640 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1942 | 26 | IMD2 |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.76	DC_12_n25-n66

6.76.1	Operating bands for DC

Table 6.76.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n25-n66 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | TDD |

Table 6.76.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A_n25A-n66A | DC_12A_n25A DC_12A_n66A |

6.76.2	Channel bandwidths per operating band for DC

Table 6.76.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_12A_n25A-n66A |  |  | 12 |  | 15 |  |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n66 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.76.3	Co-existence studies

For UE coexistence study of Band 12 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.76.3-1.

Based on the calculation, there is no IMD interference impact on the 3rd  RX band.

Table 6.76.3-1: The harmonic and IMD products caused by UL DC_12_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 1930 | 1995 |
| 3rd Band DL | 2110 | 2200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1134 | 1216 | 2549 | 2631 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 517 | 418 | 2984 | 3131 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3248 | 3347 | 4399 | 4546 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 182 | 298 | 4834 | 5046 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3947 | 4063 | 6249 | 6461 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2432 | 2268 | 5098 | 5262 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6961 | 6684 | 1014 | 881 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4347 | 4118 | 1552 | 1733 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8376 | 4646 | 4779 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7177 | 5797 | 5978 |

For UE coexistence study of Band 12 + Band n66, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.76.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD4 interference of UL DC_12_n66 may have an impact on DL Band n25.

Table 6.76.3-2: The harmonic and IMD products caused by UL DC_12_n66

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2110 | 2200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1081 | 2409 | 2496 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 | 278 | 2704 | 2861 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 | 3928 | 5829 | 6056 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 | 1988 | 4818 | 4992 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 | 6772 | 5517 | 5708 |

6.76.4	∆TIB and ∆RIB values

For DC_12_n25-n66, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.76.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12_n25-n66 | 0.8 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.76.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12_n25-n66 | 0.5 | 0.3 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.76.5	MSD requirements

The MSD values for UL harmonic interference of UL Band 12 on DL Band n66 and n25 are already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

Based on co-existence studies, MSD need to be defined for the IMD4 interference of UL DC_12_n66 on DL Band n25. The MSD requirements can be specified as follows:

Table 6.76.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n25A-n66A | 12 | 713.5 | 5 | 25 | 743.5 | N/A | N/A |
|  | n25 | N/A | 5 | N/A | 1992.5 | 6.2 | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |

6.77	DC_71_n25-n66

6.77.1	Operating bands for DC

Table 6.77.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n25-n66 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | TDD |

Table 6.77.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_71A_n25A-n66A | DC_71A_n25A DC_71A_n66A |

6.77.2	Channel bandwidths per operating band for DC

Table 6.77.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_71A_n25A-n66A |  |  | 71 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  |  | n25 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 453 |  |  |  |  |  |  |  |
|  |  |  | n66 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |  |  |  |  |  |  |  |

6.77.3	Co-existence studies

For UE coexistence study of Band 71 + Band n25, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.77.3-1.

Based on the calculation, there is no impact on the 3rd RX band.

Table 6.77.3-1: The harmonic and IMD products caused by UL DC_71_n25

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 1930 | 1995 |
| 3rd Band DL | 2110 | 2200 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1152 | 1252 | 2513 | 2613 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

For UE coexistence study of Band 71 + Band n66, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.77.3-2.

Based on the calculation, there is no impact on the 3rd RX band.

Table 6.77.3-2: The harmonic and IMD products caused by UL DC_71_n66

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 617 | 652 | 2110 | 2200 |
| 3rd Band DL | 1930 | 1995 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1012 | 1117 | 2373 | 2478 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 454 | 314 | 2722 | 2897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3036 | 3176 | 4083 | 4258 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 209 | 384 | 4432 | 4677 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3699 | 3874 | 5793 | 6038 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2234 | 2024 | 4746 | 4956 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6457 | 6142 | 1082 | 872 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4014 | 3734 | 1326 | 1571 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7503 | 7818 | 4362 | 4572 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6456 | 6736 | 5409 | 5654 |

6.77.4	∆TIB and ∆RIB values

For DC_71_n25-n66, ΔTIB,c and ΔRIB,c values are as follows.

Table 6.77.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_71_n25-n66 | 0.6 | 0.5 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.77.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_71_n25-n66 | 0.3 | 0.3 | 0.3 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.77.5	MSD requirements

No MSD value need to be defined since there is no impact on the 3rd RX band.

6.78	DC_3(n)_n77

6.78.1	Operating bands for DC

Table 6.78.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| NR and E-UTRA DC Configuration | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3(n)_n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.78.1-2: Inter-band NE-DC configurations within FR1 (three bands)

| NE-DC configuration | Uplink NE-DC configuration |
| --- | --- |
| DC_3(n)AA_n77A | DC_3(n)AA1 DC_n77A_3A |
| NOTE1: Only single switched UL is supported. |  |

6.78.2	Channel bandwidths per operating band for DC

Table 6.78.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3(n)AA_n77A |  |  | 3 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  |  | n3 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | 15 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.78.3	Co-existence studies

Same as DC_(n)3AA-n78A

6.78.4	∆TIB and ∆RIB values

For DC_3(n)_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.78.4-1: ΔTIB,c

| Inter-band NE-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3(n)_n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.78.4-2: ΔRIB

| Inter-band NE-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3(n)_n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.78.5	MSD

The MSD values are taken from DC_(n)3AA-n78A in TS 38.101-3.

Table 6.78.2.2-1: MSD test points for Scell due to dual uplink operation for NE-DC in NR FR1 (three bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NE-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3(n)AA_n77A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD24 |
|  | n3 | N/A | 5 | N/A | 1840 | [28.9] | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.79	DC_3(n)_n8

6.79.1	Operating bands for DC

Table 6.79.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| NR and E-UTRA DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3(n)_n8 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

6.79.2	Channel bandwidths per operating band for DC

Table 6.79.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR and E-UTRA DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_3(n)AA_n8A | DC_3(n)AA1 DC_n8A_3A_ | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.79.3	Co-existence studies

The studies are already performed for DC_3(n)AA and DC_3A_n8A, in TR 37.717-11-11 and TR 37.716-11-11, respectively.

6.79.4	∆TIB and ∆RIB values

Same values as DC_(n)3AA-n8A are used.

6.79.5	MSD

Similar values as for DC_(n)3AA-n8A is proposed.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NE-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3(n)AA_n8A | 3 | N/A | 5 | N/A | 1837.5 | 4.5 | IMD5 |
|  | n3 | 1747.5 | 5 | 25 | 1842.5 | 6.4 | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |

6.80	DC_8_n1-n78

6.80.1	Configurations for DC

Table 6.80.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8B_n1A-n78A5 | DC_8A_n1A DC_8B_n1A DC_8A_n78A DC_8B_n78A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

Note that the DL DC_8B_n1A-n78A with UL DC_8A_n1A and UL DC_8A_n78A configurations are already specified in the Rel.18 specification via draft CR approach.

6.80.2	Co-existence studies

The co-existence study for DL DC_8B_n1A-n78A with UL DC_8B_n1A and UL DC_8B_n78A is focused on the UL triple beat issue on the 3rd band.

Table 6.80.2-1:  BCS for CA_8B in current specification (TS 36.101 V18.4.0)

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_8B | CA_8B | 5,10 | 10 | 20 | 0 |
|  |  | 10 | 5 |  |  |
|  |  | 10 | 3, 5 | 15 | 1 |
|  |  | 3, 5 | 10 |  |  |

Then, with consideration of the minimum and maximum aggregated CBW, 3+3 and 10+10 (MHz) from Table 6.80.2-1, the triple beat IMD products between uplink CA_8B and band n1 or band n78 are analysed in Table 6.80.2-2 and Figure 6.80.2-3 below, based on the guidelines in TS 38.846.

Table 6.80.2-2: Band 8 and Band n1 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 880 | 883 | 900 | 915 |  | 3+3, 10+10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1920 | 1980 | 912 | 895 |  | 0 |
| 1st order TB | IfU3L-fU1L-fSCCL¦ | IfU2L-fU1L-fSCCH¦ | IfU2L-fU1L+fSCCL¦ | IfU3L-fU1L+fSCCH¦ |  | Max ch. separation |
| Ranges | 1900 | 1977 | 1923 | 2000 |  | 20 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L+fU1L+fSCCL¦ | IfU1H+fU2H+fSCCH¦ |  |  |
| Ranges | 217 | 93 | 3683 | 3807 |  |  |

Table 6.80.2-3: Band 8 and Band n78 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 880 | 883 | 900 | 915 |  | 3+3, 10+10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 3300 | 3800 | 912 | 895 |  | 0 |
| 1st order TB | IfU3L-fU1L-fSCCL¦ | IfU2L-fU1L-fSCCH¦ | IfU2L-fU1L+fSCCL¦ | IfU3L-fU1L+fSCCH¦ |  | Max ch. separation |
| Ranges | 3280 | 3797 | 3303 | 3820 |  | 20 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L+fU1L+fSCCL¦ | IfU1H+fU2H+fSCCH¦ |  |  |
| Ranges | 2037 | 1473 | 5063 | 5627 |  |  |

Based on the co-existence studies above, the own Rx impact on the 3rd band is listed below.

- The TB of |fU2L+fU1L+fSCCL| ~ |fU1H+fU2H+fSCCH| from DC_8B_n1A falls into the Rx frequency range of the band n78, however this can be covered by the IM3 of DL DC_8A_n1A-n78A with UL DC_8A_n1A, which MSD is already specified.

- There is no TB from DC_8B_n78A falling into the Rx frequency range of the band n1.

6.80.3	∆TIB and ∆RIB values

For the TIB,c and RIB,c values for  DC_8_n1-n78 are already specified.

6.80.4	Reference sensitivity exceptions

Based on the analysis in the co-existence study, no additional MSD requirements are expected to be specified.

6.81	DC_3_n8-n77

6.81.1	Operating bands for DC

Table 6.81.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n8-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.81.2	Configuration for DC

Table 6.81.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n8A-n77A | DC_3A_n8A DC_3A_n77A | 3A | CA_n8A-n77A |

6.81.3	Co-existence studies

The coexistence study for DC_3A_n8A-n77A was covered by TR 37.717-11-21 [2].

Co-existence analysis for DC_3_n8 shows that there are IMD3 and IMD5 impact from DC_3_n8 UL to NR Band n77 DL.

Co-existence analysis for DC_3_n77 shows that there is IMD4 impact from DC_3_n77 UL to NR Band n8 DL.

6.81.4	∆TIB and ∆RIB values

For DC_3_n8-n77 the TIB,c and RIB,c values are same as for DC_3-8_n77 and are given in the tables below.

Table 6.81.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n8-n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.81.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n8-n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.81.5	MSD

There exist IMDs impacting the reference sensitivity of NR band n8 and n77. Based on Table 5.3-1 in TR 37.717-11-21 [2] the corresponding MSD requirements shall be re-used from DC_3A_n8A-n78A. The required MSD is shown in the following Table.

Table 6.81.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n8A-n77A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.3 | IMD34 |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | n8 | N/A | 5 | N/A | 955 | 9.7 | IMD4 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

6.82	DC_8_n7-n78

6.82.1	Operating bands for DC

Table 6.82.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n7-n78 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.82.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n7A-n78A | DC_8A_n7A DC_8A_n78A |

6.82.2	Channel bandwidths per operating band for DC

Table 6.82.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n7A-n78A | 8 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  | n7 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 |  |  |  |  |  |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 |  |  |  |  |  |  |
|  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.82.3	Co-existence studies

Based on co-existence studies of DC_8_n7 and DC_8_n78, own Rx impact of the 3rd band is the followings:

-	2nd order IMD products generated by DC_8A_n7A uplink may fall into own Rx of band n78.

-	2nd order IMD products generated by DC_8A_n78A uplink may fall into own Rx of band n7.

6.82.4	∆TIB and ∆RIB values

Referring to CA_n7-n8-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.82.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8_n7-n78 | 0.6 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.82.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8_n7-n78 | 0.2 | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.82.5	MSD requirements

For UE coexistence study of Band n7 + Band 8, 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.82.5-1.

| Table 6.82.5-1: UE coexistence study of Band n7 + Band 8UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2500 | 2570 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1585 | 1690 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 670 | 4085 | 4260 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4260 | 4400 | 5880 | 6055 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 70 | 175 | 6585 | 6830 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5140 | 5315 | 8380 | 8625 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3380 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9400 | 9085 | 1160 | 950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10880 | 11195 | 6020 | 6230 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5670 | 2255 | 2500 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9260 | 9540 | 7640 | 7885 |

2nd and 4th order IMD generated by dual uplink of band n7 and band 8 may fall into Rx of band n78.

| Component | IP4(dBm) |
| --- | --- |
| Antenna switch | 56 |
| Diplexer | 55 |
| Duplexer | 55 |
| PA forward mixing | 30 |
| PA reverse mixing | 30 |
| LNA | -6 |

Table 6.82.5-2: Assumptions on MSD analysis for DC_8_n7-n78

| n7 Rx Rejection at n78 Tx | 30 |
| --- | --- |
| B8 Rx Rejection at n78 Rx | 28 |
| n7 Tx Rejection at B8 Rx | 30 |
| B8 Tx Rejection at n7 Rx | 30 |
| n78 Rx Rejection at n7 Tx | 40 |
| n78 Rx Rejection at B8 Tx | 40 |
| Diplexer isolation | 15 |
| Antenna isolation | 10 |
| PCB isolation | 60 |

|  | Primary | Diversity |
| --- | --- | --- |
| Thermal | -92.8 | -92.8 |
| IMD total | -78.3 | -88.3 |
| REFSENS | -95.8 |  |
| MRC SENS | -87.5 |  |
| MSD | 8.3 |  |

For 4th order IMD,

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8_n7-n78 | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3310 | 8.3 | IMD4 |

Referring to CA_n7-n8-n78, the MSD requirements can be specified as below:

Table 6.82.5-3: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n7A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3455 | 28.5 | IMD21 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n7 | N/A | 5 | N/A | 2650 | 28 | IMD2 |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.83	DC_28_n5-n78

6.83.1	Operating bands for DC

Table 6.83.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n5-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.83.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n5A-n78A | DC_28A_n5A DC_28A_n78A |

6.83.2	Channel bandwidths per operating band for DC

Table 6.83.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n5A-n78A |  |  | 28 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  |  | n5 |  | 15 |  |  | 5 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.83.3	Co-existence studies

For UE coexistence study of UL DC_28_n5, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.83.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD5 of UL DC_28_n5 may have an impact on DL Band n78.

Table 6.83.3-1: The harmonic and IMD products caused by UL DC_28_n5

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 869 | 894 |
| 3rd Band DL | 3300 | 3800 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4120 | 4245 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 76 | 146 | 1527 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 672 | 900 | 995 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2345 | 2351 | 2446 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1420 | 1724 | 1844 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 152 | 3054 | 3194 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3093 | 3175 | 3295 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2548 | 2168 | 1963 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 976 | 596 | 411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4144 | 3636 | 3841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4043 | 3757 | 3942 |

Based on the calculation, we identify the following interference impact:

- The IMD5 of UL DC_28_n78 may have an impact on DL Band n5.

Table 6.83.3-1: The harmonic and IMD products caused by UL DC_28_n78

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 3300 | 3800 |
| 3rd Band DL | 869 | 894 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |

6.83.4	∆TIB and ∆RIB values

For DC_28_n5-n78, ΔTIB,c and ΔRIB,c values are as follows (from CA_n5-n28-n78).

Table 6.83.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n5-n78 | 0.7 | 0.7 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.83.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n5-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.83.5	MSD requirements

The MSD values for IMD5 of UL DC_28_n5 impact on DL Band n78 is taken from DC_5A_n28A-n78A.

The MSD values for IMD5 of UL DC_28_n78 impact on DL Band n5 is taken from DC_18A-28A_n78A, where band 18 is covered by band n5.

The MSD requirements can be specified as follows:

Table 6.83.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n5A-n78A | 28 | 707 | 5 | 25 | 762 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3781 | 4.0 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n5 | N/A | 5 | N/A | 874 | 3.8 | IMD5 |
|  | n78 | 3766 | 10 | 50 | 3756 | N/A | N/A |

6.84	DC_28_n40-n77

6.84.1	Operating bands for DC

Table 6.84.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n40-n77 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.84.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n40A-n77A | DC_28A_n40A DC_28A_n77A |

6.84.2	Channel bandwidths per operating band for DC

Table 6.84.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DC_28A_n40A-n77A | 28 | 15 | 3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n40 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n77 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.84.3	Co-existence studies

Based on co-existence studies of DC_28_n77 and DC_28_n40, own Rx impact of the 3rd band is the followings.

-   3rd order IMD products generated by DC_28_n77 uplink may fall into own Rx of band n40.

-    3rd  and 4th order IMD products generated by DC_28_n40 uplink may fall into own Rx of band n77.

6.84.4	∆TIB and ∆RIB values

For DC_28_n40-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.84.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n40-n77 | 0.5 | 0.3 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.84.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n40-n77 | 0.2 | 0.4 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.84.5	MSD requirements

Table 6.84.5-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n28-n40-n77 and CA_n7-n28-n78 are reused.

Table 6.84.5-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n40A-n77A | 28 | 708 | 5 | 25 | 763 | N/A | N/A |
|  | n40 | N/A | 5 | N/A | 2334 | 15.7 | IMD3 |
|  | n77 | 3750 | 10 | 50 | 3750 | N/A | N/A |
|  | 28 | 708 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3726 | 16.0 | IMD3 |
|  | 28 | 706 | 5 | 25 | 761 | N/A | N/A |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3368 | 9.7 | IMD4 |

6.85	DC_28_n1-n5

6.85.1	Operating bands for DC

Table 6.85.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n1-n5 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

Table 6.85.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n1A-n5A | DC_28A_n1A DC_28A_n5A |

6.85.2	Channel bandwidths per operating band for DC

Table 6.85.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n1A-n5A |  |  | 28 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 95 |
|  |  |  | n1 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n5 |  | 15 |  |  | 5 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |

6.85.3	Co-existence studies

For UE coexistence study of UL DC_28_n1, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.85.3-1.

For UE coexistence study of UL DC_28_n5, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.85.3-2.

Based on the calculation, we identify the following interference impact:

- The IMD5 of UL DC_28_n1 may have an impact on DL Band n5.

- The IMD5 of UL DC_28_n5 may have an impact on DL Band n1.

Table 6.85.3-1: The harmonic and IMD products caused by UL DC_28_n1

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 2110 | 2170 |
| 3rd Band DL | 869 | 894 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 574 | 424 | 3092 | 3257 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3326 | 3476 | 4543 | 4708 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 129 | 324 | 5012 | 5237 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2554 | 2344 | 5246 | 5456 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4029 | 4224 | 6463 | 6688 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7217 | 6932 | 1072 | 832 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4534 | 4264 | 1596 | 1851 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8383 | 8668 | 4732 | 4972 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7166 | 7436 | 5949 | 6204 |

Table 6.85.3-2: The harmonic and IMD products caused by UL DC_28_n5

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 869 | 894 |
| 3rd Band DL | 2110 | 2170 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4120 | 4245 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 76 | 146 | 1527 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 672 | 900 | 995 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2345 | 2351 | 2446 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1420 | 1724 | 1844 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 152 | 3054 | 3194 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3093 | 3175 | 3295 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2548 | 2168 | 1963 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 976 | 596 | 411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4144 | 3636 | 3841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4043 | 3757 | 3942 |

6.85.4	∆TIB and ∆RIB values

For DC_28_n1-n5, ΔTIB,c and ΔRIB,c values are as follows (from DC_1-28_n5 and DC_1-5_n28, respectively).

Table 6.85.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n1-n5 | 0.6 | 0.3 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.85.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n1-n5 | 0.2 | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.85.5	MSD requirements

The MSD values for IMD5 of UL DC_28_n1 impact on DL Band n5 is re-used from DC_1A-5A_n28A.

The MSD values for IMD5 of UL DC_28_n5 impact on DL Band n1 is re-used from DC_1A-5A_n28A.

The MSD requirements can be specified as follows:

Table 6.85.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n1A-n5A | 28 | 708 | 5 | 25 | 763 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n5 | N/A | 5 | N/A | 882 | 4.6 | IMD5 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n1 | N/A | 5 | N/A | 2136 | 4 | IMD5 |
|  | n5 | 836 | 5 | 25 | 881 | N/A | N/A |

6.86	DC_28_n5-n105

6.86.1	Operating bands for DC

Table 6.86.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n5-n105 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.86.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n5A-n105A | DC_28A_n5A |

6.86.2	Channel bandwidths per operating band for DC

Table 6.86.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n5A-n105A |  |  | 28 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  |  | n5 |  | 15 |  |  | 5 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n105 |  | 15 |  |  | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |

6.86.3	Co-existence studies

For UE coexistence study of UL DC_28_n5, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.86.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD3 of UL DC_28_n5 may have an impact on DL Band n105.

Table 6.86.3-1: The harmonic and IMD products caused by UL DC_28_n5

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 869 | 894 |
| 3rd Band DL | 612 | 652 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4120 | 4245 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 146 | 76 | 1527 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 900 | 995 | 557 | 672 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2351 | 2446 | 2230 | 2345 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1724 | 1844 | 1260 | 1420 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 152 | 292 | 3054 | 3194 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3175 | 3295 | 2933 | 3093 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2168 | 1963 | 2693 | 2548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 596 | 411 | 1141 | 976 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3636 | 3841 | 3999 | 4144 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3757 | 3942 | 3878 | 4043 |

6.86.4	∆TIB and ∆RIB values

For DC_28_n5-n105, ΔTIB,c and ΔRIB,c values are as follows (from CA_n5-n28-n105).

Table 6.86.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n5-n105 | 1.0 | 0.7 | 1.0 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.86.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n5-n105 | 0.7 | 0.2 | 0.7 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.86.5	MSD requirements

The MSD values for IMD3 of UL DC_28_n5 impact on DL Band n105 is re-used from CA_n5-n28-n105.

The MSD requirements can be specified as follows:

Table 6.86.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n5A-n105A | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n105 | N/A | 5 | N/A | 635 | 25 | IMD3 |

6.87	DC_28_n1-n105

6.87.1	Operating bands for DC

Table 6.87.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n1-n105 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.87.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n1A-n105A | DC_28A_n1A |

6.87.2	Channel bandwidths per operating band for DC

Table 6.87.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n1A-n105A |  |  | 28 |  | 15 |  |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 105 |
|  |  |  | n1 |  | 15 |  |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n105 |  | 15 |  |  | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |

6.87.3	Co-existence studies

For UE coexistence study of UL DC_28_n1, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.87.3-1.

Based on the calculation, we do not identify any interference impact:

Table 6.87.3-1: The harmonic and IMD products caused by UL DC_28_n1

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 2110 | 2170 |
| 3rd Band DL | 612 | 652 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

6.87.4	∆TIB and ∆RIB values

For DC_28_n1-n105, ΔTIB,c and ΔRIB,c values are as follows (from CA_n1-n5-n28).

Table 6.87.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n1-n105 | 1 | 0.3 | 1 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.87.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n1-n105 | 0.7 | - | 0.7 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.87.5	MSD requirements

There is no additional receiver requirements for MSD.

6.88	DC_28_n78-n105

6.88.1	Operating bands for DC

Table 6.88.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n78-n105 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.88.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n78A-n105A | DC_28A_n78A |

6.88.2	Channel bandwidths per operating band for DC

Table 6.88.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n78A-n105A | 28 | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  | n78 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | n105 | 15 |  | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |

6.88.3	Co-existence studies

For UE coexistence study of UL DC_28_n78, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.88.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD5 of UL DC_28_n78 may have an impact on DL Band n105.

Table 6.88.3-1: The harmonic and IMD products caused by UL DC_28_n78

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 3300 | 3800 |
| 3rd Band DL | 612 | 652 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |

6.88.4	∆TIB and ∆RIB values

For DC_28_n78-n105, ΔTIB,c and ΔRIB,c values are as follows (from DC_28_n78 and CA_n78-n105).

Table 6.88.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n78-n105 | 1 | 0.8 | 1 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.88.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_28_n78-n105 | 0.7 | 0.5 | 0.7 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.88.5	MSD requirements

The MSD values for IMD5 of UL DC_28_n78 impact on DL Band n105 is re-used from DC_12A-71A_n77A.

The MSD requirements can be specified as follows:

Table 6.88.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n78A-n105A | 28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n105 | N/A | 5 | N/A | 628 | 3.9 | IMD5 |

Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

7.1	DC_1-38_n28-n78

7.1.1	Operating bands for DC

Table 7.1.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-38_n28-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.1.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_38A_n28A DC_38A_n78A |

7.1.2	Configuration for DC

Table 7.1.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-38A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.1.3	∆TIB and ∆RIB values

Referring to DC_1-41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.1.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-38_n28-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.1.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-38_n28-n78 | - | - | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.2	DC_3-38_n28-n78

7.2.1	Operating bands for DC

Table 7.2.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-38_n28-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.2.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-38A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3C-38A_n28A-n78A | DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |

7.2.2	Configuration for DC

Table 7.2.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-38A_n28A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-38A_n28A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.2.3	∆TIB and ∆RIB values

Referring to DC_3-41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.2.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-38_n28-n78 | 1.0 | 0.3 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.2.3-2: ΔRIB,c due to EN-DC (fourbands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-38_n28-n78 | 0,5 | 0.4 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.3	DC_1_(n)3-n257

7.3.1	Operating bands for DC

Table 7.3.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_(n)3-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n275 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.3.2	Configuration for DC

Table 7.3.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_(n)3AA-n257A DC_1A _(n)3AA-n257G DC_1A _(n)3AA-n257H DC_1A _(n)3AA-n257I DC_1A _(n)3AA-n257J DC_1A _(n)3AA-n257K DC_1A _(n)3AA-n257L DC_1A _(n)3AA-n257M | DC_1A_n3A DC_1A_n257A DC_(n)3AA1 DC_3A_n257A |
| NOTE1: Only single switched UL is supported. |  |

7.3.3	Co-existence studies

The studies are already performed for DC_1A_n3A in TR 37.716-11-11.

No study is needed for DC_1A_n257A, due to large frequency separation.

The studies are already performed for DC_(n)3AA in TR 37.717-11-11.

No study is needed for DC_3A_n257A, due to large frequency separation.

7.3.4	∆TIB and ∆RIB values

For DC_(n)3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.3.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1_(n)3-n257 | - | - | 0.6 | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.3.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1_(n)3-n257 | - | - | 0.2 | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.3.5	MSD

MSD analysis is already available for DC_(n)3AA and DC_1A_n3A, in TR 37.717-11-11 and TR 37.716-11-11, respectively.

No MSD analysis is needed for DC_1A_n257A  and DC_3A_n257A, as there is no coexistence issue.

7.4	DC_1-3_n26-n78

7.4.1	Operating bands for DC

Table 7.4.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.4.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A |
| DC_1A-3C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |

7.4.2	Channel bandwidths per operating band for DC

Table 7.4.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

7.4.3	∆TIB and ∆RIB values

Referring to DC_1-3_5-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-3_n26-n78 | 0.6 | 0.6 | 0.3 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.4.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-3_n26-n78 | 0.2 | 0.2 | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.5	DC_1-7_n26-n78

7.5.1	Operating bands for DC

Table 7.5.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.5.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |

7.5.2	Channel bandwidths per operating band for DC

Table 7.5.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-7C_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

7.5.3	∆TIB and ∆RIB values

Referring to DC_1-5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.5.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-7_n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.5.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-7_n26-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.6		DC_3-7_n26-n78

7.6.1	Operating bands for DC

Table 7.6.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n26-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.6.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-7A_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A |
| DC_3A-7C_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A |
| DC_3C-7A_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A |
| DC_3C-7C_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |

7.6.2	Channel bandwidths per operating band for DC

Table 7.6.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n26A-n78A |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3A-7C_n26A-n78A |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3C-7A_n26A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3C-7C_n26A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

7.6.3	∆TIB and ∆RIB values

Referring to DC_3-5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.6.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-7_n26-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.6.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-7_n26-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.7	DC_1-38_n7-n78

7.7.1	Operating bands for DC

Table 7.7.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-38_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.7.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-38A_n7A-n78A | DC_1A_n78A |

7.7.2	Configuration for DC

Table 7.7.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-38A_n7A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.7.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.7.4	∆TIB and ∆RIB values

For DC_1-38_n7-n78, the TIB,c and RIB,c values are given in the tables as below.

Table 7.7.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-38_n7-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.7.4-2: ΔRIB, c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-38_n7-n78 | 0.2 | - | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.7.5	MSD

All MSD requirements for DC_1-38_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.8	DC_7-20_n1-n75

7.8.1	Operating bands for DC

Table 7.8.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-20_n1-n75 | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

7.8.2	Channel bandwidths per operating band for DC

Table 7.8.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

7.8.3	UE co-existence studies

Co-existence studies of DC_7-20_n1-n75 are already covered by the fallback configurations, DC_7_n1-n75 and DC_20_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.8.4	∆TIB and ∆RIB values

For DC_7-20_n1-n75, the TIB,c and RIB,c values are reused from CA_1-7-20_32, and are given in the tables below.

Table 7.8.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_7-20_n1-n75 | 0.7 | 0.3 | 0.7 | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.8.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_7-20_n1-n75 | - | - | - | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.8.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.9	DC_3-20_n1-n75

7.9.1	Operating bands for DC

Table 7.9.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

7.9.2	Channel bandwidths per operating band for DC

Table 7.9.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n1A-n75A | DC_3A_n1A DC_20A_n1A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C-20A_n1A-n75A | DC_3A_n1A DC_3C_n1A DC_20A_n1A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

7.9.3	UE co-existence studies

Co-existence studies of DC_3-20_n1-n75 are already covered by the fallback configurations, DC_3_n1-n75 and DC_20_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.9.4	∆TIB and ∆RIB values

For DC_3-20_n1-n75, the TIB,c and RIB,c values are reused from CA_1-3-20_32, and are given in the tables below.

Table 7.9.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-20_n1-n75 | 0.5 | 0.3 | 0.5 | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.9.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-20_n1-n75 | - | - | - | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.9.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.10	 DC_3-7_n1-n75

7.10.1	Operating bands for DC

Table 7.10.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

7.10.2	Configuration for DC

Table 7.10.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n1A-n75A | DC_3A_n1A DC_7A_n1A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C-7A_n1A-n75A | DC_3C_n1A DC_3A_n1A DC_7A_n1A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

7.10.3	UE co-existence studies

Co-existence studies of DC_3-7_n1-n75 are already covered by the fallback configurations, DC_3_n1-n75 and DC_7_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.10.4	∆TIB and ∆RIB values

For DC_3-7_n1-n75, the TIB,c and RIB,c values are reused from CA_1-3-7_32, and are given in the tables below.

Table 7.10.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-7_n1-n75 | 0.6 | 0.6 | 0.6 | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.10.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-7_n1-n75 | - | - | - | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.10.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.11	DC_1-7_n75-n78

7.11.1	Operating bands for DC

Table 7.11.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n75-n78 | 1 | 1920MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500MHz |  | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n75 |  | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300MHz | – |  | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |

7.11.2	Configuration for DC

Table 7.11.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n75A-n78A | DC_1A_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.11.3	UE co-existence studies

Co-existence studies of DC_1-7_n75-n78 are already covered by the fallback configurations, DC_1-7_n78, DC_1_n75-n78 and DC_7_n75-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.11.4	∆TIB and ∆RIB values

For DC_1-7_n75-n78, the TIB,c and RIB,c values are reused from DC_1-7-32_n78, and are given in the tables below.

Table 7.11.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-7_n1-n75 | 0.2 | 0.2 | - | 0.5 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.11.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-7_n1-n75 | 0.6 | 0.6 | - | 0.8 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.11.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.12	DC_3-41_n1-n78

7.12.1	Operating bands for DC

Table 7.12.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.12.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |

7.12.2	Channel bandwidths per operating band for DC

Table 7.12.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 190 |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 210 |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.12.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.12.4	∆TIB and ∆RIB values

For DC_3-41_n1-n78, the TIB,c, and RIB,c values are reused from DC_1-3-41_n78 and given in the tables below.

Table 7.12.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.12.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.12.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.13	DC_8-41_n1-n78

7.13.1	Operating bands for DC

Table 7.13.1-1: DC band combination of NR 2DL/1UL + LTE 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-41_n1-n78 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.13.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-41A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |

7.13.2	Channel bandwidths per operating band for DC

Table 7.13.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_8A-41A_n1A-n78A | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_8A-41C_n1A-n78A | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.13.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.13.4	∆TIB and ∆RIB values

For DC_8-41_n1-n78, the TIB,c and RIB,c values are reused from DC_7-8_n1-n78 and given in the tables below.

Table 7.13.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_8-41_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.13.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_8-41_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.13.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.14	DC_20-41_n1-n78

7.14.1	Operating bands for DC

Table 7.14.1-1: DC band combination of NR 2DL/1UL + LTE 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20-41_n1-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.14.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_20A-41A_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-41C_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |

7.14.2	Channel bandwidths per operating band for DC

Table 7.14.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_20A-41A_n1A-n78A | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 190 |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_20A-41C_n1A-n78A | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 210 |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.14.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.14.4	∆TIB and ∆RIB values

For DC_20-41_n1-n78, the TIB,c, and RIB,c values are reused from DC_1-20-n41_n78 and given in the tables below.

Table 7.14.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_20-41_n1-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.14.4-2: ΔRIB,

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_20-41_n1-n78 | - | - | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.14.5	MSD

There is no need to specify additional MSD requirements for this UL DC configuration.

7.15	DC_3-7_n75-n78

7.15.1	Operating bands for DC

Table 7.15.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n75-n78 | 3 | 1710MHz |  | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz |  | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n75 |  | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300MHz | – |  | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |

7.15.2	Configuration for DC

Table 7.15.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n75A-n78A | DC_3A_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.15.3	UE co-existence studies

Co-existence studies of DC_3-7_n75-n78 are already covered by the fallback configurations, DC_3-7_n78, DC_3_n75-n78 and DC_7_n75-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.15.4	∆TIB and ∆RIB values

For DC_3-7_n75-n78, the TIB,c and RIB,c values are reused from DC_3-7-32_n78, and are given in the tables below.

Table 7.15.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-7_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.15.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-7_n75-n78 | 0.2 | 0.2 | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.15.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.16	DC_1-3_n75-n78

7.16.1	Operating bands for DC

Table 7.16.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n75-n78 | 1 | 1920MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710MHz |  | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | n75 |  | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300MHz | – |  | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |

7.16.2	Configuration for DC

Table 7.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n75A-n78A | DC_1A_n78A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.16.3	UE co-existence studies

Co-existence studies of DC_1-3_n75-n78 are already covered by the fallback configurations, DC_1-3_n78, DC_1_n75-n78 and DC_3_n75-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.16.4	∆TIB and ∆RIB values

For DC_1-3_n75-n78, the TIB,c and RIB,c values are reused from DC_1-3-32_n78, and are given in the tables below.

Table 7.16.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-3_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.16.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-3_n75-n78 | - | - | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.16.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.17	DC_1-3_n40-n77

7.17.1	Operating bands for DC

Table 7.17.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n40-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 7.17.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |
| DC_1A-3A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |

7.17.2	Channel bandwidths per operating band for DC

Table 7.17.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n40A-n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A_n40A-n77(2A) |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 340 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.17.3	UE co-existence studies

Co-existence studies of DC_1-3_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.17.4	∆TIB and ∆RIB values

Referring to DC_1-3_n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.17.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-3_n40-n77 | 0.5 | 0.6 | 0.36 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one EUTRA band and without simultaneous Rx/Tx. NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.17.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-3_n40-n77 | - | 0.2 | 0.45 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one EUTRA band and without simultaneous Rx/Tx. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.17.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.18	DC_1-5_n40-n77

7.18.1	Operating bands for DC

Table 7.18.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-5_n40-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 7.18.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-5A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |

7.18.2	Channel bandwidths per operating band for DC

Table 7.18.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-5A_n40A-n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-5A_n40A-n77(2A) |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.18.3	UE co-existence studies

Co-existence studies of DC_1-5_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.18.4	∆TIB and ∆RIB values

Referring to DC_3_n5-n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.18.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-5_n40-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.18.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-5_n40-n77 | 0.2 | 0.2 | 0.48 | 0.5 |
| NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.18.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.19	DC_3-5_n40-n77

7.19.1	Operating bands for DC

Table 7.19.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-5_n40-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 7.19.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-5A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_3A-5A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |

7.19.2	Channel bandwidths per operating band for DC

Table 7.19.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-5A_n40A-n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-5A_n40A-n77(2A) |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.19.3	UE co-existence studies

Co-existence studies of DC_3-5_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.19.4	∆TIB and ∆RIB values

Referring to DC_3_n5-n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.19.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-5_n40-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.19.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-5_n40-n77 | 0.2 | 0.2 | 0.48 | 0.5 |
| NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.19.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.20	DC_3-7_n40-n77

7.20.1	Operating bands for DC

Table 7.20.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n40-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 7.20.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |

7.20.2	Channel bandwidths per operating band for DC

Table 7.20.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n40A-n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7A_n40A-n77(2A) |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 340 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.20.3	UE co-existence studies

Co-existence studies of DC_3-7_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.20.4	∆TIB and ∆RIB values

Referring to DC_3-7_n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.20.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-7_n40-n77 | 0.6 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.20.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-7_n40-n77 | 0.2 | - | 0.48 | 0.58 |
| NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.20.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.21	DC_5-7_n40-n77

7.21.1	Operating bands for DC

Table 7.21.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-7_n40-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 7.21.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-7A_n40A-n77A | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |

7.21.2	Channel bandwidths per operating band for DC

Table 7.21.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-7A_n40A-n77A |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_5A-7A_n40A-n77(2A) |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.21.3	UE co-existence studies

Co-existence studies of DC_5-7_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.21.4	∆TIB and ∆RIB values

Referring to DC_7-28_n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.21.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_5-7_n40-n77 | 0.3 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.21.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_5-7_n40-n77 | 0.2 | - | 0.4 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.21.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.22	DC_1-5_n40-n78

7.22.1	Operating bands for DC

Table 7.22.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-5_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.22.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-5A_n40A-n78A DC_1A-5A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_5A_n40A DC_5A_n78A |

7.22.2	Channel bandwidths per operating band for DC

Table 7.22.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-5A_n40A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-5A_n40A-n78C |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.22.3	UE co-existence studies

Co-existence studies of DC_1-5_n40-n78 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.22.4	∆TIB and ∆RIB values

Referring to DC_3_n5-n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.22.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-5_n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.22.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-5_n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.22.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.23	DC_3-5_n40-n78

7.23.1	Operating bands for DC

Table 7.23.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-5_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.23.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-5A_n40A-n78A DC_3A-5A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A |

7.23.2	Channel bandwidths per operating band for DC

Table 7.23.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-5A_n40A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-5A_n40A-n78C |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.23.3	UE co-existence studies

Co-existence studies of DC_3-5_n40-n78 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.23.4	∆TIB and ∆RIB values

Referring to DC_3_n5-n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.23.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-5_n40-n78 | 0.6 | 0.6 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.23.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-5_n40-n78 | 0.2 | 0.2 | 0.48 | 0.5 |
| NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.23.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.24	DC_5-7_n40-n78

7.24.1	Operating bands for DC

Table 7.24.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-7_n40-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.24.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-7A_n40A-n78A | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A_n40A-n78C | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |

7.24.2	Channel bandwidths per operating band for DC

Table 7.24.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_5A-7A_n40A-n78A |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_5A-7A_n40A-n78C |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.24.3	UE co-existence studies

Co-existence studies of DC_5-7_n40-n78 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.24.4	∆TIB and ∆RIB values

Referring to DC_7-28_n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.24.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_5-7_n40-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.24.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_5-7_n40-n78 | 0.2 | - | 0.4 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.24.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.25	DC_1-7_n40-n77

7.25.1	Operating bands for DC

Table 7.25.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n40-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 7.25.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |

7.25.2	Channel bandwidths per operating band for DC

Table 7.25.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n40A-n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-7A_n40A-n77(2A) |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 340 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.25.3	UE co-existence studies

Co-existence studies of DC_1-7_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.25.4	∆TIB and ∆RIB values

Referring to DC_1-7_n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.25.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-7_n40-n77 | 0.6 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.25.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-7_n40-n77 | 0.2 | - | 0.4 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.25.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.26	DC_3-38_n7-n78

7.26.1	Operating bands for DC

Table 7.26.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-38_n7-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.26.2	Configuration for DC

Table 7.26.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL (MHz) |
| DC_3A-38A_n7A-n78A | DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.26.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.26.4	∆TIB and ∆RIB values

For DC_3-38_n7-n78, the TIB,c and RIB,c values are given in the tables as below.

Table 7.26.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-38_n7-n78 | 0.6 | - | - | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.26.4-2: ΔRIB, c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-38_n7-n78 | 0.2 | 0.4 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.26.5	MSD

All MSD requirements for DC_3-38_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.27	DC_8-(n)3-n77

7.27.1	Operating bands for DC

Table 7.27.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-(n)3-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.3.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_8A-(n)3AA-n77A | DC_8A_n3A DC_8A_n77A DC_(n)3AA1 DC_3A_n77A |
| DC_8A-(n)3AA-n77(2A) | DC_8A_n3A DC_8A_n77A DC_(n)3AA1 DC_3A_n77A |
| NOTE1: Only single switched UL is supported. |  |

7.27.2	Channel bandwidths per operating band for DC

Table 7.27.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-(n)3AA-n77A | DC_8A_n3A DC_8A_n77A DC_(n)3AA DC_3A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-(n)3AA-n77(2A) | DC_8A_n3A DC_8A_n77A DC_(n)3AA DC_3A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.27.3	Co-existence studies

The studies are already performed for DC_8_n3-n77, DC_(n)3-n8 and DC_(n)3-n77  captured in TR 37.717-11-21, TR 37.718-11-21, TR 37.718-11-21., respectively.

7.27.4	∆TIB and ∆RIB values

For DC_8-(n)3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.27.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_8-(n)3-n77 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.27.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_8-(n)3-n77 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.27.5	MSD

All the MSD analysis are available at DC_8_n3-n77, DC_(n)3-n8 and DC_(n)3-n77  captured in TR 37.717-11-21, TR 37.718-11-21, TR 37.718-11-21., respectively.

7.28	DC_1-(n)3-n8

7.28.1	Operating bands for DC

Table 7.28.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-(n)3AA-n8A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

Table 6.3.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-(n)3AA-n8A | DC_1A_n3A DC_1A_n8A DC_(n)3AA1 DC_3A_n8A |
| NOTE1: Only single switched UL is supported. |  |

7.28.2	Channel bandwidths per operating band for DC

Table 7.28.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-(n)3AA-n8A | DC_1A_n3A DC_1A_n8A DC_(n)3AA DC_3A_n8A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 125 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes | Yes1 |  | Yes1 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes1 |  | Yes1 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.28.3	Co-existence studies

The studies are already available for DC_1_n3-n8 in TR 37.717-11-21

7.28.4	∆TIB and ∆RIB values

The values are copied from DC_1_n3-n8 in TR 37.717-11-21

Table 7.28.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-(n)3-n8 | 0.3 | 0.3 | 0.3 | 0.3 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.28.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-(n)3-n8 | - | - | - | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.28.5	MSD

As mentioned for DC_1_n3-n8 in TR 37.717-11-21, the second harmonic of n8 may occur on the Rx of n3. The corresponding MSD values are already mentioned in 38.101-3 Table 7.3B.2.3.1-1

7.29	DC_5-7_n1-n78

7.29.1	Operating bands for DC

Table 7.29.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-7A_n1A-n78A | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.29.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-7A_n1A-n78A | DC_5A_n1A DC_5A_n78A DC_7A_n1A DC_7A_n78A |

7.29.2	Channel bandwidths per operating band for DC

Table 7.29.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_5A-7A_n1A-n78A | 5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 190 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.29.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.29.4	∆TIB and ∆RIB values

For DC_5-7_n1-n78, the TIB,c, and RIB,c values are reused from CA_n1-n5-n7-n78 and given in the tables below.

Table 7.29.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_5-7_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.29.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_5-7_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.29.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.30	DC_1-8_n7-n78

7.30.1	Operating bands for DC

Table 7.30.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.30.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_8A_n7A DC_8A_n78A |

7.30.2	Configuration for DC

Table 7.30.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A_n7A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.30.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.30.4	∆TIB and ∆RIB values

For DC_1-8_n7-n78, the TIB,c and RIB,c values are given in the tables as below, referring to CA_n1-n7-n8-n78.

Table 7.30.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-8_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.30.4-2: ΔRIB, c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-8_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.30.5	MSD

All MSD requirements for DC_1-8_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.31	DC_3-8_n7-n78

7.31.1	Operating bands for DC

Table 7.31.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n7-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.31.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-8A_n7A-n78A | DC_3A_n7A DC_3A_n78A DC_8A_n7A DC_8A_n78A |

7.31.2	Configuration for DC

Table 7.31.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A_n7A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.31.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.31.4	∆TIB and ∆RIB values

For DC_3-8_n7-n78, the TIB,c and RIB,c values are given in the tables as below, referring to CA_n3-n7-n8-n78.

Table 7.31.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-8_n7-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.31.4-2: ΔRIB, c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-8_n7-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.31.5	MSD

All MSD requirements for DC_3-8_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.32	DC_7-8_n7-n78

7.32.1	Operating bands for DC

Table 7.32.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n7-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.32.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-8A_n7A-n78A | DC_7A_n7A DC_7A_n78A DC_8A_n7A DC_8A_n78A |

7.32.2	Configuration for DC

Table 7.32.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-8A_n7A-n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.32.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.32.4	∆TIB and ∆RIB values

For DC_7-8_n7-n78, the TIB,c and RIB,c values are given in the tables as below, referring to DC_7-7-8_n78.

Table 7.32.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_7-8_n7-n78 | 0.5 | 0.6 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.32.4-2: ΔRIB, c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_7-8_n7-n78 | - | 0.2 | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.32.5	MSD

All MSD requirements for DC_7-8_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

8.1	DC_1-3-38_n28-n78

8.1.1	Operating bands for DC

Table 8.1.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-38_n28-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.1.1-2: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3C-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |

8.1.2	Configuration for DC

Table 8.1.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-38A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-38A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.1.3	∆TIB and ∆RIB values

Referring to DC_—1-41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 8.1.3-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-38_n28-n78 | 0.5 | 0.6 | 0.3 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.1.3-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-38_n28-n78 | - | 0.2 | - | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.2	DC_1-3-7_n26-n78

8.2.1	Operating bands for DC

Table 8.2.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.2.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3C-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3C-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |

8.2.2	Channel bandwidths per operating band for DC

Table 8.2.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-38A_n28A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C-38A_n28A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C-38A_n28A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C-7C_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

8.2.3	∆TIB and ∆RIB values

Referring to DC_1-3-5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 8.2.3-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n26-n78 | 0.5 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.2.3-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n26-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.3	DC_3-8-41_n1-n78

8.3.1	Operating bands for DC

Table 8.3.1-1: DC band combination of NR 2DL/1UL + LTE 3DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.3.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |

8.3.2	Channel bandwidths per operating band for DC

Table 8.3.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 200 |
|  | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 220 |
|  | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.3.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.3.4	∆TIB and ∆RIB values

For DC_3-8-41_n1-n78, the TIB,c and RIB,c values are reused from DC_3-7-8_n1-n78 and given in the tables below.

Table 8.3.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 0.5 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.3.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.3.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.4	DC_3-20-41_n1-n78

8.4.1	Operating bands for DC

Table 8.4.1-1: DC band combination of NR 2DL/1UL + LTE 3DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.4.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |

8.4.2	Channel bandwidths per operating band for DC

Table 8.4.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 210 |
|  | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 230 |
|  | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.4.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.4.4	∆TIB and ∆RIB values

For DC_3-20-41_n1-n78, the TIB,c, and RIB,c values are reused from DC_1-3-20_n41-n78 and given in the tables below.

Table 8.4.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | 0.5 | 0.3 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.4.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | - | - | - | - | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.4.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.5	DC_3-7-20_n1-n75

8.5.1	Operating bands for DC

Table 8.5.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-20_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

8.5.2	Channel bandwidths per operating band for DC

Table 8.5.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A DC_20A_n1A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

8.5.3	UE co-existence studies

Co-existence studies of DC_3-7-20_n1-n75 are already covered by the fallback configurations, DC_3-7-20_n1, DC_3_n1-n75, DC_7_n1-n75, DC_20_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.5.4	∆TIB and ∆RIB values

For DC_3-7-20_n1-n75, the TIB,c and RIB,c values are reused from CA_1-3-7-20-32, and are given in the tables below.

Table 8.5.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-20_n1-n75 | 0.7 | 0.7 | 0.3 | 0.7 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.5.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-20_n1-n75 | - | - | - | - | - |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.5.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.6	DC_1-3-7_n75-n78

8.6.1	Operating bands for DC

Table 8.6.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n75-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

8.6.2	Channel bandwidths per operating band for DC

Table 8.6.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A_n75A-n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.6.3	UE co-existence studies

Co-existence studies of DC_1-3-7_n75-n78 are already covered by the fallback configurations.

8.6.4	∆TIB and ∆RIB values

For DC_1-3-7_n75-n78, the TIB,c and RIB,c values are reused from DC_1-3-7-32_n78, and are given in the tables below.

Table 8.6.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n75-n78 | 0.7 | 0.7 | 0.7 | - | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.6.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n75-n78 | 0.3 | 0.3 | 0.3 | - | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.6.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.7	DC_1-3-8_n7-n78

8.7.1	Operating bands for DC

Table 8.7.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.7.1-2: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_8A_n7A DC_8A_n78A |

8.7.2	Configuration for DC

Table 8.7.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-8A_n7A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.7.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.7.4	∆TIB and ∆RIB values

For DC_1-3-8_n7-n78, the TIB,c and RIB,c values are given in the tables as below, referring to DC_1-3-7-8_n78.

Table 8.7.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-8_n7-n78 | 0.7 | 0.7 | 0.6 | 0.7 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.7.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-8_n7-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.7.5	MSD

All MSD requirements for DC_1-3-8_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.8	DC_1-7-8_n7-n78

8.8.1	Operating bands for DC

Table 8.8.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-8_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.8.1-2: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_7A_n7A1 DC_7A_n78A DC_8A_n7A DC_8A_n78A |
| NOTE 1: Only single switched UL is supported |  |

8.8.2	Configuration for DC

Table 8.8.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A-8A_n7A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.8.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.8.4	∆TIB and ∆RIB values

For DC_1-7-8_n7-n78, the TIB,c and RIB,c values are given in the tables as below, referring to DC_1-7-7-8_n78.

Table 8.8.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-7-8_n7-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.8.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-7-8_n7-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.8.5	MSD

All MSD requirements for DC_1-7-8_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.9	DC_3-7-8_n7-n78

8.9.1	Operating bands for DC

Table 8.9.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n7-n78 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.9.1-2: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-7A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_7A_n7A1 DC_7A_n78A DC_8A_n7A DC_8A_n78A |
| NOTE 1: Only single switched UL is supported. |  |

8.9.2	Configuration for DC

Table 8.9.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-8A_n7A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.9.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.9.4	∆TIB and ∆RIB values

For DC_3-7-8_n7-n78, the TIB,c and RIB,c values are given in the tables as below, referring to DC_3-7-7-8_n78.

Table 8.9.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-8_n7-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.9.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-8_n7-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.9.5	MSD

All MSD requirements for DC_3-7-8_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

9.1	DC_UA-VA-WA-XA_nYA-nZ

9.1.1	Operating bands for DC

9.1.2	Channel bandwidths per operating band for DC

9.1.3	Co-existence studies

9.1.4	∆TIB and ∆RIB values

9.1.5	MSD

Annex A: Change history

| Date | Meeting | TDoc | CR | Rev | Subject/Comment | Old version | New version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2022-08 | RAN4#104e | R4-2212174 |  |  | TR37.718-11-21 Skeleton for Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) |  | 0.0.1 |
| 2022-08 | RAN4#104e | R4-2212175 |  |  | TR 37.718-11-21 v0.1.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18 | 0.0.1 | 0.1.0 |
| 2022-10 | RAN4#104-bis-e | R4-2215383 |  |  | TR 37.718-11-21 v0.2.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18 | 0.1.0 | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2218026 |  |  | TR 37.718-11-21 v0.3.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18 | 0.2.0 | 0.3.0 |
| 2023-03 | RAN4#106 | R4-2300760 |  |  | TR 37.718-11-21 v0.4.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18 | 0.3.0 | 0.4.0 |
| 2023-04 | RAN4#106-bis-e | R4-2304088 |  |  | TR 37.718-11-21 v0.5.0 TR Update for DC_R18_xBLTE_2BNR_yDL2UL | 0.4.0 | 0.5.0 |
| 2023-05 | RAN4#107 | R4-2307812 |  |  | TR 37.718-11-21 v0.6.0 for DC_R18_xBLTE_2BNR_yDL2UL | 0.5.0 | 0.6.0 |
| 2023-08 | RAN4#108 | R4-2311759 |  |  | TR 37.718-11-21 v0.7.0 for DC_R18_xBLTE_2BNR_yDL2UL | 0.6.0 | 0.7.0 |
| 2023-10 | RAN4#108-bis | R4-2315068 |  |  | TR 37.718-11-21 v0.8.0 for DC_R18_xBLTE_2BNR_yDL2UL | 0.7.0 | 0.8.0 |
| 2023-11 | RAN4#109 | R4-2318098 |  |  | TR 37.718-11-21 v0.9.0 for DC_R18_xBLTE_2BNR_yDL2UL | 0.8.0 | 0.9.0 |
| 2024-02 | RAN4#110 | R4-2400605 |  |  | TR 37.718-11-21 v0.10.0 for DC_R18_xBLTE_2BNR_yDL2UL | 0.9.0 | 0.10.0 |
| 2024-04 | RAN4#110-bis | R4-2404233 |  |  | TR 37.718-11-21 v0.11.0 for DC_R18_xBLTE_2BNR_yDL2UL | 0.10.0 | 0.11.0 |
| 2024-05 | RAN4#111 | R4-2407207 |  |  | TR 37.718-11-21 v0.12.0 for DC_R18_xBLTE_2BNR_yDL2UL | 0.11.0 | 0.12.0 |