# 3GPP TSG-RAN WG4 Meeting #111	R4-2407308

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.16.7.1

Source:	Apple

Title:	On NR-NTN accuracy requirements in above 10 GHz bands

Document for:	Discussion

# Introduction

In RAN4#110bis meeting, the NR-NTN RRM requirements in above 10 GHz bands have been discussed and WF was agreed in [1]. However, the measurement accuracy requirement has not been concluded yet and it was not captured in the big CR [2]. In this contribution, we discuss the NR-NTN measurement accuracy requirements in above 10 GHz bands.

# Discussion

The previous agreement and open issue for UE accuracy requirement over 10GHz is:

### Issue 6-2-2: Above 10 GHz, Rx beam gain

Agreement: (ad-hoc) - FFS whether the RF margin for different RX beams in existing TN FR2-1 intra-frequency relative accuracy requirements can be removed or not - Existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz. - Further discuss the minimum SSB_RP condition on accuracy requirements

Since the Rx beam sweeping is not specified for NTN over 10GHz, both intra-frequency and inter-frequency measurement happens on the same satellite. Regarding the intra-frequency/inter-frequency relative accuracy, since serving cell and neighbor cells are on the same satellite and single Rx beam is assumed in the delay requirement, that means same Rx beam is required for intra-frequency measurement and the RF margin for different Rx beams in this relative accuracy can be removed.

Proposal 1: remove an RF margin for different RX beams in the relative accuracy and tighten relative accuracy requirements.

In last meeting, we were also one of the proponent companies for “Existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz”, since we assume all the conditions are same between TN and NTN FR2 accuracy. However, after last RAN4 meeting, we realized that RAN4 RF had one assumption in TS38.101-5 that, the minimum requirements on the receiver characteristics apply under one polarization.

In FR2 TN, the dual polarization is always assumed, i.e., DL signals measurement in orthogonal polarizations. RAN4 never discussed if single polarization can achieve the same accuracy as dual-polarization case, and therefore at least we need to double check if the same absolute measurement accuracy can be reused from TN to NTN FR2. Our understanding is, the same relaxation in FR1 1Rx RedCap can be reused here to reflect such degradation by losing the diversity reception, e.g., 1dB relaxation is assumed.

Proposal 2: since single polarization is assumed for FR2-1 NTN, the existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with 1dB relaxation. (i.e., revert the last meeting agreement in issue 6-2-2)

The TN measurement accuracy is defined as following example:

The parameter of SSB_RP has been specified in annex B of TS38.133, as following example:

The UE power class for legacy FR2 TN UE may not be applied for NTN UE above 10GHz. In UE RF spec session, one big CR has been agreed in last meeting in [3], and the UE class were defined quite different from legacy power class, as duplicated below,

Thus, instead of using UE power class, RAN4 shall define the minimum SSB_RP condition for accuracy requirement for 5 UE types specified in table 9.2.1.0-1 of TS38.101-5.

Proposal 3: instead of using UE power class, RAN4 shall define the minimum SSB_RP condition for accuracy requirement of five UE types specified in table 9.2.1.0-1 of TS38.101-5.

In the legacy TN FR2 accuracy requirement, the minimum SSB power is determined based on the following criteria:

In NTN above 10GHz, the same principle can be applied to derive the mini SSB_RP, which is based on the equation of Minimum SSB_RP = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n, where,

Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5 as following:

Y is the gain difference between fine and rough beams, which needs to be further discussed in RAN4; (the fine beam and rough beam gain for VSAT might be different from legacy TN FR2 UE)

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is 0 for NTN above 10GHz since CA/DC is not considered.

Proposal 4: RAN4 to define:

Minimum SSB_RP_NTN_FR2 = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n,

where,

Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5

Y is the gain difference between fine and rough beams, which needs FFS in RAN4

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is 0 for NTN above 10GHz since CA/DC is not considered.

To derive the Rx beam gain in the proposal 4, we may consider the following formulas,

- REFSENS_50MHz = -174 dBm/Hz + 10log10(50MHz) + NF + SNR (QPSK) + IM - Ant_Gain

Therefore Ant_Gain can be derived as,

- Ant_Gain = -174 dBm/Hz + 10log10(50MHz) + NF + SNR (QPSK) + IM - REFSENS_50MHz

For FR2-NTN, two REFSENS_50MHz values have been specified,

- -126.8 dBm for Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO

- -115.6 dBm for fixed VSAT communication with LEO only

With the following parameter assumptions,

- NF = 10 dB (we used a typical value here from previous FR2 discussion in RF, but can be adjusted if companies have concern)

- SNR = -1 dB

- IM = 2 dB (here, it’s just our understanding for example, may need FFS in RAN4 to decide the value among companies)

We can derive the following two Ant_Gain values,

- For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO:

  - Ant_Gain = -174dBm + 77dB + 10dB + (-1dB) + 2dB - (-126.8dBm) = 40.8dB

- For fixed VSAT communication with LEO only,

  - Ant_Gain = -174dBm + 77dB + 10dB + (-1dB) + 2dB - (-115.6dBm) = 29.6dB

Thus, based on our calculation the upper bound of Rx beam gain can be as following:

- For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO, the upper bound of Rx beam gain is 41dB.

- For fixed VSAT communication with LEO only, the upper bound of Rx beam gain is 30dB.

Proposal 5:

For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO, the upper bound of Rx beam gain is 41dB.

For fixed VSAT communication with LEO only, the upper bound of Rx beam gain is 30dB.

# Conclusion

In this contribution, we discuss the NR-NTN measurement accuracy requirements in above 10 GHz bands.

Proposal 1: remove an RF margin for different RX beams in the relative accuracy and tighten relative accuracy requirements.

Proposal 2: since single polarization is assumed for FR2-1 NTN, the existing absolute measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with 1dB relaxation. (i.e., revert the last meeting agreement in issue 6-2-2).

Proposal 3: instead of using UE power class, RAN4 shall define the minimum SSB_RP condition for accuracy requirement of five UE types specified in table 9.2.1.0-1 of TS38.101-5.

Proposal 4: RAN4 to define:

Minimum SSB_RP_NTN_FR2 = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n,

where,

Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5

Y is the gain difference between fine and rough beams, which needs FFS in RAN4

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is 0 for NTN above 10GHz since CA/DC is not considered.

Proposal 5:

For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO, the upper bound of Rx beam gain is 41dB.

For fixed VSAT communication with LEO only, the upper bound of Rx beam gain is 30dB.

# References

[1] R4-2406430, WF on RRM requirements for NR_NTN_enh, Qualcomm, RAN4 #110bis

[2] R4-2406515, Big CR to TS 38.133 on RRM requirements for NR NTN enhancement, Qualcomm, RAN4#110bis

[3] R4-2401116, Big CR on TS38.101-5 for UE RF Requirements, Samsung Electronics France SA, RAN4 #110