3GPP TSG-RAN WG4 Meeting #111	R4-2407315

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.764 | CR | 0002 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_FDD_LS_band) CR to 36.764 for IoT-NTN UE RF requirements (Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the clause 2 reference, the reference TS needs to be added. Secondly, reference tables need to be correctly referred. Finally, the ambiguous content of FDD band B254 (L+S band) needs to be modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TS 36.101 and TS 36.102 are added in the clause 2. - Table 5.3A-1 and Table 6.5A.4.2-2 are referred to TS 36.102 - In clause 6 FDD band B254 (L+S band), requirements of B256 and B255 are removed. - Correct typo in clauses 6.2.1.3, 6.2.1.4, 6.2.1.5, 6.2.1.6, 6.2.3.1 and 6.2.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference about technical specification is not provided. The reference tables are incorrectly referred. It is ambiguous about content for FDD band B254 (L+S band). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.1, 6.2.2, 6.2.3.1, 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================Start of the change 1==========================

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.741: “Non-Terrestrial Networks (NTN) L-/S-band for NR”.

[3]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”.

[4]	3GPP TS 36.102: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”.

====================End of the change 1==========================

====================Start of the change 2==========================

6.2.1.2	Emission requirements and NS values

6.2.1.2.1	Spurious emission for category M1

Table 6.2.1.2.1-1: Requirements for spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 254 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 54, 66, 70, 71, 72, 85, 87, 88, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n54, n65, n67, n74, n75, n76, n77, n78, n90, n91, n92, n93, n94, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.1.1-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in TS 36.102 [4] Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.2.1.3	Configured TX power

It is agreed the current TS 36.102 [4] requirements can be re-used for band B254.

6.2.1.4	Power control

It is agreed the current TS 36.102 [4] requirements can be re-used for band B254.

6.2.1.5	Frequency error

It is agreed the current TS 36.102 [4] requirements can be re-used for band B254.

6.2.1.6	Transmit modulation quality

It is agreed the current TS 36.102 [4] requirements can be re-used for band B254.

====================End of the change 2==========================

====================Start of the change 3==========================

6.2.2	UE receiver characteristics

6.2.2.1	Reference sensitivity for category M1 and NB1/NB2

Table 6.2.2.1-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101 [3]. |  |  |

Table 6.2.2.1-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -103.1 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101 [3]. |  |  |

Table 6.2.2.1-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 254 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3A-1 in TS 36.102 [4]). |  |  |

Table 6.2.2.1-4: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| 254 | - 108.2 |

<Unchanged sections omitted>

====================End of the change 3==========================

====================Start of the change 4==========================

6.2.3.1	Evaluation and simulation on Cat. M1

The 3GPP emission mask for the 1.4MHz channel is generally either the same or stricter comparing to the ETSI or FCC requirements, and there is one case for the ETSI in-band emission mask for the 1618.25-1626.5MHz range where the ETSI mask is stricter. To check whether existing MPR requirements still apply, we conduct a set of simulation cases with the 1.4MHz LTE channel placed in different locations. The common simulation parameters are as follows:

-	Tx power: PC3 +23dBm

-	Modulation: DFT-s-OFDM QPSK

-	LO placement: always in the centre of the carrier

-	Regulations: ETSI and FCC

As for the actual channel configurations, Figure 6.2.3.1-1 below shows three 1.4MHz channel with centre frequencies of 1610.7MHz, 1618.95MHz and 1625.8MHz.

Figure 6.2.3.1-1: Considered channels for power back-off simulations.

<Unchanged sections omitted>

====================End of the change 4==========================

====================Start of the change 5==========================

6.2.3.2	Evaluation and simulation on Cat. NB1/NB2

For A-MPR for category NB1/NB2, simulations where a QPSK signal with different allocations including 3 tones, 6 tones and 12 tones is generated and passed to a typical PC3 power amplifier. And LO is set to the center of the carrier. The simulation results are shown in the following figures from R4-2317572 and R4-2318708.

a.	3-tone [0-2] allocation, FCC  No A-MPR is required

b.	3-tone [0-2] allocation, ETSI, Fc = 1610 – 1618.25 MHz  A-MPR = 0.7dB

c.	3-tone [0-2] allocation, ETSI, Fc = 1618.25 – 1626.5 MHz  A-MPR = 1.5 dB

Fig. 6.2.3.2-1, MPR simulation results with 3-tone allocation for FCC and ETSI requirements.

d.	6-tone [0-6] allocation, FCC  No A-MPR is required

e.	6-tone [0-5] allocation, ETSI, Fc = 1610 – 1618.25 MHz  A-MPR = 0

f.	6-tone [0-5] allocation, ETSI, Fc = 1618.25 – 1626.5 MHz  A-MPR = 0.7dB

Fig. 6.2.3.2-2, MPR simulation results with 6-tone allocation for FCC and ETSI requirements.

g.	FCC

h.	ETSI, Fc = 1618.25 MHz – 1626.5 MHz

i.	ETSI, Fc = 1610 MHz – 1618.25 MHz

Fig. 6.2.3.2-3, MPR simulation results with 12-tone allocation for FCC and ETSI requirements.

<Unchanged sections omitted>

====================End of the change 5==========================