# 3GPP TSG-RAN WG4 Meeting #111	R4-2407358

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.4.3

Source:	Apple

Title:	On FG31-1 test cases for Scell activation enhancement

Document for:	Discussion

# Introduction

In last RAN4#110bis meeting, the WF for SCell activation has been agreed in [1]. Almost all the core part issues have been addressed, and there was still one small issue remained for FG31-1 TCs. In this contribution, we discuss the remaining issue for FG31-1 TCs.

# Discussion

Issue 2-2: whether to verify requirement without L3 reporting for FG31-1 TC

Issue 2-1: How to configure M to have valid L3 report for FG31-1 TC Agreement: - Calculate the M values based on UE capability in the corresponding sections. Agreement: DCI transmission timing: cover two case: - Case 1: n+3ms+THARQ+M-k2 - FFS: UE pass condition is either UE to L1 or L3 report. - Case 2: n+[7]ms+ THARQ - where k2=1 with type A mapping and startSymbolAndLength (SLIV) = 42 (L=4, and S=0). DL scheduling for data starts from n+3ms. Issue 2-2: whether to verify requirement without L3 reporting for FG31-1 TC Agreement: - For all FG31-1 TCs, only verify the activation delay when L3 report is triggered. - FFS UE reporting L1 is also a pass condition. Issue 2-3: whether to verify requirement for L1 report before and after TCI configuration in FG31-1 TC - Option 1 (CMCC): - for the test cases on L3 report based enhancement, it is proposed to consider both the case with L1 measurement and the case without L1 measurement. - Option 2 (CTC, Apple): - When TCI configuration are based on L1-RSRP reporting, legacy requirements and existing test cases can be applied. It’s proposed only to define test case for TCI configuration based on the latest valid L3-RSRP reporting, i.e., the latest valid L3-RSRP reporting is before TCI configuration. - Agreement: - Up to the conclusion of “FFS” part in issue 2-2

Technically, both L1 report and L3 reporting can used for network to activate the corresponding TCI and complete the SCell activation procedure. From requirement perspective, we have following uncertainty definition:

The requirement can accommodate adaptively to the cases of L1 report or L3 report. Thus, if UE can report L1-RSRP before the target L3 reporting time (DCI triggered PUSCH), such UE shall still be treated as “pass the test”. RAN4 can consider adding applicability clarification in the test cases.

Proposal 1: For all FG31-1 TCs, only verify the activation delay when L3 report is triggered. But if UE can report L1-RSRP earlier than the target L3 reporting time (DCI triggered PUSCH), such UE can still be treated as “pass the test” as long as it can complete the SCell activation within the delay requirement of FR2 SCell activation enhancement with FG31-1.

RAN4 can consider adding above applicability clarification in the test cases.

# Conclusion

In this contribution, we discuss the remaining issue for FG31-1 TCs.

Proposal 1: For all FG31-1 TCs, only verify the activation delay when L3 report is triggered. But if UE can report L1-RSRP earlier than the target L3 reporting time (DCI triggered PUSCH), such UE can still be treated as “pass the test” as long as it can complete the SCell activation within the delay requirement of FR2 SCell activation enhancement with FG31-1.

RAN4 can consider adding above applicability clarification in the test cases.

# References

[1] R4-2406339, Way Forward for [110bis][207] NR_RRM_enh3_part1, Apple, RAN4 #110bis