3GPP TSG-RAN WG4 Meeting #111	R4-2407373

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Murata Manufacturing Co Ltd.

Title:	TP for TR 38.894: CA_n39A-n40A-n41A

Agenda item:			6.20.2

Document for:	Approval

Introduction

This contribution is a text proposal for TR 38.894 to update the requirements of CA_n39A-n40A-n41A.

Text Proposal

<Start of Text Proposal>

5.6	CA_n39A-n40A-n41A

5.6.1	Status of the band combination

5.6.1.1	Operating bands for CA

The operating bands for CA_n39-n40-n41are specified in Table 5.6.1.1-1.

Table 5.6.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n39-n40-n41 | n39, n40, n41 |  |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n39-n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx for n39 and n41, for n40 and n41.

5.6.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n39-n40-n41 are specified in Table 5.6.1.2-1

Table 5.6.1.2-1: ΔTIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.3 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

ΔRIB,c due to CA_n39-n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.6.1.3	Existing MSD requirement

There is no Reference sensitivity exceptions due to intermodulation interference due to 2UL CA for CA_n39-n40-n41, considering no simultaneous Rx-Tx for n39 and n41, n40 and n41.

5.6.2	MSD analysis for simultaneous Rx/Tx

5.6.2.1	Reference UE architecture

In Murata paper R4-2404222, two different reference architecture options using triplexer are provided.

(a)

(b)

Figure 5.6.2.1-1: Reference architecture using shared antenna (R4-2404222, Murata)

In Huawei paper R4-2404513, the architecture of n40 and n41 sharing antenna switch is provided.

Figure 5.6.2.1-2: Reference UE architecture for CA_n39-n40-n41 (R4-2404513, Huawei)

In Qualcomm paper R4-2405452, the architecture of n40 and n41 sharing antenna switch is provided.

Figure 5.6.2.1-3: Reference UE architecture for CA_n39-n40-n41 (R4-2405452, Qualcomm)

5.6.2.2	RF component assumptions

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2404222, Murata).

Table 5.6.2.2-1: Intercept point parameters (R4-2404222)

|  | IP3 |
| --- | --- |
| PA forward | 30 |
| PA reverse | 29 |
| Switch | 68 |
| Filter (Triplexer) | 74 |
| LNA | -5 |

Table 5.6.2.2-2: Filter attenuation/isolation, PCB and antenna isolation	(R4-2404222)

| n39 filter attenuation at n40 | 40 |
| --- | --- |
| n39 filter attenuation at n41 | 40 |
| n40 filter attenuation at n39 | 35 |
| n40 filter attenuation at n41 | 40 |
| n41 filter attenuation at n39 | 35 |
| n41 filter attenuation at n40 | 35 |
| Isolation btw n39 and n40 at n39 | 45 |
| Isolation btw n39 and n40 at n40 | 40 |
| Isolation btw n39 and n41 at n39 | 45 |
| Isolation btw n39 and n41 at n41 | 40 |
| Isolation btw n40 and n41 at n40 | 40 |
| Isolation btw n40 and n41 at n41 | 40 |
| Antenna isolation | 15 |
| PCB isolation | 60 |

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2405413, Huawei).

Table 5.6.2.2-3: Intercept point parameters (R4-2405413)

| Component | IP3(dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Duplexer | 74 |
| PA forward mixing | 32 |
| PA reverse mixing | 30 |
| LNA | -6 |

Table 5.6.2.2-4: Filter attenuation/isolation, PCB and antenna isolation	(R4-2405413)

| n41 Tx Rejection at n40 Tx | 40 |
| --- | --- |
| n41 Tx Rejection at n39 Rx | 40 |
| n40 Tx Rejection at n41 Tx | 40 |
| n40 Tx Rejection at n39 Rx | 35 |
| n39 Rx Rejection at n40 Tx | 40 |
| n39 Tx Rejection at n41 Rx | 40 |
| Diplexer isolation | 15 |
| PCB isolation | 60 |

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2405452, Qualcomm).

Table 5.6.2.2-5: Intercept point parameters, filter attenuation/isolation, PCB and antenna isolation (R4-2405452)

5.6.2.3	Calculated MSD values

Proposed Option 1: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2404222, Murata).

Table 5.6.2.3-1: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2404222)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 31.6 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1917.5 | 31.6 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Proposed Option 2: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2405413, Huawei).

Table 5.6.2.3-2: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2405413)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 27.1 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1915 | 16.3 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Proposed Option 3: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2405452, Qualcomm).

Table 5.6.2.3-2: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2405452)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 |  | 5 | 25 |  | N/A | TDD | N/A |
|  | n40 |  | 5 | 25 |  | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | [30.9] | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1915 | [22.5] | TDD | IMD31 |
|  | n40 |  | 5 | 25 |  | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 |  | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.6.3	Requirements for simultaneous Rx/Tx

5.6.3.1	∆TIB and ∆RIB values

Sufficient attenuation level between bands should be considered in ΔTIB,c and ΔRIB,c , given that increased insertion loss of the triplexer compared to single filter (R4-2404222).

Table 5.6.3.1-1: ΔTIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.6 | 0.6 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.6.3.1-2: ΔRIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.6 | 0.6 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.6.3.2	Reference sensitivity requirements

Table 5.6.3.2-1: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2405452)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 30.3 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1917.5 | 27.4 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

<End of Text Proposal>