# 3GPP TSG-RAN WG4 Meeting #111	R4-2407413

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source:	Sony

Title:	UE RF requirement for NTN Redcap and eRedcap

Agenda item:	10.15.2.1

Document for:	Approval

# Background

The WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in RAN#102 [1], and the following task on support Redcap and eRedcap for NTN operation has been included:

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1]

- For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4]

- For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1]

- Depending on feasibility assessment above, define the RF and RRM requirements [RAN4]

Notes for this objective:

GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In RAN4#110bis, multiple agreements have been made which creates a basis of NTN Redcap RF requirements [2]. In this contribution, we provide our view on the remaining missing RF requirement for NTN Redcap and eRedcap in Rel-19.

# Discussion

The Rel-17 Redcap RF requirements are leveraged from the normal NR devices with applicable bandwidths, the power classes, and the numbers of antennas, with the inclusion of HD-FDD mode as well. In addition, further limitations on the RB allocation for REFSENS requirements are introduced in Rel-18 eRedcap requirement to support the reduced baseband bandwidth operation.

The following agreements have been made with respect to the UE channel BW in RAN4#110bis [2]:

- All the NR-NTN FR1-NTN bands will be considered.

- Adopt the same bandwidth as well as SCS as TN RedCap and TN eRedCap UE.

- Specify both eRedCap variants, i.e., with and without bandwidth reduction.

Based on supported bandwidth and device type for Redcap NTN, the following proposals for the corresponding transmission and reception requirements are being proposed:

## Transmission requirement

In RAN4#110bis, the following agreement has been made for the transmission requirements [2]:

- Target power class is PC3.

Therefore, for the transmission requirement, the Rel-17 Redcap and Rel-18 eRedcap devices reuse the corresponding NR requirements of PC3. For the NTN Redcap and eRedcap, the general transmission requirements might be able to be re-used. However, considering the regulatory requirements (with NS signallings) maybe very different between TN and NTN, it is suggested to further verify if the corresponding requirements can be reused (e.g., AMPR).

Observation 1: For the NTN Redcap and eRedcap, the general transmission requirements might be reusable, but further check on the regulatory requirements maybe needed.

Proposal 1: Further check if any update is needed for NTN Redcap and eRedcap with respect to regulatory requirements with considering the feasibility of Redcap and eRedcap implementations.

## Reception requirement

In RAN4#110bis, the following agreement has been made for the reception requirements [2]:

- Re-use non-RedCap NTN UE refsens for 2Rx FD-FDD NTN (e)RedCap UE

- Re-use TN (e)RedCap UE 1 Rx refsens allowance for NTN (e)RedCap UE as follows

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

- Start the discussions on the RF requirements for NTN RedCap with HD-FDD in the next meeting (RAN4#111).

Therefore, the remaining issue for the REFSENS is the requirements for HD-FDD and the RB configuration for eRedcap REFSENS.

During Rel-17 Redcap HD-FDD REFSENS discussion, the baseline is that 0.8 dB tightening compared to FD-FDD is adopted but with a few bands that have exceptions considering the less insertion loss in filter + switch compared to a duplexer, and no Tx interference in HD-FDD mode. However, a few bands are taken as exceptions with less tightening since their FD-FDD REFSENS are already very stringent (in a similar level as TDD REFSENS in a similar frequency range), which shows that such an improved sensitivity may have been considered already in the FD-FDD requirement [3]. Essentially, the rule of tightening was originally proposed as below [3]:

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Therefore, it is proposed that the same approach can be used to derive the REFSENS for NTN Redcap HD-FDD, and it will be 0 dB tightening for n255, and 0.5 dB tightening for n254 and n256 for 5MHz BW. Therefore, the REFSENS for n256, n255, and n254 will be all -100 dBm/5MHz for HD-FDD.

Proposal 2: It is proposed to adopt the same tightening rule as in Rel-17 Redcap to derive the REFSENS for NTN Redcap HD-FDD, which results in 0 dB tightening for n255, and 0.5 dB tightening for n254 and n256 for 5MHz, e.g., REFNSEN = -100 dBm for all three NR bands in NTN with 5MHz BW for 2Rx.

For larger BW, it was agreed in Rel-17 Redcap to scale the REFSENS value based on the number of configured RB, which can be calculated as REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS, n=2 for 30kHz SCS, and n=4 for 60kHz SCS. The same approach can be used for NTN as well.

Proposal 3: NTN Redcap HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS, n=2 for 30kHz SCS, and n=4 for 60kHz SCS.

For Rel-18 eRedcap that supports NOT reduced baseband bandwidth, the same REFSENS as Rel-17 Redcap is applied.

For reduced baseband bandwidth eRedcap, for 15kHz SCS, the 5MHz REFSENS requirement applies to 5, 10, 15, and 20 MHz RF bandwidth as the PDSCH is limited to 5MHz regardless of the RF bandwidth. For larger SCS, e.g., 30 kHz, where 5MHz channel RF bandwidth does not apply, the REFSENS of 10 MHz is applied with 3 dB relaxation. In our view, those principles can be directly reused for NTN eRedcap UE, and the detail description is captured in 7.3I.3 of 38.101-1.

Proposal 4: Reuse the same way as in Rel-18 eRedcap REFSENS (as in 7.3I.3 of 38.101-1) to define the NTN eRedcap REFSENS.

For other reception requirements, we believe it should be feasible to reuse the NR NTN requirements. RAN4 can further check if there is any special case that needs to be dealt with for NTN Redcap and eRedcap.

Proposal 5: RAN4 further checks if other reception requirements from NR NTN can be reused for NTN Redcap and eRedcap.

# Conclusion

In this paper, we presented our view on the RF requirement for NTN Redcap and eRedcap, where the following observation and proposals are made:

Observation 1: For the NTN Redcap and eRedcap, the general transmission requirements might be reusable, but further check on the regulatory requirements maybe needed.

Proposal 1: Further check if any update is needed for NTN Redcap and eRedcap with respect to regulatory requirements with considering the feasibility of Redcap and eRedcap implementations.

Proposal 2: It is proposed to adopt the same tightening rule as in Rel-17 Redcap to derive the REFSENS for NTN Redcap HD-FDD, which results in 0 dB tightening for n255, and 0.5 dB tightening for n254 and n256 for 5MHz, e.g., REFNSEN = -100 dBm for all three NR bands in NTN with 5MHz BW for 2Rx.

Proposal 3: NTN Redcap HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS, n=2 for 30kHz SCS, and n=4 for 60kHz SCS.

Proposal 4: Reuse the same way as in Rel-18 eRedcap REFSENS (as in 7.3I.3 of 38.101-1) to define the NTN eRedcap REFSENS.

Proposal 5: RAN4 further checks if other reception requirements from NR NTN can be reused for NTN Redcap and eRedcap.

# References

- RP-234078  New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- R4-2406610  WF on NR_NTN_Ph3_UERF  Qualcomm

- R4-2200442  RedCap UE HD-FDD REFSENS requirements   Apple