3GPP TSG-RAN WG4 Meeting # 111	R4-2407432

Fukuoka, Japan, 20 – 24 May 2024

Agenda Item:	7.8.1

Source: 	Nokia

Title: 	Proposal on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1

Document for:	Approval

# 1.	Introduction

RAN4 received a LS from RAN1 at RAN4#100 in R4-2400012 [1] about an issue on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1, and sent a reply LS in R4-2403852 [2] to acknowledge the issue and invite RAN2 to handle this in RAN2 specifications. RAN2 sent a reply LS in R4-2407118 [3] to request RAN4 to define the ‘reserved’ GSCN / ARFCN-ValueNR and NR operating band and capture in RAN4 specifications, and provide feedback, if any.

This contribution further discusses this issue and provide a proposal from RAN4 specifications viewpoint.

# 2.	Discussion

It was discussed at RAN4#100 that one proposal in RAN2 required RAN4 to reserve a GSCN value in the RAN4 specifications. The need to reserve a GSCN value to be used in the legacy list was stated in R2-2400430 [4]:

The interFreqCarrierFreqList in SIB4 is mandatory to be present with at least one element, and within that element, dl-CarrierFreq is also mandatory field. This means the legacy list cannot be omitted. And in the scenarios where all the neighbours are <5MHz, there needs to be a way to signal such that legacy UEs know how to ignore the legacy list (while the new UEs use the new list).

It was requested by RAN2 in R2-2403960 [3]:

RAN2 requests RAN4 to define these ‘reserved’ values of GSCN =2 / ARFCN-ValueNR = 250 and NR operating band = 1024, and capture in RAN4 specifications, e.g. in Table 5.4.3.1-1 and Table 5.2-1 in TS 38.101-1.

However, GSCN = 2 and NR operating band = 1024 do not match RAN4 current allocation of operating band number in different frequency ranges, as GSCN = 2 maps to 1250 kHz synchronization raster frequency which is included for the GSCN range in 0 – 3000 MHz in TS 38.104 [5] as extracted below, while NR operating band numbers >256 are reserved in RAN4 for operating bands in FR2.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 3000; N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5} (Note); 3N + (M-3)/2; 2 – 7498. 3000 – 24250; 3000 MHz + N * 1.44 MHz, N = 0:14756; 7499 + N; 7499 – 22255. 24250 – 100000; 24250.08 MHz + N * 17.28 MHz, N = 0:4383; 22256 + N; 22256 – 26639. NOTE: The default value for operating bands which only support SCS spaced channel raster(s) is M=3..

Therefore, it is proposed to reserve GSCN = 1, which is not included in any frequency range in RAN4 specifications, instead of GSCN = 2 per RAN2 request.

# 3.	Conclusion

This contribution has further discussed the issue on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1 and provided a proposal from RAN4 specifications viewpoint.

Proposal 1: To reserve GSCN = 1, which is not included in any frequency range in RAN4 specifications, instead of GSCN = 2 per RAN2 request.

# References

[1]	R4-2400012, “LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1”, Qualcomm.

[2]	R4-2403852, “Response to LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1”, Huawei.

[3]	R4-2407118, “Reply LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1”, Qualcomm.

[4]	R2-2400430, “Discussion regarding LS on inter-frequency neighbour cells supporting NR dedicated spectrum less than 5 MHz for FR1”, Qualcomm.

[5]	3GPP TS 38.104 v18.5.0: “NR; Base Station (BS) radio transmission and reception”.