3GPP TSG-RAN WG4 Meeting # 111	R4-2407440

Fukuoka, Japan, 20 – 24 May 2024

Agenda Item:	10.14.3

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#103 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#110bis, and the Way Forward proposals were agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

A few issues were FFS in the agreed WF as follows:

FFS: - The applicable BS types. e.g. BS type 1-C, BS type 1-H and BS type 1-O - Power degradation of RBs other than LP-WUS signal within the carrier - Whether BS power boosting is declaration based or not - Others

## The applicable BS types. e.g. BS type 1-C, BS type 1-H and BS type 1-O

As listed in the topic summary [3], there are different views on the applicable frequency range and bands for LP-WUS. Therefore, it is proposed to decide on the applicable BS types after the applicable frequency range and bands for LP-WUS have been decided.

## Power degradation of RBs other than LP-WUS signal within the carrier

It was agreed in the WF:

Way forward: - Concept of LP-WUS dynamic range/power boosting is considered as starting point - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - With following assumption to guide future discussion, RAN4 will revisit the above concept after BS core requirements are settled. - Transmission power is shared between NR OFDM signal and NR LP-WUS for the same carrier. - The rated carrier output power and rated total output power are not changed with LP-WUS power boosting.

Therefore, the available power of RBs other than LP-WUS signal within the carrier will be reduced to provide the power required for LP-WUS power boosting. Some analyses on the potential impact, depending on the power and number of boosted LP-WUS RBs, were provided in tables 1 to 3 below. It can be seen that the power reduction for other RBs varies with the the number and power of boosted LP-WUS RBs. Therefore, it is proposed to consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1.

Table 1: Impact of 3 dB LP-WUS power boosting

| LP-WUS power boosting (dB) | SCS (KHz) | Number of RBs | LP-WUS RBs | Rated carrier output power (dBm) | Channel bandwidth (MHz) | Power reduction for other RBs (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 25 | 24 | 46 | 5 | N/A |
| 3 | 15 | 52 | 24 | 46 | 10 | -8.33 |
| 3 | 15 | 79 | 24 | 46 | 15 | -2.47 |
| 3 | 15 | 106 | 24 | 46 | 20 | -1.50 |
| 3 | 15 | 133 | 24 | 46 | 25 | -1.07 |
| 3 | 15 | 160 | 24 | 46 | 30 | -0.84 |
| 3 | 15 | 188 | 24 | 46 | 35 | -0.68 |
| 3 | 15 | 216 | 24 | 46 | 40 | -0.58 |
| 3 | 15 | 242 | 24 | 46 | 45 | -0.50 |
| 3 | 15 | 270 | 24 | 46 | 50 | -0.44 |
| 3 | 30 | 162 | 12 | 46 | 60 | -0.36 |
| 3 | 30 | 189 | 12 | 46 | 70 | -0.30 |
| 3 | 30 | 217 | 12 | 46 | 80 | -0.26 |
| 3 | 30 | 245 | 12 | 46 | 90 | -0.23 |
| 3 | 30 | 273 | 12 | 46 | 100 | -0.20 |

Table 2: Impact of 4 dB LP-WUS power boosting

| LP-WUS power boosting (dB) | SCS (KHz) | Number of RBs | LP-WUS RBs | Rated carrier output power (dBm) | Channel bandwidth (MHz) | Power reduction for other RBs (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 15 | 25 | 24 | 46 | 5 | N/A |
| 4 | 15 | 52 | 24 | 46 | 10 | N/A |
| 4 | 15 | 79 | 24 | 46 | 15 | -4.68 |
| 4 | 15 | 106 | 24 | 46 | 20 | -2.54 |
| 4 | 15 | 133 | 24 | 46 | 25 | -1.76 |
| 4 | 15 | 160 | 24 | 46 | 30 | -1.35 |
| 4 | 15 | 188 | 24 | 46 | 35 | -1.09 |
| 4 | 15 | 216 | 24 | 46 | 40 | -0.91 |
| 4 | 15 | 242 | 24 | 46 | 45 | -0.79 |
| 4 | 15 | 270 | 24 | 46 | 50 | -0.69 |
| 4 | 30 | 162 | 12 | 46 | 60 | -0.56 |
| 4 | 30 | 189 | 12 | 46 | 70 | -0.47 |
| 4 | 30 | 217 | 12 | 46 | 80 | -0.40 |
| 4 | 30 | 245 | 12 | 46 | 90 | -0.35 |
| 4 | 30 | 273 | 12 | 46 | 100 | -0.31 |

Table 3: Impact of 6 dB LP-WUS power boosting

| LP-WUS power boosting (dB) | SCS (KHz) | Number of RBs | LP-WUS RBs | Rated carrier output power (dBm) | Channel bandwidth (MHz) | Power reduction for other RBs (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 15 | 25 | 24 | 46 | 5 | N/A |
| 4 | 15 | 52 | 24 | 46 | 10 | N/A |
| 4 | 15 | 79 | 24 | 46 | 15 | N/A |
| 4 | 15 | 106 | 24 | 46 | 20 | -8.95 |
| 4 | 15 | 133 | 24 | 46 | 25 | -4.64 |
| 4 | 15 | 160 | 24 | 46 | 30 | -3.24 |
| 4 | 15 | 188 | 24 | 46 | 35 | -2.49 |
| 4 | 15 | 216 | 24 | 46 | 40 | -2.02 |
| 4 | 15 | 242 | 24 | 46 | 45 | -1.73 |
| 4 | 15 | 270 | 24 | 46 | 50 | -1.49 |
| 4 | 30 | 162 | 12 | 46 | 60 | -1.18 |
| 4 | 30 | 189 | 12 | 46 | 70 | -0.98 |
| 4 | 30 | 217 | 12 | 46 | 80 | -0.83 |
| 4 | 30 | 245 | 12 | 46 | 90 | -0.72 |

## Whether BS power boosting is declaration based or not

Since support of LP-WUS power boosting would increase BS implementation complexity, similar to NB-IoT power boosting in TS 36.141 [4] and TS 38.141-1 [5], it is proposed to allow BS manufacturer to declare the supported LP-WUS power boosting level in the manufacturer declarations for BS supporting LP-WUS operation, and the power boosting levels allowed to be declared by BS manufacturer can be further discussed.

# 3.	Conclusion

This contribution has provided proposals to progress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: To decide on the applicable BS types after the applicable frequency range and bands for LP-WUS have been decided.

Proposal 2: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1.

Proposal 3: To allow BS manufacturer to declare the supported LP-WUS power boosting level in the manufacturer declarations for BS supporting LP-WUS operation, and the power boosting levels allowed to be declared by BS manufacturer can be further discussed.

# References

[1]	RP-240801, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony.

[2]	R4-2406140, “Way Forward for [110bis][314] NR_LPWUS”, Huawei.

[3]	R4-2405834, “Topic summary for [110bis][314] NR_LPWUS”, Moderator (Huawei).

[4]	3GPP TS 36.141 v18.4.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[5]	3GPP TS 38.141-1 v18.5.0: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.