3GPP TSG-RAN WG4 Meeting #110bis                                                                                        R4-2407476

Fukuoka, Japan,  May 20  – May 24, 2024

Source: 	CATT

Title: 	Simulation results for SAN demodulation requirements for above 10 GHz bands

Agenda item:	7.16.8.1

Document for:	Approval

- Introduction

In RAN4#110bis meeting, the simulation assumption on demodulation requirements for NR NTN enhancements was agreed [1].

This contribution provides initial simulation results for SAN demodulation requirements for alignment.

- Discussion

Based on the agreed simulation assumptions [1], presented below are our preliminary simulation outcomes for PUSCH and PRACH concerning SAN demodulation:

2.1 Ka-band PUSCH

The simulation results for PUSCH are shown in Table2.1-1.

Table 2.1-1: Simulation results for PUSCH

| Normal PUSCH with CP-OFDM |  | CATT |  |
| --- | --- | --- | --- |
|  |  | Ideal | Impairment |
| MCS2 | 1x1 | -2.6 | -0.1 |
| MCS2 | 1x2 | -6.0 | -3.5 |
| MCS12 | 1x1 | 6.4 | 8.9 |
| MCS12 | 1x2 | 3.0 | 5.5 |
| Normal PUSCH with DFT-s-OFDM |  | CATT |  |
|  |  | Ideal | Impairment |
| MCS2 | 1x1 | -2.5 | 0.0 |
| MCS2 | 1x2 | -5.8 | -3.3 |
| PUSCH repetition Type A |  | CATT |  |
|  |  | Ideal | Impairment |
| MCS2 | 1x1 | -5.6 | -5.2 |
| MCS2 | 1x2 | -8.7 | -8.3 |

2.2 Ka-band PRACH

The simulation results for PRACH missed detection are shown in Table 2.2-1.

Table 2.2-1: Simulation results for PRACH missed detection

| PRACH |  | CATT |  |
| --- | --- | --- | --- |
|  |  | Ideal | Impairment |
| B4 | 1x1 | 0.52 | 3.0 |
| B4 | 1x2 | -7.4 | -4.9 |
| C2 | 1x1 | 6.28 | 8.8 |
| C2 | 1x2 | -3.65 | -1.2 |

- Conclusion

This contribution provides our simulation results for SAN demodulation requirement for above 10GHz bands for alignment.

- References

- R4-2405146, Simulation assumption on demodulation requirements for NR NTN enhancements, Huawei, HiSilicon, RAN4#110bis