3GPP TSG-RAN WG4 Meeting #111                                                                                                      R4-2407478

Fukuoka, Japan, May 20th – May 24th, 2024

Source: 	CATT

Title: 	Discussion on co-existence elevation for ambient-IoT in NR

Agenda item:	10.13.2.2

Document for:	Approval

- Introduction

In RAN4#110-bis meeting, a WF [1] on Ambient IoT in NR was agreed and the remaining issues on co-existence elevation in WF require further discussion.

This contribution, we provide our views on co-existence elevation for ambient-IoT in NR.

- Discussion

2.1 Evaluation methodology

Performance metric for AIOT

As per the WF [1], the remaining open issue concerning performance metric for AIoT is shown as below:

Issue 2-4-2: Performance metric for AIOT Agreement: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded

For BLER and Rx power in option 1, we think that it is hard to find appropriate BLER degradation or Rx power degradation threshold value as performance metric for A-IoT like 5% throughput loss threshold value for legacy NR.

For SINR degradation in option 2, let’s first analyze SINR degradation corresponding to the 5% throughput loss for legacy NR. Referring to the link level performance for 5G NR existence in sub-clause 5.2.7 of TR 38.803, the SINR degradation  can be achieved using the following equations:

(Equation 1)

Where,

(Equation 2)

According to the Table 5.2.7-1 of TR 38.803, the value for DL and UL is 0.6 and 0.4, respectively, and  for DL and UL is 30dB and 20dB, respectively. Using equation 1, we can get that SINR degradation is 1.4959dB and 0.9589dB, respectively.

Observation 1: SINR degradation for DL and UL corresponding to 5% throughput loss for legacy NR coexistence evaluation is 1.4959dB and 0.9589dB, respectively.

Referring to Table 6.4.13-1 of TR 36.802, the Criteria for NB-IoT coexistence evaluation is that average of SNR value loss is less or equal to 1 dB, the SNR degradation (SNR loss) is as performance metric for NB-IoT existence evaluation.

Observation 2: 1 dB SNR degradation (SNR loss) was used as performance metric for NB-IoT existence evaluation in TR 36.802.

Referring to above analysis for SNR degradation in legacy NR and NB-IoT coexistence evaluation, it is reasonable to adopt SINR degradation as performance metric for A-IoT co-existence evaluation. For A-IoT, SINR degradation can directly indicate that the coverage distance is decreased, and coverage distance loss is more important performance requirement for A-IoT, so we propose to adopt SINR degradation (Option 2) as performance metric for A-IoT coexistence evaluation.

Proposal 1: Adopt SINR degradation (Option 2) as performance metric for A-IoT coexistence evaluation.

From our understanding, the 10% coverage distance loss is coverage performance loss threshold for A-IoT, so SINR degradation is corresponding to degradation of path loss due to 10% coverage distance (d3D) loss. As per WF[1], the NR indoor industrial scenario (InF-xx) and NR InH-Office are defined for A-IoT co-existence scenario, according to Table 7.4.1-1 of TR 38.901, degradation of path loss  due to 10% coverage distance (d3D) loss can be calculated with following equation:

(Equation 3)

Where,

is from scenarios (InH-Office, InF-SL, InF-DL, InF-SH, and InF-DH) in Table 7.4.1-1 of TR 38.901.

Using above Equation 3, we can get:

The in dB for InH-Office is 1.7525dB, which is calculated with.

The in dB for InF-SL is 1.1668dB, which is calculated with.

The in dB for InF-DL is 1.6335dB, which is calculated with.

The in dB for InF-SH is 1.0524dB, which is calculated with.

The in dB for InF-DH is 1.0021dB, which is calculated with.

So, the SINR degradation  corresponding to degradation of path loss due to 10% coverage distance (d3D) loss for InH-Office, InF-SL, InF-DL, InF-SH, and InF-DH is 1.7525dB, 1.1668dB, 1.6335dB, 1.0524dB, and 1.0021dB, respectively. The minimum 1.0021dB SNR degradation for InF-DH can apply to all scenarios (InH-Office and InF-xx).

Based on above analysis, for legacy NR coexistence evaluation, SINR degradation for DL and UL corresponding to 5% throughput loss performance metric is 1.4959dB and 0.9589dB, respectively. For NB-IoT, performance metric is 1dB SNR loss (SNR degradation), so it is reasonable to adopt 1dB SNR degradation as performance metric for A-IoT coexistence evaluation.

From our understanding, the 5% CDF SINR distribution means that devices are located at cell edge, and the 50% CDF SINR distribution means that devices are located at cell center. So we propose that it is reasonable to define 1dB SNR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation.

Proposal 2: Use 1dB SINR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation for DL and UL.

Evaluation methodology

As per the WF [1], the remaining open issue concerning evaluation methodology is shown as below:

Issue 2-4-1: Evaluation methodology Agreement: - Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803 - Depending on the discussion on deployment scenarios, for some cases, calculation for the worst interference link may be enough. - FFS on whether RAN4 needs to perform link level simulation

Based above discussion, we propose to define 1dB SNR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation, the simulation steps using SNR degradation for Monte-Carlo method for A-IoT coexistence evaluation is shown as below:

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	Devices are distributed randomly across the network.

2.	Device associations: Devices are associated to Reader based on coupling loss.

3.	SINRICI is computed in the victim systems without considering ACI as below:

-	SINRICI=SN+IICI, where IICI is the inter-cell interference.

-    Calculate CDF of SINRICI, then get SINRICI,@5%CDF and SINRICI,@50%CDF, where SINRICI,@5%CDF is SINRICI at 5% CDF point, and SINRICI,@50%CDF is SINRICI at 50% CDF point.

4.	SINRICI+ACI is computed in the victim systems considering ACI as below:

-	SINRICI+ACI=SN+IICI+IACI, where IACI is the adjacent channel interference.

-    Calculate CDF of SINRICI+ACI, then get SINRICI+ACI,@5%CDF and SINRICI+ACI,@50%CDF, where SINRICI+ACI,@5%CDF is SINRICI+ACI at 50% CDF point, and SINRICI+ACI,@50%CDF is SINRICI+ACI  at 50% CDF point.

5.	RF parameters are determined based on the SNR degradation cause by ACI as below:

-	SNRdegradationACI,@%5CDF=SINRICI,@5%CDF-SINRICI+ACI,@5%CDF

-	SNRdegradationACI,@%50CDF=SINRICI,@50%CDF-SINRICI+ACI,@50%CDF

Proposal 3: Adopt following simulation steps using SNR degradation for Monte-Carlo method for A-IoT coexistence evaluation:

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	Devices are distributed randomly across the network.

2.	Device associations: Devices are associated to Reader based on coupling loss.

3.	SINRICI is computed in the victim systems without considering ACI as below:

-	SINRICI=SN+IICI, where IICI is the inter-cell interference.

-    Calculate CDF of SINRICI, then get SINRICI,@5%CDF and SINRICI,@50%CDF, where SINRICI,@5%CDF is SINRICI at 5% CDF point, and SINRICI,@50%CDF is SINRICI at 50% CDF point.

4.	SINRICI+ACI is computed in the victim systems considering ACI as below:

-	SINRICI+ACI=SN+IICI+IACI, where IACI is the adjacent channel interference.

-    Calculate CDF of SINRICI+ACI, then get SINRICI+ACI,@5%CDF and SINRICI+ACI,@50%CDF, where SINRICI+ACI,@5%CDF is SINRICI+ACI at 50% CDF point, and SINRICI+ACI,@50%CDF is SINRICI+ACI  at 50% CDF point.

5.	RF parameters are determined based on the SNR degradation cause by ACI as below:

-	SNRdegradationACI,@%5CDF=SINRICI,@5%CDF-SINRICI+ACI,@5%CDF

-	SNRdegradationACI,@%50CDF=SINRICI,@50%CDF-SINRICI+ACI,@50%CDF

As above discussion, the 1dB SNR degradation as performance metric for A-IoT coexistence evaluation is reasonable, BLERE is not appropriate as performance metric A-IoT coexistence evaluation, the BLER vs SNR curve is not needed for A-IoT coexistence evaluation, so the link level simulation for BLER vs SNR curve is not needed.  Only ICI from co-channel adjacent cell was considered in Monte-Carlo method for A-IoT coexistence evaluation, the ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum is unknown and is not needed to be considered in Monte-Carlo method for A-IoT coexistence evaluation.  So link level simulation is needed to determine guard RB size to cancel ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum and ensure A-IoT performance.

Observation 3: Link level simulation is needed to determine guard RB size to cancel ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum and ensure A-IoT performance.

2.2 Evaluation parameters

Intermediate UE dropping for D2T2

Since the number of intermediate UE is limit, the intermediate UE dropping will affect coexistence performance, it need more intermediate UEs dropping to reduce coexistence performance impact. To reduce intermediate UE dropping impact, it is reasonable to use fixed 18 intermediate UEs deployment for D2T2 for InF-DH and InF-SH as show in following table.

Table 2.3-1: intermediate UE deployment for D2T2 for InF-DH and InF-SH

| Parameter | Values |
| --- | --- |
| Scenario | InF-SH, InF-DH |
| Hall size | InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| intermediate UE deployment | 18 intermediate UEs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |

Proposal 4: For Intermediate UE dropping for D2T2, to adopt 18 intermediate UEs on a square lattice with spacing D, located D/2 from the walls as shown in Table 2.3-1.

Table 2.3-1: intermediate UE deployment for D2T2 for InF-DH and InF-SH

| Parameter | Values |
| --- | --- |
| Scenario | InF-SH, InF-DH |
| Hall size | InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| intermediate UE deployment | 18 intermediate UEs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |

NR network layout model and parameters

As per the WF [1], the agreed deployment scenario can summarized as following:

For A-IoT:

D1T1: AIoT BS reader/CW/devices are all indoors.

D2T2: AIoT intermediate UE reader/CW/devices are all indoors.

For NR:

D1T1: outdoor macro legacy NR gNB, Legacy NR UE is only allowed outdoors

D1T1: outdoor macro legacy NR gNB, Legacy NR UE is indoor accessing to outdoor NR marco gNB

D2T2: outdoor macro legacy NR gNB, Legacy NR UE is only allowed outdoor.

D2T2: outdoor macro legacy NR gNB, Legacy NR UE is indoor, which is accessing to outdoor NR marco gNB

So, based on above agreements, the urban macro is applicable for NR, and Indoor UE ratio can be 0% and 100%. Referring to Table 6.2-1 of TR 36.802, we propose to define network layout model parameters for NR, NR BS parameter for co-existence for urban macro, and NR UE parameter for co-existence for urban macro in tables 2.3-2, 2.3-3, and 2.3-4, respectively.

Table 2.3-2: Network layout model parameters for NR

| Parameter |  | NR |
| --- | --- | --- |
| Carrier frequency in GHz |  | 0.9 |
| Size of each nominal channel BW in MHz |  | DL:10 UL: 10 |
| Transmission bandwidth in MHz |  | DL: 9 UL: 9 |
| subcarrier spacing |  | DL: 15kHz UL: 15kHz |
| Waveform |  | DL:CP-OFDM UL: SC-FDMA |
| Environment |  | Urban macro |
| Network layout |  | 1-sites, 3 sectors |
| Inter-site distance in meter |  | 750 for 0.9GHz band |
| System loading and activity |  | Full buffer 100% |
| Network location |  | (see IoT) |
| DL power control |  | NO |
| UL power control |  | YES |
| Frequency reuse |  | 1 |
| Number of scheduled UE per cell (DL) |  | 1 |
| Number of scheduled UE per cell (UL) |  | 3 |
| BS antenna height in meter |  | 30 |
| UE antenna height in meter |  | 1.5 |
| Cell selection margin in dB |  | 3 |
| BS-MS min couple loss in dB |  | 70 |
| BS-UE path-loss model |  | TR38.901 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB |  | 10 |
| Shadowing correlation |  | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  | As in Annex A.1 in 36.942 |
| UE location | Outdoor/indoor | Outdoor or indoor |
|  | Indoor UE ratio | 0%, 100% |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS |
| UE distribution (horizontal) |  | Uniform(horizontal) |

Table 2.3-3: NR BS parameter for co-existence for urban macro

| Parameter | NR |
| --- | --- |
| BS max TX power in dBm | 46 |
| BS antenna gain including feeder loss in dBi | 15 |
| BS antenna pattern | Horizontal (36.942) |
| BS antenna front-back ratio in dB | 20 |
| BS noise figure in dB | 5 |

Table 2.3-4: NR UE parameter for co-existence for urban macro

| Parameter | NR |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| UE noise figure in dB | 9 |
| UT antenna configurations | 1 element (vertically polarized),Omni-direction antenna |

Based on above discussion, we propose,

Proposal 5: Adopt following NR parameters in tables 2.3-2, 2.3-3, and 2.3-4 for coexistence.

Table 2.3-2: Network layout model parameters for NR

| Parameter |  | NR |
| --- | --- | --- |
| Carrier frequency in GHz |  | 0.9 |
| Size of each nominal channel BW in MHz |  | DL:10 UL: 10 |
| Transmission bandwidth in MHz |  | DL: 9 UL: 9 |
| subcarrier spacing |  | DL: 15kHz UL: 15kHz |
| Waveform |  | DL:CP-OFDM UL: SC-FDMA |
| Environment |  | Urban macro |
| Network layout |  | 1-sites, 3 sectors |
| Inter-site distance in meter |  | 750 for 0.9GHz band |
| System loading and activity |  | Full buffer 100% |
| Network location |  | (see IoT) |
| DL power control |  | NO |
| UL power control |  | YES |
| Frequency reuse |  | 1 |
| Number of scheduled UE per cell (DL) |  | 1 |
| Number of scheduled UE per cell (UL) |  | 3 |
| BS antenna height in meter |  | 30 |
| UE antenna height in meter |  | 1.5 |
| Cell selection margin in dB |  | 3 |
| BS-MS min couple loss in dB |  | 70 |
| BS-UE path-loss model |  | TR38.901 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB |  | 10 |
| Shadowing correlation |  | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  | As in Annex A.1 in 36.942 |
| UE location | Outdoor/indoor | Outdoor or indoor |
|  | Indoor UE ratio | 0%, 100% |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS |
| UE distribution (horizontal) |  | Uniform(horizontal) |

Table 2.3-3: NR BS parameter for co-existence for urban macro

| Parameter | NR |
| --- | --- |
| BS max TX power in dBm | 46 |
| BS antenna gain including feeder loss in dBi | 15 |
| BS antenna pattern | Horizontal (36.942) |
| BS antenna front-back ratio in dB | 20 |
| BS noise figure in dB | 5 |

Table 2.3-4: NR UE parameter for co-existence for urban macro

| Parameter | NR |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| UE noise figure in dB | 9 |
| UT antenna configurations | 1 element (vertically polarized),Omni-direction antenna |

- Conclusion

This contribution provides analysis on coexistence evaluation for A-IoT. The following proposals and observations are concluded as follows:

Observation 1: SINR degradation for DL and UL corresponding to 5% throughput loss for legacy NR coexistence evaluation is 1.4959dB and 0.9589dB, respectively.

Observation 2: 1dB SNR degradation (SNR loss) was used as performance metric for NB-IoT existence evaluation in TR 36.802.

Proposal 1: Adopt SINR degradation (Option 2) as performance metric for A-IoT coexistence evaluation.

Proposal 2: Use 1dB SINR degradation for 5% and 50% CDF SINR point as performance metric for A-IoT co-existence evaluation for DL and UL.

Proposal 3: Adopt following simulation steps using SNR degradation for Monte-Carlo method for A-IoT coexistence evaluation:

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	Devices are distributed randomly across the network.

2.	Device associations: Devices are associated to Reader based on coupling loss.

3.	SINRICI is computed in the victim systems without considering ACI as below:

-	SINRICI=SN+IICI, where IICI is the inter-cell interference.

-    Calculate CDF of SINRICI, then get SINRICI,@5%CDF and SINRICI,@50%CDF, where SINRICI,@5%CDF is SINRICI at 5% CDF point, and SINRICI,@50%CDF is SINRICI at 50% CDF point.

4.	SINRICI+ACI is computed in the victim systems considering ACI as below:

-	SINRICI+ACI=SN+IICI+IACI, where IACI is the adjacent channel interference.

-    Calculate CDF of SINRICI+ACI, then get SINRICI+ACI,@5%CDF and SINRICI+ACI,@50%CDF, where SINRICI+ACI,@5%CDF is SINRICI+ACI at 50% CDF point, and SINRICI+ACI,@50%CDF is SINRICI+ACI  at 50% CDF point.

5.	RF parameters are determined based on the SNR degradation cause by ACI as below:

-	SNRdegradationACI,@%5CDF=SINRICI,@5%CDF-SINRICI+ACI,@5%CDF

-	SNRdegradationACI,@%50CDF=SINRICI,@50%CDF-SINRICI+ACI,@50%CDF

Observation 3: Link level simulation is needed to determine guard RB size to cancel ICI between A-IoT carriers and NR carriers in same cell for A-IoT in-band or guard band operation in NR spectrum and ensure A-IoT performance.

Proposal 4: For Intermediate UE dropping for D2T2, to adopt 18 intermediate UEs on a square lattice with spacing D, located D/2 from the walls as shown in Table 2.3-1.

Table 2.3-1: intermediate UE deployment for D2T2 for InF-DH and InF-SH

| Parameter | Values |
| --- | --- |
| Scenario | InF-SH, InF-DH |
| Hall size | InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| intermediate UE deployment | 18 intermediate UEs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |

Proposal 5: Adopt following NR parameters in tables 2.3-2, 2.3-3, and 2.3-4 for coexistence.

Table 2.3-2: Network layout model parameters for NR

| Parameter |  | NR |
| --- | --- | --- |
| Carrier frequency in GHz |  | 0.9 |
| Size of each nominal channel BW in MHz |  | DL:10 UL: 10 |
| Transmission bandwidth in MHz |  | DL: 9 UL: 9 |
| subcarrier spacing |  | DL: 15kHz UL: 15kHz |
| Waveform |  | DL:CP-OFDM UL: SC-FDMA |
| Environment |  | Urban macro |
| Network layout |  | 1-sites, 3 sectors |
| Inter-site distance in meter |  | 750 for 0.9GHz band |
| System loading and activity |  | Full buffer 100% |
| Network location |  | (see IoT) |
| DL power control |  | NO |
| UL power control |  | YES |
| Frequency reuse |  | 1 |
| Number of scheduled UE per cell (DL) |  | 1 |
| Number of scheduled UE per cell (UL) |  | 3 |
| BS antenna height in meter |  | 30 |
| UE antenna height in meter |  | 1.5 |
| Cell selection margin in dB |  | 3 |
| BS-MS min couple loss in dB |  | 70 |
| BS-UE path-loss model |  | TR38.901 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB |  | 10 |
| Shadowing correlation |  | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  | As in Annex A.1 in 36.942 |
| UE location | Outdoor/indoor | Outdoor or indoor |
|  | Indoor UE ratio | 0%, 100% |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |
|  | LOS/NLOS | LOS and NLOS |
| UE distribution (horizontal) |  | Uniform(horizontal) |

Table 2.3-3: NR BS parameter for co-existence for urban macro

| Parameter | NR |
| --- | --- |
| BS max TX power in dBm | 46 |
| BS antenna gain including feeder loss in dBi | 15 |
| BS antenna pattern | Horizontal (36.942) |
| BS antenna front-back ratio in dB | 20 |
| BS noise figure in dB | 5 |

Table 2.3-4: NR UE parameter for co-existence for urban macro

| Parameter | NR |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| UE noise figure in dB | 9 |
| UT antenna configurations | 1 element (vertically polarized),Omni-direction antenna |

- References

- R4-2406714, WF on Ambient IoT in NR, CMCC, RAN4#110bis