3GPP RAN WG4 Meeting #111	R4-2407493

Fukuoka, Japan, May 20 – May 24, 2024

Title: 	Discussion on Performance requirements of RedCap UE positioning

Source: 	CATT

Agenda item:	7.12.2.4

Document for:	Discussion

Introduction

In RAN4#110bis meeting, fruitful progress regarding the performance requirements was achieved which are captured in the WF [1]. In this paper, we will further discuss a few open issues related to performance requirements.

Discussion

BW and structure for measurement accuracy requirements with Rx FH

Issue1: Bandwidths and structure for accuracy requirements with Rx FH

In RAN4#110bis meeting, the parameters that shall be considered when defining measurement accuracy requirements are determined and the agreement is shown below. However, the structure of the accuracy table and the total PRS bandwidths for which accuracy requirements shall be defined are not decided. We present our proposals on these issues since it is better to align the structure in different clauses contributed by multiple companies.

# Bandwidth for accuracy requirement for RedCap positioning Agreement: - Option 2c: combination (total measured PRS BW during all hops, per-hop BW), regardless of the overlapping BW size

Based on the above agreement, and the bandwidth per hop and the total bandwidth across all hops in the simulation assumption for Rx FH [2], the following table structure and total PRS bandwidths are proposed for measurements with Rx FH for 2Rx RedCap in AWGN in FR1 as an example:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 10 | ≥ 52 |
| TBD |  |  |  | ≥ 104 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 30 | 20 | ≥ 48 |
| TBD |  |  |  | ≥ 132 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 60 | 20 | ≥ 24 |
| TBD |  |  |  | ≥ 64 |
| TBD |  |  |  | ≥ 132 |

Similarly, an example for the accuracy table in FR2 is shown below:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 50 | ≥ 64 |
| TBD |  |  |  | ≥ 132 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 120 | 100 | ≥ 64 |
| TBD |  |  |  | ≥ 128 |
| TBD |  |  |  | ≥ 272 |

The rest of structure can reuse the applicable parts in the existing accuracy tables, for example, Io range, etc.

Proposal 1: The structure of accuracy tables for measurements with FH in different clauses shall be aligned, and example tables for accuracy requirements with FH for 2Rx RedCap in FR1 and FR2 are given below:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 10 | ≥ 52 |
| TBD |  |  |  | ≥ 104 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 30 | 20 | ≥ 48 |
| TBD |  |  |  | ≥ 132 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 60 | 20 | ≥ 24 |
| TBD |  |  |  | ≥ 64 |
| TBD |  |  |  | ≥ 132 |

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 50 | ≥ 64 |
| TBD |  |  |  | ≥ 132 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 120 | 100 | ≥ 64 |
| TBD |  |  |  | ≥ 128 |
| TBD |  |  |  | ≥ 272 |

The rest of structure can reuse the applicable parts in the existing accuracy tables, for example, Io range, etc.

Conclusions

This paper discussed the accuracy requirements for RedCap UE positioning, and the following proposals are provided:

Proposal 1: The structure of accuracy tables for measurements with FH in different clauses shall be aligned, and example tables for accuracy requirements with FH for 2Rx RedCap in FR1 and FR2 are given below:

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | 10 | ≥ 52 |
| TBD |  |  |  | ≥ 104 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 30 | 20 | ≥ 48 |
| TBD |  |  |  | ≥ 132 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 60 | 20 | ≥ 24 |
| TBD |  |  |  | ≥ 64 |
| TBD |  |  |  | ≥ 132 |

| Accuracy | PRS Ês/Iot | PRS SCS | BW per hop | Total PRS bandwidth |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | MHz | PRB |
| TBD | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | 50 | ≥ 64 |
| TBD |  |  |  | ≥ 132 |
| TBD |  |  |  | ≥ 272 |
| TBD |  | 120 | 100 | ≥ 64 |
| TBD |  |  |  | ≥ 128 |
| TBD |  |  |  | ≥ 272 |

The rest of structure can reuse the applicable parts in the existing accuracy tables, for example, Io range, etc.

References

[1] R4-2406383, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson, RAN4#110bis.

[2] R4-2403471, Updated simulation assumptions for RedCap Positioning with Rx FH, Ericsson, RAN4#110.