3GPP RAN WG4 Meeting #111	R4-2407497

Fukuoka, Japan, May 20 – May 24, 2024

Title: 	Discussion on testability and interoperability issues for BM

Source: 	CATT

Agenda item:	10.11.3

Document for:	Discussion

Introduction

In RAN4#110bis meeting, RAN4 discussed on Rel-19 AI/ML for NR air interface and agreements are captured in [1]. There are still some issues related to testability and interoperability for beam management that needs to be further discussed. In this contribution, we will present our views on the following issue:

-	Metrics/KPIs for Beam management

Discussion

Metrics/KPIs for Beam management

Metrics/KPIs for beam management are discussed in previous RAN4 meetings to align companies’ understandings, and no down-selection is made so far. The progresses in RAN4 side are summarized below for convenience:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options RAN4#110bis: Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

Though further discussions on this topic are hold on until RAN1 have conclusions on the schemes, we want to present our analysis to align the understandings. First, parts of RAN1 agreements achieved in RAN1#116bis are copied below for reference:

RAN1#116bis: Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

According to the RAN1 agreement/assumption, an observation can be made:

Observation 1: The RSRP of predicted beam(s) in the report of inference results can be predicted RSRP or measured RSRP, which depends on whether the beam is configured for corresponding measurement.

In the following section, we will analyse each option to check if it is reasonable to be metrics/KPIs for BM tests.

Option 1: RSRP accuracy

For option 1, there is no clear mechanism to describe how it works, which needs to be clarified. From the perspective of test, a straightforward definition of RSRP accuracy is the difference between measured RSRP and predicted RSRP for the same beam.

We will illustrate the details by an example shown in Figure 1, where 4 predicted beams with predicted RSRP are reported and beam 3, the strongest beam among the Top-4 predicted beams, is configured to be measured by UE. The measured RSRP3 is also reported which is assumed as ground truth here. Therefore the option 1 becomes that the UE is confirmed to successfully pass the tests when the absolute difference between predicted RSRP and measured RSRP is smaller than a specific value, e.g., Y dB, which is described by the equation

AbsMeasured RSRP3-Predicted RSRP3<Y dB

Figure1. An illustration of example of Option 1

Option 3: The successful rate

For Option 3, the mechanism is quite clear. For each inference, it can be described by the following equation

MaxPredicted RSRP1, RSRP2, RSRP3,RSRP4>Measured RSRP3-X dB

All predicted RSRPs and measured RSRPs are shown in Figure 2 in which the beam 2 is the strongest beam among the Top-4 predicted beams and the real strongest beam is beam 3 according to the measurement results. If the above equation is met, the UE is confirmed to successfully predict the result for this inference, and the UE passes the tests if the overall successful rate is higher than a threshold, e.g., 90%.

Figure2. An illustration of example of Option 3

Based on the above analysis on option 3, an identified issue is that the RSRPs used for comparison may be not from the same beam, i.e., the maximum predicted RSRP is from beam 2 and the maximum measured RSRP is from beam 3. Even though the equation is met, the predicted best beam still could be the worst beam among the Top-K measured beams. Besides, this mechanism is not straightforward comparing to Option 1.

Therefore we propose to merge Option 1 and Option 3 and redefine as follows:

-	Option 1a: The absolute difference between the predicted RSRPs of Top-K predicted beams and the measured RSRPs of the corresponding beams shall less than X dB

-	K should be aligned with RAN1 agreements.

-	Related measurement accuracy can be considered to determine X.

Observation 2: In Option 3, the RSRPs used for comparison are not from the same beam, which may not ensure the RSRP prediction accuracy.

Proposal 1: Option 1 and Option 3 can be merged and redefined as follows:

-	Option 1a: The absolute difference between the predicted RSRPs of Top-K predicted beams and the measured RSRPs of the corresponding beams shall be less than X dB,

-	K should be aligned with RAN1 agreements.

-	Related measurement accuracy can be considered to determine X.

Option 2: Beam prediction accuracy

Both Option 1 and Option 3 consider the RSRP accuracy while Option 2 is applicable for the classifiers that can output integers only, i.e., beam IDs in beam management use cases.

Proposal 2: Both KPIs/metrics based on RSRP accuracy and beam ID accuracy should be defined, considering the capability of different types of AI/ML models.

There are three sub-options which are depicted in Figure 3 for reference:

Figure3. Illustrations of three sub-options for Option 2

First, according to Figure 3, we think Option 2-2 and 2-3 have similar effect as KPIs/metrics. The main difference is reflected in inference, measurement and reporting scheme designs which are out of RAN4 scope. Therefore RAN4 shall wait for RAN1 progress.

Proposal 3: Down-selection between option 2-2 and 2-3 which have similar effect as KPIs/metrics can be made after RAN1 have progresses.

Regarding the Option 2-1 and Option 2-2/2-3, the difference is that whether a margin shall be considered for beam ID prediction. At least Option 2-2/2-3 should be supported if the performance for Option 2-1 and Option 2-2/2-3 are comparable.

Observation 3: The difference between Option 2-1 and 2-2/3 is that whether a margin shall be considered for beam ID prediction.

Proposal 4: At least Option 2-2/2-3 should be supported if the performance for Option 2-1 and Option 2-2/2-3 are comparable.

Conclusions

This paper discussed some issues related to AI/ML for beam management, and following proposals are provided:

Observation 1: The RSRP of predicted beam(s) in the report of inference results can be predicted RSRP or measured RSRP, which depends on whether the beam is configured for corresponding measurement.

Observation 2: In Option 3, the RSRPs used for comparison are not from the same beam, which may not ensure the RSRP prediction accuracy.

Observation 3: The difference between Option 2-1 and 2-2/2-3 is that whether a margin shall be considered for beam ID prediction.

Proposal 1: Option 1 and Option 3 can be merged and redefined as follows:

-	Option 1a: The absolute difference between the predicted RSRPs of Top-K predicted beams and the measured RSRPs of the corresponding beams shall be less than X dB,

-	K should be aligned with RAN1 agreements.

-	Related measurement accuracy can be considered to determine X.

Proposal 2: Both KPIs/metrics based on RSRP accuracy and beam ID accuracy should be defined, considering the capability of different types of AI/ML models.

Proposal 3: Down-selection between option 2-2 and 2-3 which have similar effect as KPIs/metrics can be made after RAN1 have progresses.

Proposal 4: At least Option 2-2/2-3 should be supported if the performance for Option 2-1 and Option 2-2/2-3 are comparable.

References

[1] R4-2406617, WF on AI/ML, Qualcomm Incorporated, RAN4#110bis.