3GPP TSG-RAN WG4 Meeting # 111                                                                R4-2407524

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Title: 	                 Discussion on RF requirements of Ambient IoT intermediate node

Source: 	CATT

Agenda item:	10.13.3.3

Document for:	Discussion

- Introduction

According to the SID [1], RF requirements for Ambient IoT need to be analyzed, in this contribution, we provide the potential RF requirements analysis of A-IoT intermediate node.

- Potential RF requirements of A-IoT intermediate node

In the SID[1], the RF requirements objectives for Ambient IoT can be seen below:

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

A-IoT intermediate node includes R2D interrogation signal transmitters, D2R backscattering signal receivers and CW transmitter if CW is transmitted from inside the topology. CW is transmitted to device 1 or device 2a.

In this contribution, we only analyze potential requirements of device 1 and device 2a, whether it is necessary to set requirements for CW is TBD, requirements of device 2b will be analyzed later. We analyze A-IoT requirements based on RFID standard [2] and regulation [3], NB-IoT BS standard [4], as well as A-IoT topology, scenario deployment, device architecture, etc.[5], the potential requirements for R2D transmitters and D2R receivers are the same as for A-IoT BS.

2.1 A-IoT intermediate node potential Tx requirements

Table 1: A-IoT intermediate node potential Tx requirements

| No. | ISO-IEC 18000-6C Reader RF Tx requirements | RFID EN 302 208 Interrogator transmitter conformance requirement | NB-IoT BS RF Tx requirements | A-IoT intermediate node potential Tx requirements |
| --- | --- | --- | --- | --- |
| 1 | Max EIRP | Effective radiated power | Output power | Output power |
| 2 |  |  | NB-IoT RB power dynamic range | Power dynamic range |
| 3 |  |  | Transmit ON/OFF power | Transmit ON/OFF power |
| 4 |  |  | Frequency error | Frequency error |
| 5 |  |  | EVM | TBD |
| 6 | Occupied bandwidth |  | Occupied bandwidth | Occupied bandwidth |
| 7 |  |  | ACLR | ACLR |
| 8 | Spectrum masks | Transmitter spectrum masks | Operating band unwanted emissions | Operating band unwanted emissions |
| 9 | Spurious emissions | Transmitter spurious emissions | Transmitter spurious emissions | Transmitter spurious emissions |
| 10 |  |  | Transmitter intermodulation | Transmitter intermodulation |
| 11 |  | Transmission times |  | RAN4 needs further analysis |
| 12 |  | Frequency stability under low voltage conditions |  | Not necessary |
| 13 |  | Transmitter antenna beam-width |  | Not necessary |

A-IoT intermediate node potential Tx requirements are shown in table 1. EVM requirement is TBD now, and further analysis is needed according to the progress of discussion on waveform, modulation, etc. The No.12 and No.13 requirements are not related to A-IoT intermediate node, and definitions are not necessary.

Observation 1: A-IoT intermediate node potential Tx requirements are shown in Table 1, the requirements only consider device 1 and device 2a.

2.2 A-IoT intermediate node potential Rx requirements

Table 2: A-IoT intermediate node potential Rx requirements

| No. | RFID EN 302 208[3] Interrogator receiver conformance requirement | NB-IoT BS RF Rx requirements | A-IoT intermediate node potential Rx requirements |
| --- | --- | --- | --- |
| 1 | Receiver sensitivity | Reference sensitivity level | Reference sensitivity level |
| 2 |  | Dynamic range | Dynamic range |
| 3 |  | In-channel selectivity | In-channel selectivity |
| 4 | Adjacent channel selectivity | Adjacent Channel Selectivity and narrow-band blocking | Adjacent Channel Selectivity Narrow-band blocking |
| 5 | Blocking or desensitization | In-band blocking | In-band blocking |
| 6 |  | Out-band blocking | Out-band blocking |
| 7 |  | Receiver spurious emissions | Receiver spurious emissions |
| 8 | Receiver radio-frequency intermodulation | Receiver intermodulation | Receiver intermodulation |
| 9 | Receiver spurious response rejection | Narrowband intermodulation | Narrowband intermodulation |

A-IoT intermediate node potential Rx requirements are shown in Table 2, all Rx requirements of RFID reader and NB-IoT BS need to be defined as A-IoT intermediate node reception.

Observation 2: A-IoT intermediate node potential Rx requirements are shown in Table 2, the requirements only consider device 1 and device 2a.

2.3 A-IoT intermediate node

There're two links for intermediate UE, i.e. NR link and A-IoT link. The two links work simultaneously. There could be some interference for some DL/UL spectrum usage choice. The RF requirements for the links works simultaneously may need discussion according to the different DL/UL spectrum usage for A-IoT.

Proposal 1: The RF requirements for the links working simultaneously need discussion according to the different DL/UL spectrum usage for A-IoT.

- Summary

This contribution provides our preliminary analysis for RF requirements of A-IoT intermediate node. We have the following proposals.

Observation 1: A-IoT intermediate node potential Tx requirements are shown in Table 1, the requirements only consider device 1 and device 2a.

Observation 2: A-IoT intermediate node potential Rx requirements are shown in Table 2, the requirements only consider device 1 and device 2a.

Proposal 1: The RF requirements for the links working simultaneously need discussion according to the different DL/UL spectrum usage for A-IoT.

Reference

[1] RP-234058, “Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#102

[2] ISO/IEC 18000-6C

[3] Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum, ETSI EN 302 208

[4] TS 36.104, Base Station (BS) radio transmission and reception

[5] R1-2403782, “LS on Ambient-IoT evaluation scenarios and assumptions”, RAN1#116bis