3GPP TSG-RAN WG4 Meeting #111				                                                                            R4-2407553

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	CATT

Title: 	Discussion on the remaining issues for ATG enhancement BS RF

Agenda item:	10.8.3

Document for:	Discussion

- Introduction

This contribution provides the further discussion on the remaining issues for BS RF requirements for ATG enhancement.

- Discussion

According to the WF [1] in last RAN4 meeting, for BS RF requirements, the following issue is the only open issue.

#### Issue 1-2:  How to handle CA TAE requirements for ATG BS in TS38.104

Way Forward

- Option 1: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3

- Option 2: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, then add the following in 6.5.3.2 and 9.6.3.2,

  - For ATG BS, for intra-band contiguous carrier aggregation, without MIMO, TAE shall not exceed 260ns.

  - For ATG BS, inter-band carrier aggregation, without MIMO, TAE shall not exceed 3µs.

- Option 3: Find a place to add a general clarification that for ATG, intra-band contiguous CA and inter-band CA are supported but DL MIMO is not supported.

For the three options, option 1 is to make ATG BS support both DL MIMO and CA while option 2 and option 3 are supposing WID is strictly followed that DL MIMO is not supported. However, although the agreement made in R18 ATG WI is that DL MIMO is not supported by ATG feature considering the LOS channel is the most possible prorogation channel between ATG BS and ATG UE, R19 WI is studying UL MIMO now. So in order to align DL and UL feature, it’s reasonable to update R19 WID to support DL MIMO if the work for DL MIMO is not that much. BS RF DL MIMO requirement is relatively easy to discuss and agree but DL MIMO funtion is also related to UE demodulation requirements, more discussion is needed.

Observation: UE DEMOD performance requirement needs to be added if DL MIMO is supported in this WI. It’s a RAN plenary discussion.

Considering the above reason, we have the following proposals for the BS RF specification handling,

Option 1: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, if WID is updated to include DL MIMO in the scope.

Option 2: Replace the sentence “There is no TAE requirement for ATG BS” with “The requirements for MIMO transmission are not applicable to ATG BS”.

- Conclusion

We have the following observation and proposal to handle BS RF requirements for R19 ATG.

Observation: UE DEMOD performance requirement needs to be added if DL MIMO is supported in this WI. It’s a RAN plenary discussion.

Option 1: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, if WID is updated to include DL MIMO in the scope.

Option 2: Replace the sentence “There is no TAE requirement for ATG BS” with “The requirements for MIMO transmission are not applicable to ATG BS”.

- References

[1] R4-2406089, “Way Forward for [110bis][312] NR_ATG_enh”, ZTE