3GPP RAN WG4 Meeting #111		R4-2407558

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	10.15.2.1

Source:	Apple

Title:	On NR NTN for (e)RedCap UE REFSENS requirements

WI/SI:	NR_NTN_Ph3-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

Considering that the support of RedCap devices operating in FR1 band NR-NTN networks can offer enhanced service capabilities (wideband/broadband) compared to IoT-NTN while ensuring low-complexity devices and the global coverage would clearly benefit RedCap devices, an objective on the “support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands” is included in the Rel-19 new WID on “Non-Terrestrial Networks (NTN) for NR Phase 3” led by RAN2 [1]. In last RAN4 meeting, the following aspects on NR NTN for (e)RedCap UE were captured in the approved “WF on NR_NTN_Ph3_UERF” [2]:

- HD-FDD

- 1Rx and 2Rx

- eRedCap UE BW reduction

- Target power class

- Channel BW and SCS

- Specifications structure

- Operation bands

- REFSENS

In this contribution, we share our views on the REFSENS requirements for NR NTN (e)RedCap UE.

# Discussion

## 2.1	REFSENS for NR NTN RedCap UE

According to the “WF on NR_NTN_Ph3_UERF” [2], the 2Rx FD-FDD REFSENS will reuse the requirements for non-RedCap NTN UE [3] which are summarized in Table 2.1-1 and Table 2.1-2 for REFSENS and the corresponding UL configurations.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -99.5 | -96.3 | -94.5 |  |
| n254 | 30 |  | -96.6 | -94.6 |  |
|  | 60 |  | -97.0 | -94.9 |  |

Table 2.1-1 NR NTN RedCap UE 2Rx FD-FDD REFSENS requirements

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | 25 | 50 | 75 | 100 | FD-FDD |
| n256 | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | [75] | FD-FDD |
| n255 | 30 |  | 24 | 36 | [36] |  |
|  | 60 |  | 10 | 18 | [18] |  |
|  | 15 | 25 | 50 | 75 |  | FD-FDD |
| n254 | 30 |  | 24 | 36 |  |  |
|  | 60 |  | 10 | 18 |  |  |

Table 2.1-2 Uplink configuration for NR NTN RedCap UE 2Rx FD-FDD REFSENS requirements

For 2Rx HD-FDD, the 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS can follow the previous agreement for RedCap UE [4] which is summarized in Table 2.1-3.

| 2Rx FD-FDD 5MHz REFSENS | 2Rx HD-FDD 5MHz REFSENS Tightening |
| --- | --- |
| ≤ -100 dBm | 0 dB |
| > -100 dBm and ≤ -99 dBm | 0.5 dB |
| > -99 dBm | 0.8 dB |

Table 2.1-3 2Rx HD-FDD 5MHz REFSENS tightening relative to 2Rx FD-FDD 5MHz REFSENS

For HD-FDD, since there is no UL self-interference to DL, the REFSENS for channel BW wider than 5MHz can be linearly scaled over 5MHz and calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n = 1 for 15kHz SCS, n = 2 for 30kHz SCS, and n = 4 for 60kHz SCS. On the other hand, UL RB restriction would not be needed for HD-FDD REFSENS as there is no UL self-interference to DL, therefore, the UL configuration for HD-FDD REFSENS shall be specified with full allocations. Based on the above REFSENS tightening and the scaling over channel BW, the 2Rx HD-FDD REFSENS and UL configuration can be derived and specified as in Table 2.1-4 and Table 2.1-5 respectively.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n256 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Table 2.1-4 NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | 25 | 50 | 75 | 100 | HD-FDD |
| n256 | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 100 | HD-FDD |
| n255 | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 |  | HD-FDD |
| n254 | 30 |  | 24 | 36 |  |  |
|  | 60 |  | 10 | 18 |  |  |

Table 2.1-5 Uplink configuration for NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements

For 1Rx REFSENS relative to 2Rx, according to the “WF on NR_NTN_Ph3_UERF” [2], the allowance specified for RedCap UE [5] as recaptured in Table 2.1-6 can be reused.

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Table 2.1-6 RedCap UE 1Rx REFSENS allowance relative to 2Rx

Therefore, the 1Rx FD-FDD REFSENS and 1Rx HD-FDD REFSENS can be derived and specified in Table 2.1-7 and Table 2.1-8 respectively.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.0 | -93.3 | -91.5 | -90.3 |
| n256 | 30 |  | -93.6 | -91.6 | -90.5 |
|  | 60 |  | -94.0 | -91.9 | -90.7 |
|  | 15 | -97.5 | -93.8 | -92.0 | -90.8 |
| n255 | 30 |  | -94.1 | -92.1 | -91.0 |
|  | 60 |  | -94.5 | -92.4 | -91.2 |
|  | 15 | -97.0 | -93.3 | -91.5 |  |
| n254 | 30 |  | -93.6 | -91.6 |  |
|  | 60 |  | -94.0 | -91.9 |  |

Table 2.1-7 NR NTN RedCap UE 1Rx FD-FDD REFSENS requirements

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.5 | -93.8 | -92.0 | -90.8 |
| n256 | 30 |  | -94.1 | -92.1 | -91.0 |
|  | 60 |  | -94.5 | -92.4 | -91.2 |
|  | 15 | -97.5 | -93.8 | -92.0 | -90.8 |
| n255 | 30 |  | -94.1 | -92.1 | -91.0 |
|  | 60 |  | -94.5 | -92.4 | -91.2 |
|  | 15 | -97.5 | -93.8 | -92.0 |  |
| n254 | 30 |  | -94.1 | -92.1 |  |
|  | 60 |  | -94.5 | -92.4 |  |

Table 2.1-8 NR NTN RedCap UE 1Rx HD-FDD REFSENS requirements

The UL configurations for 1Rx FD-FDD REFSENS and 1Rx HD-FDD REFSENS are the same as specified in Table 2.1-2 and Table 2.1-5 respectively.

Proposal 1: NR NTN RedCap UE 2Rx FD-FDD REFSENS requirements and UL configuration are specified as in Table 2.1-1 and Table 2.1-2 respectively.

Proposal 2: NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements and UL configuration are specified as in Table 2.1-4 and Table 2.1-5 respectively.

Proposal 3: NR NTN RedCap UE 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are specified in Table 2.1-7 and Table 2.1-8 respectively.

Proposal 4: The UL configurations for NR NTN RedCap UE 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are specified in Table 2.1-2 and Table 2.1-5 respectively.

## 2.2	REFSENS for NR NTN eRedCap UE

For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for NR NTN RedCap UE proposed in section 2.1 apply.

For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not IE eRedCapNotReducedBB-BW-r18, the 2Rx FD-FDD REFSENS requirements and UL configuration are specified as in Table 2.2-1 and Table 2.2-2 respectively.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n256 | 15 | -99.5 | -99.5 | -99.5 | -99.5 |
|  | 30 |  | -99.6 | -99.6 | -99.6 |
| n255 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.1 | -100.1 | -100.1 |
| n254 | 15 | -99.5 | -99.5 | -99.5 |  |
|  | 30 |  | -99.6 | -99.6 |  |
| NOTE：The Rx RBs shall be allocated within the range from RBlow = ceil(NRB/2 – NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 – 1, where RBlow and RBhigh are the lowest and highest RB position and NRB_PR3 is 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS. |  |  |  |  |  |

Table 2.2-1 NR NTN eRedCap UE 2Rx FD-FDD REFSENS requirements

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n256 | 15 | 25 | 25 | 25 | 25 | FD-FDD |
|  | 30 |  | 12 | 12 | 12 |  |
| n255 | 15 | 25 | 25 | 25 | 25 | FD-FDD |
|  | 30 |  | 12 | 12 | 12 |  |
| n254 | 15 | 25 | 25 | 25 |  | FD-FDD |
|  | 30 |  | 12 | 12 |  |  |
| NOTE：The Tx RBs shall be allocated within the range from RBlow = ceil(NRB/2 – NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 – 1, where RBlow and RBhigh are the lowest and highest RB position and NRB_PR3 is 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS. |  |  |  |  |  |  |

Table 2.2-2 Uplink configuration for NR NTN eRedCap UE 2Rx FD-FDD REFSENS

For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not IE eRedCapNotReducedBB-BW-r18, the 2Rx HD-FDD REFSENS requirements and UL configuration are specified as in Table 2.2-3 and Table 2.2-4 respectively.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n256 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.1 | -100.1 | -100.1 |
| n255 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.1 | -100.1 | -100.1 |
| n254 | 15 | -100.0 | -100.0 | -100.0 |  |
|  | 30 |  | -100.1 | -100.1 |  |
| NOTE：The Rx RBs shall be allocated within the range from RBlow = ceil(NRB/2 – NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 – 1, where RBlow and RBhigh are the lowest and highest RB position and NRB_PR3 is 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS. |  |  |  |  |  |

Table 2.2-3 NR NTN eRedCap UE 2Rx HD-FDD REFSENS requirements

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n256 | 15 | 25 | 25 | 25 | 25 | HD-FDD |
|  | 30 |  | 12 | 12 | 12 |  |
| n255 | 15 | 25 | 25 | 25 | 25 | HD-FDD |
|  | 30 |  | 12 | 12 | 12 |  |
| n254 | 15 | 25 | 25 | 25 |  | HD-FDD |
|  | 30 |  | 12 | 12 |  |  |
| NOTE：The Tx RBs shall be allocated within the range from RBlow = ceil(NRB/2 – NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 – 1, where RBlow and RBhigh are the lowest and highest RB position and NRB_PR3 is 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS. |  |  |  |  |  |  |

Table 2.2-4 Uplink configuration for NR NTN eRedCap UE 2Rx HD-FDD REFSENS

For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not IE eRedCapNotReducedBB-BW-r18, the 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are specified as in Table 2.2-5 and Table 2.2-6 respectively.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n256 | 15 | -97.0 | -97.0 | -97.0 | -97.0 |
|  | 30 |  | -97.1 | -97.1 | -97.1 |
| n255 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.6 | -97.6 | -97.6 |
| n254 | 15 | -97.0 | -97.0 | -97.0 |  |
|  | 30 |  | -97.1 | -97.1 |  |
| NOTE：The Rx RBs shall be allocated within the range from RBlow = ceil(NRB/2 – NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 – 1, where RBlow and RBhigh are the lowest and highest RB position and NRB_PR3 is 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS. |  |  |  |  |  |

Table 2.2-5 NR NTN eRedCap UE 1Rx FD-FDD REFSENS requirements

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n256 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.6 | -97.6 | -97.6 |
| n255 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.6 | -97.6 | -97.6 |
| n254 | 15 | -97.5 | -97.5 | -97.5 |  |
|  | 30 |  | -97.6 | -97.6 |  |
| NOTE：The Rx RBs shall be allocated within the range from RBlow = ceil(NRB/2 – NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 – 1, where RBlow and RBhigh are the lowest and highest RB position and NRB_PR3 is 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS. |  |  |  |  |  |

Table 2.2-6 NR NTN eRedCap UE 1Rx HD-FDD REFSENS requirements

For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not IE eRedCapNotReducedBB-BW-r18, the UL configuration for 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are the same as specified in Table 2.2-2 and Table 2.2-4 respectively.

Proposal 5: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the 2Rx FD-FDD REFSENS requirements and UL configuration are specified as in Table 2.2-1 and Table 2.2-2 respectively.

Proposal 6: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the 2Rx HD-FDD REFSENS requirements and UL configuration are specified as in Table 2.2-3 and Table 2.2-4 respectively.

Proposal 7: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are specified as in Table 2.2-5 and Table 2.2-6 respectively.

Proposal 8: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the UL configuration for 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are the same as specified in Table 2.2-2 and Table 2.2-4 respectively.

# 3	Conclusion

In this contribution, we share our views on the REFSENS requirements for NR NTN (e)RedCap UE.

Proposal 1: NR NTN RedCap UE 2Rx FD-FDD REFSENS requirements and UL configuration are specified as in Table 2.1-1 and Table 2.1-2 respectively.

Proposal 2: NR NTN RedCap UE 2Rx HD-FDD REFSENS requirements and UL configuration are specified as in Table 2.1-4 and Table 2.1-5 respectively.

Proposal 3: NR NTN RedCap UE 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are specified in Table 2.1-7 and Table 2.1-8 respectively.

Proposal 4: The UL configurations for NR NTN RedCap UE 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are specified in Table 2.1-2 and Table 2.1-5 respectively.

Proposal 5: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the 2Rx FD-FDD REFSENS requirements and UL configuration are specified as in Table 2.2-1 and Table 2.2-2 respectively.

Proposal 6: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the 2Rx HD-FDD REFSENS requirements and UL configuration are specified as in Table 2.2-3 and Table 2.2-4 respectively.

Proposal 7: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are specified as in Table 2.2-5 and Table 2.2-6 respectively.

Proposal 8: For NR NTN eRedCap UE supporting IE supportOfERedCap-r18 but not supporting IE eRedCapNotReducedBB-BW-r18, the UL configuration for 1Rx FD-FDD and 1Rx HD-FDD REFSENS requirements are the same as specified in Table 2.2-2 and Table 2.2-4 respectively.

# 4	References

RP-234078 “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Huawei, 3GPP TSG-RAN Meeting #102, Edinburgh, Scotland, December 11th – 15th, 2023

R4-2406610 “WF on NR_NTN_Ph3_UERF”, Qualcomm, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024

3GPP TS 38.101-5 V18.5.0 (2024-03)

R4-2203692 “RedCap UE HD-FDD REFSENS requirements”, Apple, 3GPP TSG RAN WG4 Meeting #102-e, Online, February 21st – March 3rd, 2022

3GPP TS 38.101-1 V18.5.0 (2024-03) clause 7.3I.2