3GPP TSG-RAN4 Meeting #111	R4-2407590

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Source:	Samsung

Title:	Discussion on MPR reduction for FR1 single carrier

Agenda item:	10.1.1.3.1

Document for:	Discussion

Introduction

In RAN4#110-bis, a WF [1] is approved which focus on power domain enhancements. The following WF was captured, which focus on power domain enhancements for single carrier.

## 1.1 Scenarios for power domain enhancements for single carrier

Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator)

Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators)

Scenario 2: Narrower UE channel BW within wider BS bandwidth

Way forward:

- Prioritize scenario 1-1 and scenario 2 for initial study of power domain enhancements for single carrier in terms of relaxed requirements

  - FFS on sub-scenarios of scenario 2.

- Scenario 1-2 will be studied after scenario 1-1 and scenario 2

## 1.2. relaxed requirements from co-existence/regulation perspective

Way forward:

- For regions where at least ACLR and SEM can be relaxed for the identified scenarios, FFS whether SE could be relaxed, or under which conditions can be relaxed.

## 1.4 Evaluation of relaxed requirements

Way forward:

- [No relaxation of ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum for scenario 2, i.e. Narrower UE channel BW within wider BS bandwidth]

- FFS whether outer, edge or inner RB allocation is prioritized for further evaluation

- FFS impact on MPR by relaxing ACLR w/ or w/o relaxing SEM/Spurious Emissions

- FFS whether IBE is considered instead of the relaxed ACLR/SEM/SE for the region between UE CBW and BS CBW.

Discussion

When narrower UE channel BW is within wider BS bandwidth (i.e., Scenario 2), there is big room to relax ACLR/SEM. However, when ACLR/SEM/SE are outside of one operator holding spectrum, no relaxation should be considered since there are potential adjacent co-existence issues.

The relationship between the BS CBW and one operator holding spectrum is shown in Figure 1. In WF, it is said that

[No relaxation of ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum for scenario 2, i.e. Narrower UE channel BW within wider BS bandwidth]

In our understanding, it should be “ACLR/SEM/SE outside of one operator holding spectrum” instead of “ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum”, and ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum can be relaxed since the BS CBW is within one operator holding spectrum as shown in Figure 1.

Figure 1: Illustration of BS CBW for one operator holding spectrum

Proposal 1: No relaxation of ACLR/SEM/SE outside of one operator holding spectrum for scenario 2 will be considered since there are potential adjacent co-existence issues.

As discussed in last meeting, Scenario 2 mainly focuses on when there are other BSs of adjacent operators. Scenario 2-1 can be when non-RedCap UE CBW is located at the edge of BS BW; Scenario 2-2 can be when RedCap UE CBW is located at the edge of BS BW with the Shifted Channel Edge approaches, as mentioned by MTK’s paper [3] of last meeting; Scenario 2-3 can be when non-RedCap UE CBW is located around the center of BS BW; Scenario 2-4 can be when RedCap UE CBW is located around the center of BS BW with the Shifted Channel Edge approaches, as mentioned by MTK’s paper [3] of last meeting.

Proposal 2:

Discuss the following sub-scenarios of scenario 2:

Scenario 2-1 can be when non-RedCap UE CBW is located at the edge of BS BW;

Scenario 2-2 can be when RedCap UE CBW is located at the edge of BS BW with the Shifted Channel Edge approaches;

Scenario 2-3 can be when non-RedCap UE CBW is located around the center of BS BW;

Scenario 2-4 can be when RedCap UE CBW is located around the center of BS BW with the Shifted Channel Edge approaches.

As mentioned by MTK’s paper [3] of last meeting, for RedCap UE, the Shifted Channel Edge approaches can be applied when narrower UE channel BW is within wider BS bandwidth (i.e., Scenario 2). In this context, IBE requirements can be considered for the region between actual UE channel edge and the shifted channel edge instead of relaxed ACLR/SEM. In our understanding, the Shifted Channel Edge approaches should be mainly for RedCap UE rather than for both RedCap UE and non-RedCap UE, as non-Red Cap UE can achieve same effect as the Shifted Channel Edge approaches simply by scheduling only inner RB in some cases. More specific, for non-RedCap UE with CBW> 50 MHz, if the Shifted Channel Edge approaches are applied, the equivalent extended CBW will be larger than 100 MHz for which there are no corresponding requirements specified for FR1. For non-RedCap UE with CBW<= 50 MHz, there is no need to apply the Shifted Channel Edge approaches since NW can directly/simply schedule larger CBW with inner RB allocations for non-RedCap UE. Overall, such Shifted Channel Edge approaches should not be considered for non-RedCap UE. On the contrary, such Shifted Channel Edge approaches could bring some benefits for RedCap UE in principle since the maximum UE CBW is limited to 20 MHz thus NW is not able to schedule larger CBW beyond this.

Proposal 3: The Shifted Channel Edge approaches should not be considered for non-RedCap UE.

Proposal 4: For RedCap UE, the Shifted Channel Edge approaches can be considered and IBE requirements can be applied to the new extended CBW.

As already discussed in our paper [2] last meeting, different from SEM and ACLR, SE is deemed as regulatory requirements to our understanding. Though 3GPP also has general SE requirements, the regulator usually have additional requirements and could be more stringent. Correspondingly 3GPP general SE requirements are more like a reference, and the regulatory SE requirements determine when it comes to certification. Besides, SE also servers the purpose for protection of remote frequency spectrum, relaxing SE has more risk of interfering the deployment of other operators. Therefore, in our view, SE requirements should not be relaxed in lack of justification.

Proposal 5: SE requirements should not be relaxed in lack of justification.

In the discussion of Rel-18, some technologies were considered and specified, such as FDSS without SE, for power boosting/MPR reduction, which are limited to inner RB allocations. Furthermore, the specified MPR values are relatively small for inner RB allocations, thus the room for improvement is less compared to outer RB allocations.

Observation 1: In the discussion of Rel-18, some power boosting/MPR reduction technologies were limited to inner RB allocations, such as FDSS without SE.

Observation2: The MPR values are relatively small for inner RB allocations.

In addition, in this WID, only QPSK and 16QAM are considered for MPR reduction with ACLR/SEM/spurious emission relaxation on FR1 single carrier. In this context, as mentioned by vivo’s paper [4] of last meeting, ACLR and SEM are the major limiting factor for outer RB allocations; SEM are the major limiting factor for edge RB allocations; SEM are the major limiting factor for the inner RB allocations close to the edge of BW. When ACLR and SEM are relaxed, outer RB allocations can gain the most.

As also mentioned by MTK’s paper [3] of last meeting, from analysis in Rel-15, it has been shown that for ACLR/SEM is generally the limiting factor driving Outer RB allocation MPR requirements for QPSK and 16QAM.

Observation 3: When ACLR and SEM are relaxed, outer RB allocations can gain the most.

We take a few simulation points to further verify the viewpoints from vivo and MTK.

For QPSK outer RB allocations with 100RB (RBStart = 3) on 20 MHz channel bandwidth:

MPR can reduce by about 0.361 dB with 1 dB ACLR relaxation and reduce by about 0.697 dB with 2 dB ACLR relaxation;

MPR can reduce by about 0 dB with 1 dB SEM relaxation and reduce by about 0 dB with 2 dB SEM relaxation;

MPR can reduce by about 0.361 dB with 1 dB ACLR and SEM relaxation and reduce by about 0.697 dB with 2 dB ACLR and SEM relaxation;

As can found, the major limiting factor is ACLR for QPSK outer RB allocations with 100RB (RBStart = 3) on 20 MHz channel bandwidth.

For QPSK outer RB allocations with 60 RB (RBStart = 23) on 20 MHz channel bandwidth:

MPR can reduce by about 0 dB with 1 dB ACLR relaxation and reduce by about 0 dB with 2 dB ACLR relaxation;

MPR can reduce by about 0 dB with 1 dB SEM relaxation and reduce by about 0 dB with 2 dB SEM relaxation;

MPR can reduce by about 0.309 dB with 1 dB ACLR and SEM relaxation and reduce by about 0.542 dB with 2 dB ACLR and SEM relaxation;

As can found, the major limiting factor is ACLR and SEM for QPSK outer RB allocations with 60 RB (RBStart = 23) on 20 MHz channel bandwidth.

For QPSK outer RB allocations with 20 RB (RBStart = 80) on 20 MHz channel bandwidth:

MPR can reduce by about 0 dB with 1 dB ACLR relaxation and reduce by about 0 dB with 2 dB ACLR relaxation;

MPR can reduce by about 0.234 dB with 1 dB SEM relaxation and reduce by about 0.464 dB with 2 dB SEM relaxation;

MPR can reduce by about 0.234 dB with 1 dB ACLR and SEM relaxation and reduce by about 0.464 dB with 2 dB ACLR and SEM relaxation;

As can found, the major limiting factor is SEM for QPSK outer RB allocations with 20 RB (RBStart = 80) on 20 MHz channel bandwidth.

Observation 4: ACLR is the major limiting factor for QPSK outer RB allocations with large RB numbers.

Observation 5: ACLR and SEM are the major limiting factor for QPSK outer RB allocations with medium RB numbers.

Observation 6: SEM is the major limiting factor for QPSK outer RB allocations with small RB numbers.

Although the major limiting factor is different for outer RB allocations with different RB numbers, it is not necessary to identify the major bottleneck for each allocation and define the relaxed condition and requirements accordingly, which will lead to more complicated process and fragmented requirements which is not preferred by RAN4 in a procedure perspective as well as UE/NW implementation perspective. Therefore, for simplify, ACLR/SEM can be relaxed for MPR reduction together.

Observation 7:  It is not necessary to identify the major bottleneck for each allocation and define the relaxed condition and requirements accordingly.

Proposal 6: For simplify, ACLR/SEM can be relaxed for MPR reduction together.

Observation 8: When ACLR/SEM are relaxed 1 dB, MPR can reduce 0.234-0.361 dB for QPSK outer RB allocations.

Observation 9: When ACLR/SEM are relaxed 2 dB, MPR can reduce 0.464-0.697 dB for QPSK outer RB allocations.

- 3 Conclusion

In the document, the following Observations and Proposals are made:

Proposal 1: No relaxation of ACLR/SEM/SE outside of one operator holding spectrum for scenario 2 will be considered since there are potential adjacent co-existence issues.

Proposal 2:

Discuss the following sub-scenarios of scenario 2:

Scenario 2-1 can be when non-RedCap UE CBW is located at the edge of BS BW;

Scenario 2-2 can be when RedCap UE CBW is located at the edge of BS BW with the Shifted Channel Edge approaches;

Scenario 2-3 can be when non-RedCap UE CBW is located around the center of BS BW;

Scenario 2-4 can be when RedCap UE CBW is located around the center of BS BW with the Shifted Channel Edge approaches.

Proposal 3: The Shifted Channel Edge approaches should not be considered for non-RedCap UE.

Proposal 4: For RedCap UE, the Shifted Channel Edge approaches can be considered and IBE requirements can be applied to the new extended CBW.

Proposal 5: SE requirements should not be relaxed in lack of justification.

Observation 1: In the discussion of Rel-18, some power boosting/MPR reduction technologies were limited to inner RB allocations, such as FDSS without SE.

Observation2: The MPR values are relatively small for inner RB allocations.

Observation 3: When ACLR and SEM are relaxed, outer RB allocations can gain the most.

Observation 4: ACLR is the major limiting factor for QPSK outer RB allocations with large RB numbers.

Observation 5: ACLR and SEM are the major limiting factor for QPSK outer RB allocations with medium RB numbers.

Observation 6: SEM is the major limiting factor for QPSK outer RB allocations with small RB numbers.

Observation 7:  It is not necessary to identify the major bottleneck for each allocation and define the relaxed condition and requirements accordingly.

Proposal 6: For simplify, ACLR/SEM can be relaxed for MPR reduction together.

Observation 8: When ACLR/SEM are relaxed 1 dB, MPR can reduce 0.234-0.361 dB for QPSK outer RB allocations.

Observation 9: When ACLR/SEM are relaxed 2 dB, MPR can reduce 0.464-0.697 dB for QPSK outer RB allocations.

- References

- R4-2406584, WF on power domain enhancements, Huawei.

- R4-2404588, Discussion on MPR reduction for FR1 and FR2, Samsung.

- R4-2405619, Discussion on power domain enhancement for NR single carrier, MediaTek Inc.

- R4-2404642, Discussion on MPR reduction for NR single carrier and NR intra-band UL CA, vivo.