3GPP TSG-RAN4 Meeting #111	R4-2407596

# Fukuoka City, Fukuoka, Japan, 20th May 2024 - 24th May 2024

Title:	TP to TR 38.898 Addition of PC2 for DC_1A-18A_n77A

Source:	Samsung, KDDI Corporation, Murata

Agenda item:	6.15.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.898 to add PC2 for DC_1A-18A_n77A. All PC2 fallbacks have been specified.

************************************* Start of TP*****************************************

## 5.x	DC_1-18_n77

#### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-18A_n77A5,14 | DC_1A_n77A14 DC_18A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. |  |

#### 5.x.2	Maximum output power for DC

Since the maximum output power requirements for PC2 UL DC_1_n77 and DC_18_n77 are already specified in the specification, this section can be omitted.

#### 5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- IMD5 of dual UL DC_1_n77 may fall into Rx frequencies of band 18.

- IMD3 of dual UL DC_18_n77 may fall into Rx frequencies of band 1.

New PC2 MSDs are defined in the following table.

Table 5.x.3-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-18A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 18 | N/A | 5 | N/A | 870 | 15.8 | IMD5 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | N/A |
|  | 1 | N/A | 5 | N/A | 2120 | 25.0 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |

#### 5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

************************************* End of TP*****************************************

# Reference

[1] RP-240241 Revised WID: Core part: High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)