| 3GPP TR 38.894 V0.7.0 (2024-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	11

3.3	Abbreviations	11

4	Background	12

4.1	Background information in Rel-17	12

4.2	WI Objective in Rel-18	12

5	Specific Band combinations	13

5.1	CA_n34A-n41A	13

5.1.1	Status of the band combination	13

5.1.1.1	Operating bands for CA	13

5.1.1.2	Power class of the BC	13

5.1.1.3	∆TIB and ∆RIB	13

5.1.2	MSD analysis for simultaneous Rx/Tx	14

5.1.2.1	Reference UE architecture	14

5.1.2.2	RF component assumptions	14

5.1.2.3	Calculated MSD values	15

5.1.3	Requirements for simultaneous Rx/Tx	16

5.1.3.1	Reference sensitivity requirements	16

5.2	CA_n39A-n41A	16

5.2.1	Status of the band combination	16

5.2.1.1	Operating bands for CA	16

5.2.1.2  	Power class of the BC	16

5.2.2	MSD analysis for simultaneous Rx/Tx	17

5.2.2.1	Reference UE architecture	17

5.2.2.2	RF component assumptions	17

5.2.2.3	Calculated MSD values for cross band isolation	18

5.2.2.4	Calculated MSD values for harmonic mixing	19

5.2.3    	 Requirements for simultaneous Rx/Tx	19

5.2.3.1	∆TIB and ∆RIB values	19

5.2.3.2	Reference sensitivity requirements for cross band isolation	20

5.2.3.3	Reference sensitivity requirements for Harmonic mixing	20

5.3	CA_n40A-n41A	21

5.3.1	Status of the band combination	21

5.3.1.1	Operating bands for CA	21

5.3.1.2	Power class of the BC	21

5.3.1.3	∆TIB and ∆RIB	22

5.3.2	MSD analysis for simultaneous Rx/Tx	22

5.3.2.1	Reference UE architecture	22

5.3.2.2	RF component assumptions	22

5.3.2.3	Calculated MSD values	24

5.3.3	Requirements for simultaneous Rx/Tx	26

5.3.3.1	∆TIB and ∆RIB values	26

5.3.3.2	Reference sensitivity requirements	26

5.4	CA_n7A-n40A	27

5.4.1	Status of the band combination	27

5.4.1.1	Operating bands for CA	27

5.4.1.2	Power class of the BC	27

5.4.1.3	∆TIB and ∆RIB	27

5.4.1.4	Existing MSD requirement	27

5.4.2	MSD analysis for simultaneous Rx/Tx	28

5.4.2.1	Reference UE architecture	28

5.4.2.2	RF component assumptions	28

5.4.2.3	Calculated MSD values	30

5.4.3	Requirements for simultaneous Rx/Tx	31

5.4.3.1	Reference sensitivity requirements	31

5.5	CA_n28A-n39A-n41A	31

5.5.1	Status of the band combination	31

5.5.1.1	Operating bands for CA	31

5.5.1.2	∆TIB and ∆RIB	31

5.5.1.3	Existing MSD requirement	32

5.5.2	MSD analysis for simultaneous Rx/Tx	32

5.5.2.1	Reference UE architecture	32

5.5.2.2	RF component assumptions	33

5.5.2.3	Calculated MSD values	33

5.5.3	Requirements for simultaneous Rx/Tx	34

5.5.3.1	∆TIB and ∆RIB values	34

5.5.3.2	Reference sensitivity requirements	35

5.6	CA_n39A-n40A-n41A	35

5.6.1	Status of the band combination	35

5.6.1.1	Operating bands for CA	35

5.6.1.2	∆TIB and ∆RIB	35

5.6.1.3	Existing MSD requirement	36

5.6.2	MSD analysis for simultaneous Rx/Tx	36

5.6.2.1	Reference UE architecture	36

5.6.2.2	RF component assumptions	37

5.6.2.3	Calculated MSD values	39

5.6.3	Requirements for simultaneous Rx/Tx	40

5.6.3.1	∆TIB and ∆RIB values	40

5.6.3.2	Reference sensitivity requirements	40

5.7	CA_n39A-n41A-n79A	41

5.7.1	Status of the band combination	41

5.7.1.1	Operating bands for CA	41

5.7.1.2	∆TIB and ∆RIB	41

5.7.1.3	Existing MSD requirement	41

5.7.2	MSD analysis for simultaneous Rx/Tx	42

5.7.2.1	Reference UE architecture	42

5.7.2.2	RF component assumptions	42

5.7.2.3	Calculated MSD values	43

5.7.3	Requirements for simultaneous Rx/Tx	44

5.7.3.1	∆TIB and ∆RIB values	44

5.7.3.2	Reference sensitivity requirements	44

5.8	CA_n8A-n40A-n41A	45

5.8.1	Status of the band combination	45

5.8.1.1	Operating bands for CA	45

5.8.1.2	∆TIB and ∆RIB	45

5.8.1.3	Existing MSD requirement	45

5.8.2	MSD analysis for simultaneous Rx/Tx	46

5.8.2.1	Reference UE architecture	46

5.8.2.2	RF component assumptions	46

5.8.2.3	Calculated MSD values	48

5.8.3	Requirements for simultaneous Rx/Tx	49

5.9	CA_n28A-n40A-n41A	49

5.9.1	Status of the band combination	49

5.9.1.1	Operating bands for CA	49

5.9.1.2	∆TIB and ∆RIB	49

5.9.1.3	Existing MSD requirement	49

5.9.2	MSD analysis for simultaneous Rx/Tx	50

5.9.2.1	Reference UE architecture	50

5.9.2.2	RF component assumptions	50

5.9.2.3	Calculated MSD values	53

5.9.3	Requirements for simultaneous Rx/Tx	53

5.10	CA_n40A-n41A-n79A	53

5.10.1	Status of the band combination	53

5.10.1.1	Operating bands for CA	53

5.10.1.2	∆TIB and ∆RIB	54

5.10.1.3	Existing MSD requirement	54

5.10.2	MSD analysis for simultaneous Rx/Tx	55

5.10.2.1	Reference UE architecture	55

5.10.2.2	RF component assumptions	55

5.10.2.3	Calculated MSD values	58

5.10.3	Requirements for simultaneous Rx/Tx	59

5.x	CA_nA-nB	59

5.x.1	Status of the band combination	59

5.x.2	MSD analysis	59

5.x.3	Requirements for simultaneous Rx/Tx	59

Annex A (informative): Change history	60

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for requirements on simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC.

Requested FDD-TDD, TDD-TDD CA, SUL, MR-DC and NR-DC band combinations for evaluation of supporting simultaneous Rx/Tx capability/operation are provided in the table below.

Table 1: Band combinations for evaluation of simultaneous Rx/Tx

| Band combination | Power Class |
| --- | --- |
| CA_n34A-n41A | PC3 |
| CA_n39A-n41A | PC3 |
| CA_n40A-n41A | PC3, PC2 |
| CA_n7A-n40A | PC3 |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-210890, “New WID on New WID on simultaneous Rx/Tx band combinations for CA, SUL, MR-DC and NR-DC”.

[3]	RP-222648, “New WID: Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18”.

[4]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[5]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[6]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component Carrier

DC	Dual Connectivity

EIRP	Equivalent Isotropically Radiated Power

EN-DC	E-UTRA/NR DC

EVM	Error Vector Magnitude

FDM	Frequency Division Multiplexing

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MCG	Master Cell Group

NR	New Radio

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control Channel

PSSCH	Physical Sidelink Shared Channel

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SL	Sidelink

SUL	Supplementary uplink

TDM	Time Division Multiplex

Tx	Transmitter

UE	User Equipment

UL MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

4	Background

4.1	Background information in Rel-17

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

In Rel-17, the principles for judging the mandatory capability for a band combination have been discussed, and the cases include:

-	FR1+FR1 FDD-TDD band combination

-	FR1+FR1 TDD-TDD band combination

-	FR1+FR2 FDD-TDD band combination

-	FR1+FR2 TDD-TDD band combination

-	FR2+FR2 TDD-TDD band combination

For some categories, the capability should be checked case by case. Therefore, to facilitate the analysis of specific band combinations, a dedicated WI is needed to avoid the ambiguity for application of the general principles agreed in Rel-17 and the analysis and conclusion should be captured in the TR.

In addition, as the capability is defined for CA, SUL, MR-DC and NR-DC band combinations, and applicability of the corresponding requirements cover different specifications, e.g. TS 38.101-1 and TS 38.101-3, the way to treat simultaneous Rx/Tx capability as well as the requirements should be aligned among the specifications.

The fallback rules are captured as below for reference:

-	Request for additions of band combinations to this WI shall be provided using an agreed template and sent to the 3GPP_TSG_RAN_WG4_NR_BANDS email reflector before a RAN4 Tdoc submission deadline and no new band combinations are allowed to be requested after the deadline except to correct the missing fallback and add more supporting companies for the proposed band combinations.

-	When a proponent requests a new band combination, all the next level fallback configurations shall be listed and recorded in the request template and the status (“New”, “Ongoing”, “Completed”) of all the fallback configurations shall be declared accurately and clearly. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination in the same meeting.

-	A band combination configuration can only be considered as completed when all of the fallback configurations are completed and specified in advance or at the same meeting. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Rapporteurs and other companies are encouraged to check the status of all of the fallback configurations once the higher order band combinations are declared as completed.

4.2	WI Objective in Rel-18

The objectives of the Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC WI in Rel-18 are as follows:

1.	Identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values.

Note 1: Band combinations considered in this WI have to be introduced first via basket WIs (see 2.3) or completed in previous releases if necessary.

Note 2: Whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case.

2.	Align the specification treatment of simultaneous Rx/Tx capability for CA, SUL, MR-DC and NR-DC band combinations.

5	Specific Band combinations

5.1	CA_n34A-n41A

5.1.1	Status of the band combination

5.1.1.1	Operating bands for CA

The operating bands for CA_n34-n41 are specified in Table 5.1.1.1-1.

Table 5.1.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n34-n419 | n34, n41 |  |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

For CA_n34-n41, the current spec only considers scenarios without simultaneous Rx/Tx operation.

5.1.1.2	Power class of the BC

The UE Power Class for uplink CA_n34A-n41A are specified in Table 5.1.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

Table 5.1.1.2-1 UE Power Class for uplink CA_n34A-n41A

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n34A-n41A |  |  |  |  | 23 | +2/-3 |  |  |

5.1.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n34-n41 are specified in Table 5.1.1.3-1.

Table 5.1.1.3-1: ΔTIB,c due to CA_n34-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n34-n41 | n34 | 0.3 |
|  | n41 | 0.3 |

ΔRIB,c due to CA_n34-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.1.2	MSD analysis for simultaneous Rx/Tx

5.1.2.1	Reference UE architecture

The following reference UE architecture for CA_n34A-n41A is considered for simultaneous Rx/Tx operation.

Figure 5.1.2.1-1: Reference UE architecture for CA_n34-n41

5.1.2.2	RF component assumptions

Typical duplexer performance of n41 is shown in the figure below:

Figure 5.1.2.2-1: Example duplexer performance of band n41

According to the available data from vendors, band n41 Rx/Tx filter rejection at n34 is around 27dB.

Figure 5.1.2.2-2: Example filter performance of band n34

In the analysis, we adopt value of 28dB for the band n34 Rx/Tx filter rejection at n41.

Table 5.1.2.2-1: Assumptions on MSD analysis for CA_n34-n41

| PA far end noise floor (dBm/Hz) | -130 |
| --- | --- |
| Transceiver noise floor at PA output (dBm/Hz) | -132 |
| PA output power at antenna port (dBm) | 23 |
| n41 Rx filter rejection at n34 Tx (dB) | 27 |
| n34 Tx filter rejection at n41Rx (dB) | 28 |
| n34 Rx filter rejection at n41Tx (dB) | 28 |
| n41Tx filter rejection at n34 Rx (dB) | 27 |
| RFIC IIP2 (dBm) | 50 |
| Band n40 Tx LO phase noise (dBc/Hz) | -155 |
| Band n41 Rx LO phase noise (dBc/Hz) | -155 |
| Antenna ISO (dB) | 10 |
| Band n34/n41 Tx FE Loss (dB) | 4 |
| Band n34/n41 Rx FE Loss (dB) | 4 |
| Band n34 Rx noise figure (dB) | 9 |
| Band n41 Rx noise figure (dB) | 11 |
| FFS: Other parameters |  |

5.1.2.3	Calculated MSD values

Proposed Option 1:

Table 5.1.2.3-1: Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A [R4-2300347]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n41 | 2017.5 | 15 | 15 | 78 (RBstart=0) | 2501 | 10 | 2.3 | >ACLR2 |
| n34 | n41 | 2017.5 | 15 | 15 | 78 (RBstart=0) | 2546 | 100 | 2.3 | >ACLR2 |
| n41 | n34 | 2546 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 9.2 | >ACLR2 |
| n41 | n34 | 2456 | 100 | 30 | 270 (RBstart=0) | 2017.5 | 15 | 9.4 | >ACLR2 |

Proposed Option 2:

Table 5.1.2.3-2: Cross band isolation for simultaneous Rx-Tx with CA_n34A-n41A [R4-2302054]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n41 | 2012.5 | 5 | 15 | 25 (RBstart=0) | 2501 | 10 | 4.1 | >ACLR2 |
| n34 | n41 | 2012.5 | 5 | 15 | 25 (RBstart=0) | 2546 | 100 | 2.8 | >ACLR2 |
| n41 | n34 | 2521 | 50 | 30 | 128 (RBstart=0) | 2022.5 | 5 | 5.2 | >ACLR2 |

The agreed cross band isolation for simultaneous Rx-Tx with CA_n34A-n41A are as follows in Table 5.1.2.3-4.

Table 5.1.2.3-4: Cross band isolation for simultaneous Rx-Tx with CA_n34A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n41 | 2017.5 | 15 | 15 | 78 (RBstart=0) | 2501 | 10 | [3.2] | >ACLR2 |
| n41 | n34 | 2456 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | [7.2] | >ACLR2 |

5.1.3	Requirements for simultaneous Rx/Tx

5.1.3.1	Reference sensitivity requirements

Table 5.1.3.1-1: Cross band isolation for simultaneous Rx-Tx with CA_n34A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2501 | 10 | 3.2 | >ACLR2 |
| n41 | n34 | 2456 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 7.2 | >ACLR2 |

5.2	CA_n39A-n41A

5.2.1	Status of the band combination

5.2.1.1	Operating bands for CA

The operating bands for CA_n39-n41 are specified in Table 5.2.1.1-1.

Table 5.2.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.x-1) |
| --- | --- |
| CA_n39-n41 | n39, n41 |

For CA_n39-n41, the current spec only considers scenarios without simultaneous Rx/Tx.

5.2.1.2  	Power class of the BC

The UE Power Class for uplink CA_n39A-n41A are specified in Table 5.2.1.2-1.

Table 5.2.1.2-1 UE Power Class for uplink CA_n39A-n41A

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n39A-n41A | 23 | +2/-3 |

5.2.2	MSD analysis for simultaneous Rx/Tx

5.2.2.1	Reference UE architecture

The following reference UE architecture for CA_n39A-n41A is considered for simultaneous Rx/Tx operation.

Figure 5.2.2.1-1a: Reference UE architecture for CA_n39-n41 [R4-2216660]

5.2.2.2	RF component assumptions

Typical duplexer performance of n41 is shown in the figure below:

Figure 5.2.2.2-1: Example duplexer performance of band n41

According to the available data from vendors, band n41 Rx/Tx filter rejection at n39 is around 40dB.

Figure 5.2.2.2-2: Example duplexer performance of band n39

In the analysis, we adopt value of 55dB for the band n39 Rx/Tx filter rejection at n41.

5.2.2.3	Calculated MSD values for cross band isolation

Proposed Option 1:

Table 5.2.2.3-1: Cross band isolation for simultaneous Rx-Tx with CA_n39A-n41A [R4-2218106]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.2 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.0 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.5 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 1.5 | >ACLR2 |

Proposed Option 2:

Table 5.2.2.3-2: Cross band isolation for simultaneous Rx-Tx with CA_n39A-n41A [R4-2219212]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.5 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.5 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.1 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 0.8 | >ACLR2 |

Proposed Option 3:

Table 5.2.2.3-3: Cross band isolation for simultaneous Rx-Tx with CA_n39A-n41A [R4-2219417]

| CBW (MHz) | 10 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- |
| n41 MSD (dB) | 3.0 | 1.8 | 1.6 | 1.5 |
| n39 MSD (dB) | 2.4 | 2.3 | 2.3 | 2.3 |

The agreed cross band isolation for simultaneous Rx-Tx with CA_n39A-n41A are as follows in Table 5.2.2.3-4.

Table 5.2.2.3-4: Cross band isolation for simultaneous Rx-Tx with CA_n39A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | >ACLR2 |

5.2.2.4	Calculated MSD values for harmonic mixing

Table 5.2.4-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n39-n41.

Table 5.2.4-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there is no harmonic issue for the band combination of n39 and n41.

Table 5.2.4-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there are two types of, i.e. UL3/DL4 and UL4/DL3 order harmonic mixing issue for the band combination of n39 and n41, i.e.  3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL, and 4th harmonic of band n39 UL may fall into 3rd harmonic of band n41 DL.

5.2.3    		Requirements for simultaneous Rx/Tx

5.2.3.1	∆TIB and ∆RIB values

For CA_n39-n41 supporting simultaneous Rx/Tx, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 5.2.3.1-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n39-n41 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.2.3.2-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n39-n41 | 0.2 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.2.3.2	Reference sensitivity requirements for cross band isolation

The cross band isolation MSD for CA_n39-n41 supporting simultaneous Rx/Tx are defined in table 5.2.3-3.

Table 5.2.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | >ACLR2 |

5.2.3.3	Reference sensitivity requirements for Harmonic mixing

The harmonic mixing MSD requirements for CA_n39-n41 supporting simultaneous Rx/Tx are defined in table 5.2.3-4.

Table 5.2.3-4. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 4.3 | NOTE x | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 0.8 | NOTE x | UL3/DL4 |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE y | UL4/DL3 |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.3 | NOTE y | UL4/DL3 |
| NOTE x: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE y: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

5.3	CA_n40A-n41A

5.3.1	Status of the band combination

5.3.1.1	Operating bands for CA

The operating bands for CA_n40-n41 are specified in Table 5.3.1.1-1.

Table 5.3.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n40-n41 | n40, n41 | No |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx, which are limited by the applicable requirements of ΔTIB,c and ΔRIB,c.

5.3.1.2	Power class of the BC

The UE Power Class for uplink CA_n40A-n41A are specified in Table 5.3.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

Table 5.3.1.2-1 UE Power Class for uplink CA_n40A-n41A

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n40A-n41A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports PC3 within an NR TDD or FDD band and supports PC2 within a second NR TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported [powerClassPerBand] if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

5.3.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n40-n41 are specified in Table 5.3.1.3-1.

Table 5.3.1.3-1: ΔTIB,c due to CA_n40-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n40-n41 | n40 | 0.53 |
|  | n41 | 0.53 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

ΔRIB,c due to CA_n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.3.2	MSD analysis for simultaneous Rx/Tx

5.3.2.1	Reference UE architecture

The following reference UE architecture for CA_n40A-n41A is considered for simultaneous Rx/Tx operation.

Figure 5.3.2.1-1: Reference UE architecture for CA_n40-n41

5.3.2.2	RF component assumptions

Typical duplexer performance of n41 is shown in the figure below:

Figure 5.3.2.2-1: Example duplexer performance of band n41

According to the available data from vendors, band n41 Rx/Tx filter rejection at n40 is around 35~39dB. We take the value of 35dB for the MSD calculation

Figure 5.3.2.2-2: Example filter performance of band n40

In the analysis of Proposed Option 2[R4-2300916] in section 5.3.2.3, adopt value of 35dB for the band n40 Rx/Tx filter rejection at n41 referring to the filter performance of band n40.

Figure 5.3.2.2-3 IMD landscapes sketch for CA_n40A-n41A with non-simultaneous RxTx. Left: n41 10MHz MSD due to 80MHz UL CBW in band n40, Right: n40 5MHz MSD due to 100MHz UL CBW in band n41 [R4-2300916]

Interference levels are measured using a mid-band PA for both PC3 and PC2, with the following assumptions:

-	Power class 3 operation: the PA is calibrated to meet -30dBc at 26dBm output power using a 20 MHz, SCS15 kHz, QPSK, DFT-S-OFDM, waveform with Lcrb=100 RB at lower channel edge,

-	Power class 2 operation: the PA is calibrated to meet -31dBc at 29dBm output power using a 20 MHz, SCS15 kHz, QPSK, DFT-S-OFDM, waveform with Lcrb=100 RB at lower channel edge,

-	Post PA losses: 					 4dB

-	Diplexer to antenna IL:			1.5dB

-	Local Oscillator (LO) leakage: 	-28dBc

-	IQ Image rejection: 				-28dB

-	RF transceiver impairments: 		C-IM3: -60dBc, C-IM5: -70dBc

-	For n40 MSD: The PA spurious emission levels are measured in the band n40 upper 5MHz channel with the UL carrier in band n41 configured to the lowest 100MHz CBW channel, and LCRB = 270 (RBstart=0)

-	For n41 MSD: The PA spurious emission levels are measured in the band n41 lower 10MHz channel with the UL carrier in band n40 configured to the highest 80MHz CBW channel, and LCRB = 216 (RBstart=1)

Figure 5.3.2.2-4 summarizes the measured PA noise levels and estimated MSD for n40, n41, and for PC2 and PC3 operations [R4-2300916]:

-	For UL n41 100MHz UL CBW and LCRB=270 (RBstart=0), we measure at the band n40 5MHz CBW upper DL channel, an integrated PA noise level of -25.5dBm for PC3, and -22.4dBm for PC2 operation (Figure 5.3.2.2-4 -left)

-	For UL n40 80MHz UL CBW and LCRB=216 (RBstart=1), we measure at the band n41 10MHz CBW lower DL channel, an integrated PA noise level of -23.5dBm for PC3, and -20.3dBm for PC2 operation (Figure 5.3.2.2-4 -right)

At these levels:

-	Figure 5.3.2.2-4 -left shows that the band n40 5MHz MSD is 32.4dB for PC3, and 35.5dB for PC2

-	Figure 5.3.2.2-4 -right shows that the band n41 10MHz MSD is 24.3dB for PC3, and 27.4dB for PC2

Figure 5.3.2.2-4 CA_n40-n41 non-simultaneous RxTx MSD. Left: band n40 5MHz upper channel MSD, Right: band n41 10MHz lower channel MSD [R4-2300916]

Table 5.3.2.2-1: Assumptions on MSD analysis for CA_n40-n41 for Proposed Option 4[R4-2305436]

| PA far end noise floor (dBm/Hz) | -130 |
| --- | --- |
| Transceiver noise floor at PA output (dBm/Hz) | -132 |
| n41 PA Tx noise regrowth beyond ACLR2 (dBc) | 40 for 10MHz @2501M |
| n40 PA Tx noise regrowth beyond ACLR2 (dBc) | 40 for 10MHz @2397.5M |
| PA output power at antenna port (dBm) | 26/23 |
| n41 Rx filter rejection at n40 Tx (dB) | 35 |
| n40 Tx filter rejection at n41Rx (dB) | 35 |
| n40 Rx filter rejection at n41Tx (dB) | 35 |
| n41Tx filter rejection at n40 Rx (dB) | 35 |
| RFIC IIP2 (dBm) | 50 |
| Band n40 Tx LO phase noise (dBc/Hz) | -155 |
| Band n41 Rx LO phase noise (dBc/Hz) | -155 |
| Antenna ISO (dB) | 10 |
| Band n40/n41 Tx FE Loss (dB) | 4 |
| Band n40/n41 Rx FE Loss (dB) | 4 |
| Band n40 Rx noise figure (dB) | 9 |
| Band n41 Rx noise figure (dB) | 11 |
| FFS: Other parameters |  |

5.3.2.3	Calculated MSD values

Proposed Option 1:

Table 5.3.2.3-1: Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A [R4-2304319]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 32.3 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 24.4 | ACLR2 |

Proposed Option 2:

Table 5.3.2.3-2a: PC3 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A [R4-2305741]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 25.7 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 32.4 | ACLR2 |

Table 5.3.2.3-2b: PC2 Cross band isolation for simultaneous RxTx in CA_n40A-n41A.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.9 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 35.5 | ACLR2 |

Proposed Option 3:

Table 5.3.2.3-3: Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A [R4-2304871]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 25.7 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 35 | ACLR2 |

Proposed Option 4:

Table 5.3.2.3-4: Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A [R4-2305436]

| DL CBW (MHz) | 5 |
| --- | --- |
| n40 MSD (dB) | 25.6 |

| DL CBW (MHz) | 10 |
| --- | --- |
| n41 MSD (dB) | 20.6 |

The agreed cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A are as follows in Table 5.4.2.3-5a and Table 5.3.2.3-5b

Table 5.3.2.3-5a: PC3 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | [28.1] | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | [31.4] | ACLR2 |

Table 5.3.2.3-5b: PC2 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | [31.1] | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | [34.4] | ACLR2 |

5.3.3	Requirements for simultaneous Rx/Tx

5.3.3.1	∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c applied to UE supporting inter-band carrier aggregation with simultaneous Rx/Tx are as follows.

Table 5.3.3.1-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n40-n41 | 0.53 | 0.53 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.3.3.1-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n40-n41 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.3.3.2	Reference sensitivity requirements

Table 5.3.3.2-1a: PC3 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | ACLR2 |

Table 5.3.3.2-1b: PC2 Cross band isolation for simultaneous Rx-Tx with CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 31.1 | ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 34.4 | ACLR2 |

5.4	CA_n7A-n40A

5.4.1	Status of the band combination

5.4.1.1	Operating bands for CA

The operating bands for CA_n7-n40 are specified in Table 5.4.1.1-1.

Table 5.4.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n7-n40 | n7, n40 |  |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n7-n40, the current spec has no limitation on scenarios with or without simultaneous Rx/Tx operation.

5.4.1.2	Power class of the BC

The UE Power Class for uplink CA_n7A-n40A are specified in Table 5.4.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

Table 5.4.1.2-1 UE Power Class for uplink CA_n7A-n40A

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n40A |  |  |  |  | 23 | +2/-3 |  |  |

5.4.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n7-n40 are specified in Table 5.4.1.3-1.

Table 5.4.1.3-1: ΔTIB,c due to CA_n7-n40

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n7-n40 | n7 | 0.5 |
|  | n40 | 0.6 |

The ΔRIB,c due to CA_n7-n40 are specified in Table 5.4.1.3-2.

Table 5.4.1.3-2: ΔRIB,c due to CA_n7-n40

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n7-n40 | n40 | 0.5 |

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.4.1.4	Existing MSD requirement

MSD values due to crossband isolation specified in Rel-17 are in brackets, which should be further studied.

Table 5.4.1.4-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | [21.9] | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | [13.5] | >ACLR2 |

5.4.2	MSD analysis for simultaneous Rx/Tx

5.4.2.1	Reference UE architecture

The following reference UE architecture for CA_n7A-n40A is considered for simultaneous Rx/Tx operation.

Figure 5.4.2-1: Reference UE architecture for CA_n7-n40

5.4.2.2	RF component assumptions

MSD at band n7 with interference from n40:

-	n7 Rx filter rejection at n40 Tx (IM2, reciprocal mixing)

-	n40 Tx filter rejection at n7 Rx (Tx noise at Rx frequency)

MSD at band n40 with inference from n7:

-	n40 Rx filter rejection at n7 Tx (IM2, reciprocal mixing)

-	n7 Tx filter rejection at n40 Rx (Tx noise at Rx frequency)Typical duplexer performance of n7 is shown in the figure below:

Figure 5.4.2.2-1: Example duplexer performance of band n7

According to the available data from vendors, band n7 Tx filter rejection at n40 is around 35~40dB, while n7 Rx filter rejection to n40 Tx is around 30~45dB.

Figure 5.4.2.2-2: Example filter performance of band n40

Rejection of n40 filter to frequency range of band n7 (2500-2570MHz UL/ 2620-2690MHz DL) could be as low as 28dB for the worst case. In the analysis, we adopt slightly better value of 30dB for the MSD calculation.

An example PA measurement curve is shown in Figure below. It can be seen that ACLR2 is roughly 10dB lower than ACLR1, and the downward trend is not stopped at ACLR2, in other words, PA noise floor is could be lower for frequency beyond ACLR2.

Figure 5.4.2.2-3: Example PA performance

Table 5.4.2.2-1: Assumptions on MSD analysis for CA_n7-n40

| PA far end noise floor (dBm/Hz) | -130 |
| --- | --- |
| Transceiver noise floor at PA output (dBm/Hz) | -132 |
| PA Tx noise regrowth beyond ACLR2 (dBc) | 40 for 5MHz @2622.5M 45 for 50MHz@2645M |
| PA output power at antenna port (dBm) | 23 |
| n7 Rx filter rejection at n40 Tx (dB) | 35 |
| n40 Tx filter rejection at n7 Rx (dB) | 30 |
| n40 Rx filter rejection at n7 Tx (dB) | 30 |
| n7 Tx filter rejection at n40 Rx (dB) | 35 |
| Diplexer (dB) | 10 |
| RFIC IIP2 (dBm) | 50 |
| Band n7 Tx LO phase noise (dBc/Hz) | -150 |
| Band n40 Rx LO phase noise (dBc/Hz) | -150 |
| Antenna ISO (dB) | 10 |
| Band n7/n40 Tx FE Loss (dB) | 4 |
| Band n7/n40 Rx FE Loss (dB) | 4 |
| Band n7 Rx noise figure (dB) | 11 |
| Band n40 Rx noise figure (dB) | 9 |

5.4.2.3	Calculated MSD values

Proposed Option 1:

Table 5.4.2.3-1: Cross band isolation for simultaneous Rx-Tx with CA_n7A-n40A [R4-2300347]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | 21.9 | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | 13.5 | >ACLR2 |

Proposed Option 2:

Table 5.4.2.3-2: Cross band isolation for simultaneous Rx-Tx with CA_n7A-n40A [R4-2302378]

| DL CBW (MHz) | 5 | 20 | 40 | 50 |
| --- | --- | --- | --- | --- |
| n7 MSD (dB) | 22.6 | 20.7 | 18.7 | 17.7 |

The averaging values of Option 1 and Option 2 are agreed as

Table 5.4.2.3-3: Cross band isolation for simultaneous Rx-Tx with CA_n7A-n40A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | 22.3 | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | 15.6 | >ACLR2 |

5.4.3	Requirements for simultaneous Rx/Tx

5.4.3.1	Reference sensitivity requirements

Table 5.4.3-1: Cross band isolation for simultaneous Rx-Tx with CA_n7A-n40A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | 22.3 | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | 15.6 | >ACLR2 |

5.5	CA_n28A-n39A-n41A

5.5.1	Status of the band combination

5.5.1.1	Operating bands for CA

The operating bands for CA_n28-n39-n41 are specified in Table 5.5.1.1-1.

Table 5.5.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n28-n39-n41 | n28, n39, n41 |  |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n28-n39-n41, the current spec only considers scenarios without simultaneous Rx/Tx for n39 and n41.

5.5.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n28-n39-n41 are specified in Table 5.5.1.2-1

Table 5.5.1.2-1: ΔTIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n28-n39-n41 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The ΔRIB,c due to CA_n28-n39-n41 are specified in Table 5.5.1.2-2.

Table 5.5.1.2-2: ΔRIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n28-n39-n41 | - | 0.2 | 0.2 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.5.1.3	Existing MSD requirement

Reference sensitivity exceptions due to intermodulation interference due to 2UL CA were specified for UL of n39 and n41 to DL of n28, without simultaneous Rx/Tx for n39 and n41.

Table 5.5.1.3-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n39-n41 | n28 | N/A | 5 | N/A | 762 | 29.3 | FDD | IMD2 |
|  | n39 | 1923 | 5 | 25 | 1923 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |

5.5.2	MSD analysis for simultaneous Rx/Tx

5.5.2.1	Reference UE architecture

The following two reference architectures can be considered for CA_n28A-n39A-n41A for simultaneous Rx/Tx operation.

Figure 5.5.2.1-1: Reference architecture using shared antenna (R4-2400583, Murata)

Figure 5.5.2.1-2: Reference architecture using separate antenna (R4-2400583, Murata)

5.5.2.2	RF component assumptions

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2400583, Murata).

Table 5.5.2.2-1: Intercept point parameters

|  | IP2 | IP4 | IP5 |
| --- | --- | --- | --- |
| PA forward | 27 | 31 | 27 |
| PA reverse | 38 | 33 | 32 |
| Switch | 112 | 55 | 53 |
| Filter/Duplexer | 100 | 55 | 53 |
| Diplexer | 115 | 55 | 53 |
| LNA | 6 | -5 | -10 |

Table 5.5.2.2-2: Filter attenuation/isolation, PCB and antenna isolation

| n41 filter attenuation at n28 | 35 |
| --- | --- |
| n41 filter attenuation at n39 | 30 |
| n39 filter attenuation at n28 | 30 |
| n39 filter attenuation at n41 | 35 |
| n28 filter attenuation at n39 | 45 |
| n28 filter attenuation at n41 | 45 |
| n79 filter attenuation at n41 | 40 |
| Diplexer isolation | 20 |
| Antenna isolation | 15 |
| PCB isolation | 60 |

5.5.2.3	Calculated MSD values

Proposed Option 1: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2400583, Murata).

Table 5.5.2.3-1: MSD due to 2UL IMD for simultaneous Rx/Tx using shared antenna structure (R4-2400583)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 32.6 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 32.6 | TDD | IMD2 |

Table 5.5.2.3-2: MSD due to 2UL IMD for simultaneous Rx/Tx using seperate antenna structure (R4-2400583)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 45.0 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 42.7 | TDD | IMD2 |

Table 5.5.2.3-3: MSD due to 2UL IMD for simultaneous Rx/Tx with averaging MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 42.2 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 40.1 | TDD | IMD2 |

Proposed Option 2: n39 MSD 32.2dB, n41 MSD 31.5dB (Qualcomm)

5.5.3	Requirements for simultaneous Rx/Tx

5.5.3.1	∆TIB and ∆RIB values

Table 5.5.3.1-1: ΔTIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n28-n39-n41 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.5.3.1-2: ΔRIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n28-n39-n41 | - | 0.2 | 0.2 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.5.3.2	Reference sensitivity requirements

Table 5.5.3.2-1: The MSD value for CA_n28A-n39A-n41A with simultaneous Rx-Tx

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 39.6 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 37.6 | TDD | IMD2 |

CA_n28A-n39A-n41A is only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.

5.6	CA_n39A-n40A-n41A

5.6.1	Status of the band combination

5.6.1.1	Operating bands for CA

The operating bands for CA_n39-n40-n41are specified in Table 5.6.1.1-1.

Table 5.6.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n39-n40-n41 | n39, n40, n41 |  |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n39-n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx for n39 and n41, for n40 and n41.

5.6.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n39-n40-n41 are specified in Table 5.6.1.2-1

Table 5.6.1.2-1: ΔTIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.3 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

ΔRIB,c due to CA_n39-n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.6.1.3	Existing MSD requirement

There is no Reference sensitivity exceptions due to intermodulation interference due to 2UL CA for CA_n39-n40-n41, considering no simultaneous Rx-Tx for n39 and n41, n40 and n41.

5.6.2	MSD analysis for simultaneous Rx/Tx

5.6.2.1	Reference UE architecture

In Murata paper R4-2404222, two different reference architecture options using triplexer are provided.

(a)

(b)

Figure 5.6.2.1-1: Reference architecture using shared antenna (R4-2404222, Murata)

In Huawei paper R4-2404513, the architecture of n40 and n41 sharing antenna switch is provided.

Figure 5.6.2.1-2: Reference UE architecture for CA_n39-n40-n41 (R4-2404513, Huawei)

In Qualcomm paper R4-2405452, the architecture of n40 and n41 sharing antenna switch is provided.

Figure 5.6.2.1-3: Reference UE architecture for CA_n39-n40-n41 (R4-2405452, Qualcomm)

5.6.2.2	RF component assumptions

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2404222, Murata).

Table 5.6.2.2-1: Intercept point parameters (R4-2404222)

|  | IP3 |
| --- | --- |
| PA forward | 30 |
| PA reverse | 29 |
| Switch | 68 |
| Filter (Triplexer) | 74 |
| LNA | -5 |

Table 5.6.2.2-2: Filter attenuation/isolation, PCB and antenna isolation	(R4-2404222)

| n39 filter attenuation at n40 | 40 |
| --- | --- |
| n39 filter attenuation at n41 | 40 |
| n40 filter attenuation at n39 | 35 |
| n40 filter attenuation at n41 | 40 |
| n41 filter attenuation at n39 | 35 |
| n41 filter attenuation at n40 | 35 |
| Isolation btw n39 and n40 at n39 | 45 |
| Isolation btw n39 and n40 at n40 | 40 |
| Isolation btw n39 and n41 at n39 | 45 |
| Isolation btw n39 and n41 at n41 | 40 |
| Isolation btw n40 and n41 at n40 | 40 |
| Isolation btw n40 and n41 at n41 | 40 |
| Antenna isolation | 15 |
| PCB isolation | 60 |

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2405413, Huawei).

Table 5.6.2.2-3: Intercept point parameters (R4-2405413)

| Component | IP3(dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Duplexer | 74 |
| PA forward mixing | 32 |
| PA reverse mixing | 30 |
| LNA | -6 |

Table 5.6.2.2-4: Filter attenuation/isolation, PCB and antenna isolation	(R4-2405413)

| n41 Tx Rejection at n40 Tx | 40 |
| --- | --- |
| n41 Tx Rejection at n39 Rx | 40 |
| n40 Tx Rejection at n41 Tx | 40 |
| n40 Tx Rejection at n39 Rx | 35 |
| n39 Rx Rejection at n40 Tx | 40 |
| n39 Tx Rejection at n41 Rx | 40 |
| Diplexer isolation | 15 |
| PCB isolation | 60 |

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2405452, Qualcomm).

Table 5.6.2.2-5: Intercept point parameters, filter attenuation/isolation, PCB and antenna isolation (R4-2405452)

5.6.2.3	Calculated MSD values

Proposed Option 1: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2404222, Murata).

Table 5.6.2.3-1: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2404222)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 31.6 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1917.5 | 31.6 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Proposed Option 2: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2405413, Huawei).

Table 5.6.2.3-2: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2405413)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 27.1 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1915 | 16.3 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Proposed Option 3: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2405452, Qualcomm).

Table 5.6.2.3-2: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2405452)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 |  | 5 | 25 |  | N/A | TDD | N/A |
|  | n40 |  | 5 | 25 |  | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | [30.9] | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1915 | [22.5] | TDD | IMD31 |
|  | n40 |  | 5 | 25 |  | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 |  | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.6.3	Requirements for simultaneous Rx/Tx

5.6.3.1	∆TIB and ∆RIB values

Sufficient attenuation level between bands should be considered in ΔTIB,c and ΔRIB,c , given that increased insertion loss of the triplexer compared to single filter (R4-2404222).

Table 5.6.3.1-1: ΔTIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.6 | 0.6 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.6.3.1-2: ΔRIB,c due to CA_n39-n40-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| CA_n39-n40-n41 | 0.3 | 0.6 | 0.6 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.6.3.2	Reference sensitivity requirements

Table 5.6.3.2-1: M3DL/2UL MSDs for CA_n39A-n40A-n41A (R4-2405452)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n40-n41 | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2685 | 30.3 | TDD | IMD3 |
|  | n39 | N/A | 5 | N/A | 1917.5 | 27.4 | TDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.7	CA_n39A-n41A-n79A

5.7.1	Status of the band combination

5.7.1.1	Operating bands for CA

The operating bands for CA_n39-n41-n79 are specified in Table 5.7.1.1-1.

Table 5.7.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n39-n41-n79 | n39, n41, n79 | No |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n39-n41-n79, the current spec only considers scenarios without simultaneous Rx/Tx for n39 and n41.

5.7.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n39-n41-n79 are specified in Table 5.7.1.2-1

Table 5.7.1.2-1: ΔTIB,c due to CA_n39-n41-n79

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n39-n41-n79 | 0.3 | 0.3 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The ΔRIB,c due to CA_n39-n41-n79 are specified in Table 5.7.1.2-2.

Table 5.7.1.2-2: ΔRIB,c due to CA_n39-n41-n79

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n39-n41-n79 | - | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.7.1.3	Existing MSD requirement

Reference sensitivity exceptions due to intermodulation interference due to 2UL CA were specified for UL of n39 and n41 to DL of n79, without simultaneous Rx/Tx for n39 and n41.

Table 5.7.1.3-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39-n41-n79 | n39 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n41 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | TDD | IMD29 |
| NOTE 9: There is no IMD2 product in band n79 downlink for n79 operating in 4800 – 5000 MHz frequency range. |  |  |  |  |  |  |  |  |

5.7.2	MSD analysis for simultaneous Rx/Tx

5.7.2.1	Reference UE architecture

The following two reference architectures can be considered for CA_n39A-n41A-n79A for simultaneous Rx/Tx operation.

Figure 5.7.2.1-1: Reference architecture using shared antenna (R4-2400583, Murata)

Figure 5.7.2.1-2: Reference architecture using seperate antenna (R4-2400583, Murata)

5.7.2.2	RF component assumptions

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2400583, Murata).

Table 5.7.2.2-1: Intercept point parameters

|  | IP2 | IP4 | IP5 |
| --- | --- | --- | --- |
| PA forward | 27 | 31 | 27 |
| PA reverse | 38 | 33 | 32 |
| Switch | 112 | 55 | 53 |
| Filter/Duplexer | 100 | 55 | 53 |
| Diplexer | 115 | 55 | 53 |
| LNA | 6 | -5 | -10 |

Table 5.7.2.2-2: Filter attenuation/isolation, PCB and antenna isolation

| n41 filter attenuation at n39 | 30 |
| --- | --- |
| n41 filter attenuation at n79 | 40 |
| n39 filter attenuation at n41 | 35 |
| n39 filter attenuation at n79 | 40 |
| n79 filter attenuation at n39 | 40 |
| n79 filter attenuation at n41 | 40 |
| Diplexer isolation | 20 |
| Antenna isolation | 15 |
| PCB isolation | 60 |

5.7.2.3	Calculated MSD values

Proposed Option 1: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2400583, Murata).

Table 5.7.2.3-1: MSD due to 2UL IMD for simultaneous Rx/Tx using shared antenna structure (R4-2400583)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n39-n41-n79 | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 26.0 | TDD | IMD2 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | 1890 | 5 | 25 | 1890 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2680 | 9.2 | TDD | IMD5 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 24.8 | TDD | IMD2 |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 3.1 | TDD | IMD5 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |

Table 5.7.2.3-2: MSD due to 2UL IMD for simultaneous Rx/Tx using seperate antenna structure (R4-2400583)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n39-n41-n79 | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 33.6 | TDD | IMD2 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | 1890 | 5 | 25 | 1890 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2680 | 18.2 | TDD | IMD5 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 32.0 | TDD | IMD2 |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 4.7 | TDD | IMD5 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |

Table 5.7.2.3-3: MSD due to 2UL IMD for simultaneous Rx/Tx with averaging MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n39-n41-n79 | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 31.3 | TDD | IMD21 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 29.7 | TDD | IMD21 |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Proposed Option 2: n41 MSD 27.4dB, n39 MSD 27.8dB (Qualcomm)

5.7.3	Requirements for simultaneous Rx/Tx

5.7.3.1	∆TIB and ∆RIB values

Table 5.7.3.1-1: ΔTIB,c due to CA_n39-n41-n79

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n39-n41-n79 | 0.3 | 0.3 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.7.3.1-2: ΔRIB,c due to CA_n39-n41-n79

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n39-n41-n79 | - | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.7.3.2	Reference sensitivity requirements

Table 5.7.3.2-1: The MSD value for CA_n39A-n41A-n79A with simultaneous Rx-Tx

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n39-n41-n79 | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 29.8 | TDD | IMD21 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 28.9 | TDD | IMD21 |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.8	CA_n8A-n40A-n41A

5.8.1	Status of the band combination

5.8.1.1	Operating bands for CA

The operating bands for CA_n8-n40-n41 are specified in Table 5.8.1.1-1.

Table 5.8.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n8-n40-n41 | n8, n40, n41 |  |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n8-n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx for n40 and n41.

5.8.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n8-n40-n41 are specified in Table 5.8.1.2-1

Table 5.8.1.2-1: ΔTIB,c due to CA_n8-n40-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n8-n40-n41 | 0.3 | 0.3 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

ΔRIB,c due to CA_n8-n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.8.1.3	Existing MSD requirement

There is no Reference sensitivity exceptions due to intermodulation interference due to 2UL CA for CA_n8-n40-n41, considering no simultaneous Rx-Tx for n40 and n41.

5.8.2	MSD analysis for simultaneous Rx/Tx

5.8.2.1	Reference UE architecture

Figure 5.8.2.1-1: Reference UE architecture for CA_n8A-n40A-n41A

5.8.2.2	RF component assumptions

Typical duplexer performance of n8 is shown in the figure below:

Figure 5.8.2.2-1: Example filter performance of band n8 Tx

Typical duplexer performance of n40 is shown in the figure below:

Figure 5.8.2.2-2: Example filter performance of band n40

Typical duplexer performance of n41 is shown in the figure below:

Figure 5.8.2.2-3: Example duplexer performance of band n41

Table 5.8.2.2-1: Assumptions on MSD analysis for CA_n8A-n40A-n41A

| n8 Tx Rejection at n40 Rx | 45 |
| --- | --- |
| n8 Tx Rejection at n41 Tx | 40 |
| n40 Rx Rejection at n8 Tx | 40 |
| n40 Rx Rejection at n41 Tx | 40 |
| n41 Tx Rejection at n8 Tx | 40 |
| n41 Tx Rejection at n40 Rx | 40 |
| Diplexer isolation | 15 |
| PCB isolation | 60 |

For UE coexistence study of Band n8 + Band n41, 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.8.2.2-2.

Table 5.8.2.2-2 IMD analysis for UL n8+n41-> DL n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2496 | 2690 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1581 | 1810 | 3376 | 3605 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 | 666 | 4077 | 4500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4520 | 5872 | 6295 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 50 | 55 | 6573 | 7190 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5136 | 5435 | 8368 | 8985 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3162 | 3620 | 6752 | 7210 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9880 | 9069 | 1164 | 830 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10864 | 11675 | 6016 | 6350 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6310 | 5658 | 2247 | 2740 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9248 | 9900 | 7632 | 8125 |

5th order IMD generated by dual uplink of Band n41 + Band n8 may fall into Rx of Band n40.

5.8.2.3	Calculated MSD values

Table 5.8.2.3-1 IMD for simultaneous Rx-Tx with CA_n8A-n40A-n41A (Huawei)

| CA_n8-n40-n41 | n8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n40 | N/A | 5 | N/A | 2355 | 3.58 | TDD | IMD5 |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | TDD | N/A |

Table 5.8.2.3-2 IMD for simultaneous Rx-Tx with CA_n8A-n40A-n41A (Murata)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n8-n40-n41 | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2360 | 6.2 | TDD | IMD5 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |

IMD for simultaneous Rx-Tx with CA_n8A-n40A-n41A provided by Qualcomm：n40MSD 4.4 dB.

5.8.3	Requirements for simultaneous Rx/Tx

Table 5.8.3-1 IMD for simultaneous Rx-Tx with CA_n8A-n40A-n41A

| CA_n8-n40-n41 | n8 | 895 | 5 | 25 | 940 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n40 | 2355 | 5 | 25 | 2355 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2520 | 4.9 | TDD | IMD5 |

5.9	CA_n28A-n40A-n41A

5.9.1	Status of the band combination

5.9.1.1	Operating bands for CA

The operating bands for CA_n28-n40-n41 are specified in Table 5.9.1.1-1.

Table 5.9.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n28-n40-n41 | n28, n40, n41 |  |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n28-n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx for n40 and n41.

5.9.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n28-n40-n41 are specified in Table 5.9.1.2-1

Table 5.9.1.2-1: ΔTIB,c due to CA_n28-n40-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n28-n40-n41 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

ΔRIB,c due to CA_n28-n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

5.9.1.3	Existing MSD requirement

Reference sensitivity exceptions due to intermodulation interference due to 2UL CA were specified for UL of n40 and n41 to DL of n28, without simultaneous Rx/Tx for n40 and n41.

Table 5.9.1.3-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n40-n41 | n28 | N/A | 5 | N/A | 765 | 7.6 | FDD | IMD4 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |

5.9.2	MSD analysis for simultaneous Rx/Tx

5.9.2.1	Reference UE architecture

Figure 5.9.2.1-1: Reference UE architecture for CA_n28A-n40A-n41A

5.9.2.2	RF component assumptions

Typical duplexer performance of n28 is shown in the figure below:

Figure 5.9.2.2-1: Example filter performance of band n28

Typical duplexer performance of n40 is shown in the figure below:

Figure 5.9.2.2-2: Example filter performance of band n40

Typical duplexer performance of n41 is shown in the figure below:

Figure 5.9.2.2-3: Example duplexer performance of band n41

Table 5.9.2.2-1: Assumptions on MSD analysis for CA_n28A-n40A-n41A

| n28 Rx Rejection at n40 Tx | 41 |
| --- | --- |
| n28 Tx Rejection at n41 Rx | 41 |
| n40 Tx Rejection at n28 Tx | 40 |
| n40 Tx Rejection at n41 Rx | 35 |
| n41 Rx Rejection at n28 Tx | 40 |
| n41 Rx Rejection at n40 Tx | 35 |
| Diplexer isolation | 10 |
| PCB isolation | 60 |

For UE coexistence study of Band n28 + Band n40, 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.9.2.2-2.

Table 5.9.2.2-2 IMD analysis for UL n28+n40-> DL n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2300 | 2400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1552 | 1697 | 3003 | 3148 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 994 | 804 | 3852 | 4097 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3706 | 3896 | 5303 | 5548 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 291 | 156 | 6152 | 6497 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4409 | 4644 | 7603 | 7948 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3104 | 3394 | 6006 | 6296 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8897 | 8452 | 692 | 412 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9903 | 10348 | 5112 | 5392 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5794 | 5404 | 2356 | 2691 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8306 | 8696 | 6709 | 7044 |

5th order IMD generated by dual uplink of Band n40 + Band n28 may fall into Rx of Band n41

5.9.2.3	Calculated MSD values

Table 5.9.2.3-1 IMD for simultaneous Rx-Tx with CA_n28A-n40A-n41A (Huawei)

| CA_n28-n40-n41 | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2540 | 3.52 | TDD | IMD5 |

Table 5.9.2.3-2 IMD for simultaneous Rx-Tx with CA_n28A-n40A-n41A (Murata)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n40-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2545 | 15.3 | TDD | IMD5 |

IMD for simultaneous Rx-Tx with CA_n28A-n40A-n41A provided by Qualcomm: n41 MSD 7.4dB.

5.9.3	Requirements for simultaneous Rx/Tx

Table 5.9.3-1 IMD for simultaneous Rx-Tx with CA_n28A-n40A-n41A

| CA_n28-n40-n41 | n28 | 740 | 5 | 25 | 795 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n40 | 2380 | 5 | 25 | 2380 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2540 | 11.4 | TDD | IMD5 |

5.10	CA_n40A-n41A-n79A

5.10.1	Status of the band combination

5.10.1.1	Operating bands for CA

The operating bands for CA_n40-n41-n79 are specified in Table 5.10.1.1-1.

Table 5.10.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n40-n41-n791,2 | n40, n41, n79 | No for CA n40-n79, CA n41-n79 |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n40-n41-n79, the current spec only considers scenarios without simultaneous Rx/Tx for n40 and n41.

5.10.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n40-n41-n79 are specified in Table 5.10.1.2-1

Table 5.10.1.2-1: ΔTIB,c due to CA_n40-n41-n79

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n40-n41-n79 | 0.5 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The ΔRIB,c due to CA_n40-n41-n79 are specified in Table 5.10.1.2-2.

Table 5.10.1.2-2: ΔRIB,c due to CA_n40-n41-n79

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n40-n41-n79 | - | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.10.1.3	Existing MSD requirement

Reference sensitivity exceptions due to intermodulation interference due to 2UL CA were specified for UL of n40 and n41 to DL of n79, without simultaneous Rx/Tx for n40 and n41.

Table 5.10.1.3-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41-n79 | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4940 | 30.5 | TDD | IMD2 |

5.10.2	MSD analysis for simultaneous Rx/Tx

5.10.2.1	Reference UE architecture

Figure 5.10.2.1-1: Reference UE architecture for CA_n40-n41-n79

5.10.2.2	RF component assumptions

Typical duplexer performance of n40 is shown in the figure below:

Figure 5.10.2.2-1: Example filter performance of band n40

Typical duplexer performance of n41 is shown in the figure below:

Figure 5.10.2.2-2: Example duplexer performance of band n41

Typical duplexer performance of n79 is shown in the figure below:

Figure 5.10.2.2-3: Example duplexer performance of band n79

Table 5.10.2.2-1: Assumptions on MSD analysis for CA_n40-n41-n79

| n41 Tx Rejection at n40 Tx | 40 |
| --- | --- |
| n41 Tx Rejection at n79 Rx | 35 |
| n40 Tx Rejection at n41 Tx | 40 |
| n40 Tx Rejection at n79 Rx | 35 |
| n79 Rx Rejection at n40 Tx | 40 |
| n79 Tx Rejection at n41 Rx | 40 |
| Diplexer isolation | 15 |
| PCB isolation | 60 |

For UE coexistence study of Band n41 + Band n79, 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.10.2.2-2.

Table 5.10.2.2-2 IMD analysis for UL n41+n79-> DL n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2506 | 2690 | 4800 | 5000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2110 | 2494 | 7306 | 7690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 12 | 580 | 6910 | 7494 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9812 | 10380 | 12106 | 12690 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2518 | 3070 | 11710 | 12494 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 12318 | 13070 | 16906 | 17690 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4220 | 4988 | 14612 | 15380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17494 | 16510 | 5960 | 5024 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 21706 | 22690 | 14824 | 15760 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9988 | 9020 | 1530 | 2482 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19412 | 20380 | 17118 | 18070 |

For UE coexistence study of Band n40 + Band n79, 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.10.2.2-3.

Table 5.10.2.2-3 IMD analysis for UL n40+n79-> DL n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 4800 | 5000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2400 | 2700 | 7100 | 7400 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 400 | 0 | 7200 | 7700 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9400 | 9800 | 11900 | 12400 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1900 | 2200 | 12000 | 12700 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11700 | 12200 | 16700 | 17400 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4800 | 5400 | 14200 | 14800 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17700 | 16800 | 4800 | 4200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 21500 | 22400 | 14000 | 14600 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10400 | 9600 | 2400 | 3100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19000 | 19800 | 16500 | 17200 |

2nd order and 5th order IMD generated by dual uplink of Band n41 + Band n79 may fall into Rx of Band n40.

2nd order and 5th order IMD generated by dual uplink of Band n40 + Band n79 may fall into Rx of Band n41.

5.10.2.3	Calculated MSD values

Table 5.10.2.3-1 IMD for simultaneous Rx-Tx with CA_n40A-n41A-n79A (Huawei)

| CA_n40-n41-n79 | n40 | N/A | 5 | N/A | 2335 | 23.5 | TDD | IMD2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2545 | 10 | 50 | 2545 | N/A | TDD | N/A |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2340 | 4.35 | TDD | IMD5 |
|  | n41 | 2540 | 10 | 50 | 2540 | N/A | TDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2540 | 26.2 | TDD | IMD2 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n40 | 2330 | 5 | N/A | 2330 | N/A | TDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | 4.19 | TDD | IMD5 |
|  | n79 | 4830 | 40 | 216 | 4830 | N/A | TDD | N/A |

Table 5.10.2.3-2 IMD for simultaneous Rx-Tx with CA_n40A-n41A-n79A (Murata)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n40-n41-n79 | n40 | 2330 | 5 | 25 | 2330 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2520 | 31.3 | TDD | IMD2 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2550 | 5.7 | TDD | IMD4 |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | TDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2550 | 3.4 | TDD | IMD5 |
|  | n79 | 4800 | 40 | 216 | 4800 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2330 | 33 | TDD | IMD2 |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | TDD | N/A |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2330 | 3.0 | TDD | IMD5 |
|  | n41 | 2530 | 10 | 50 | 2540 | N/A | TDD | N/A |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | TDD | N/A |

IMD for simultaneous Rx-Tx with CA_n40A-n41A-n79A provided by Qualcomm: n40 MSD 32 dB, n41 MSD 29.4dB

5.10.3	Requirements for simultaneous Rx/Tx

Table 5.10.3-1 IMD for simultaneous Rx-Tx with CA_n40A-n41A-n79A

| CA_n40-n41-n79 | n40 | N/A | 5 | N/A | 2335 | 31 | TDD | IMD21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2545 | 10 | 50 | 2545 | N/A | TDD | N/A |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2600 | 29.4 | TDD | IMD21 |
|  | n79 | 4880 | 40 | 216 | 4940 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.x	CA_nA-nB

For reference of the TP.

5.x.1	Status of the band combination

5.x.2	MSD analysis

5.x.3	Requirements for simultaneous Rx/Tx

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | RAN4#104-bis-e | R4-22xxxxx |  |  |  | TR skeleton | 0.0.1 |
| 2022-11 | RAN4#105 | R4-2219415 |  |  |  | Implemented TP’s from RAN4#105 R4-2220484 TP for TR 38.894 to include simultaneous Rx/Tx CA_n39-n41, ZTE R4-2220485 TP for TR38.894: CA_n39A-n41A ,CA_n34A-n41A, CA_n40A-n41A, CA_n7A-n40A Huawei, HiSilicon TR 38.894 | 0.1.0 |
| 2023-04 | RAN4#106-bis-e |  |  |  |  | Updates | 0.2.0 |
| 2023-05 | RAN4#107 | R4-2307170 |  |  |  | Updates | 0.3.0 |
| 2023-11 | RAN4#109 |  |  |  |  | Updates | 0.4.0 |
| 2024-03 | RAN4#110 | R4-2400854 |  |  |  | Updates for the structure for the 3-band combos | 0.5.0 |
| 2024-04 | RAN4#110bis | R4-2404512 |  |  |  | Updates for the analysis of 5 band combinations | 0.6.0 |