3GPP TSG-RAN WG4 Meeting # 111	R4-2407624

# Fukuoka, Japan, 20th - 24th, May 2024

Source: 	Samsung, KDDI

Title: 	Views on b/n28 full-band duplex

Agenda Item:	6.19.2

Document for:	Discussion

- Introduction

In RAN4#110-bis meeting, the WF [1] on the recommendation of a Rel-19 new spectrum WI for b/n28 full-band duplexer with PC3/PC2 was discussed but not approved. In this paper we share our further consideration and real implementation on this item, particularly for PC3.

- Discussion

The WF is reproduced as following,

The PC2 n28 was requested and included in the Rel-18 HPUE_NR_FR1_FDD_R18 WI and has been discussed for a couple of meetings but not finalized yet. The WF[1] mentions that the industry has demand to enable full-band duplex operation for b/n28, which is to replace “b/n28A+b/n28B” implementation, while practically our UEs have already implemented full-band b/n28 duplex (From Galaxy S22) in Japan around three years ago for sake of cost reduction, so far no issue identified in the field nor in the conformance test to meet existing PC3 regulatory requirements(i.e., A-MPR=0) for DTV protection(NS_17) in Japan, and the margin is quite enough particularly for LTE. The commercial full-band duplex can provide good suppression to meet PC3 requirements from our observation.

Observation 1: From real UE implementation perspective, Samsung UEs(From Galaxy S22) already implemented full-band b/n28 duplex in Japan around three years ago, so far no issue identified in the field nor in the conformance test assuming existing PC3 regulatory requirements (i.e. A-MPR=0) for DTV protection, and the margin is quite enough particularly for LTE.

From NW perspective, the legacy NW is designed assuming A-MPR=0 for PC3. If PC3 A-MPR requirements are re-visited and a new set of A-MPR requirements(A-MPR ≠0) is introduced for n28 full-band duplex, theoretically a signalling must be introduced/utilized to indicate this new implementation to NW, and a new NW would be able to understand there are different MPR requirements corresponding to different UE implementations, and the maximum configured transmission power of a UE with full-band duplex might be reduced. Nevertheless, the legacy NW (assuming A-MPR=0) is not able to understand such information and cannot predict the UE behaviors, which leads to NBC issue, as the new UE(s) and legacy UE(s) at same location with same scheduling would have different UE behaviors.

Observation 2: From NW implementation perspective, introducing a new set of PC3 A-MPR requirements (i.e., A-MPR≠0) for b/n28 full-band duplex bring NBC issues to legacy NW.

Further, it makes full-band duplex an inferior solution than “b/n 28A+28B” due to the coverage shrink with large A-MPR relaxation, which has adverse impact to popularize this implementation.

Observation 3: It has adverse impact to popularize full-band b/n28 duplex if A-MPR relaxation (i.e., A-MPR≠0) is introduced for PC3 for full-band b/n28 duplex.

With above, we propose:

Proposal 1: Exiting PC3 A-MPR requirements (i.e., A-MPR=0) should be applicable for the UE with b/n28 full-band duplex implementation as well, for DTV protection in Japan.

- 3. Conclusion

Observation 1: From real UE implementation perspective, Samsung UEs(From Galaxy S22) already implemented full-band b/n28 duplex in Japan around three years ago, so far no issue identified in the field nor in the conformance test assuming existing PC3 regulatory requirements (i.e. A-MPR=0) for DTV protection, and the margin is quite enough particularly for LTE.

Observation 2: From NW implementation perspective, introducing a new set of PC3 A-MPR requirements (i.e., A-MPR≠0) for b/n28 full-band duplex bring NBC issue to legacy NW.

Observation 3: It has adverse impact to popularize full-band b/n28 duplex if A-MPR relaxation (i.e., A-MPR≠0) is introduced for PC3 for full-band b/n28 duplex.

Proposal 1: Exiting PC3 A-MPR requirements (i.e., A-MPR=0) should be applicable for the UE with b/n28 full-band duplex implementation as well, for DTV protection in Japan.

- 4. Reference

[1] R4-2406708, “WF on band n28 full band duplexer”, Apple, R4#110-bis