3GPP TSG-RAN WG4 Meeting #111	R4-2407663
Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	7.10.1

Source:	CAICT

Title:	Summary of 3GPP Rel-18 FR2 MIMO OTA lab alignment results

Document for:	Discussion

1	Introduction

The Rel-18 FR2 MIMO OTA lab alignment activity is expected to be concluded at this meeting. Four volunteer labs have submitted their measurement results of the performance alignment devices (PADs) [1]-[4]. Besides, Lab A indicated that their results are 3 dB more optimistic due to the way the power is set at the center of the test zone, and thus their MASC results should be corrected by 3-dB offset [5].

According to the Framework [6], the reference value of each PAD is derived by the linear average (in dB) of the measurement results submitted by labs. The pass/fail limit was specified as 0.75*preliminary MU.

This contribution presents a summary of the Rel-18 FR2 MIMO OTA lab alignment result. The detailed analysis data is attached in the Excel worksheet of this contribution.

2	Discussion

The Rel-18 FR2 MIMO OTA lab alignment measurement results are summarized and analysed, as shown in Table 1 (with Lab A’s original data) and Table 2 (with Lab A’s corrected data with 3-dB offset).

Table 1. Summary of the Rel-18 FR2 MIMO OTA lab alignment results (with Lab A’s original data)

| Device | Band | MASC70 measurement result [dBm/120kHz] |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab A (original results) | Lab B | Lab C | Lab D | Lab E |  |  |  |
| PAD 1 | n261 | -104.97 | -101.79 | -100.64 | Results not available in Rel-18 | NA | Linear average (in dB) | -102.47 | 4.33 |
| PAD 2 | n261 | -108.56 | -104.92 | -103.87 |  | -105.41 |  | -105.69 | 4.69 |
| PAD 3 | n261 | -105.40 | -101.92 | -101.22 |  | -103.31 |  | -102.96 | 4.18 |
| PAD 4 | n257 | -108.76 | -105.80 | -104.59 |  | -106.01 |  | -106.29 | 4.17 |
| Device | Band | TRMS offset [dBm/120kHz] |  |  |  |  | Pass/fail limit |  |  |
|  |  | Lab A | Lab B | Lab C | Lab D | Lab E |  |  |  |
| PAD 1 | n261 | -2.51 | 0.68 | 1.83 | NA | NA | ± 0.75*preliminary MU, i.e., ± 3.79 dB |  |  |
| PAD 2 | n261 | -2.87 | 0.77 | 1.82 | NA | 0.28 |  |  |  |
| PAD 3 | n261 | -2.44 | 1.04 | 1.74 | NA | -0.35 |  |  |  |
| PAD 4 | n257 | -2.47 | 0.49 | 1.70 | NA | 0.28 |  |  |  |
| Lab alignment conclusion |  | Pass | Pass | Pass | NA | Pass |  |  |  |

Table 2. Summary of the Rel-18 FR2 MIMO OTA lab alignment results (with Lab A’s corrected data)

| Device | Band | MASC70 measurement result [dBm/120kHz] |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab A (with 3dB offset) | Lab B | Lab C | Lab D | Lab E |  |  |  |
| PAD 1 | n261 | -101.97 | -101.79 | -100.64 | Results not available in Rel-18 | NA | Linear average (in dB) | -101.47 | 1.33 |
| PAD 2 | n261 | -105.56 | -104.92 | -103.87 |  | -105.41 |  | -104.94 | 1.69 |
| PAD 3 | n261 | -102.40 | -101.92 | -101.22 |  | -103.31 |  | -102.21 | 2.09 |
| PAD 4 | n257 | -105.76 | -105.80 | -104.59 |  | -106.01 |  | -105.54 | 1.41 |
| Device | Band | TRMS offset [dBm/120kHz] |  |  |  |  | Pass/fail limit |  |  |
|  |  | Lab A | Lab B | Lab C | Lab D | Lab E |  |  |  |
| PAD 1 | n261 | -0.51 | -0.32 | 0.83 | NA | NA | ± 0.75*preliminary MU, i.e., ± 3.79 dB |  |  |
| PAD 2 | n261 | -0.62 | 0.02 | 1.07 | NA | -0.47 |  |  |  |
| PAD 3 | n261 | -0.19 | 0.29 | 0.99 | NA | -1.10 |  |  |  |
| PAD 4 | n257 | -0.22 | -0.26 | 0.95 | NA | -0.47 |  |  |  |
| Lab alignment conclusion |  | Pass | Pass | Pass | NA | Pass |  |  |  |

Figure 2 presents the MASC measurement results, and Figure 3 visualizes the deviations between each MASC measurement results and average values.

(a) with Lab A’s original data

(b) with Lab A’s corrected data

Figure 2. The Rel-18 FR2 MIMO OTA lab alignment measurement results

(a) with Lab A’s original data

(b) with Lab A’s corrected data

Figure 3. Deviation between each measurement result and the average value

Observation 1: Four volunteer labs provided their measurement results for FR2 MIMO OTA lab alignment in Rel-18 MIMO OTA lab alignment activity. With Lab A’s corrected data, the maximum deviation between different labs is 2.09 dB, which is within 0.5 preliminary MU. No outlier value is observed.

Observation 2: The performance rankings of the PADs are consistent across different labs.

Observation 3: With Lab A’s corrected data, the maximum deviation between measurement results and average values is -1.10 dB, which is about 0.22*preliminary MU.

Observation 4: The maximum deviation between measurement results and reference values is within the agreed pass/fail limit, +/- 0.75*preliminary MU (3.79 dB). The four labs can be confirmed as aligned.

To provide a comprehensive overview of the measurement results, the measured sensitivity values @ 70% TP at each of test points are illustrated in Annex.

3	Conclusion

This contribution presents a summary of the Rel-18 FR2 MIMO OTA lab alignment result.

Observation 1: Four volunteer labs provided their measurement results for FR2 MIMO OTA lab alignment in Rel-18 MIMO OTA lab alignment activity. With Lab A’s corrected data, the maximum deviation between different labs is 2.09 dB, which is within 0.5 preliminary MU. No outlier value is observed.

Observation 2: The performance rankings of the PADs are consistent across different labs.

Observation 3: With Lab A’s corrected data, the maximum deviation between measurement results and average values is -1.10 dB, which is about 0.22*preliminary MU.

Observation 4: The maximum deviation between measurement results and reference values is within the agreed pass/fail limit, +/- 0.75*preliminary MU (3.79 dB). The four labs can be confirmed as aligned.

Annex: Measurement results at each test point

To provide a comprehensive overview of the measurement results, the measured sensitivity values @ 70% TP at each of test points are shown in Fig. A-1 (with Lab A’s original data).

(a) PAD 1

(b) PAD 2

(c) PAD 3

(d) PAD 4

Figure A-1. Measured sensitivity values at each test point (with Lab A’s original data)

References

- R4-2400194, On FR2 MIMO OTA Lab Alignment PADs measurement results, Apple, 3GPP RAN4#110, Feb. 2024.

- R4-2405463, Measurement results for 3GPP FR2 MIMO OTA Lab Alignment Activity, CAICT, 3GPP RAN4#110bis, Apr. 2024.

- R4-2405312, Measurement results for 3GPP FR2 MIMO OTA Lab Alignment Activity, CMCC, 3GPP RAN4#110bis, Apr. 2024.

- R4-2406133, Measurement results for 3GPP FR2 MIMO OTA Lab Alignment Activity, Huawei, HiSilicon, RAN4#110bis, Apr. 2024.

- R4-2407064, On FR2 MIMO OTA measurement results, Apple, 3GPP RAN4#111, May 2024.

- R4-2407662, Updated Framework and time plan for FR2 MIMO OTA performance requirements development (May 2024), CAICT, 3GPP RAN4#111, May 2024.