3GPP TSG-RAN WG4 Meeting #111	R4-2407708

# Fukuoka, Japan, May 20-24, 2024

Source:	T-Mobile USA

Title:	TP for TR 38.850: DL CA_n71(2A) UL PC2 n71A

Agenda item:	6.18.2

Document for:	Approval

- Introduction

This contribution is a text proposal to update the PC2 n71 for DL CA_n71(2A). This is based on input from several companies. It is anticipated that it will need to be updated in RAN4#111. For now the proposed values are [TBD].

- Reference

[1]

- Text Proposal

# 6	High Power UE for Intra-band DL CA with PC2 on single FDD band

## 6.1	CA_n71(2A)

### 6.1.1	UE maximum output power

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | n713 | 5, 10, 15, 20 | 5,10,15, 20 |  |  | 30 | 0 |
|  |  | See n71 channel bandwidths in Table 5.3.5-1 for each carrier up to 25 MHz per carrier |  |  |  | 30 | 4 and 5 |
| NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |  |  |  |

### 6.1.2	Reference sensitivity requirements

#### 6.1.2.0	General

For PC2, CA_n71(2A) has self-interference for UL n71. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD.

#### 6.1.2.1	Reference sensitivity requirements with PC2 on n71 without TxD

For CA_n71(2A), this is the configuration and MSD for UL n71 with PC3 based on the proposal in R4-2407158

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 |  |  |  |  |  |
|  |  |  |  | 15MHz + 10MHz |  | Wgap = 10.0 |  | 5 (RBstart = 2) |  | 22.2 |  |  |
|  |  |  |  | 25MHz + 5MHzx |  | Wgap = 5.0 |  | 20 (RBstart = 19) |  | [TBD] |  |  |
| NOTE x: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |  |  |  |  |  |  |

The following is proposed as a the PC2 single Tx MSD which would require a new table in 38.101-1:

Table 7.3A.2.2-1a: Power class 2 intra-band non-contiguous CA reference sensitivity with one uplink carrier.

| CA configuration |  |  |  |  |  |  |  |  | SCS (kHz) |  | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) |  |  |  |  |  |  |  |  | 15/15 |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | [[TBD] | [TBD] | FDD |
|  |  | 25MHz + 5MHz1 | Wgap = 5.0 | 20 (RBstart = 8) | [[TBD] | [TBD] |  |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with dual Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |  |

#### 6.1.2.2	Reference sensitivity requirements with PC2 on n71 with TxD

[TBD]